3GPP TSG-RAN5 Meeting #90-e	R5-211620

# Electronic Meeting, 22nd Feb – 5th Mar 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1109 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 4.7.5.1 EN-DC FR1 SFTD measurement accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Netherlands B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Recently concluded RAN4 added TRS Configuration to this test |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added TRS Configuration to Table 4.7.5.1.5-1 r1: Added TRS config values in the Table 4.7.5.1.5-1, instead of deleting and adding new Table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test case 4.7.5.1 will not be in sync with core RAN4 Spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Reference RAN4 Tdoc is R4-2104086 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-210955r1 R5-211620 |  |  |  |  |  |  |  |  |

<<Start of Changes>>

#### 4.7.5.1	EN-DC FR1 SFTD measurement accuracy

4.7.5.1.1	Test purpose

The purpose of this test is to verify that that SFTD measurement accuracy is within the specified limits for all bands.

4.7.5.1.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from Release 15 onwards. Applicability requires support of SFTP measurements between a E-UTRA PCell and an NR PSCell.

4.7.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.7.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.7.5.1.

4.7.5.1.4	Test description

4.7.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.7.5.1.4.1-1.

Table 4.7.5.1.4.1-1: Test configurations

| Test Case ID | Description |
| --- | --- |
| 4.7.5.1-1 | LTE FDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.7.5.1-2 | LTE FDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 4.7.5.1-3 | LTE FDD, NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| 4.7.5.1-4 | LTE TDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 4.7.5.1-5 | LTE TDD, NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 4.7.5.1-6 | LTE TDD, NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 4.7.5.1.4.1-2.

Table 4.7.5.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 4.7.5.1.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is the NR FR1 cell. Cell 2 is the PSCell. The connection setup is done according to the settings in Annex C.1.1.

3.	The SFTD between PCell and PSCell shall be set to one of the differences in Table 4.7.5.1.5-2.

4.7.5.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 4.7.5.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit a MeasurementReport containing the MeasResultCellListSFTD-NR for the PSCell with SFTD measurements.

6.	The SS shall check the reported SFN offset and frame boundary offset between the E-UTRA PCell and the NR PSCell. If the reported values fulfil the accuracy requirements in clause 4.7.5.1.5, the number of passed iterations is increased by one. If the reported fulfil are outside the limits in clause 4.7.5.1.5 or the UE does not report the SFTD measurements, the number of failed iterations is increased by one.

7.	The SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

8.	The SS then shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

9.	If any the reconfiguration fails, switch off and on the UE and ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

10.	Repeat steps 3-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

4.7.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.7.5.1.4.3-1: Common exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.4-1 Table H.3.4-2 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 4.7.1.1.1.-1 and 4.7.1.1.1-4 | Table H.3.1-3 with Condition Asynchronous cells |
| Specific message contents exceptions for Test Configuration 4.7.1.1.1-2, 4.7.1.1.1-3, 4.7.1.1.1-5 and 4.7.1.1.1-6 | Table H.3.1-3 with Condition Synchronous cells |

Table 4.7.5.1.4.3-2: ReportConfigNR-DEFAULT

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| reportSFTD SEQUENCE { |  |  |  |
| reportSFTD-Meas | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

4.7.5.1.5	Test requirement

Table 4.7.5.1.5-1 defines the primary level settings including test tolerances.

Table 4.7.5.1.5-1: Test parameters for SFTD accuracy

| Parameter | Config | Unit | Test 1 |
| --- | --- | --- | --- |
| SSB GSCN | 1~6 |  | freq1 |
| Duplex mode | 1,4 |  | FDD |
|  | 2,5 |  | TDD |
|  | 3,6 |  | TDD |
| TDD Configuration | 1,4 |  | N/A |
|  | 2,5 |  | TDDConf.1.1 |
|  | 3,6 |  | TDDConf.2.1 |
| BWchannel | 1,4 | MHz | 10: NRB,c = 52 |
|  | 2,5 |  | 10: NRB,c = 52 |
|  | 3,6 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1,4 |  | SR.1.1 FDD |
|  | 2,5 |  | SR.1.1 TDD |
|  | 3,6 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1,4 |  | CR.1.1 FDD |
|  | 2,5 |  | CR.1.1 TDD |
|  | 3,6 |  | CR.2.1 TDD |
| RMC CORESET Reference Channel | 1,4 |  | CCR.1.1 FDD |
|  | 2,5 |  | CCR.1.1 TDD |
|  | 3,6 |  | CCR.2.1 TDD |
| SSB configuration | 1,4 |  | SSB.1 FR1 |
|  | 2,5 |  | SSB.1 FR1 |
|  | 3,6 |  | SSB.2 FR1 |
| SMTC configuration | 1~6 |  | SMTC.1 |
| DL BWP configuration | 1~6 |  | DLBWP.1.1 |
| UL BWP configuration | 1~6 |  | ULBWP.1.1 |
| CSI-RS for tracking | 1,4 |  | TRS.1.1 FDD |
|  | 2,5 |  | TRS.1.1 TDD |
|  | 3,6 |  | TRS.1.2 TDD |
| OCNG Patterns | 1~6 |  | OP.1 |
| EPRE ratio of PSS to SSS | 1~6 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note2 | 1~6 | dBm/15kHz | -104 |
| Note2 | 1,2,4,5 | dBm/SSB SCS | -104 |
|  | 3,6 |  | -101 |
|  | 1~6 | dB | -2.7 |
|  | 1~6 | dB | -2.7 |
| SS-RSRP Note3 | 1,2,4,5 | dBm/SCS | -106.7 |
|  | 3,6 |  | -103.7 |
| Io Note3 | 1,2,4,5 | dBm/9.36 MHz | -74.18 |
|  | 3,6 | dBm/38.16 MHz | -68.08 |
| Propagation condition | 1~6 |  | AWGN |
| Antenna configuration | 1~6 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |

The SFTD reported by the UE consists of two elements, the SFN offset and the frame boundary offset between PCell and PSCell. Table 4.7.5.1.5-2 defines the timing offsets for the SFTD accuracy test. The reported SFTD shall match the values in Table 4.7.5.1.5-2 ±40 Ts to fulfil the accuracy requirements.

Table 4.7.5.1.5-2: Timing offsets for SFTD accuracy test

| Configuration | SFN offset between PCell and PSCell | Frame boundary offset between PCell and PSCell (Ts) |
| --- | --- | --- |
| 1 | 100 | -122000 |
| 2 | 300 | -60540 |
| 3 | 500 | 1000 |
| 4 | 700 | 62540 |
| 5 | 900 | 124000 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

<<End of Changes>>