3GPP TSG-RAN5 Meeting #90-e                      	R5-211641

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1130 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new RMCs and OCNGs into Annex A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN4 in R4-2100068 has added new RMCs and OCNG to address the issue of not enough dB range to implement RRM test cases using Spherical Coverage direction. |  |  |  |  |  |  |  |  |
| R4- |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align Annex A RMCs and OCNG of 38.533 with corrections provided in R4-2100068 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Newly added RMCs and OCNG will remain missing in 38.533 annex A, Some FR2 test cases using Spherical Coverage direction cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revised from: R5-211225 R1: corrected Allocated resource blocks for PDSCH for configuration SR.3.3 TDD |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

# A.1	Reference measurement channels

This section contains the Reference Measurement Channels (RMC) to be used for the RRM test scenarios in Sections 4 to 7 of this document.

## A.1.1	PDSCH

### A.1.1.1	FDD

Table A.1.1.1-1: PDSCH Reference Measurement Channels for SCS = 15 kHz for FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | SR.1.1 FDD |
| Channel bandwidth | MHz | 10 |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |
| Allocated slots per Radio Frame |  | 10 |
| Radio frame containing SSB | slots | Note 5 |
| Radio frame not containing SSB | slots | 10 |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI Note 2 | Bits | 1608 |
| For slots without RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

### A.1.1.2	TDD

Table A.1.1.2-1: PDSCH Reference Measurement Channels for SCS = 15 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | SR.1.1 TDD |
| Channel bandwidth | MHz | 10 |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |
| Allocated slots per Radio Frame |  |  |
| Radio frame containing SSB | slots | Note 5 |
| Radio frame not containing SSB | slots | 4 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI Note 2 | Bits | 1608 |
| For slots without RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

Table A.1.1.2-2: PDSCH Reference Measurement Channels for SCS = 30 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | SR.2.1 TDD |
| Channel bandwidth | MHz | 40 |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |
| Allocated slots per Radio Frame |  |  |
| Radio frame containing SSB | slots | Note 5 |
| Radio frame not containing SSB | slots | 10 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI Note 2 | Bits | 1608 |
| For slots without RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

Table A.1.1.2-3: PDSCH Reference Measurement Channels for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | SR.3.1 TDD | SR.3.2 TDD | SR.3.3 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 |
| Number of transmitter antennas |  | 1 | 1 | 1 |
| Allocated resource blocks for PDSCH |  | 24Note 1 | 24Note 7 | 48Note 7 |
| Allocated slots per Radio Frame |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 8 | Note 5 |
| Radio frame not containing SSB | slots | 48 | 48 | 48 |
| MCS table |  | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Number of control symbols |  | 2 | 2 | 2 |
| PDSCH mapping type |  | Type A | Type A | Type A |
| Information Bit Payload |  |  |  |  |
| For slots with RMSI | Bits | 1608 | 1608 | 3104 |
| For slots without RMSI | Bits | 1864 | 1864 | 3624 |
| Number of Code Blocks per slot |  | 1 | 1 | 1 |
| Binary Channel Bits Per slot |  |  |  |  |
| For slots with RMSI Note 4 | Bits | 5184 | 5184 | 10368 |
| For slots without RMSI Note 6 | Bits | 6048 | 6048 | 12096 |
| Note 1: Allocated in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: Void Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditionalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: Allocated in the same resource blocks as the CORESET. |  |  |  |  |

## A.1.2	CORESET for RMSI scheduling

### A.1.2.1	FDD

Table A.1.2.1-1: RMSI CORESET Reference Measurement Channels for SCS = 15 kHz for FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.1.1 FDD |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing for RMSI CORESET | kHz | 15 |
| Allocated resource blocks for RMSI CORESETNote 7 |  | 24 |
| Subcarrier spacing for SSB | kHz | 15 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESETNote 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| Note 1: DCI formats are defined in TS 38.212 [31]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [8]. Note 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

### A.1.2.2	TDD

Table A.1.2.2-1: RMSI CORESET Reference Measurement Channels for SCS = 15 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.1.1 TDD |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing for RMSI CORESET | kHz | 15 |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |
| Index of transmitted SSB within an SS-Burst |  | #0 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESET Note 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| Note 1: DCI formats are defined in TS 38.212 [31]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [8]. Note 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

Table A.1.2.2-2: RMSI CORESET Reference Measurement Channels for SCS = 30 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.2.1 TDD |
| Channel bandwidth | MHz | 40 |
| Subcarrier spacing for RMSI CORESET | kHz | 30 |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |
| Index of transmitted SSB within an SS-Burst |  | #0 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESET Note 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| Note 1: DCI formats are defined in TS 38.212 [31]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-6 in TS 38.213 [8]. Note 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

Table A.1.2.2-3: RMSI CORESET Reference Measurement Channels for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | CR.3.1 TDD | CR.3.2 TDD |
| Channel bandwidth | MHz | 100 | 100 |
| Subcarrier spacing for RMSI CORESET | kHz | 120 | 120 |
| Allocated resource blocks for RMSI CORESET |  | 24Note 7 | 48Note 9 |
| Subcarrier spacing for SSB | kHz | 120 | 120 |
| Index of transmitted SSB within an SS-Burst |  | #0 | #0 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1Note 7 | Pattern 1Note 9 |
| Offset between SSB and RMSI CORESET Note 3 | RB | 0 (Note 8) Note 7 | 0 (Note 8)Note 9 |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 |
| Number of transmitter antennas |  | 1 | 1 |
| Duration of RMSI CORESET | symbols | 2Note 7 | 2Note 9 |
| DCI Format Note 1 |  | Note 2 | Note 2 |
| Aggregation level | CCE | 8 | 8 |
| DMRS precoder granularity |  | 6 | 6 |
| REG bundle size |  | 6 | 6 |
| Mapping from REG to CCE |  | Distributed | Distributed |
| Cell ID |  | Note 5 | Note 5 |
| Payload (without CRC) | Bits | Note 6 | Note 6 |
| Note 1: DCI formats are defined in TS 38.212 [31]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-8 in TS 38.213 [8]. Note 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC Note 9: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 2 in Table 13-10 in TS 38.213 [8]. |  |  |  |

## A.1.3	CORESET for RMC scheduling

### A.1.3.1	FDD

Table A.1.3.1-1: Control Channel RMC for SCS = 15 kHz for FDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 FDD | CCR.1.2 FDD | CCR.1.3 FDD | CCR.1.4 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |
| Subcarrier spacing for RMSI CORESET | kHz | 15 | 15 | 15 | 15 |
| Allocated resource blocks for CORESET |  | 24 | 18 | 24 | 18 |
| Number of transmitter antenna |  | 1 | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 |
| REG bundle size |  | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 2 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |

### A.1.3.2	TDD

Table A.1.3.2-1: Control Channel RMC for SCS = 15 kHz for TDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 TDD | CCR.1.2 TDD | CCR.1.3 TDD | CCR.1.4 TDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Allocated resource blocks for CORESET Note3 |  | 24 | 18 | 24 | 18 |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 |
| REG bundle size |  | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 2 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |

Table A.1.3.2-2: Control Channel RMC for SCS = 30 kHz for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 TDD | CCR.2.2 TDD |
| Channel bandwidth | MHz | 40 | 40 |
| Subcarrier spacing for RMSI CORESET | kHz | 30 | 30 |
| Allocated resource blocks for CORESET3 |  | 24 | 24 |
| Number of transmitter antenna |  | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 |
| REG bundle size |  | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 |
| Interleave size |  | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A |
| Aggregation level | CCE | 4 | 8 |
| DCI formats |  | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |

Table A.1.3.2-3: Control Channel RMC for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.3.1 TDD | CCR.3.2 TDD | CCR.3.3 TDD | CCR.3.4 TDD | CCR.3.5 TDD | CCR.3.6 TDD |  | CCR.3.7 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 | 100 | 100 | 100 |  | 100 |
| Subcarrier spacing for RMSI CORESET | kHz | 120 | 120 | 120 | 120 | 120 | 120 |  | 120 |
| Allocated resource blocks for CORESETNote 3 |  | 24 | 24 | 24 | 24 | 24 | 24 |  | 48 |
| Number of transmitter antenna |  | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 |
| monitoringSlotPeriodicityAndOffset |  | sl160 0 | sl160 0 | sl160 80 | sl160 0 | sl160 0 | sl160 80 |  | sl160 0 |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 |  | 1100000 0000000 |
| Duration of CORESET | symbols | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 |  | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |  | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |  | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 |  | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| Aggregation level | CCE | 4 | 4 | 4 | 8 | 8 | 8 |  | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |  | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |  | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the same resource blocks where the associated PDSCH RMC is scheduled. |  |  |  |  |  |  |  |  |  |  |

## A.1.4	CSI-RS

### A.1.4.1	FDD

Table A.1.4.1-1: CSI-RS Reference Measurement Channels for SCS = 15 kHz for FDD

|  | CSI-RS.1.1 FDD | CSI-RS.1.2 FDD | CSI-RS.1.3 FDD | CSI-RS.1.4 FDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 6 | 6 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | N/A | N/A |
| Offset | 1 | 1 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

### A.1.4.2	TDD

Table A.1.4.2-1: CSI-RS Reference Measurement Channels for SCS = 15 kHz for TDD

|  | CSI-RS.1.1 TDD | CSI-RS.1.2 TDD | CSI-RS.1.3 TDD | CSI-RS.1.4 TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 6 | 6 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | N/A | N/A |
| Offset | 1 | 1 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

Table A.1.4.2-2: CSI-RS Reference Measurement Channels for SCS = 30 kHz for TDD

|  | CSI-RS.2.1 TDD | CSI-RS.2.2 TDD | CSI-RS.2.3 TDD | CSI-RS.2.4 TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 6 | 6 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot10 | slot20 | N/A | N/A |
| Offset | 2 | 2 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

Table A.1.4.2-3: CSI-RS Reference Measurement Channels for SCS = 120 kHz for TDD

|  | CSI-RS.3.1 TDD | CSI-RS.3.2 TDD | CSI-RS.3.3 TDD | CSI-RS.3.4 TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 6 | 6 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot40 | slot80 | N/A | N/A |
| Offset | 8 | 8 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

## A.1.4A	CSI-RS for tracking

### A.1.4A.1	FR1

#### A.1.4A.1.1	FDD

Table A.1.4A.1.1-1: CSI-RS for tracking FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.1 FDD | TRS.1.2 FDD |
| Bandwidth |  | BW of Active BWP1 | BW of Active BWP1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | -3 | -3 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

#### A.1.4A.1.2	TDD

Table A.1.4A.1.2-1: CSI-RS for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.1 TDD | TRS.1.2 TDD |
| Bandwidth |  | BW of Active BWP1 | BW of Active BWP1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | -3 | -3 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

### A.1.4A.2	FR2

#### A.1.4A.2.1	TDD

Table A.1.4A.2.1-1: CSI-RS for tracking for TDD FR2

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.2.1 TDD | TRS.2.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1, 2 | BW of Active BWPNote 1,2 |
| SCS | kHz | 120 | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 | l0 = 2 for CSI-RS resource 1 and 3 l0 = 6 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | -3 | -3 |
| TCI state |  | TCI.State.0 | TCI.State.1 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |  |

## A.1.5	TDD UL/DL configuration

Table A.1.5-1: TDD UL/DL configuration for SCS=15kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 |  |  |
| referenceSubcarrierSpacing | kHz | 15 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=‘10DL:2GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 |  |  |
| nrofDownlinkSlots |  | 1 |  |  |
| nrofDownlinkSymbols |  | 10 |  |  |
| nrofUplinkSlot |  | 2 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 |  |  |
| nrofDownlinkSlots |  | 1 |  |  |
| nrofDownlinkSymbols |  | 0 |  |  |
| nrofUplinkSlot |  | 0 |  |  |
| nrofUplinkSymbols |  | 0 |  |  |
| Note 1: As specified in TS 38.213 [8] and TS 38.331 [13]. Note 2: For information |  |  |  |  |

Table A.1.5-2: TDD UL/DL configuration for SCS=30kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 |  |  |
| referenceSubcarrierSpacing | kHz | 30 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=‘6DL:4GP:4UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 6 |  |  |
| nrofUplinkSlot |  | 4 |  |  |
| nrofUplinkSymbols |  | 4 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 |  |  |
| nrofDownlinkSlots |  | 2 |  |  |
| nrofDownlinkSymbols |  | 0 |  |  |
| nrofUplinkSlot |  | 0 |  |  |
| nrofUplinkSymbols |  | 0 |  |  |
| Note 1: As specified in TS 38.213 [8] and TS 38.331 [13]. Note 2: For information |  |  |  |  |

Table A.1.5-3: TDD UL/DL configuration for SCS=120kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.3.1 |  |  |
| referenceSubcarrierSpacing | kHz | 120 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DDDSU’ S=‘10DL:2GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 0.625 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 10 |  |  |
| nrofUplinkSlot |  | 1 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |  |  |
| dl-UL-TransmissionPeriodicity | ms | Not configured |  |  |
| nrofDownlinkSlots |  | Not configured |  |  |
| nrofDownlinkSymbols |  | Not configured |  |  |
| nrofUplinkSlot |  | Not configured |  |  |
| nrofUplinkSymbols |  | Not configured |  |  |
| Note 1: As specified in TS 38.213 [8] and TS 38.331 [13]. Note 2: For information |  |  |  |  |

## A.1.6	PUSCH

This rule applies to NR cell(s), which the UE is connected to. The UE is in RRC_CONNECTED mode.

When signalling or data payloads are expected to be sent on the PUSCH, the UE may be provided in advance with PUSCH resources by the SS. For sake of simplicity, the PUSCH scheduling may also occur continuously over many consecutive subframes. These options shall not be used if:
1) stated otherwise in the test description, or 
2) the transmission of PUSCH and UL scheduling information affects the test purpose (e.g. DRX, PUCCH reception etc.)

# A.2	Reference OCGN configuration

## A.2.1 Generic OFDMA channel noise generator (OCGN)

The OCGN pattern is used in a test for modelling the allocation of unused resourced in the channel bandwidth to virtual UEs (UEs that are not under test). The OCNG pattern simulates both PDCCH and PDSCH transmissions to the virtual UEs.

Table A.2.1-1: OP.1: Generic OCNG pattern for all unused REs

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. |  |  |

Table A.2.1-2: OP.2: Generic OCNG pattern for all unused REs for 2AoA setup

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Probe | Transmitting the serving beam |  |
| Resource allocation | Unused REs (Note 1) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. | Unused REs (Note 2) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. Note 3: No OCNG is transmitted from the probe transmitting non-serving beam. |  |  |

Table A.2.1-3: OP.3: Generic OCNG pattern for unused REs in the same BW as CORESET

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the allocated bandwidth of the CORESET of the serving cell. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. |  |  |

Table A.2.1-4: OP.4: Generic OCNG pattern for all unused REs outside SSB slot(s)

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated in the slot(s) containing SSB of the respective cell. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. REs for OCNG shall not be allocated in the slot(s) containing SSB of the respective cell. |  |  |

Table A.2.1-4: OP.5: Generic OCNG pattern for unused REs in the same BW as CORESET for 2AoA setup

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Probe | Transmitting the serving beam |  |
| Resource allocation | Unused REs (Note 1) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. | Unused REs (Note 2) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the allocated bandwidth of the CORESET of the serving cell. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. Note 3: No OCNG is transmitted from the probe transmitting non-serving beam. |  |  |

///End of changes