3GPP TSG-RAN5 Meeting #90-e	R5-211671

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1707 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct reportOffsetList in CSI-ReportConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | reportOffseList in CSI-ReportConfig is a list, but it is treated as a single value |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the format of reportOffsetList to a sequence of elements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The format of reportOffsetList will continue to be incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: corrected the formatting of the reportOffsetList IE to match PRD13 |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### –	CSI-ReportConfig

Table 4.6.3-39: CSI-ReportConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId |  |  |
| carrier | ServCellIndex |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId |  |  |
| nzp-CSI-RS-ResourcesForInterference | CSI-ResourceConfigId |  |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER { | 1 entry |  |  |
| INTEGER[1] | 14 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RI-PMI-CQI | NULL, |  | FR1 |
| cri-RI-LI-PMI-CQI | NULL |  | FR2 |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| cqi-FormatIndicator | widebandCQI |  |  |
| pmi-FormatIndicator | widebandPMI |  |  |
| csi-ReportingBand | Not present |  |  |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | notConfigured |  |  |
| timeRestrictionForInterferenceMeasurements | notConfigured |  |  |
| codebookConfig | CodebookConfig |  |  |
| dummy | Not present |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n1 |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-Table | table2 |  | FR1 |
|  | table1 |  | FR2 |
| subbandSize | value2 |  |  |
| non-PMI-PortIndication | Not present |  |  |
| } |  |  |  |

< End of changes >