3GPP TSG-RAN5 Meeting #90-e	R5-211756

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5266 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.6.2.3_s ACLR for sTTI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_sTTIandPT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test case 6.6.2.3_s is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP would not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.3_s(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-210781r1 |  |  |  |  |  |  |  |  |

<Start of Changes>

6.6.2.3_s	Adjacent Channel Leakage power Ratio for subslot/slot TTI

6.6.2.3_s.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.6.2.3_s.2	Test applicability

This test case applies to all types of E-UTRA Power Class 3 UE release 15 and forward that support subslot/slot TTI.

6.6.2.3_s.3	Minimum conformance requirements

ACLR requirements are specified for two scenarios for an adjacent E-UTRAACLR and UTRAACLR1/2 as shown in Figure 6.6.2.3_s.3-1.

Figure 6.6.2.3_s.3-1: Adjacent Channel Leakage Power Ratio requirements

6.6.2.3_s.3.1	Minimum conformance requirements for E-UTRA

E-UTRA ACLR (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA channel power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidth specified in Table 6.6.2.3_s.3.1-1.

If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the valued specified in Table 6.6.2.3_s.3.1-1.

Table 6.6.2.3_s.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

The normative reference for this requirement is TS 36.101 [2] subclause 6.6.2.3.1.

6.6.2.3_s.3.2	 Minimum conformance requirements for UTRA

UTRA ACLR (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent UTRA channel frequency.

UTRA ACLR is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2).The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor = 0.22. The assigned E-UTRA channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3_s.3.2-1.

If the measured UTRA channel power is greater than –50dBm then the UTRAACLR1, and UTRAACLR2 shall be higher than the valued specified in Table 6.6.2.3_s.3.2-1.

Table 6.6.2.3_s.3.2-1: General requirements for UTRAACLR1/2

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 subclause 6.6.2.3.2.

6.6.2.3_s.4	Test description

6.6.2.3_s.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.3_s.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in AnnexeA.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.3_s.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for ACLR testing |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM | 6 | 6 |
| 1.4MHz |  |  |  | 16QAM | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM | 15 | 15 |
| 3MHz |  |  |  | 16QAM | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | 16QAM | 25 | 25 |
| 5MHz |  |  |  | 16QAM | 8 | 8 |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | 16QAM | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  |  |  | 16QAM | 12 | 12 |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | 16QAM | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  |  |  | 16QAM | 16 | 16 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | 16QAM | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  |  |  | 16QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB#0 and RB# (max + 1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories 2-5 |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.3_s.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.6.2.3_s.4.3.

6.6.2.3_s.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.6.2.3_s.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.3_s.5-1. The period of the measurement shall be at least the continuous duration of 7 OS (0.5ms) for slot TTI and 2/3 OS for subslot TTI. For TDD slots with transient periods are not under test.

4.	Measure the rectangular filtered mean power for E-UTRA.

5.	Measure the rectangular filtered mean power of the first E-UTRA adjacent channel on both lower and upper side of the E-UTRA channel, respectively.

6.	Measure the RRC filtered mean power of the first and the second UTRA adjacent channel on both lower and upper side of the E-UTRA channel, respectively.

7.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper E-UTRAACLR, respectively.

8.	Calculated the ratios of the power between the values measured in step 4 over step 6 for lower and upper UTRAACLR1, UTRAACLR2, respectively.

6.6.2.3_s.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 6.6.2.3_s.4.3-1: PhysicalConfigDedicated-DEFAULT (preamble)

| Derivation Path: 36.508 Table 4.8.2.1.6-1 with condition Short-TTI |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| PhysicalConfigDedicatedSTTI-r15 SEQUENCE { |  |  |  |  |
| shortTTI-r15 SEQUENCE { |  |  |  |  |
| dl-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 OR FDD_sTTI-27 |
| ul-STTI-Length-r15 |  | subslot |  | FDD_sTTI-22 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD_sTTI-22 | FDD cell environment and UEs supporting {subslot, subslot} combination |
| FDD_sTTI-27 | FDD cell environment and UEs supporting {subslot, slot} combination |

6.6.2.3_s.5	Test requirement

6.6.2.3_s.5.1	Test requirements E-UTRA

-	The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.3.5-1 as appropriate,

and

-	if the measured adjacent channel power is greater than –50 dBm then the measured E-UTRAACLR, derived in step 7, shall be higher than the limits in table 6.6.2.3_s.5.1-1.

Table 6.6.2.3_s.5.1-1: E-UTRA UE ACLR

|  | Channel bandwidth / E-UTRAACLR1 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB | 29.2 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UE channel | +1.4 MHz or -1.4 MHz | +3 MHz or -3 MHz | +5MHz or -5MHz | +10MHz or -10MHz | +15MHz or -15MHz | +20MHz or -20MHz |

6.6.2.3_s.5.2 Test requirements UTRA

If the measured UTRA channel power is greater than –50dBm then the measured UTRAACLR1, UTRAACLR2, derived in step 8, shall be higher than the limits in table 6.6.2.3_s.5.2-1.

Table 6.6.2.3_s.5.2-1: UTRA UE ACLR

|  | Channel bandwidth / UTRAACLR1/2 / measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB | 32.2 dB |
| Adjacent channel centre frequency offset (in MHz) | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | 2.5+BWUTRA/2 / -2.5-BWUTRA/2 | 5+BWUTRA/2 / -5-BWUTRA/2 | 7.5+BWUTRA/2 / -7.5-BWUTRA/2 | 10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 35.2 dB | 35.2 dB | 35.2 dB | 35.2 dB |
| Adjacent channel centre frequency offset (in MHz) | - | - | 2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | 5+3*BWUTRA/2 / -5-3*BWUTRA/2 | 7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | 10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth1 | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth2 | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28 MHz |
| Note 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. Note 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. Note 3: BWUTRA for UTRA FDD is 5MHz and for UTRA TDD is 1.6MHz. |  |  |  |  |  |  |

<End of Changes>