3GPP TSG-RAN5 Meeting #90-e	R5-211794

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2565 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 7.3.88 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRM Test Case 7.3.88 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage has been introduced in RAN4 [4] core-spec, but not added in RAN5 TS 36.521-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Test Case 7.3.88 and TT reused from HD-FDD Test Cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case 7.3.88 will be still missed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.88 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Put all non-test case titles to H6. - Adding information in summary of change R2: - Correct word from “HD-FDD” to “TDD” in test purpose R3: - Removing redundant statement from Section 7.3.88.4.1 Initial conditions R4: - Put all non-test case titles to H6 with official Arial 10. - Correct typos This is the final outcome of R5-210887 and the result of 1 revision |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

7.3.88.1	Test purpose

The purpose of this test is to verify that the TDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT TDD radio link monitoring requirements in clause TS36.133 clause 7.23.

7.3.88.2	Test applicability

This test applies to all types of E-UTRA TDD UE release 15 and forward of UE Category NB1 and NB2.   Applicability requires support for FGI bit 5.

7.3.88.3	Minimum conformance requirements

Same minimum conformance requirements as defined in 7.3.60.3.

7.3.88.4	Test description

Same test description as defined in 7.3.60.4

7.3.88.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-4 and TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: E-UTRA Bandwidth is 10 MHz, as defined in TS 36.508 [7] clause 4.3.1.1. Ncell bandwidth is as specified in Table 7.3.88.5-1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A figure A.10 using only main Tx/Rx antenna and without using the faders.

2.	The general test parameter settings are set up according to Table 7.3.88.4.1-1.

3.	Propagation conditions are set according to Annex B clauses B.0.

4.	Message contents are defined in clause 7.3.88.4.3

5.	There are two cells specified in this test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test. Cell 1 is a EUTRAN cell.

Table 7.3.88.4.1-1: General test parameters for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | nCell 1 |  |
| Neighbour cell |  | eCell 1 |  |
| CP length |  | Normal |  |
| Deployment Mode |  | In-band |  |
| NPDCCH transmission parameters Rmax |  | 8 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.3.60.3-1 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 [16] |
| DRX cycle | ms | 256 | See Table 7.3.60.5-2 |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 1000 | T311 is enabled |
| T1 | s | 5.12 |  |
| dT | S | 0.8 |  |
| T2 | s | 10.24 |  |
| dT | S | 0.7 |  |
| T3 | s | 5.12 |  |
| dT | S | 1.4 |  |
| T4 | s | 5.12 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331 [2]. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

7.3.88.4.2	Test procedure

Same test procedure as in clause 7.3.60.4.2.

7.3.88.4.3	Message contents

Same message contents as in clause 7.3.60.4.3.

7.3.88.5	Test requirement

Table 7.3.88.5-1: nCell specific test parameters for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 180 |  |  |  |  |  |  |
| PRB location within eCell |  | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.5.7 |  | R.14 NB-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table 7.3.88.5-1a |  |  |  |  |  |  |
| SNR Note 4, 5 | dB | -2.5 | Note 6 | -8.5 | Note 7 | -14.7 | Note 8 | -2.5 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |
| Configuration |  | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.88.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Table 7.3.88.5-1a: eCell 1 specific test parameters for TDD Out-of-sync radio link monitoring test in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T4 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table 7.3.88.5-2: DRX-Configuration for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.7.3 in TS 36.331 [2] |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| drx-StartOffset | 0 |  |

Table 7.3.88.5-3: TimeAlignmentTimer -Configuration for NB-IoT TDD out-of-sync testing for UE category NB1 In-band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

## << End of changes >>