3GPP TSG-RAN5 Meeting #90-e	R5-211865

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0475 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TX Test Cases for UL MIMO in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Since the test point analysis was mentioned in R5-206874 in RAN5#89-e, so the test configuration needs to update in test spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of test configuration for TX Test Cases for UL MIMO in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuration for UL MIMO in FR2 will be still incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Update according to the SISO TCs in CR# R5-210547 R2: - keep the editor’s note about the test point analysis This is the final outcome of R5-210893 and the result of 1 revision |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.5D.2	Out of band emission for UL MIMO

6.5D.2.1	Spectrum Emission Mask for UL MIMO

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- OTA test procedure for UL MIMO is still under investigation

- TRP Measurement Uncertainty is FFS.

- 38.905 TP analysis for UL MIMO is pending

6.5D.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

6.5D.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward supporting UL MIMO.

6.5D.2.1.3	Minimum conformance requirements

For UE(s) supporting UL MIMO, the Spectrum Emission Mask requirements in clause 6.5.2.1.3 apply. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-3.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.5D.2.

6.5D.2.1.4	Test description

6.5D.2.1.4.1	Initial condition

Same initial condition in clause 6.5.2.1.4.1 with following exceptions:

-	Instead of Table 6.5.2.1.4.1-1 use Table 6.5D.2.1.4.1-1.

-	Instead of Table 6.5.2.1.4.1-2  use Table 6.5D.2.1.4.1-2

Table 6.5D.2.1.4.1-1: Test Configuration Table (Power Class 1)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Spectrum Emission Mask test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  | CP-OFDM QPSK | 8@0 |
| 2 | High |  | CP-OFDM QPSK | 8@NRB-8 |
| 3 | Default |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low |  | CP-OFDM 16 QAM | 8@0 |
| 5 | High |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 6 | Default |  | CP-OFDM 16 QAM | Outer_Full |
| 7 | Low |  | CP-OFDM 64 QAM | 8@0 |
| 8 | High |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 9 | Default |  | CP-OFDM 64 QAM | Outer_Full |
| 10 | Low |  | CP-OFDM 256 QAM | 8@0 |
| 11 | High |  | CP-OFDM 256 QAM | 8@NRB-8 |
| 12 | Default |  | CP-OFDM 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-2 for PC1. NOTE 2: All test points in this table must also exist in table 6.2.2.4.1-1, table 6.2.2.4.1-2, table 6.2.2.4.1-3 (MPR). |  |  |  |  |

Table 6.5D.2.1.4.1-2: Test Configuration Table (Power Class 2, 3 and 4)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  |  |  |  | N/A for Spectrum Emission Mask test case |  | Modulation |  | RB allocation (NOTE 1) |  |
| 1 |  |  | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |  |
| 2 |  |  | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |  |
| 3 |  |  | Default |  |  |  | CP-OFDM QPSK |  | Outer Full |  |
| 4 |  |  | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |  |
| 5 |  |  | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |  |
| 6 |  |  | Default |  |  |  | CP-OFDM 16 QAM |  | Outer Full |  |
| 7 |  |  | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |  |
| 8 |  |  | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |  |
| 9 |  |  | Default |  |  |  | CP-OFDM 64 QAM |  | Outer Full |  |
| 10 |  |  | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |  |
| 11 |  |  | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |  |
| 12 |  |  | Default |  |  |  | CP-OFDM 256 QAM |  | Outer Full |  |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4. NOTE 2: All test points in this table must also exist in table 6.2.2.4.1-7, table 6.2.2.4.1-8, table 6.2.2.4.1-9 (MPR). |  |  |  |  |  |  |  |  |  |  |

6.5D.2.1.4.2	Test procedure

Same test procedure as in clause 6.5.2.1.4.2.

6.5D.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.2.1.5	Test requirements

The test requirement is the same as in clause 6.5.2.1.5.

6.5D.2.2	Adjacent channel leakage ratio for UL MIMO

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- OTA test procedure for UL MIMO is still under investigation

- TRP Measurement Uncertainty is FFS.

6.5D.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

6.5.2.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward supporting UL MIMO.

6.5D.2.2.3	Minimum conformance requirements

For UE(s) supporting UL MIMO, the Adjacent channel leakage ratio requirements in clause 6.5.2.3.3 apply. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-3.

The normative reference for this requirement is TS 38.101-2 [3] clause 6.5D.2.

6.5D.2.2.4	Test description

6.5D.2.2.4.1	Initial condition

Same initial condition in clause 6.5.2.3.4.1 with following exceptions:

-	Instead of Table 6.5.2.3.4.1-1 use Table 6.5D.2.2.4.1-1.

-	Instead of Table 6.5.2.3.4.1-2  use Table 6.5D.2.2.4.1-2.

Table 6.5D.2.2.4.1-1: Test Configuration Table (Power Class 1)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 2 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 3 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 5 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 6 | Default |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 7 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 8 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 9 | Default |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 10 | Low |  |  |  | CP-OFDM 256 QAM | 8@0 |
| 11 | High |  |  |  | CP-OFDM 256 QAM | 8@NRB-8 |
| 12 | Default |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Following Test IDs shall be skipped for FR2b - All Test IDs for 400MHz Channel Bandwidth - Test ID 10-13 for FR2b 200MHz Channel Bandwidth - Test ID 10 for FR2b 100MHz Channel Bandwidth NOTE 3: All test points in this table must also exist in table 6.2.2.4.1-1, table 6.2.2.4.1-2, table 6.2.2.4.1-3 (MPR). |  |  |  |  |  |  |

Table 6.5D.2.2.4.1-2: Test Configuration Table (Power Class 2, 3 and 4)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 2 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 4 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 5 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 6 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 7 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 8 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 10 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Following Test IDs shall be skipped for FR2b - All Test IDs for 400MHz Channel Bandwidth - Test ID 9-12 for FR2b 200MHz Channel Bandwidth - Test ID 9 for FR2b 100MHz Channel Bandwidth NOTE 3: All test points in this table must also exist in table 6.2.2.4.1-7, table 6.2.2.4.1-8, table 6.2.2.4.1-9 (MPR). |  |  |  |  |  |  |

6.5D.2.2.4.2	Test procedure

Same test procedure as in clause 6.5.2.3.4.2.

6.5D.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.5D.2.2.5	Test requirements

The test requirement is the same as in clause 6.5.2.3.5.

## << End of changes >>