3GPP TSG-RAN5 Meeting #90-e	R5-210063r2

# Electronic Meeting, 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1931 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The execution of steps 0Aa1 and 0Aa1 is dependent on PICS paramters pc_EN_DC, pc_NGEN_DC and pc_NG_RAN_NR. Since these are UE capabilities it is possible that a UE may support more than one 5G deployment option, e.g. pc_EN_DC set as TRUE does not necessarily mean the test case is executing in EN-DC mode. This can cause the test steps to be executed incorrectly causing test case failure. - Vedict missing from step 19 and incorrect TP indicated for verdict at step 22 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replaced “IF pc_EN_DC…” with “For EN-DC….”, as is done in other test cases applicable for more than one deployment option. - Corrected the verdicat assignments at step 19 and 22 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases due to some steps being executed when they are not applicable for the current operation mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1->chnages taken R5-210158 to corrected the verdict assignements at step 19 and 22 R2->Changed the text “For EN-DC ..” to “IF Connectivity is EN-DC ..” in order to refer to the standard terms for referring to different 5G deployment options. Also removed the PICS reference at Step 0Ab1. In addition, made couple of editorial changes at Step 3, 4 and 8. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.1.8.1	Bandwidth Part (BWP) operation UL/DL

7.1.1.8.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { BandwidthPart-Config IE is included in RRC Message received on SpCell (i.e. PSCell in case of EN-DC or PCell in case of SA)}

then { UE starts normal MAC operation in the FirstActive UL and DL Bandwidth part }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a DL DCI format 1_1 assigning a BWP different than the previously configured BWP }

then { UE starts normal MAC operation in the received new BWP }

}

(3)

with { UE in RRC_CONNECTED }

ensure that {

when { UE receives a UL DCI format 0_1 assigning a BWP different than the previously configured BWP }

then { UE starts normal MAC operation in the received new BWP }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { RACH Procedure is triggered in SpCell (i.e. PSCell in case of EN-DC or PCell in case of SA) and PRACH occasions are not configured}

then { UE initiates the PRACH procedure in the initial BWP }

}

(5)

with { UE in RRC_Connected State with defaultDownlinkBWP configured }

ensure that {

when { bwp-InactivityTimer expires }

then { UE performs BWP switching to a BWP indicated by the defaultDownlinkBWP }

}

(6)

with { UE in RRC_Connected State with defaultDownlinkBWP configured and Active BWP is different than defaultDownlinkBWP and bwp-InactivityTimer is running }

ensure that {

when { UE receives UL assignment or DL grant addressed to its C-RNTI }

then { UE restarts the bwp-InactivityTimer }

}

7.1.1.8.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.211 clause 4.4.5, TS 38.212 clause 7.3.1.1.2 and 7.3.1.2.2, TS 38.321 clause 5.15 and TS 38.331 clause 5.3.5.3 and 6.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.211, clause 4.4.5]

A bandwidth part is a subset of contiguous common resource blocks defined in subclause 4.4.4.3 for a given numerology  in bandwidth part  on a given carrier. The starting position  and the number of resource blocks  in a bandwidth part shall fulfil  and , respectively. Configuration of a bandwidth part is described in clause 12 of [5, TS 38.213].

A UE can be configured with up to four bandwidth parts in the downlink with a single downlink bandwidth part being active at a given time. The UE is not expected to receive PDSCH, PDCCH, or CSI-RS (except for RRM) outside an active bandwidth part.

A UE can be configured with up to four bandwidth parts in the uplink with a single uplink bandwidth part being active at a given time. If a UE is configured with a supplementary uplink, the UE can in addition be configured with up to four bandwidth parts in the supplementary uplink with a single supplementary uplink bandwidth part being active at a given time. The UE shall not transmit PUSCH or PUCCH outside an active bandwidth part. For an active cell, the UE shall not transmit SRS outside an active bandwidth part.

Unless otherwise noted, the description in this specification applies to each of the bandwidth parts. When there is no risk of confusion, the index μ may be dropped from NBWP,istart,μ, NBWP,isize,μ, Ngrid,xstart,μ, and Ngrid,xsize,μ.

[TS 38.212, clause 7.3.1.1.2]

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator – 0 or 3 bits, as defined in Subclause 10.1 of [5, TS 38.213].

-	UL/SUL indicator – 0 bit for UEs not configured with SUL in the cell or UEs configured with SUL in the cell but only PUCCH carrier in the cell is configured for PUSCH transmission; 1 bit for UEs configured with SUL in the cell as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs  configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

[TS 38.212, clause 7.3.1.2.2]

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Carrier indicator – 0 or 3 bits as defined in Subclause 10.1 of [5, TS 38.213].

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL BWPs  configured by higher layers, excluding the initial DL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

[TS 38.321, clause 5.15]

In addition to clause 12 of TS 38.213 [6], this subclause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id for SpCell or activation of an SCell, the DL BWP and/or UL BWP indicated by firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id respectively (as specified in TS 38.331 [5]) is active without receiving PDCCH indicating a downlink assignment or an uplink grant. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

For each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	if a BWP is activated:

2>	transmit on UL-SCH on the BWP;

2>	transmit on RACH on the BWP, if PRACH occasions are configured;

2>	monitor the PDCCH on the BWP;

2>	transmit PUCCH on the BWP, if configured;

2>	report CSI for the BWP;

2>	transmit SRS on the BWP, if configured;

2>	receive DL-SCH on the BWP;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2.

1>	if a BWP is deactivated:

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not monitor the PDCCH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	not report CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not receive DL-SCH on the BWP;

2>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP;

2>	suspend any configured uplink grant of configured grant Type 1 on the inactive BWP.

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in subclause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	switch the active UL BWP to BWP indicated by initialUplinkBWP;

2>	if the Serving Cell is a SpCell:

3>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is a SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in subclauses 5.1.4 and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the Serving Cell.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching.

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer:

1>	if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id; or

1>	if the defaultDownlinkBWP-Id is not configured, and the active DL BWP is not the initialDownlinkBWP:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment:

3>	if there is no ongoing random access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP-Id is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP-Id.

3>	else:

4>	perform BWP switching to the initialDownlinkBWP.

NOTE:	If a Random Access procedure is initiated on an SCell, both this SCell and the SpCell are associated with this Random Access procedure.

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id; or

2>	if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

[TS 38.331, clause 5.2.1]

System Information (SI) is divided into the MIB and a number of SIBs where:

-	...

-	For a UE in RRC_CONNECTED, the network can provide system information through dedicated signalling using the RRCReconfiguration message, e.g. if the UE has an active BWP with no common search space configured to monitor system information or paging.

-	For PSCell and SCells, the network provides the required SI by dedicated signalling, i.e. within an RRCReconfiguration message. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI for SCell, RAN releases and adds the concerned SCell. For PSCell, SI can only be changed with Reconfiguration with Sync.

NOTE:	The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

...

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (MCG is E-UTRA):

2>	if RRCReconfiguration was received via SRB1:

3>	submit the RRCReconfigurationComplete via the EUTRA MCG embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10];

3>	if reconfigurationWithSync was included in spCellConfig of an SCG:

4>	initiate the random access procedure on the SpCell, as specified in TS 38.321 [3];

...

NOTE:	For EN-DC, in the case RRCReconfiguration is received via SRB1, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

[TS 38.331, clause 6.3.2]

7.1.1.8.1.3	Test description

7.1.1.8.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.8.1.3.2	Test procedure sequence

Table 7.1.1.8.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS transmits RRCReconfiguration to configure the dedicated BWPs incl. the FirstActive BWP. (Note 1) (Note 4). | <-- | (RRCReconfiguration) | - | - |
| - | EXCEPTION: Steps 0Aa1 to 0Ab2 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 0Aa1 | IF Connectivity is EN-DC or NGEN-DC, the UE sends RRCReconfigurationComplete (Note 2). | --> | (RRCReconfigurationComplete) | - | - |
| 0Ab1 | IF Connectivity is NR, the SS allocates (transmitted in FirstActiveDownlinkBWP) an UL Grant with DCI format 0_1 indicating FirstActiveUplinkBWP (BWP#1). | <-- | UL Grant | - | - |
| 0Ab2 | Check: Does the UE send RRCReconfigurationComplete in the FirstActive BWP configured. (Note 2) (Note 3) (Note 5) | --> | (RRCReconfigurationComplete) | 1 | P |
| 1 | The SS transmits a valid MAC PDU containing RLC PDU in the configured FirstActive Downlink BWP configured. | <-- | MAC PDU | - | - |
| 2 | After 100ms from step 1, the SS allocates (transmitted in FirstActiveDownlinkBWP) an UL Grant. | <-- | UL Grant | - | - |
| 3 | Check: Does the UE transmit a MAC PDU including one RLC SDU in the FirstActive BWP configured? (Note 5) | --> | MAC PDU | 1 | P |
| 4 | Void | - | - | - | - |
| 5 | The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id (= BWP #2) and transmits a MAC PDU containing RLC PDU on the newly configured BWP (i.e. Downlink BWP#2). | <-- | MAC PDU | - | - |
| 6 | After 100ms from step5, the SS allocates (transmitted in Downlink BWP#2) an UL Grant (with DCI indicating BWP#2), sufficient for loopback of the RLC SDU from step 5 in a Slot. (Note 3) | <-- | UL Grant | - | - |
| 7 | Check: Does the UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#2)? (Note 5) | --> | MAC PDU | 2 | P |
| 8 | Void | - | - | - | - |
| 9 | The SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (i.e. Downlink BWP#2). | <-- | MAC PDU | - | - |
| 10 | After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) UL DCI format 0_1 with new BWP Id (= BWP #3) and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9 in a Slot. | <-- | UL Grant | - | - |
| 11 | Check: Does the UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#3 for FDD and for TDD)? (Note 5) | --> | MAC PDU | 3 | P |
| 11A | The SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (i.e. Downlink BWP#2 for FDD or Downlink BWP#3 for TDD). | <-- | MAC PDU | - | - |
| 12 | After 100ms from step 11A the SS indicates PDCCH order for contention-based random access (transmitted in Downlink BWP#2 for FDD resp. Downlink BWP#3 for TDD). | <-- | PDCCH Order (ra-PreambleIndex = '000000'B) | - | - |
| 13 | Check: Does the UE send PRACH Preamble in the initial BWP (UL BWP#0)? | --> | PRACH Preamble | 4 | P |
| 13A | The SS transmits (in Downlink BWP #0) a MAC PDU addressed to UE RA-RNTI, containing RAR with matching RAPID in MAC sub header. | <-- | Random Access Response | - | - |
| 13B | The UE sends (in UL BWP#0) a msg3 in the grant associated to the received Random Access Response. | --> | msg3 (C-RNTI MAC CONTROL ELEMENT) | - | - |
| 13C | SS schedules (in Downlink BWP#0) PDCCH transmission for UE C-RNTI and allocates UL grant sufficient for the UE to loop back the data received at step 11a. | <-- | Contention Resolution | - | - |
| 13D | Check: Does the UE transmit a MAC PDU including one RLC SDU in the initial BWP (i.e. Uplink BWP#0)? (Note 5) | --> | MAC PDU | 4 | P |
| 14-15 | Void | - | - | - | - |
| 16 | The SS indicates on PDCCH (transmitted in Downlink BWP#0) DL DCI format 1_1 with BWP Id (= BWP #1) and transmits a MAC PDU containing RLC PDU on the configured BWP (i.e. Downlink BWP#1). | <-- | MAC PDU | - | - |
| 17 | After 400 ms from step 16, the SS transmits another valid MAC PDU containing RLC PDU in the active BWP (i.e. Downlink BWP#1). | <-- | MAC PDU | - | - |
| 18 | After 400 ms from step 17, the SS allocates (transmitted in Downlink BWP#1) an UL Grant, sufficient for loopback of a MAC PDU containing both RLC SDUs from steps 16 and 17 in a Slot. | <-- | UL Grant | - | - |
| 19 | Check: Does the UE transmit a MAC PDU containing both RLC SDUsin the active BWP (i.e. Uplink BWP#1)? (Note 5) | --> | MAC PDU | 5 | P |
| 20 | The SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (= Downlink BWP#2). | <-- | MAC PDU | - | - |
| 21 | The SS allocates (transmitted in the defaultDownlinkBWP, i.e. Downlink BWP#2) an UL Grant, sufficient for loopback of the RLC SDU from step 20 in a Slot. | <-- | UL Grant | - | - |
| 22 | Check: Does the UE transmit a MAC PDU in Uplink BWP#2 (= BWP Id of the defaultDownlinkBWP)? (Note 5) | --> | MAC PDU | 6 | P |
| Note 1: for EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: for EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: In paired spectrum (= FDD), the switching of Downlink BWP and Uplink BWP can happen independently. Whereas in TDD, the switching of BWP for Downlink and Uplink is always at the same time instance. Currently, the scope of the Test Purposes (TP) is considered to not cover checking of a BWP deviation which results from non-synchronized Downlink and Uplink BWP switching in FDD. Note 4: After the preamble the UE is in RRC_CONNECTED, therefore SRBs and DRBs are already established. The RRCReconfiguration message in step 1 and step 14 shall not contain any elements like e.g. "rlc-BearerToAddModList" whose value(s) remain unchanged since the preamble. The sole purpose of the RRCReconfiguration message in step 1 and 14 is to configure BWPs and related fields for switching of BWPs. Note 5: When the UE does not use the expected BWP for the UL transmission the SS shall not receive the data what implicitly fails the test case. |  |  |  |  |  |

7.1.1.8.1.3.3	Specific message contents

Table 7.1.1.8.1.3.3-1: RRCReconfiguration (step 0)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-13 (see also Note 4 in Table 7.1.1.8.1.3.2-1) |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.8.1.3.3-1A: CellGroupConfig (Table 7.1.1.8.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
|  | ServCellIndex |  | EN-DC |
| spCellConfigCommon | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig-Dedicated | Table 7.1.1.8.1.3.3-2 |  |
| } |  |  |  |
| reportUplinkTxDirectCurrent-v1530 | true |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2: ServingCellConfig-Dedicated (Table 7.1.1.8.1.3.3-1A)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
|  | TDD-UL-DL-ConfigDedicated |  | TDD |
| .. initialDownlinkBWP | Not present |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) BWP-Downlink { | 3 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink-BWP-N with condition BWP#1 | entry 1 |  |
| BWP-Downlink[2] | BWP-Downlink-BWP-N with condition BWP#2 | entry 2 |  |
| BWP-Downlink[3] | BWP-Downlink-BWP-N with condition BWP#3 | entry 3 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 |  |  |
| bwp-InactivityTimer | ms500 |  |  |
| defaultDownlinkBWP-Id | 2 |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 3 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink-BWP-N with condition BWP#1 | entry 1 |  |
| BWP-Uplink[2] | BWP-Uplink-BWP-N with condition BWP#2 | entry 2 |  |
| BWP-Uplink[3] | BWP-Uplink-BWP-N with condition BWP#3 | entry 3 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 |  |  |
| pusch-ServingCellConfig | Not present |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig | Not present |  |  |
| pdsch-ServingCellConfig | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP#1 | Bandwidth part 1 |
| BWP#2 | Bandwidth part 2 |
| BWP#3 | Bandwidth part 3 |

Table 7.1.1.8.1.3.3-2A: BWP-Downlink-BWP-N (Table 7.1.1.8.1.3.3-2 and Table 7.1.1.8.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP#1 |
|  | 2 |  | BWP#2 |
|  | 3 |  | BWP#3 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | 1381 | NOTE | BWP#1 |
|  | 1387 | NOTE | BWP#2 |
|  | 1393 | NOTE | BWP#3 |
| } |  |  |  |
| pdcch-ConfigCommon | Not present | no cell specific configuration for dedicated BWP |  |
| pdsch-ConfigCommon | Not present | no cell specific configuration for dedicated BWP |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config-BWP-N |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config-BWP-N |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=6 and RBStart=6,12,18 for BWP#1,2,3 |  |  |  |

Table 7.1.1.8.1.3.3-2B: PDCCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE (SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet-BWP-N with condition BWP#N | entry 1 |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE (SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace-BWP-N with condition BWP#N | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2C: PDSCH-Config (Table 7.1.1.8.1.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList | PDSCH-TimeDomainResourceAllocationList | TS 38.508-1 [4] Table 4.6.3-103 |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2D: ControlResourceSet-BWP-N (Table 7.1.1.8.1.3.3-2B)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 9 |  | BWP#1 |
|  | 10 |  | BWP#2 |
|  | 11 |  | BWP#3 |
| frequencyDomainResources | 10000000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 6 RBs of each BWP |  |
| duration | 2 | SearchSpace duration of 2 symbols |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2E: SearchSpace-BWP-N (Table 7.1.1.8.1.3.3-2B)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 37 |  | BWP#1 |
|  | 38 |  | BWP#2 |
|  | 39 |  | BWP#3 |
| controlResourceSetId | 9 |  | BWP#1 |
|  | 10 |  | BWP#2 |
|  | 11 |  | BWP#3 |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n0 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2F: BWP-Uplink-BWP-N (Table 7.1.1.8.1.3.3-2 and Table 7.1.1.8.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP#1 |
|  | 2 |  | BWP#2 |
|  | 3 |  | BWP#3 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | 1381 | NOTE | BWP#1 |
|  | 1387 | NOTE | BWP#2 |
|  | 1393 | NOTE | BWP#3 |
| } |  |  |  |
| rach-ConfigCommon | Not present | No cell specific configuration for dedicated BWP |  |
| pusch-ConfigCommon | Not present | no cell specific configuration for dedicated BWP |  |
| pucch-ConfigCommon | Not present | no cell specific configuration for dedicated BWP |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config-BWP-N |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config-BWP-N |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE: According to TS 38.214 [21] clause 6.1.2.2.2 with =275, LRBs=6 and RBStart=6,12,18 for BWP#1,2,3 |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP#1 | Bandwidth part 1 |
| BWP#2 | Bandwidth part 2 |
| BWP#3 | Bandwidth part 3 |

Table 7.1.1.8.1.3.3-2G: PUCCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2F)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config::= SEQUENCE { |  |  |  |
| resourceSetToAddModList SEQUENCE (SIZE (1..4)) OF PUCCH-ResourceSet { | 1 entry |  |  |
| PUCCH-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| pucch-ResourceSetId | 0 |  |  |
| resourceList SEQUENCE (SIZE (1..32)) OF PUCCH-ResourceId { | 1 entry |  |  |
| PUCCH-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| maxPayloadSize | 256 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceToAddModList SEQUENCE (SIZE (1..128)) OF PUCCH-Resource { | 1 entry |  |  |
| PUCCH-Resource[1] SEQUENCE { |  | entry 1 |  |
| pucch-RessourceId | 0 |  |  |
| startingPRB | 0 |  | BWP#1 |
|  | 0 |  | BWP#2 |
|  | 0 |  | BWP#3 |
| intraSlotFrequencyHopping | disabled |  |  |
| secondHopPRB | Not Present |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP#1 | Bandwidth part 1 |
| BWP#2 | Bandwidth part 2 |
| BWP#3 | Bandwidth part 3 |

Table 7.1.1.8.1.3.3-2H: PUSCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2F)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList | PUSCH-TimeDomainResourceAllocationList | TS 38.508-1 [4] Table 4.6.3-122 |  |
| } |  |  |  |