3GPP TSG-RAN5 Meeting #90-e	R5-210121r1

# Electronic Meeting, 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0077 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS over 5GS test case 7.30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NR TC 7.30 Session Timer / MO Call / Remote end supports but does not use Session Timer / 5GS is scheduled on the work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding TC of Session Timer / MO Call / Remote end supports but does not use Session Timer / 5GS as CL 7.30. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.30(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0278 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Came to common style for Conformance Requirements. - Changed the description style of Main Behaviour. - Updated the MO INVITE request to carry the condition to indicate NR access. - Editorial corrections. |  |  |  |  |  |  |  |  |

7.30	Session Timer / MO Call / Remote end supports but does not use Session Timer / 5GS

7.30.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and being configured to use Session Timer and preconditions and to be the refresher }

ensure that {

when { UE is being made to initiate a voice call }

then { UE sends INVITE for voice call }

}

(2)

with { UE having sent INVITE for voice call and continuing the call setup }

ensure that {

when { UE receives 200 OK for INVITE with timer tag and without Session-Expires }

then { UE sends ACK }

}

(3)

with { UE having sent ACK }

ensure that {

when { 1860 seconds passed without the UE refreshing the session }

then { UE receives BYE and sends 200 OK for BYE }

}

7.30.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.2A.1.1]

A UE supporting RFC 4028 [58], when it receives a 422 (Session Interval Too Small) to an INVITE request where the response contains a Min-SE header field, shall retry the request in accordance with RFC 4028 [58] subclause 7.4.

[TS 24.229 clause 5.2.7.2]

When the P-CSCF receives from the UE an INVITE request, the P-CSCF may require the periodic refreshment of the session to avoid hung states in the P-CSCF. If the P-CSCF requires the session to be refreshed, then the P-CSCF shall apply the procedures described in RFC 4028 [58] clause 8.

NOTE 1:	Requesting the session to be refreshed requires support by at least one of the UEs. This functionality cannot automatically be granted, i.e. at least one of the involved UEs needs to support it.

[TS 24.229 clause 5.2.7.3]

When the P-CSCF receives an INVITE request destined for the UE the P-CSCF may require the periodic refreshment of the session to avoid hung states in the P-CSCF. If the P-CSCF requires the session to be refreshed, then the P-CSCF shall apply the procedures described in RFC 4028 [58] clause 8.

NOTE 1:	Requesting the session to be refreshed requires support by at least one of the UEs. This functionality cannot automatically be granted, i.e. at least one of the involved UEs needs to support it in order to make it work.

[TS 24.229 clause 5.4.5.3]

If the S-CSCF requested the session to be refreshed periodically, and the S-CSCF got the indication that the session will be refreshed, when the session timer expires, the S-CSCF shall delete all the stored information related to the dialog.

7.30.3	Test description

7.30.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use Session Timer and preconditions.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

7.30.3.2	Test procedure sequence

Table 7.30.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt an IMS voice call. | - | - |  |  |
| 2-7 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - |  |  |
| 8 | UE sends INVITE indicating support for Session Timer, with either the Session-Expires value set to 1800 or no Session-Expires header. | --> | INVITE | 1 | P |
| 9-17 | Steps 2-10 of Annex A.4.1 happen. | - | - |  |  |
| 18 | SS sends 200 OK for INVITE, with timer tag in Supported headers but without Session-Expires header. | <-- | 200 OK |  |  |
| 19 | UE sends ACK. | --> | ACK | 2 | P |
| 20 | SS sends BYE to release the call within 1860 seconds after step 19. (Step 1 of Annex A.8) | <-- | BYE | 3 | P |
| 21 | UE sends 200 OK for BYE. (Step 2 of Annex A.8) | --> | 200 OK | 3 | P |

7.30.3.3	Specific message contents

Table 7.30.3.3-1: INVITE (step 8, table 7.30.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.2.1, Conditions A1, A3, A4, A26 and A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Session-Expires |  | (if present) |  | RFC 4028 [37] |
| delta-seconds |  | 1800 |  |  |
| refresher |  | uac (if present) |  |  |

Table 7.30.3.3-2: 200 OK (step 18, table 7.30.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.3.1, Conditions A1 and A10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Supported |  | timer |  | RFC 4028 [37] |