3GPP TSG-RAN5 Meeting #90-e	R5-210123r1

# Electronic Meeting, 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0079 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS over 5GS test case 7.32 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NR TC 7.32 Session Timer / MT Call / Remote end sends Session-Expires but does not choose refresher / 5GS is scheduled on the work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding TC of Session Timer / MT Call / Remote end sends Session-Expires but does not choose refresher / 5GS as CL 7.32. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.32(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0278 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Came to common style for Conformance Requirements. - Changed the description style of Main Behaviour. - Editorial corrections. |  |  |  |  |  |  |  |  |

7.32	Session Timer / MT Call / Remote end sends Session-Expires but does not choose refresher / 5GS

7.32.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and being configured to use Session Timer and preconditions }

ensure that {

when { UE receives INVITE for voice call containing timer tag and Session-Expires value 1800 }

then { UE continues setup of voice call and finally sends 200 OK for INVITE with Session-Expires being 1800 and setting refresher to uac }

}

(2)

with { Call having been set up }

ensure that {

when { 900 seconds have passed and UE receives UPDATE to refresh the session }

then { UE sends 200 OK for UPDATE }

}

7.32.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.2A.1.1]

A UE supporting RFC 4028 [58], when it receives a 422 (Session Interval Too Small) to an INVITE request where the response contains a Min-SE header field, shall retry the request in accordance with RFC 4028 [58] subclause 7.4.

[TS 24.229 clause 5.2.7.2]

When the P-CSCF receives from the UE an INVITE request, the P-CSCF may require the periodic refreshment of the session to avoid hung states in the P-CSCF. If the P-CSCF requires the session to be refreshed, then the P-CSCF shall apply the procedures described in RFC 4028 [58] clause 8.

NOTE 1:	Requesting the session to be refreshed requires support by at least one of the UEs. This functionality cannot automatically be granted, i.e. at least one of the involved UEs needs to support it.

[TS 24.229 clause 5.2.7.3]

When the P-CSCF receives an INVITE request destined for the UE the P-CSCF may require the periodic refreshment of the session to avoid hung states in the P-CSCF. If the P-CSCF requires the session to be refreshed, then the P-CSCF shall apply the procedures described in RFC 4028 [58] clause 8.

NOTE 1:	Requesting the session to be refreshed requires support by at least one of the UEs. This functionality cannot automatically be granted, i.e. at least one of the involved UEs needs to support it in order to make it work.

[TS 24.229 clause 5.4.5.3]

If the S-CSCF requested the session to be refreshed periodically, and the S-CSCF got the indication that the session will be refreshed, when the session timer expires, the S-CSCF shall delete all the stored information related to the dialog.

7.32.3	Test description

7.32.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	UE is configured to use Session Timer and preconditions.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

7.32.3.2	Test procedure sequence

Table 7.32.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - |  |  |
| 9 | SS sends INVITE with timer tag set in Supported header and Session-Expires value set to 1800. | <-- | INVITE |  |  |
| 10-18 | Steps 2-10 of Annex A.5.1 happen. | - | - |  |  |
| 19 | UE sends 200 OK for INVITE with Session-Expires value set to 1800 and refresher value set to uac. | --> | 200 OK | 1 | P |
| 20 | SS sends ACK. (Step 12 of Annex A.5.1) | <-- | ACK |  |  |
| 21 | 900 seconds after step 19, SS sends an UPDATE request to refresh the session. | <-- | UPDATE |  |  |
| 22 | UE sends 200 OK for UPDATE. | --> | 200 OK | 2 | P |
| 23-24 | SS releases the call. (Steps 1-2 of Annex A.8) | - | - |  |  |

7.32.3.3	Specific message contents

Table 7.32.3.3-1: INVITE (step 9, table 7.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.2.9, Conditions A1, A3 and A4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Supported |  | timer |  | RFC 4028 [37] |
| Session-Expires |  |  |  | RFC 4028 [37] |
| delta-seconds |  | 1800 |  |  |

Table 7.32.3.3-2: 200 OK (step 19, table 7.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.3.1, Conditions A2 and A11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Require |  | timer |  | RFC 4028 [37] |
| Session-Expires |  |  |  | RFC 4028 [37] |
| delta-seconds |  | 1800 |  |  |
| refresher |  | uac |  |  |

Table 7.32.3.3-3: UPDATE (step 21, table 7.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.2.5, Condition A3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Supported |  | timer |  | RFC 4028 [37] |
| Session-Expires |  |  |  | RFC 4028 [37] |
| delta-seconds |  | 1800 |  |  |
| refresher |  | uac |  |  |
| Content-Type |  | not present |  | RFC 4028 [37] |

Table 7.32.3.3-4: 200 OK (step 22, table 7.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.3.1, Conditions A2 and A11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Supported |  | timer |  | RFC 4028 [37] |
| Session-Expires |  |  |  | RFC 4028 [37] |
| delta-seconds |  | 1800 |  |  |
| refresher |  | uac |  |  |