3GPP TSG RAN WG5 Meeting #90-e	R5-210148r1

# Electronic Meeting, 22nd February - 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5001 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial updates of TS 36.523-1 Sections 10 to 13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are redundant spaces in test case titles and TPs, incorrect usage of plurals and Capital Vs Small letters inconsistent usage in test case titles. The way to represent Void test cases is inconsistent within 35.523-1 but also between 36.523-1 and 36.523-2. In addition from the problem from standards editorial point of view they also represent inconsistency with the titles used in 36.523-2 where the issues do not exist. Other minor editorial issues have been found around the places with the issues described above (i.e. the entire spec was not deliberately searched for editorial issues): Missing empty row between end of table and following sub-clause title, missing spaces, syntactically incorrect TPs (PRD13) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Redundant spaces in test case titles and TPs have been removed, Capital Vs Small letters inconsistent usage has been corrected; plural replaced with singular. Ranges of void test cases have been replaced with single Void per test case. Other minor editorial issues have been resolved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficulties finding exact matches between tests definition and test applicability. Poor quality of 3GPP specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.1.1, 10.9.1, 11.2.8a, 12.2.1.3.2, 12.2.2, 12.2.3, 12.3.1, 12.3.2, 12.3.3, 13.1.17 – 13.1.18, 13.4.3.39, 13.4.3.41, 13.4.4.1 – 13.4.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Due to its editorial nature, and to avoid multiple very short CRs, the present CR is a CR to the entire 36523-1-g70_s10-s13.doc file which comprises content relevant to multiple RAN5#90-e AIs - the CR has been allocated to the AI relevant to the first change in the CR. The present CR is intermediate revision r1 to change the MS Word file format to DOCX. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 10.2.1	Dedicated EPS bearer context activation / Success

10.2.1.1	Test Purpose (TP)

(1)

with { UE is in EMM-REGISTERED state and a PDN address for an active default EPS bearer was received in an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message }

ensure that {

when { UE receives an ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message linked to the existing default EPS bearer }

then { UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message }

}

[Text skipped here]

## 10.9	UE routing of uplink packets

### 10.9.1	UE routing of uplink packets

10.9.1.1	Test Purpose (TP)

(1)

with { UE is in BEARER CONTEXT ACTIVE STATE state and in EMM-CONNECTED mode with a default EPS bearer and two dedicated EPS bearers active }

ensure that {

when { UE has IP packets for transmission where each IP packet matches at least one of the different packet filters configured in the UL TFTs for the dedicated EPS bearers }

then { UE evaluates the packet filters in the correct evaluation order and transmits IP packets in uplink on the dedicated EPS bearer associated with the matched packet filter }

}

[Text skipped here]

### 11.2.8a	Attach for emergency bearer services / Rejected / No suitable cells in tracking area / Emergency call using the CS domain / CDMA2000 1xRTT

11.2.8a.1	Test Purpose (TP)

Same Test Purpose as in clause 11.2.8.1 except: UE establishes the emergency call using the CDMA2000 1xRTT as CS domain.

11.2.8a.2	Conformance requirements

Same conformance requirements as in clause 11.2.8.2. For the present TCs the CS domain shall be understood as the CDMA2000 1xRTT CS domain.

[Text skipped here]

12.2.1.3.2	Test procedure sequence

Table 12.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | EXCEPTION: Step 1 is repeated for execution counter k=1 to 4 | - | - | - | - |
| 1 | Generic test procedure in 12.1.1 is executed for execution k with number of AM DRB (n) and number of UM DRB (m) as specified in table 12.2.1.3.1-1. | - | - | - | - |

12.2.1.3.3	Specific Message Contents

None

### 12.2.2	Data transfer of E-UTRA radio bearer combinations 2, 4, 7 and 10

12.2.2.1	Test Purpose (TP)

See 12.1.1.1

[Text skipped here]

### 12.2.3	Data transfer of E-UTRA radio bearer combinations 5, 8, 11 and 12

12.2.3.1	Test Purpose (TP)

See 12.1.1.1

[Text skipped here]

## 12.3	MIMO configured

### 12.3.1	Data transfer of E-UTRA radio bearer combinations 1, 3, 6 and 9 / MIMO

12.3.1.1	Test Purpose (TP)

See 12.1.2.1

[Text skipped here]

### 12.3.2	Data transfer of E-UTRA radio bearer combinations 2, 4, 7 and 10 / MIMO

12.3.2.1	Test Purpose (TP)

See 12.1.2.1

[Text skipped here]

### 12.3.3	Data transfer of E-UTRA radio bearer combinations 5, 8, 11 and 12 / MIMO

12.3.3.1	Test Purpose (TP)

See 12.1.2.1

[Text skipped here]

### 13.1.17	Void

### 13.1.18	Void

#### 13.4.2.8	Inter-RAT PS Handover / Synchronised / From GPRS Packet_transfer to E-UTRA cell (NC2 mode)

13.4.2.8.1	Test Purpose (TP)

(1)

with { UE in GPRS Registered state with active packet data transfer in NC2 mode }

ensure that {

when { UE receives a PS HANDOVER COMMAND message configured for already synchronised EUTRAN Cell }

then { UE performs Tracking Area update on EUTRAN cell and continues data transfer }

}

[Text skipped here]

#### 13.4.3.39	Inter-system mobility / UTRA CS voice + PS data to E-UTRA voice + PS data / rSRVCC

13.4.3.39.1	Test Purpose (TP)

(1)

with { UE in UTRA CC state U10 }

ensure that {

when { UE receives a HANDOVER FROM UTRAN COMMAND and an CS call+ PS data  is ongoing, PS bearer + Speech combination is configured for an E-UTRA cell }

then { UE transmits a RRCConnectionReconfigurationComplete message and performs Tracking Area update on EUTRAN }

}

[Text skipped here]

#### 13.4.3.41	Inter-system mobility / E-UTRA PS voice to GSM CS voice / HO cancelled / Notification procedure / SRVCC

13.4.3.41.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a NOTIFICATION message and an IMS voice call is ongoing and an GERAN Speech RAB combination is configured for an GERAN cell }

then { UE transmits a SIP re-INVITE message on the e-utra cell }

}

[Text skipped here]

### 13.4.4	Inter-system session management

#### 13.4.4.1	Void

#### 13.4.4.2	Void

#### 13.4.4.3	Void

#### 13.4.4.4	Void

#### 13.4.4.5	Void

[End of changes]