3GPP TSG RAN WG5 Meeting #90-e	R5-210150r2

# Electronic Meeting, 22nd February - 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5003 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial updates of TS 36.523-1 Sections 22 and 24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are redundant spaces in test case titles and TPs. Duplicated words and Capital Vs Small letters inconsistent usage in test case titles. In addition from the problem from standards editorial point of view they also represent inconsistency with the titles used in 36.523-2 where the issues do not exist. Other minor editorial issues have been found around the places with the issues described above (i.e. the entire spec was not deliberately searched for editorial issues): Test Purposes text does not follow the agreed RAN5 way of specifying a TP (PRD13). Test Purpose (TP) sub-section have incorrect styles. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Redundant spaces in test case titles and TPs have been removed, Capital Vs Small letters inconsistent usage has been corrected. Test Purposes text has been modified to follow the agreed RAN5 way of specifying a TP. Test Purpose (TP) sub-section styles have been modified to H6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficulties finding exact matches between tests definition and test applicability. Poor quality of 3GPP specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.1, 22.5.10, 22.5.22, 22.6.5, 24.1.17, 24.1.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The present CR is intermediate revision r1 to change the MS Word file format to DOCX. r1 - few more spelling errors corrected |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 22.2	Idle mode operations

### 22.2.1	NB-IoT / PLMN selection of RPLMN, HPLMN / EHPLMN, UPLMN and OPLMN / Automatic mode

#### 22.2.1.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and RPLMN, HPLMN, UPLMN and OPLMN cells available and UE is fitted with a USIM indicating RPLMN should be selected }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the RPLMN and UE attempts Tracking area update on the selected cell and when successfully registered indicates the selected PLMN to the user }

}

(2)

with { UE camped on a VPLMN cell and cells of a higher priority PLMN available }

ensure that {

when { UE higher priority PLMN search timer T expires }

then { UE selects and camps on a cell of the highest priority PLMN and UE attempts Tracking area update on the selected cell and when successfully registered indicates the selected PLMN to the user }

}

(3)

with { UE in Automatic network selection mode and HPLMN, UPLMN and OPLMN cells available and UE is fitted with a USIM with Access Technology data files for each PLMN and there are no equivalent HPLMNs defined }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the highest priority PLMN and UE attempts Tracking area update on the selected cell and when successfully registered indicates the selected PLMN to the user }

}

(4)

with { UE camped on a VPLMN cell and cells of a HPLMN available }

ensure that {

when { UE higher priority PLMN search timer T expires }

then { UE selects and camps on a cell of HPLMN and UE attempts Tracking area update on the selected cell and when successfully registered indicates the selected PLMN to the user }

}

[Text skipped here]

### 22.5.10	NB-IoT / EPS NAS integrity and encryption / SNOW 3G

#### 22.5.10.1	Test Purpose (TP)

(1)

with { UE having successfuly completed EPS authentication and key agreement (AKA) procedure }

ensure that {

when { UE in NB-S1 modereceives an integrity protected and ciphering SECURITY MODE COMMAND message instructing to start integrity protection and ciphering algorithm with SNOW 3G }

then { UE transmits an integrity protected with SNOW 3G and ciphering SECURITY MODE COMPLETE and starts applying the NAS Integrity protection and NAS ciphering in both UL and DL }

}

[Text skipped here]

### 22.5.22	NB-IoT / Tracking area update/Inter-RAT change between NB-IoT and E-UTRA

#### 22.5.22.1	Test Purpose (TP)

(1)

with { UE receives an ATTACH ACCEPT message without TAI of E-UTRA cell and camps on a NB-IoT cell successfully }

ensure that {

when { UE enters a new E-UTRA cell with different TAI }

then { UE requests tracking area update on the new E-UTRA cell with the last visited TAI and GUTI allocated }

}

(2)

with { UE receives an TRACKING AREA UPDATE ACCEPT message without TAI of NB-IoT cell and camps on a E-UTRA cell successfully }

ensure that {

when { UE enters a new NB-IoT cell with different TAI }

then { UE requests tracking area update on the new NB-IoT cell with the last visited TAI and GUTI allocated }

}

[Text skipped here]

### 22.6.4

### 22.6.5	NB-IoT / UE requested PDN connectivity procedure not accepted / UE requested PDN connectivity accepted Dual priority T3396 override UE requested PDN connectivity accepted / Dual priority / T3346 override

#### 22.6.5.1	Test Purpose (TP)

(1)

with { UE has sent a PDN CONNECTIVITY REQUEST message to an additional PDN }

ensure that {

when { UE receives an PDN CONNECTIVITY REJECT message with PTI matching the PDN CONNECTIVITY REQUEST message and including a ESM cause value }

then { UE enters the state PROCEDURE TRANSACTION INACTIVE }

}

[Text skipped here]

### 24.1.17	V2X Sidelink Communication / Pre-configured authorisation / UE in RRC_IDLE on an E-UTRAN cell operating on the anchor carrier frequency provisioned for V2X configuration / UE uses Tx resource pool which is associated with the synchronization reference source selected

#### 24.1.1.17	Test Purpose (TP)

(1)

with { UE supporting V2X sidelink communication }

ensure that {

when { UE is triggered by an upper layer application to transmit V2X sidelink communication }

then { UE shall only use the pool which is associated with the synchronization reference source selected }

}

[Text skipped here]

### 24.1.20	V2X Sidelink Communication / Pre-configured authorisation / UE in limited service state on the anchor carrier frequency provisioned for V2X configuration /Transmission

#### 24.1.20.1	Test Purpose (TP)

(1)

with { UE supporting V2X sidelink communication is authorized to use V2X communication over PC5 reference point }

ensure that {

when { UE in limited service state because of receiving a "PLMN not allowed" response to a registration request, and is triggered by an upper layer application to transmit V2X sidelink communication }

then { UE shall only use the radio resources and procedure available in IDLE mode for V2X communication }

}

(2)

with { UE supporting V2X sidelink communication is authorized to use V2X communication over PC5 reference point }

ensure that {

when { UE in limited service state because of receiving a "EPS services not allowed" response to a registration request, and is triggered by an upper layer application to transmit V2X sidelink communication }

then { UE shall only use the radio resources and procedure available in IDLE mode for V2X communication }

}

[End of changes]