3GPP TSG RAN WG5 Meeting #90-e	R5-210157r2

# Electronic Meeting, 22nd February - 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1942 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial changes to 38.523-1 Section 6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Editorial issues have been identified in 38.523-1 Section 6 - Styles applied to some sub-sections are incorrect making the section titles not appearing in the ToC (e.g. test case titles), or, appearing in the ToC when not needed (e.g. sub-sections of test case definition - Redundant paragraph marks - Redundant/missing spaces e.g. in test case titles - grama errors (plural Vs singular use) - long forgotten Editor's note and "FFS" in 6.1 are redundant - The section 6 title does not reflect the fact that it contains not only Idle mode tests but also tests for RRC Inactive state operations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial issues have been resolved - Styles applied to some sub-sections - Redundant paragraph marks - Redundant/missing spaces e.g. in test case titles - Corrected few more grama errors (plural Vs singular use) - long forgotten Editor's note and "FFS" in 6.1 removed - The section 6 title updated - "RRC Inactive state" added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Poor quality of 3GPP specifications. Difficulties matching between ToC and tests can lead to confusion and in some cases to misinterpretations that a test has not yet been defined. Redundant titles in ToC make ToC very long and difficult to read. Not clear what the section contains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.5, 6.1.2.19, 6.1.2.21, 6.1.2.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1 - Corrects few more grama errors (plural Vs singular use) r2 - Removed redundant text and update section 6 title |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 6	Idle mode and RRC Inactive state operations

## 6.1	NR idle mode operations

[Text skipped here]

#### 6.1.2.5	Cell reselection for interband operation using Pcompensation / Between FDD and TDD

6.1.2.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state on a FDD band }

ensure that {

when { UE detects the cell selection using Pcompensation & reselection criteria is met for the cell on a TDD band }

then { UE reselects the new cell }

}

[Text skipped here]

#### 6.1.2.19	Speed-dependent cell reselection

##### 6.1.2.19.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and in high mobility state }

ensure that {

when { UE detects the cell ranked as the best cell taking into account scaling factor }

then { UE reselects the new cell }

}

(2)

with { UE in NR RRC_IDLE state and in medium mobility state }

ensure that {

when { UE detects the cell ranked as the best cell taking into account scaling factor }

then { UE reselects the new cell }

}

##### 6.1.2.19.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.304, clauses 5.2.4.3.0 and 5.2.4.3.1. TS 38.133, clause 4.2.2.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.304, clause 5.2.4.3.0]

The UE mobility state is determined if the parameters (TCRmax, NCR_H, NCR_M and TCRmaxHyst) are broadcasted in system information for the serving cell.

State detection criteria:

…

Medium-mobility state criteria:

-	If number of cell reselections during time period TCRmax is greater than or equal to NCR_M but less than or equal to NCR_H.

High-mobility state criteria:

-	If number of cell reselections during time period TCRmax is greater than NCR_H.

The UE shall not consider consecutive reselections where a cell is reselected again right after one reselection for mobility state detection criteria.

State transitions:

The UE shall:

-	if the criteria for High-mobility state is detected:

-	enter High-mobility state.

-	else if the criteria for Medium-mobility state is detected:

-	enter Medium-mobility state.

…

If the UE is in High- or Medium-mobility state, the UE shall apply the speed dependent scaling rules as defined in subclause 5.2.4.3.1.

[TS 38.304, clause 5.2.4.3.1]

UE shall apply the following scaling rules:

-	If neither Medium- nor High-mobility state is detected:

-	no scaling is applied.

-	If High-mobility state is detected:

-	Add the sf-High of "Speed dependent ScalingFactor for Qhyst" to Qhyst if broadcasted in system information;

-	For NR cells, multiply TreselectionNR by the sf-High of "Speed dependent ScalingFactor for TreselectionNR" if broadcasted in system information;

…

-	If Medium-mobility state is detected:

-	Add the sf-Medium of "Speed dependent ScalingFactor for Qhyst" to Qhyst if broadcasted in system information;

-	For NR cells, multiply TreselectionNR by the sf-Medium of "Speed dependent ScalingFactor for TreselectionNR" if broadcasted in system information;

…

In case scaling is applied to any TreselectionRAT parameter, the UE shall round up the result after all scalings to the nearest second.

[TS 38.133, clause 4.2.2.3]

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 within Tdetect,NR_Intra when that Treselection= 0 . An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1) for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1 provided that:

-	when rangeToBestCell is not configured, the cell has at least [3]dB in FR1 or [TBD]dB in FR2 better ranked or

-	when rangeToBestCell is configured, the cell which has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among the cells whose cell-ranking criterion R value as specified in TS 38.304 [1, Section 5.2.4.6] is within rangeToBestCell of the R value of the best cell where the best cell has at least [TBD] in FR1 or [TBD]dB in FR2 better ranked, and if there are multiple such cells the UE shall perform cell reselection to the highest ranked cell among them.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non-zero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [1], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | [8] | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | [5] | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | [4] | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | [TBD] | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = [8] for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

##### 6.1.2.19.3	Test Description

6.1.2.19.3.1	Pre-test conditions

System Simulator:

-	 3 NR Cells: NR Cell 1, 2 and 4 as specified in TS 38.508-1 [4] table 4.4.2-3.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	None.

Preamble:

-	NR Cell 1 is set to "Serving Cell" and NR Cell 2 and NR Cell 4 are set to non-suitable "Off" as defined in TS 38.508-1 [4] table 6.2.2.1-3 for FR1 and table 6.2.2.2-2 for FR2.

-	The UE is in NR RRC_IDLE state (state 1N-A with PDU session inactive) as defined in TS 38.508-1 [4], table 4.4A.2-1 on NR Cell 1.

6.1.2.19.3.2	Test procedure sequence

Table 6.1.2.19.3.2-1 and Table 6.1.2.19.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.1.2.19.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -88 | -94 | NR Cell 2 becomes the highest ranked cell. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -94 | -88 | NR Cell 4 becomes the highest ranked cell. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 | -94 | NR Cell 1 becomes the highest ranked cell. |

Table 6.1.2.19.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | NR Cell 2 becomes the highest ranked cell. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | NR Cell 4 becomes the highest ranked cell. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | NR Cell 1 becomes the highest ranked cell. |

Table 6.1.2.19.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level setting according to the row "T1". | - | - | - | - |
| 2 | Wait for 40s for FR1 or 136s for FR2 for the UE to perform cell reselection procedure. (Note 1, Note 5) | - | - | - | - |
| 3 | The SS changes the power level setting according to the row "T2". | - | - | - | - |
| 4 | Wait for 14s for FR1 or 33s for FR2 for the UE to perform cell reselection procedure. (Note 2, Note 6) | - | - | - | - |
| 5 | The SS changes the power level setting according to the row "T3". | - | - | - | - |
| 6 | Wait for 14s for FR1 or 33s for FR2 for the UE to perform cell reselection procedure. (Note 3, Note 6) | - | - | - | - |
| 7-8 | Void | - | - | - | - |
| 9 | The SS changes the TAC of NR Cell 2 to 2. | - | - | - | - |
| 10 | The SS changes the power level setting according to the row "T1". | - | - | - | - |
| 11 | Check: Does the UE send an RRCSetupRequest on NR Cell 2 within 3s? (Note 7) | --> | NR RRC: RRCSetupRequest | 2 | F |
| 12 | Check: Does the UE send an RRCSetupRequest on NR Cell 2 within 6s for FR1 and 27s for FR2 after step 11? (Note 8, Note 9) | --> | NR RRC: RRCSetupRequest | 2 | P |
| 13-17 | Steps 2 to 6 of the mobility registration updating procedure described in TS 38.508-1[4] Table 4.9.5.2.2-1 are performed on NR Cell 2. | - | - | - | - |
| 18 | Void | - | - | - | - |
| 19 | The SS changes the power level setting according to the row "T2". | - | - | - | - |
| 20-25 | Void | - | - | - | - |
| 26 | Check: Does the UE send an RRCSetupRequest on NR Cell 4 within 2s? (Note 12) | --> | NR RRC: RRCSetupRequest | 1 | F |
| 27 | Check: Does the UE send an RRCSetupRequest on NR Cell 4 within 7s for FR1 and 26s for FR2 after step 26? (Note 13) | --> | NR RRC: RRCSetupRequest | 1 | P |
| 28-32 | Steps 2 to 6 of the mobility registration updating procedure described in TS 38.508-1[4] Table 4.9.5.2.2-1 are performed on NR Cell 4. | - | - | - | - |
| Note 1: NCR (number of cell reselection) is 1. Note 2: NCR is 2. Note 3: NCR is 3 and the UE enters medium mobility state because number of cell reselections during time period TCRmax equals NCR_M. Note 4: Void. Note 5: The wait time for reselection to a newly detected intra frequency cell is selected to cover Tdetect,NR_Intra(25*1280ms=32s for FR1 and 25*4*1280ms=128s for FR2) + Treselection (7s) + 320 ms system information block type scheduling = 39.32s rounded up to 40s for FR1 and 135.32s rounded up to 136s for FR2. Note 6: The wait time for reselection to an already detected intra frequency cell is selected to cover Tevaluate,NR_intra(6.4s for FR1 and 25.6 for FR2) + Treselection (7s) + 320 ms system information block type scheduling = 13.72s rounded up to 14s for FR1 and 32.92s rounded up to 33s for FR2. Note 7: The wait time for evaluating an intra-frequency cell is Treselection after scaling (3.5s). This wait time is rounded down to nearest second (i.e. 3s in this case). Note 8: The wait time for cell reselection procedure is selected to cover Tevaluate,NR_intra (6.4s for FR1 and 25.6 for FR2) + Treselection after scaling (7*0.5) + 320 ms system information block type scheduling = 10.22s rounded up to 11s for FR1 and 29.42s rounded up to 30s for FR2. The actual wait time is 6s which covers 9s - 3s (wait time in step 11) for FR1 and 27s which covers 30s – 3s (wait time in step 11) for FR2. Note 9: NCR is 4 and the UE enters high mobility state because number of cell reselections during time period TCRmax greater than NCR_H Note 10: Void. Note 11: Void. Note 12: The wait time for evaluating an intra-frequency cell is Treselection after scaling (2s). Note 13: The wait time for cell reselection procedure is selected to cover Tevaluate,NR_intra (6.4s for FR1 and 25.6 for FR2) + Treselection after scaling (7*0.25) + 320 ms system information block type scheduling = 8.47s rounded up to 9s for FR1 and 27.92s rounded up to 28s for FR2. The actual wait time is 7s which covers 9s - 2s (wait time in step 26) for FR1 and 26s which covers 28s – 2s (wait time in step 26) for FR2. |  |  |  |  |  |

[Text skipped here]

#### 6.1.2.21	Cell reselection, SIntraSearchQ and SnonIntraSearchQ

6.1.2.21.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { UE receives non-zero SIntraSearchQ in system information }

then { UE performs measurement and reselects the highest ranked cell upon Squal < SIntraSearchQ }

}

(2)

with { UE in NR RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { UE receives non-zero SnonIntraSearchQ in system information }

then { UE performs measurement and reselects the cell which belongs to the equal priority frequency cell upon Squal < SnonIntraSearchQ }

}

(3)

with { UE in NR RRC_IDLE state, and the UE is not in high mobility state }

ensure that {

when { UE receives non-zero SnonIntraSearchQ in system information }

then { UE performs measurement and reselects the cell which belongs to the high priority frequency cell upon Squal > SnonIntraSearchQ }

}

[Text skipped here]

#### 6.1.2.22	Inter-frequency cell reselection based on common priority information with parameters ThreshX, HighQ, ThreshX, LowQ and ThreshServing, LowQ

6.1.2.22.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority frequency }

then { UE reselects the cell which belongs to the higher priority frequency }

}

(2)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority frequency }

then { UE reselects the cell which belongs to the lower priority frequency }

}