3GPP TSG RAN WG5 Meeting #90-e	R5-210167r1

Electronic Meeting, 22nd Feb 2021 - 5th Mar 2021

Agenda Item:	6.3.2.20

Source:	Ericsson

Title:	MR-DC

Document for:	Endorsement

# 1	Introduction

The definition of MR-DC has been defined and EN-DC has been enhanced in TS 38.331 [1] since the first release 15 versions. Some of the definition details are not yet considered in the RAN5 documentation.

# 2	Discussion

Definition MR-DC

According release 16 TS 38.331 [1], 5.1.3 Requirements for UE in MR-DC

“In this specification, the UE considers itself to be in:

-	EN-DC, if and only if it is configured with nr-SecondaryCellGroupConfig according to TS 36.331[10], and it is connected to EPC,

-	NGEN-DC, if and only if it is configured with nr-SecondaryCellGroupConfig according to TS 36.331[10], and it is connected to 5GC,

-	NE-DC, if and only if it is configured with mrdc-SecondaryCellGroup set to eutra-SCG,

-	NR-DC, if and only if it is configured with mrdc-SecondaryCellGroup set to nr-SCG,

-	MR-DC, if and only if it is in (NG)EN-DC, NE-DC or NR-DC.”

Abbreviations

The following connectivity options are used in TS 38.508-1 [2]

EN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to EPC

E-UTRA/EPC	E-UTRA connected to EPC

NR-DC	NR-NR Dual Connectivity

Observation 2-1:

EN-DC is defined (“the UE considers itself to be in”) if and only if it is configured with nr-SecondaryCellGroupConfig. I.e. MCG and SCG are always configured.

Observation 2-2:

NR-DC is defined (“the UE considers itself to be in”) if and only if it is configured with mrdc-SecondaryCellGroup set to nr-SCG.  I.e. MCG and SCG are always configured.

Observation 2-3:

E-UTRA/EPC is defined as E-UTRA connected to EPC.  I.e. MCG configured but not SCG.

Issue 2-1:

The definition of generic procedure parameter “Bearers” in TS 38.508-1 [2] is not aligned with TS 38.331 [1].

Table 4.5.1-1: Generic procedure parameters

| Parameter | Values | Description | Parameter condition |
| --- | --- | --- | --- |
| Connectivity | E-UTRA/5GC | E-UTRA connected to 5GC | Mandatory |
|  | NR | NR connected to 5GC |  |
|  | EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |  |
|  | NGEN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to 5GC |  |
|  | NE-DC | NR E-UTRA Dual Connectivity |  |
|  | NR-DC | NR-NR Dual Connectivity |  |
|  | WLAN | Un trusted non 3GPP access over WLAN |  |
|  | E-UTRA/EPC | E-UTRA connected to EPC |  |
| Bearers | MCG(s) and SCG | MCG and SCG |  |
|  | MCG(s) and split | MCG and split |  |
|  | MCG(s) only | MCG only |  |

Proposal 2-1:

Align the definition of generic procedure parameter “Bearers” in TS 38.508-1 [2] with TS 38.331 [1], e.g. update parameter Bearers as

Table 4.5.1-1: Generic procedure parameters

| Parameter | Values | Description | Parameter condition |
| --- | --- | --- | --- |
| Connectivity | E-UTRA/5GC | E-UTRA connected to 5GC | Mandatory |
|  | NR | NR connected to 5GC |  |
|  | EN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to EPC |  |
|  | NGEN-DC | E-UTRA-NR Dual Connectivity with E-UTRA connected to 5GC |  |
|  | NE-DC | NR E-UTRA Dual Connectivity |  |
|  | NR-DC | NR-NR Dual Connectivity |  |
|  | WLAN | Un trusted non 3GPP access over WLAN |  |
|  | E-UTRA/EPC | E-UTRA connected to EPC |  |
| Bearers | MCG(s) and SCG | MCG and SCG |  |
|  | MCG(s) and split | MCG and split |  |

Proposal 2-2:

Update the generic procedures accordingly.

Proposal 2-3:

Update the TCs accordingly. For TCs previously used the parameter “Bearers” and MCG(s) only corrected to use parameter “Connectivity” and E-UTRA/EPC.

Examples TCs updates

E-UTRA/EPC TC example

8.2.2.4.1.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1 is the PCell and NR Cell 1 is the PSCell.

UE:

-	None

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (E-UTRA/EPC) according to TS 38.508-1 [4], clause 4.5.4.

-	Else, the UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (E-UTRA/EPC) and Test Mode (On) associated with UE test loop mode B configured on E-UTRA Cell 1 according to TS 38.508-1 [4], clause 4.5.4.

# 3	Proposal

It’s proposed to implement the proposals 2-1, 2-2and 2-3. If these proposals are accepted, then Ericsson are willing to create the necessary CRs for the next meeting.

# 4	References

[1] 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[2] 3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".