3GPP TSG-RAN WG5 Meeting #90-e	R5-210204r1

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0100 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of a generic procedure for MCPTT radio bearer establishment for use of pre-established session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no generic procedure yet to establish an RRC connection with one DRB associated with the default EPS bearer and another DRB associated with the dedicated EPS bearer which has been created at establishment of the pre-established session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 5.4.13 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No verification of pre-established session test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.13 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: taking into account other PDNs being registered during initial registration |  |  |  |  |  |  |  |  |

<Start of new clause>

5.4.12.4	Specific message contents

None.

### 5.4.13	Generic Test Procedure for MCPTT radio bearer establishment for use of pre-established session

5.4.13.1	Initial conditions

System Simulator:

-	SS (MCPTT server)

-	SS E-UTRA related parameters are set to the default parameters for the basic single cell environment, as defined in TS 36.508 [6] clause 4.4, unless otherwise specified in the test case. Requirements in regard to the PLMN which the simulated Cell(s) belongs to are specified in the test case using the present procedure.

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in clause 5.5.10 is inserted.

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in clause 5.4.2 and is in E-UTRA Registered, Idle Mode state with the MCPTT Client being active. During the attach a default EPS bearer context #3 (QCI 69) according to table 6.6.1-1, TS 36.508 [6] is established for MCPTT and SIP signalling.

NOTE 1:	The assumptions for the PDN support of a MCPTT capable UE, including the default EPS bearer context QCI requirements in regard to the different PDN are described in 5.4.1A.

-	The UE has performed the Generic Test Procedure for MCPTT pre-established session establishment CO as specified in clause 5.3.3.

-	Detailed initial conditions for the UE (MCPTT client) shall be specified in the TC referring to the present procedure.

5.4.13.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.4.13.3	Procedure

Table 5.4.13.3-1: MCPTT radio bearer establishment for use of pre-established session

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS sends a Paging message to the UE | <-- | RRC: Paging (PCCH) |
| 2 | The UE transmits an RRCConnectionRequest message | --> | RRC: RRCConnectionRequest |
| 3 | The SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures all DRBs, associated with the default EPS bearers established during the attach procedure and another DRB, associated with the dedicated EPS bearer established during pre-established session establishment. The RRCConnectionReconfiguration message is using condition SRB2-DRB(n, 1) as specified in TS 36.508 [6] clause 4.8.2.2.1, with n=1..3 depending on the number of PDNs (see clause 5.4.1A). => The DRB associated with MCPTT default EPS bearer obtained during the attach procedure and the DRB associated with dedicated EPS bearer obtained during the establishment of the pre-established session get established | <-- | RRC: RRCConnectionReconfiguration |
| - | EXCEPTION: In parallel to step 8, depending on the test case, the UE may start signalling on the DRBs | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of radio bearers. | --> | RRC: RRCConnectionReconfigurationComplete |

5.4.13.4	Specific message contents

All specific EUTRA/EPS signalling message contents shall be referred to TS 36.508 [6] clauses 4.6 and 4.7.

<End of new clause>