3GPP TSG-RAN WG5 Meeting #90-e	R5-210212r1

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0108 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Default Message Content - REFER and Resource-List |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Resource list and other IEs in REFER message missing - Content of resource list incomplete - Message body incorrect – hname body missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Resource list and other IEs in REFER message added - Content of resource list corrected - Message body corrected – and hname body added Rev1: - port number in From header restored to be "not present" (same as for INVITE) - P-Preferred-Identity does not need condition PRIVAT-CALL as it is "If present" already - Decision to handle ambient listening in specific msaage content for test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong message content results in incorrect MCPTT test case implementation and test case verification not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.12, 5.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: message content and condition corrections |  |  |  |  |  |  |  |  |

<Start of modification>

#### 5.5.2.12	SIP REFER

This message is sent by the UE outside of a dialog.

Table 5.5.2.12-1: SIP REFER

| Derivation Path: TS 24.229 [16] clause A.2.1.4.11, A.2.2.4.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| Method | "REFER" |  |  |  |
| Request-URI | tsc_MCX_SessionID_B | session identity of the pre-established session |  |  |
| SIP-Version | "SIP/2.0" |  |  |  |
| Via |  |  | RFC 3261 [22] RFC 3581 [55] |  |
| sent-protocol | "SIP/2.0/UDP" |  |  | UDP |
|  | "SIP/2.0/TCP" |  |  | TCP |
| sent-by |  |  |  |  |
| host | IP address or FQDN | Either the UE’s IP address or its home domain name |  |  |
| port | protected server port of the UE |  |  |  |
| via-branch | Value starting with 'z9hG4bK' |  |  |  |
| Route |  |  | RFC 3261 [22] |  |
| addr-spec[1] | SIP URI |  |  |  |
| user-info and host | P-CSCF address of the SS | P-CSCF address as assigned to the UE via NAS signalling or P-CSCF discovery |  |  |
| port | protected server port of the SS | as assigned during registration |  |  |
| uri-parameters | "lr" |  |  |  |
| addr-spec[2] | SIP URI |  |  |  |
| user-info and host | "scscf.3gpp.org" |  |  |  |
| port | not present |  |  |  |
| uri-parameters | "lr" |  |  |  |
| From |  |  | RFC 3261 [22] |  |
| addr-spec |  |  |  |  |
| user-info and host | Default public user id (px_MCX_SIP_PublicUserId_A_1) |  |  |  |
| port | not present |  |  |  |
| tag | any allowed value |  |  |  |
| To |  |  | RFC 3261 [22] RFC 5031 [54] |  |
| addr-spec |  |  |  |  |
| user-info and host | Same URI as request URI |  |  |  |
| port | not present |  |  |  |
| tag | not present |  |  |  |
| Call-ID |  |  | RFC 3261 [22] |  |
| callid | any allowed value |  |  |  |
| CSeq |  |  | RFC 3261 [22] |  |
| value | any allowed value |  |  |  |
| method | "REFER" |  |  |  |
| Supported |  |  | RFC 3261 [22] RFC 6442 [62] RFC 4488 [36] |  |
| option-tag | "norefersub" |  |  |  |
| Refer-Sub |  |  | RFC 4488 [36] |  |
| refer-sub-value | "false" |  |  |  |
| Target-Dialog |  |  | RFC 4538 [37] |  |
| callid | Callid of the pre-established session | Callid as used by the UE in the INVITE for establishment of the pre-established session |  |  |
| Require |  |  | RFC 3261 [22] RFC 3312 [56] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| option-tag | "multiple-refer" |  |  |  |
| Proxy-Require |  |  | RFC 3261 [22] RFC 3329 [53] |  |
| option-tag | "sec-agree" |  |  |  |
| Security-Verify |  |  | RFC 3329 [53] |  |
| sec-mechanism | same value as Security -Server header sent by SS during registration |  |  |  |
| Contact |  |  | RFC 3261 [22 RFC 3840 [33] |  |
| addr-spec | SIP URI |  |  |  |
| user-info and host | IP address or FQDN |  |  |  |
| feature-param | "+g.3gpp.mcptt" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. |  |  |
|  | "+g.3gpp.mcvideo" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Video (MCVideo) communication. |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Data (MCData) communication. |  | MCDATA |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" | This URN indicates that the device has the capabilities to support the mission critical push to talk (MCPTT) service. |  |  |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" | This URN indicates that the device has the capabilities to support the mission critical video (MCVideo) service. |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" | This URN indicates that the device has the capabilities to support the mission critical data (MCData) service. |  | MCDATA |
| feature-param | "audio" | This feature tag indicates that the device supports audio as a streaming media type. |  | MCPTT OR MCVIDEO |
| feature-param | "video" | This feature tag indicates that the device supports video as a streaming media type. |  | MCVIDEO |
| feature-param | "text" | This feature tag indicates that the device supports text as a streaming media type. |  | MCDATA |
| Refer-To |  |  | RFC 3515 [38] |  |
| addr-spec | a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 that points to an application/resource-lists+xml MIME body as specified in IETF RFC 5366 |  |  |  |
| Refer-To |  |  | RFC 3515 [38] | METHOD-BYE |
| addr-spec | tsc_MCX_SessionID_B | The session identity of the pre-established session to leave. |  |  |
| method | "BYE" | NOTE: "method" is URI parameter but not header parameter |  |  |
| Max-Forwards |  |  | RFC 3261 [22] |  |
| value | any allowed value | Non-zero value |  |  |
| P-Access-Network-Info |  |  | RFC 7315 [52] |  |
| access-net-specs | Access network technology and, if applicable, the cell ID |  |  |  |
| P-Preferred-Service |  |  | RFC 6050 [31] |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "urn:urn-7:3gpp-service.ims.icsi.mcdata" |  |  | MCDATA |
| P-Preferred-Identity | If present |  | RFC 3325 [32] |  |
| PPreferredID-value | same URI as in From-header |  |  |  |
| Resource-Priority |  |  | RFC 4412 [40] RFC 7134 [57] RFC 8101 [45] TS 24.379 [9] clause 6.2.8.1.15 | EMERGENCY-CALL AND (GROUP-CALL OR PRIVATE-CALL) |
| r-value |  |  |  |  |
| namespace | value of the <resource-priority-namespace> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents | As configured in Table 5.5.8.4-1 | TS 24.484 [14] |  |
| r-priority | value of the <resource-priority-priority> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document | As configured in Table 5.5.8.4-1 | TS 24.484 [14] |  |
| Resource-Priority |  |  | RFC 4412 [40] RFC 7134 [57] RFC 8101 [45] TS 24.379 [9] clause 6.2.8.1.15 | IMMPERIL-CALL AND (GROUP-CALL OR PRIVATE-CALL) |
| r-value |  |  |  |  |
| namespace | value of the <resource-priority-namespace> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration documents | As configured in Table 5.5.8.4-1 | TS 24.484 [14] |  |
| r-priority | value of the <resource-priority-priority> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCX service configuration document | As configured in Table 5.5.8.4-1 | TS 24.484 [14] |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| Content-Length | present in case of TCP and when there is a message body (otherwise optional) |  | RFC 3261 [22] |  |
| Value | any value | length of message-body |  |  |
| Message-body | not present |  |  | METHOD-BYE |
| Message-body |  |  | RFC 3261 [22] |  |
| MIME body part |  | Resource list | RFC 5366 [35] |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/resource-lists+xml" |  |  |  |
| Content-ID | same value as the cid URL in the Refer-To header field | Unique URL identifying the Resource-lists XML MIME body; used as reference in the signature MIME body too | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Resource-lists as described in Table 5.5.3.3.1-1 with condition PRE-ESTABLISH and the uri attribute of the single <entry> element extended with the headers of Table 5.5.2.12-2 |  |  |  |
|  | Resource-lists as described in Table 5.5.3.3.1-2 |  |  | MCVIDEO |
|  | Resource-lists as described in Table 5.5.3.3.1-3 |  |  | MCDATA |
| MIME body part |  | Location info |  | (MCPTT OR MCVIDEO) AND PRIVATE-CALL |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-location-info+xml" |  |  |  |
|  | "application/vnd.3gpp.mcvideo-location-info+xml" |  |  | MCVIDEO |
| Content-ID | any value | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.1-1 |  | TS 24.379 [9] clause F.3 |  |
|  | Location-info as described in Table 5.5.3.4.1-2 |  | TS 24.281 [86] clause F.3 | MCVIDEO |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-signed+xml" |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

Table 5.5.2.12-2: SIP header fields extending the uri attribute of the resource-lists' single entry

| Derivation Path: TS 24.379 [9] clause 10.1.1.2.2.1, 10.1.2.2.2.1, 11.1.1.2.2.1, 11.1.6.2.2.1 Editor's note: references for MCVIDEO and MCDATA to be added |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  | RFC 3841 [29] | GROUP-CALL OR CHAT-GROUP-CALL |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.icsi-ref=urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" |  |  | MCDATA |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  |  |
|  | "+g.3gpp.mcvideo" |  |  | MCVIDEO |
|  | "+g.3gpp.mcdata.sds" |  |  | MCDATA |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Answer-Mode | not present |  |  |  |
| Answer-Mode |  |  | RFC 5373 [34] TS 24.379 [9] cl. 11.1.1.2.2.1, 8) | PRIVATE-CALL AND (NOT FORCE) |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-value | "Manual" |  |  | MANUAL |
| Priv-Answer-Mode | not present |  |  |  |
| Priv-Answer-Mode |  |  | RFC 5373 [34] TS 24.379 [9] clause 11.1.1.2.2.1, 8) and clause 11.1.6.2.2.1, 8) | PRIVATE-CALL AND FORCE |
| answer-mode-value | "Auto" | if force of automatic commencement mode at the invited MCPTT client is requested by the MCPTT user, |  |  |
| Content-Type |  |  | RFC 5621 [58] |  |
| media-type | "multipart/mixed" |  |  |  |
| body |  | NOTE: Characters that are not formatted as ASCII characters are escaped in the following parameters in the headers portion of the SIP URI. | RFC 3261 [22] |  |
| MIME body part |  | MCPTT/MCVideo/MCData Info |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  | MCPTT |
|  | "application/vnd.3gpp.mcvideo-info+xml" |  |  | MCVIDEO |
|  | "application/vnd.3gpp.mcdata-info+xml" |  |  | MCDATA |
| Content-ID | any value | Unique URL identifying the MCPTT/MCVideo/MCData Info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.5.3.2.1-1 |  | TS 24.379 [9] clause F.1 |  |
|  | MCVideo-Info as described in Table 5.5.3.2.1-2 |  | TS 24.281 [86] clause F.1 | MCVIDEO |
|  | MCData-Info as described in Table 5.5.3.2.1-3 |  | TS 24.282 [87] clause D.1 | MCDATA |
| MIME body part |  | Location info |  | (MCPTT OR MCVIDEO) AND ALLOW-LOCATION-INFO |
| MIME-part-headers |  |  |  |  |
| Content-Type | “application/vnd.3gpp.mcptt-location-info+xml” |  |  |  |
|  | “application/vnd.3gpp.mcvideo-location-info+xml” |  |  | MCVIDEO |
| Content-ID | any value | Unique URL identifying the Location-info XML MIME body; used as reference in the signature MIME body | TS 24.379 [9] clause 6.6.3.1 |  |
| MIME-part-body | Location-info as described in Table 5.5.3.4.1-1 |  | TS 24.379 [9] clause F.3 |  |
|  | Location-info as described in Table 5.5.3.4.1-2 |  | TS 24.281 [86] clause F.3 | MCVIDEO |
| MIME body part |  | Signature |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | “application/vnd.3gpp.mcptt-signed+xml” |  | TS 24.379 [9] |  |
| MIME-part-body | Signatures for XML MIME bodies as described in Table 5.5.13.1-1 |  | TS 24.379 [9] |  |

| Condition | Explanation |
| --- | --- |
| MANUAL | Call establishment with manual commencement mode |
| FORCE | force of automatic commencement mode at the invited MCPTT client is requested by the MCPTT user |
| ALLOW-LOCATION-INFO | Implicit floor control is requested AND <allow-location-info-when-talking> element of the <ruleset> element of the MCPTT user profile document set to “true” in TS 36.579-1 [2] Table 5.5.8.3-1 |
| For further conditions see table 5.5.1-1 |  |

<End of modification>

<Start of modification>

#### 5.5.3.3	Resource-lists

##### 5.5.3.3.1	Resource-lists from the UE

##### -	MCPTT

Table 5.5.3.3.1-1: Resource-lists from the UE for MCPTT

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | encrypted (NOTE 4) |  |  |  |
| list[1] | encrypted (NOTE 4) |  |  |  |
| name attribute | Not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4,5 |  |  |  |
| uri attribute | px_MCPTT_ID_User_B | The MCPTT ID of the invited user |  |  |
|  | SIP-URI with px_MCPTT_Group_A_ID (NOTE 8) extended with SIP URI header fields as specified for the SIP REFER message | SIP-URI: prearranged MCPTT group identit or chat group identity extended with header fields |  | PRE-ESTABLISH AND (GROUP-CALL OR CHAT-GROUP-CALL) |
|  | SIP-URI with px_MCPTT_ID_User_B (NOTE 8) extended with SIP URI header fields as specified for the SIP REFER message | SIP-URI: MCPTT ID of the called user extended with header fields |  | PRE-ESTABLISH AND PRIVATE-CALL |
| display-name | not present |  |  |  |
| resource-lists | encrypted (NOTE 4) |  | TS 24.481 [11] TS 24.484 [14] | CONFIG OR GROUPCONFIG |
| list[1] | encrypted (NOTE 4) |  |  |  |
| name attribute | not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID1 & "/users/" & XUID & "/" & MCSUEID & "/" | UE Configuration document (NOTE 1a, 2, 3) |  |  |
|  | "AUID1 & "/users/" & XUID & "/" | Editor's note: It is not clear in the core specs whether both options are allowed or only one of both; if the UE is allowed not to include the MCSUEID, it is not clear where the MC server gets it from |  |  |
| display-name | Not present |  |  |  |
| entry[2] | NOTE 4, 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID2 & "/users/" & XUID & "/" | UE User Profile document (NOTE 1b, 2) |  |  |
| display-name | Not present |  |  |  |
| entry[3] | NOTE 4, 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID3 & "/global/service-config.xml" | UE Service Configuration document (NOTE 1c) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.484 [14] | GROUPCONFIG |
| uri attribute | "org.openmobilealliance.groups/global/byGroupID/" & px_MCPTT_Group_A_ID | UE Group Configuration document |  |  |
| display-name | Not present |  |  |  |
| entry[2] | optional, NOTE 4, 5 |  | TS 24.481 [11] | GROUPCONFIG |
| uri attribute | Doc-Sel & "~~" & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 4, 5 |  | TS 24.481 [11] | GROUPKEY |
| uri attribute | Doc-Sel & "~~" & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| NOTE 1a: AUID1 = "org.3gpp.mcptt.ue-config" NOTE 1b: AUID2 = "org.3gpp.mcptt.user-profile" NOTE 1c: AUID3 = "org.3gpp.mcptt.service-config" NOTE 2: XUID = "sip:" & px_MCPTT_ID_User_A NOTE 3: MCSUEID = Instance id of the UE (derived from the IMEI according to 23.003 [69] clause 13.8) NOTE 4: XML encryption may be done by - element content encryption of the root element <resource-lists> as described in Table 5.5.13.2-1 - element content encryption of (each) <list> element as described in Table 5.5.13.2-1 - attribute URI encryption of the entry's uri attribute as described in Table 5.5.13.3-1 NOTE 5: When a resource-lists document contains more than one entry, the entries may be in any order NOTE 6: Doc-Sel = “org.3gpp.MCPTT-GKTP/global/byGroupID/” & px_MCPTT_Group_A_ID & "/" NOTE 7: Node-Sel = "/group/list-service/mgktp:GKTPs?xmlns(mgktp=urn:3gpp:ns:mcpttGKTP:1.0)” NOTE 8: TS 23.179 [8] specifies MCPTT ID and MCPTT group ID (clause 8.1.3.1) to be a URIs but does not mandate them to be a SIP URIs; nevertheless according to TS 24.379 [9] (clauses 10.1.1.2.2.1, 10.1.2.2.2.1) the URI in the uri attribute of the resource-lists' <entry> element needs to be a SIP URI. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| PRE-ESTABLISH | Call using a pre-established session |
| For further conditions see table 5.5.1-1 |  |

##### -	MCVideo

Table 5.5.3.3.1-2: Resource-lists from the UE for MCVideo

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists |  |  |  | PRIVATE-CALL GROUP-CALL EMERGENCY-CALL IMMPERIL-CALL EMERGENCY-ALERT |
| list[1] |  |  |  |  |
| name attribute | Not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 5 |  |  |  |
| uri attribute | px_MCVideo_ID_User_B | The MCVideo ID of the invited user |  |  |
| display-name | Not present |  |  |  |
| resource-lists | encrypted (NOTE 4) |  | TS 24.481 [11] TS 24.484 [14] | CONFIG OR GROUPCONFIG |
| list[1] | encrypted (NOTE 4) |  |  |  |
| name attribute | "uri: mcvideo-op.gov:resource-lists" | Editor's note: to be removed |  | CONFIG |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID1 & "/users/" & XUID & "/" & MCSUEID & "/" | UE Configuration document (NOTE 1a, 2, 3) |  |  |
|  | "AUID1 & "/users/" & XUID & "/" | Editor's note: It is not clear in the core specs whether both options are allowed or only one of both; if the UE is allowed not to include the MCSUEID, it is not clear where the MC server gets it from |  |  |
| display-name | Not present |  |  |  |
| entry[2] | NOTE 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID2 & "/users/" & XUID & "/" | UE User Profile document (NOTE 1b, 2) |  |  |
| display-name | Not present |  |  |  |
| entry[3] | NOTE 5 |  | TS 24.484 [14] | CONFIG |
| uri attribute | AUID3 & "/global/service-config.xml" | UE Service Configuration document (NOTE 1c) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 5 |  | TS 24.481 [11] | GROUPCONFIG |
| uri attribute | "org.openmobilealliance.groups/global/byGroupID/" & px_MCVideo_Group_A_ID | UE Group Configuration document |  |  |
| display-name | Not present |  |  |  |
| entry[2] | optional NOTE 5 |  | TS 24.481 [11] | GROUPCONFIG |
| uri attribute | Doc-Sel & "~~" & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| entry[1] | NOTE 5 |  | TS 24.481 [11] | GROUPKEY |
| uri attribute | Doc-Sel & "~~" & Node-Sel | MCPTT-GKTP document (NOTE 6, 7) |  |  |
| display-name | Not present |  |  |  |
| NOTE 1a: AUID1 = "org.3gpp.mcvideo.ue-config" NOTE 1b: AUID2 = "org.3gpp.mcvideo.user-profile" NOTE 1c: AUID3 = "org.3gpp.mcvideo.service-config" NOTE 2: XUID = "sip:" & px_MCVideo_ID_User_A NOTE 3: MCSUEID = Instance id of the UE (derived from the IMEI according to 23.003 [69] clause 13.8) NOTE 4: XML encryption may be done by - element content encryption of the root element <resource-lists> as described in Table 5.5.13.2-1 - element content encryption of (each) <list> element as described in Table 5.5.13.2-1 - attribute URI encryption of the entry's uri attribute as described in Table 5.5.13.3-1 NOTE 5: When a resource-lists document contains more than one entry, the entries may be in any order NOTE 6: Doc-Sel = “org.3gpp.MCPTT-GKTP/global/byGroupID/” & px_MCVideo_Group_A_ID & "/" NOTE 7: Node-Sel = "/group/list-service/mgktp:GKTPs?xmlns(mgktp=urn:3gpp:ns:mcpttGKTP:1.0)” |  |  |  |  |

##### -	MCData

Table 5.5.3.3.1-3: Resource-lists from the UE for MCData

| Derivation Path: RFC 5366 [35] / RFC 4826 [83] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists | Editor's note: XML element content encryption to be added |  |  |  |
| list |  |  |  |  |
| entry | px_MCData_ID_User_B | The MCData ID of the target MCData user |  |  |

<End of modification>