3GPP TSG-RAN WG5 Meeting #90-e	R5-210224r1

Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0110 | rev | - | Current version: | 14.9.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MCPTT Test Case 5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Adaptation to generic test procedures in TS 36.579-1 cl. 5.3 - SIP ACK from UE and from SS missing, Trying from SS missing - Incorrect or incomplete specific message content in SIP UPDATE, SIP re-INVITE, SIP OK - Incorrect session pixit - Modification of pre-established session by user dependent on MCPTT client possibilities |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Call of generic test procedures - Missing SIP messages added - Specific message contents corrected and conditions applied, - removal of IEs that are covered in default message content - proposal for pre-established session modification: UL implicit floor request, DL priority change in SDP OFFER - Incorrect pixit replaced by correct constant - Introduction of a pics to check whether modification of pre-established session by MCPTT user is possible |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading procedures, conditions and message contents may result in incorrect MCPTT test case implementation and the test case is not verifiable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.579-4 CR 0012 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: adaptation of the test case to MCPTT-Client capability and PRD13 adaptations, some wording corrections, clarification in changes by SS |  |  |  |  |  |  |  |  |

<Start of modification>

## 5.4	Configuration / Pre-established Session Establishment / Pre-established Session Modification / Pre-established Session Release

5.4.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorized for MCPTT Service }

ensure that {
  when { MCPTT User requests the creation of a pre-established session }

then { UE (MCPTT Client) requests the creation of a pre-establish session by sending a SIP INVITE message }

}

(2)

with { the MCPTT client already having a pre-established session created }

ensure that {
  when { MCPTT User requests the modification of a pre-established session }

then { UE (MCPTT Client) requests the modification of a pre-establish session by sending a SIP re-INVITE message or a SIP UPDATE message (depending on UE implementation) }

}

(3)

with { the MCPTT client already having a pre-stablished session created }

ensure that {
  when { MCPTT Server requests the modification of a pre-established session by sending either a SIP UPDATE message or a SIP re-INVITE message }

then { UE (MCPTT Client) responds to the pre-established session modification request by sending a SIP 200 (OK) message }

}

(4)

with { the MCPTT client already having a pre-stablished session created }

ensure that {
  when { MCPTT User requests the release of a pre-established session }

then { UE (MCPTT Client) requests the release of a pre-establish session by sending a SIP BYE message }

}

(5)

with { the MCPTT client already having a pre-stablished session created }

ensure that {
  when { MCPTT Server requests the release of a pre-established session by sending a SIP BYE message }

then { UE (MCPTT Client) responds to the pre-established session release request by sending a SIP 200 (OK) message }

}

5.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.379, clauses 8.2.1, 8.3.1.1, 8.3.1.2, 8.4.1.1, 8.4.1.2. Unless otherwise stated, these are Rel-13 requirements.

[TS 24.379, clause 8.2.1]

When the MCPTT client initiates a pre-established session the MCPTT client shall:

1)	gather ICE candidates according to IETF RFC 5245 [17]; and

NOTE 1:	ICE candidates are only gathered on interfaces that the MCPTT UE uses to obtain MCPTT service.

2)	generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	shall set the Request-URI of the SIP INVITE request to the public service identity of the participating MCPTT function serving the MCPTT user;

2)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

3)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

4)	shall include an Accept-Contact header field with the media feature tag g.3gpp.mcptt along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

5)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP INVITE request;

6)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

7)	shall include the "timer" option tag in the Supported header field;

8)	should include the Session-Expires header field according to IETF RFC 4028 [7] and should not include the "refresher" header field. The "refresher" header field parameter shall be set to "uac" if included;

9)	shall include an SDP offer according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.1, and include ICE candidates in the SDP offer as per IETF RFC 5245 [17]; and

10)	shall send the SIP INVITE request according to 3GPP TS 24.229 [4].

Upon receiving a SIP 2xx response to the SIP INVITE request the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

NOTE 2:	If ICE candidate evaluation results in candidate pairs other than the default candidate pair being selected a further offer answer exchange using the procedures in subclause 8.3 will be needed.

[TS 24.379, clause 8.3.1.1]

When the MCPTT client needs to modify the pre-established session outside of an MCPTT session, the MCPTT client:

1)	shall generate a SIP UPDATE request or a SIP re-INVITE request according to 3GPP TS 24.229 [4];

2)	shall include an SDP offer according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.1, and include ICE candidates in the SDP offer as per IETF RFC 5245 [17], if required; and

3)	shall send the SIP request towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4].

On receipt of the SIP 200 (OK) response the MCPTT client:

1)	shall interact with media plane as specified in 3GPP TS 24.380 [5], if there is change in media parameters or codecs in the received SDP answer, compared to those in the previously agreed SDP; and

2)	shall interact with media plane as specified in 3GPP TS 24.380 [5], if there is a media stream that is currently used in the pre-established session, marked as rejected in the received SDP answer.

NOTE:	The MCPTT client keeps resources for previously agreed media stream, media parameters and codecs until it receives a SIP 200 (OK) response.

[TS 24.379, clause 8.3.1.2]

Upon receiving a SIP UPDATE request or a SIP re-INVITE request to modify an existing pre-established session without associated MCPTT session, the MCPTT client:

1)	shall validate that the received SDP offer includes at least one media stream for which the media parameters and at least one codec is acceptable by the MCPTT client and if not reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps; and

2)	shall generate a SIP 200 (OK) response as follows:

a)	shall include an SDP answer according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.2, and include ICE candidates in the SDP answer as per IETF RFC 5245 [17]. if required.

[TS 24.379, clause 8.4.1.1]

NOTE:	The MCPTT client needs to be prepared to release the pre-established session when receiving a SIP BYE request generated by the SIP core (e.g. due to network release of media plane resources).

When a MCPTT client needs to release a pre-established session as created in subclause 8.2.1, the MCPTT client:

1)	shall generate a SIP BYE request according to rules and procedures of 3GPP TS 24.229 [4];

2)	shall set the Request-URI of the SIP BYE request to the URI that identifies the pre-established session;

3)	shall send the SIP BYE request towards the participating MCPTT function within the SIP dialog of the pre-established session according to rules and procedures of the 3GPP TS 24.229 [4]; and

4)	shall, upon receiving a SIP 200 (OK) response to the SIP BYE request interact with the media plane as specified in 3GPP TS 24.380 [5].

[TS 24.379, clause 8.4.1.2]

Upon receiving a SIP BYE request from the participating MCPTT function within a pre-established session the MCPTT client shall check whether there are any MCPTT sessions using the pre-established session, and:

1)	if there is an established MCPTT session then the MCPTT client shall remove the MCPTT client from the MCPTT session by performing the procedures for session release for each MCPTT session as specified in 3GPP TS 24.380 [5]; and

2)	if there is no MCPTT session using the pre-established session, then the MCPTT client shall:

a)	interact with the media plane as specified in 3GPP TS 24.380 [5] for disconnecting the media plane resources towards the participating MCPTT function; and

b)	shall generate and send a SIP 200 (OK) response to the SIP BYE request according to rules and procedures of 3GPP TS 24.229 [4].

5.4.3	Test description

5.4.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

5.4.3.2	Test procedure sequence

Table 5.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCPTT User request the creation of a pre-established session without implicit floor control (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) perform Generic Test Procedure for MCPTT pre-established session establishment CO as described in TS 36.579-1 [2] Table 5.3.3.3-1 to create a pre-established session? | - | - | 1 | - |
| 2A-3A | Void | - | - | - | - |
| - | EXCEPTION: Steps 4a1-5b1 describe behavior that depends on the UE implementation. The "lower case letter" identifies a step sequence that takes place if the UE is capable to provide pre-established session modification triggered by the user. | - | - | - | - |
| 4a1 | IF pc_MCX_UserinitiatedPreestablishedSessionModification: Make the MCPTT User request to modify the pre-established session by requesting usage of implicit floor control (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Steps 5a1-5b1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE uses the SIP UPDATE message or the SIP re-INVITE message to modify an existing pre-established session. | - | - | - | - |
| 5a1 | Check: Does the UE (MCPTT Client) send a SIP UPDATE message to modify a pre-established session? | --> | SIP UPDATE | 2 | P |
| 5a2 | The SS responds to the SIP UPDATE message with a SIP 200 (OK) message | <-- | SIP 200 (OK) | - | - |
| 5b1 | Check: Does the UE (MCPTT Client) perform Step 1-4 of the Generic Test Procedure for MCPTT CO session modification with implicit Floor Control as described in TS 36.579-1 [2] Table 5.3.14.3-1 to modify a pre-established session? | - | - | 2 | - |
| 6 | Void | - | - |  |  |
| 7 | The SS sends a SIP UPDATE message to modify the Floor priority of the pre-established session. | <-- | SIP UPDATE | - | - |
| 8 | Check: Does the UE (MCPTT Client) respond to the SIP UPDATE message with a SIP 200 (OK) message? | --> | SIP 200 (OK) | 3 | P |
| 9 | Check: Is the Generic Test Procedure for MCPTT CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to modify the Floor priority of the pre-established session correctly performed? | - | - | 3 | - |
| 10 | Void | - | - | - | - |
| 11 | Make the MCPTT User request to release the pre-established session (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCPTT Client) perform Generic Test Procedure for MCPTT CO call release as described in TS 36.579-1 [2] Table 5.3.10.3-1 to release the pre-established session? | - | - | 4 | - |
| 13 | Void | - | - | - | - |
| 14 | Make the MCPTT User request the creation of a pre-established session (NOTE 1) | - | - | - | - |
| 15 | The Generic Test Procedure for MCPTT pre-established session establishment CO as described in TS 36.579-1 [2] Table 5.3.3.3-1 to create a pre-established session is performed. | - | - | - | - |
| 15A-16A | Void | - | - | - | - |
| 17 | Check: Is the Generic Test Procedure for MCPTT CT call release as described in TS 36.579-1 [2] Table 5.3.12.3-1 to release the pre-established session correctly performed? | - | - | 5 | - |
| 18 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.4.3.3	Specific message contents

Table 5.4.3.3-1: SIP INVITE (steps 2, 15, Table 5.4.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1 condition GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP message | SDP message as described in Table 5.4.3.3-1A |  |  |  |

Table 5.4.3.3-1A: SDP in SIP INVITE (Table 5.4.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-1, conditions INITIAL_SDP_OFFER, NOT WITH_FLOORCONTROL

Table 5.4.3.3-2: SIP 200 (OK) (steps 2, Table 5.4.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP message | SDP message as described in Table 5.4.3.3-2A |  |  |  |

Table 5.4.3.3-2A: SDP in SIP 200 (OK) (Table 5.4.3.3-2)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1, condition NOT WITH_FLOORCONTROL

Table 5.4.3.3-3: Void

Table 5.4.3.3-4: Void

Table 5.4.3.3-5: Void

Table 5.4.3.3-5A: SIP UPDATE (step 5a1, Table 5.4.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.15.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | SDP Message as described in Table 5.4.3.3-5B |  |  |  |

Table 5.4.3.3-5B: SDP in SIP UPDATE (Table 5.4.3.3-5A) or re-INVITE (Table 5.4.3.3-6)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-1 with condition IMPLICIT_GRANT_REQUESTED

Table 5.4.3.3-6: SIP re-INVITE (step 5b1, Table 5.4.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1 with condition re-INVITE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact | Contact header with the same Contact URI and the same mandatory feature parameters as in the INVITE creating the dialog |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP Message | SDP Message as described in Table 5.4.3.3-5B |  |  |  |

Table 5.4.3.3-6A: SIP 200 (OK) (step 5a2 or 5b3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1 with condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCX_SessionID_B | The URI that identifies the pre-established session |  |  |

Table 5.4.3.3-6B: SIP UPDATE (step 7, Table 5.4.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.15.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | SDP Message as described in Table 5.4.3.3-6D |  |  |  |

Table 5.4.3.3-6C: SIP 200 (OK) (step 8)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1 with condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact | Contact header with the same Contact URI and the same mandatory feature parameters as in the INVITE creating the dialog |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP Message | SDP message as described in Table TS 36.579-1 [2] 5.5.3.1.1-1 |  |  |  |

Table 5.4.3.3-6D: SDP in SIP UPDATE (Table 5.4.3.3-6B)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Media descriptions |  |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format specific parameters |  |  |  |  |
| mc_priority | "2" |  |  |  |

Table 5.4.3.3-7: SIP re-INVITE (step 9, Table 5.4.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 with condition re_INVITE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact | same as in the response for the INVITE creating the dialog |  |  |  |
| Message-body |  |  |  |  |
| SDP Message | SDP Message as described in Table 5.4.3.3-7A |  |  |  |

Table 5.4.3.3-7A: SDP in SIP re-INVITE (Table 5.4.3.3-7)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Media descriptions |  |  |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format specific parameters |  |  |  |  |
| mc_priority | "3" |  |  |  |

Table 5.4.3.3-7B: SIP 200 (OK) (step 9, Table 5.4.3.2-1; step 4, TS 36.579-1 [2] Table 5.3.4.3-1)

Derivation Path: Table 5.4.3.3-6C

Table 5.4.3.3-8: SIP BYE (step 12, Table 5.4.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.10.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | tsc_MCX_SessionID_B | The URI that identifies the pre-established session |  |  |

Table 5.4.3.3-9: SIP BYE (step 17, Table 5.4.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.12.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | tsc_MCX_SessionID_B | The URI that identifies the pre-established session |  |  |

<End of modification>