3GPP TSG-RAN5 Meeting #90-e	R5-210288r2

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1684 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of definition of common environment for R16 NR Immediate MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of definition of common environment for R16 NR Immediate MDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add includeCommonLocationInfo and ul-DelayValueConfig IEs in ReportConfigNR for R16 NR Immediate MDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The contents of ReportConfigNR is not completed for NR Immediate MDT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3, 4.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Add the related IEs of Immediate MDT r2: Update IE BT-NameList and IE WLAN-NameList. Correct clauses affected on the coversheet. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

–	ReportConfigNR

Table 4.6.3-142: ReportConfigNR(Thres1, Thres2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| rsType | ssb |  |  |
| reportInterval | ReportInterval |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
|  | true |  | pc_ss_SINR_Meas |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityRS-Indexes | Not present |  |  |
| maxNrofRS-IndexesToReport | Not present |  |  |
| includeBeamMeasurements | false |  |  |
| useWhiteCellList | false |  |  |
| measRSSI-ReportConfig-r16 | Not present |  |  |
| includeCommonLocationInfo-r16 | Not present |  |  |
| includeBT-Meas-r16 | Not present |  |  |
| includeBT-Meas-r16 CHOICE { |  |  | MDT_BT |
| setup | BT-NameList |  |  |
| } |  |  |  |
| includeWLAN-Meas-r16 | Not present |  |  |
| includeWLAN-Meas-r16 CHOICE { |  |  | MDT_WLAN |
| setup | WLAN-NameList |  |  |
| } |  |  |  |
| includeSensor-Meas-r16 | Not present |  |  |
| includeSensor-Meas-r16 CHOICE { |  |  | MDT_SENSOR |
| setup | Sensor-NameList |  |  |
| } |  |  |  |
| ul-DelayValueConfig-r16 | Not present |  |  |
| ul-DelayValueConfig-r16 CHOICE { |  |  | MDT_DELAY |
| setup | UL-DelayValueConfig |  |  |
| } |  |  |  |
| reportAddNeighMeas-r16 | Not present |  |  |
| } |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA1 SEQUENCE { |  |  | EVENT_A1 |
| a1-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| eventA2 SEQUENCE { |  |  | EVENT_A2 |
| a2-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| eventA3 SEQUENCE { |  |  | EVENT_A3 |
| a3-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useWhiteCellList | false |  |  |
| } |  |  |  |
| eventA4 SEQUENCE { |  |  | EVENT_A4 |
| a4-Threshold CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useWhiteCellList | false |  |  |
| } |  |  |  |
| eventA5 SEQUENCE { |  |  | EVENT_A5 |
| a5-Threshold1 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| a5-Threshold2 CHOICE { |  |  |  |
| rsrp | Thres2 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useWhiteCellList | false |  |  |
| } |  |  |  |
| eventA6 SEQUENCE { |  |  | EVENT_A6 |
| a6-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| reportOnLeave | false |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| useWhiteCellList | false |  |  |
| } |  |  |  |
| } |  |  |  |
| rsType | ssb |  |  |
| reportInterval | ReportInterval |  |  |
| reportAmount | r2 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | true |  |  |
| sinr | false |  |  |
|  | true |  | pc_ss_SINR_Meas |
| } |  |  |  |
| maxReportCells | 8 |  |  |
| reportQuantityRS-Indexes | Not present |  |  |
| maxNrofRS-IndexesToReport | Not present |  |  |
| includeBeamMeasurements | false |  |  |
| reportAddNeighMeas | Not present |  |  |
| measRSSI-ReportConfig-r16 | Not present |  |  |
| useT312-r16 | Not present |  |  |
| includeCommonLocationInfo-r16 | Not present |  | MDT |
| includeBT-Meas-r16 | Not present |  |  |
| includeBT-Meas-r16 CHOICE { |  |  | MDT_BT |
| setup | BT-NameList |  |  |
| } |  |  |  |
| includeWLAN-Meas-r16 | Not present |  |  |
| includeWLAN-Meas-r16 CHOICE { |  |  | MDT_WLAN |
| setup | WLAN-NameList |  |  |
| } |  |  |  |
| includeSensor-Meas-r16 | Not present |  |  |
| includeSensor-Meas-r16 CHOICE { |  |  | MDT_SENSOR |
| setup | Sensor-NameList |  |  |
| } |  |  |  |
| } |  |  |  |
| reportCGI SEQUENCE { |  |  | CGI |
| cellForWhichToReportCGI | PhysCellId |  |  |
| } |  |  |  |
| condTriggerConfig SEQUENCE { |  |  | CHO. CPC |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  | EVENT_A3 |
| a3-Offset CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| condEventA5 SEQUENCE { |  |  | EVENT_A5 |
| a5-Threshold1 CHOICE { |  |  |  |
| rsrp | Thres1 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| a5-Threshold2 CHOICE { |  |  |  |
| rsrp | Thres2 | Thres is an entry value into a mapping table in TS 38.133 [13]. |  |
| } |  |  |  |
| hysteresis | Hysteresis |  |  |
| timeToTrigger | TimeToTrigger |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EVENT_A1 | Configuration of Event A1 |
| EVENT_A2 | Configuration of Event A2 |
| EVENT_A3 | Configuration of Event A3 |
| EVENT_A4 | Configuration of Event A4 |
| EVENT_A5 | Configuration of Event A5 |
| EVENT_A6 | Configuration of Event A6 |
| PERIODICAL | Configuration of periodical reporting |
| CGI | Configuration of CGI measurement |
| CHO | Configuration of conditional handover |
| CPC | Conditional PScell change |
| MDT | Configuration of MDT |
| MDT_BT | Configuration of MDT including Bluetooth measurements |
| MDT_WLAN | Configuration of MDT including WLAN measurements |
| MDT_SENSOR | Configuration of MDT including Sensor measurements |
| MDT_DELAY | Configuration of UL PDCP Packet Delay per DRB |

[Skip unchanged text]

–	UL-DelayValueConfig

Table 4.6.3-200A: UL-DelayValueConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UL-DelayValueConfig-r16 ::= SEQUENCE { |  |  |  |
| delay-DRBlist | Set according to specific message content |  |  |
| } |  |  |  |

[Skip unchanged text]

–	BT-NameList

Table 4.6.5-0C: BT-NameList

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BT-NameList-r16 ::= SEQUENCE (SIZE (1..maxBT-Name-r16)) OF { | 1 entry |  |  |
| BT-Name-r16[1] | Set according to specific message content | OCTET STRING (SIZE (1..248)) |  |
| } |  |  |  |

[Skip unchanged text]

–	Sensor-NameList

Table 4.6.5-13: Sensor-NameList

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Sensor-NameList-r16 ::= SEQUENCE { |  |  |  |
| measUncomBarPre-r16 | Set according to specific message content |  |  |
| measUeSpeed | Set according to specific message content |  |  |
| measUeOrientation | Set according to specific message content |  |  |
| } |  |  |  |

[Skip unchanged text]

–	WLAN-NameList

Table 4.6.5-18: WLAN-NameList

| Derivation Path: TS 38.331 [6], clause 6.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| WLAN-NameList-r16 ::= SEQUENCE (SIZE (1..maxWLAN-Name-r16)) OF { | 1 entry |  |  |
| WLAN-Name-r16[1] | Set according to specific message content | OCTET STRING (SIZE (1..32)) |  |
| } |  |  |  |

<<< END OF CHANGES >>>