3GPP TSG-RAN5 Meeting #90-e	R5-210298r3

# Electronic Meeting, 22nd Feb – 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0844 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spectrum emissions mask for intra-band non-contiguous EN-DC Test Definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Wgap for intra-band non-contiguous EN-DC is now defined in 38.508-1, section 4.3.1.4.3. In addition, in R4-1904989, RAN4 confirmed how the minimum requirements apply. As a consequence, test case definition can be completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Updated initial conditions for test frequency and channel BW according to Wgap definition. -Updated initial conditions to define RB allocation and modulation to test. -Updated the test procedure and test requirements according to the minimum requirements exceptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Test Point analysis provided in a separated CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Corrected references to test configuration table. Clarified E-UTRA RB allocation depends on both channel BW and modulation. To ensure test points are a subset of MPR test point, Edge_XRB allocations in LTE replaced by Edge_1RB allocations. TP analysis revised accordingly. Added editor’s note clarifying testing of none simultaneous LTE-NR transmission needs further study. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

##### 6.5B.2.2.1	Spectrum emissions mask for intra-band non-contiguous EN-DC

Editor's note:	The following aspects are either missing or not yet determined:

-		Testing of none simultaneous LTE-NR transmission is FFS.

6.5B.2.2.1.1	Test purpose

Same test purpose as in clause 6.5.2.2 in TS 38.521-1 [8] for the NR carrier.

6.5B.2.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC.

6.5B.2.2.1.3	Minimum conformance requirements

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier. A composite spectrum emission mask is a combination of individual CC spectrum emissions masks. Where two masks overlap the most relaxed limit is used. Composite spectrum emission mask applies to frequencies up to  ΔfOOB starting from the edges of the sub-blocks. If for some frequency an individual CC spectrum emission mask overlaps with the bandwidth of another CC then the emission mask does not apply for that frequency.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5B.2.2.1.

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask and therefore LTE anchor agnostic approach is not applied.

6.5B.2.2.1.4	Test description

6.5B.2.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in TS 38.508-1 [6] clause 4.3.1.4.3, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in TS 38.508-1 [6] clause 4.3.1.4.3 and are shown in table 6.5B.2.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 6.5B.2.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1.4.3 for cases with single E-UTRA carrier |  |  | Low with maxWgap (NR low – E-UTRA high); High with maxWgap (E-UTRA low – NR high) |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [6] subclause 4.3.1.4.3 |  |  | Lowest for E-UTRA, lowest for NR; Highest for E-UTRA, highest for NR |  |  |  |
| Test SCS as specified as specified in TS 38.508-1 [6] subclause 4.3.1.4.3 |  |  | Lowest, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | NR Uplink Configuration |  | E-UTRA Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) | Modulation | RB allocation (NOTE 3) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | QPSK | Edge_1_RB_Right |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | QPSK | Edge_1_RB_Left |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full | QPSK | Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | QPSK | Edge_1_RB_Right |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | QPSK | Edge_1_RB_Left |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full | QPSK | Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | 16QAM | Edge_1_RB_Right |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | 16QAM | Edge_1_RB_Left |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full | 16QAM | Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left | 16QAM | Edge_1_RB_Right |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right | 16QAM | Edge_1_RB_Left |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full | 16QAM | Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left | 16QAM | Edge_1_RB_Right |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right | 16QAM | Edge_1_RB_Left |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full | 16QAM | Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | QPSK | Edge_1_RB_Right |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right | QPSK | Edge_1_RB_Left |
| 18 | Default |  | CP-OFDM QPSK | Outer Full | QPSK | Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | 16QAM | Edge_1_RB_Right |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | 16QAM | Edge_1_RB_Left |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full | 16QAM | Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left | 16QAM | Edge_1_RB_Right |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right | 16QAM | Edge_1_RB_Left |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full | 16QAM | Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left | 16QAM | Edge_1_RB_Right |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right | 16QAM | Edge_1_RB_Left |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full | 16QAM | Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For a given E-UTRA channel BW and modulation, Full allocation refers to the higher allocation defined in table 6.6.2.1.4.1-1 in 36.521-1 [10]. |  |  |  |  |  |  |

For Initial conditions as in clause 6.5.2.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.6-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 6.5.2.2.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG according to TS 38.508 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

6.5B.2.2.1.4.2	Test Procedure

- For NR carrier, SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.2.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

- For E-UTRA carrier, SS sends uplink scheduling information via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 6.5B.2.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

- Send continuously uplink power control "up" commands to the UE for NR and E-UTRA carrier until the UE transmits at its PUMAX level; allow at least 200 ms from the first TPC command starting in this step for the UE to reach PUMAX level.

- For E-UTRA carrier, measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1.5-1 or 6.6.2.1.5-2 in TS 36.521-1 [10], as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs. If for some frequency an individual spectrum emission mask overlaps with the bandwidth of the NR carrier then the emission mask does not apply for that frequency.

- For NR carrier, measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.5.2.2.5-1 in TS 38.521-1 [8]. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs. If for some frequency an individual spectrum emission mask overlaps with the bandwidth of the E-UTRA carrier then the emission mask does not apply for that frequency.

NOTE 1: When switching to DFT-s-OFDM waveform, send an NR RRCReconfiguration message according to TS 38.508-1 [6] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5B.2.2.1.5	Test requirement

-For NR carrier frequency masks not overlapping with E-UTRA carrier frequency masks, Power of any UE emission shall fulfil requirements in Table 6.5.2.2.5-1 defined in TS 38.521-1 [8].

-For E-UTRA carrier frequency masks not overlapping with NR carrier frequency masks, Power of any UE emission shall fulfil requirements in Table 6.6.2.1.5-1 or 6.6.2.1.5-2 in TS 36.521-1 [10], as applicable.

-For NR carrier frequency masks overlapping with E-UTRA carrier frequency masks, the most relaxed limit is used between requirements in Table 6.5.2.2.5-1 defined in TS 38.521-1 [8] and Table 6.6.2.1.5-1 or 6.6.2.1.5-2 in TS 36.521-1 [10], as applicable.

## <<< END OF CHANGES >>>