3GPP TSG-RAN5 Meeting #90-e 	R5-210376r1

# Electronic Meeting, 22nd Feb 2021 – 8th Mar 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CR |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | - | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test function Set UE Capability Info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 01/29/2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of Set UE Capability Info Function |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Set UE Capability Info description and message names "SET UE CAPABILITY INFO REQUEST" and SET UE CAPABILITY INFO RESPONSE" In Rel-16 RACS WI, the UE capability Information message can be segmented if the capability size > Max PDCP SDU size. Since there is no capability indicator from the UE on whether the capability size > Max PDCP SDU size, it can be challenging to find a definitive way to test this. On the same, UE capability message size depends on various parameters and operator requirements. To overcome this and to find a definitive way to test part of conformance requirement (TS38.331, cl 5.7.7.2 and 5.7.7.3), a test mode function which can indicate to the UE to use a preconstructed message which is > Max PDCP SDU size. This will also indicate to conformance tests on the number of segments that will be received from the UE. A test command can set the UE's UEcapabilityInformation message, to be included in the rrc-MessageSegmentContainer IE of the UlDedicatedMessageSegment message(s). Based the on this test message, UE will then trigger the UECapabilityInformation message based on the RRC-segAllowed-r16 IE of the UECapabilityEnquiry message, thus providing test coverage for this feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some aspect of the RACS WP wont be testable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9 (new), 6.9 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Spec will have to move to Rel-16 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In revision r1 : The message is updated to handle future use cases. |  |  |  |  |  |  |  |  |

<Start of Changes>

## 5.9	Set UL Message test function

### 5.9.1	General

The SS uses the Set UL message test procedure to indicate to the UE whether to use a preconfigured uplink message, see figure 5.9.1-1. [FFS].

Figure 5.9.1-1: Set UL Message test procedure

### 5.9.2	Initiation

The SS requests the UE to use the preconfigured UL message by transmitting a SET UL MESSAGE REQUEST message and the UE confirms by responding with a a SET UL MESSAGE RESPONSE message.

### 5.9.3	Reception of SET UL MESSAGE REQUEST message by UE

When UE receives SET UL MESSAGE REQUEST message then the UE shall:

- if the UE is operating in RRC_CONNECTED state:

2>	if E0 = 1.

3>	use the predefined UE capability as defined in [FFS] to create the subsequent UECapabilityInformation message to be transmitted upon receiving a UECapabilityEnquiry message.

3>	Transmit SET UL MESSAGE RESPONSE message.

2>	else

3>	the UE behaviour is unspecified.

1>	else:

2>	the UE behaviour is unspecified.

<Unchanged Sections Omitted>

## 6.9	SET UL MESSAGE

### 6.9.1	SET UL MESSAGE REQUEST

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| Use Preconfigured UL Message |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | octet 1 |

where Use Preconfigured UL Message is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  | E0 | octet 2 |

EO:”Use Preconfigured UE Capability”

### 6.9.2	SET UL MESSAGE RESPONSE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | octet 1 |

<End of Changes>