3GPP TSG-RAN5 Meeting #90-e	R5-210393r2

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0147 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition and update of PICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility, Qualcomm, Mediatek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of PICS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added missing pics for UE support for RRC inactiveState. Revision 1: Revert pc_nrMIMO_NonCB_PUSCH and pc_nrMIMO_CB_PUSCH to exclude onelayer support. Convert pc_bwp_SwitchingDelay to pc_bwp_SwitchingDelay_Type1 and pc_bwp_SwitchingDelay_Type2. Revision 2: pc_pdsch_MappingTypeA is mandatory to be supported as per 38.306 clause 4.2.1 note 1, hence added ‘yes’ in column ‘If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conforment UE may Fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2, A.4.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| 2 | Support 256QAM for PDSCH for FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_256QAM_FR1 | Yes |  |  |
| 3 | Support 256QAM for PDSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pdsch_256QAM_FR2 | No |  |  |
| 4 | Support 256QAM for PUSCH for at least one NR FR1 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR1 | No |  |  |
| 4a | Support 256QAM for PUSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR2 | No |  |  |
| 5 | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeA | Yes | Yes |  |
| 6 | Support receiving PDSCH using PDSCH mapping type B | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeB | Yes |  |  |
| 7 | Support resource allocation Type 0 for PUSCH | 38.306, 4.2.7.10 | Rel-15 | pc_ra_Type0_PUSCH | No |  |  |
| 8 | Support scaling factor 0.75 is applied to the band in the max data rate calculation | 38.306, 4.2.7 | Rel-15 | pc_scalingFactor0dot75 |  |  |  |
| 9 | Support reconfiguration with sync using a contention free random access on PRACH resources that are associated with CSI-RS resources of the target cell | 38.306, 4.2.7.10 | Rel-15 | pc_csi_RS_CFRA_ForHO | No |  |  |
| 10 | Support Type 1 PUSCH transmissions with configured grant | 38.306, 4.2.7.10 | Rel-15 | pc_configuredUL_GrantType1 | No |  |  |
| 11 | Support Type 2 PUSCH transmissions with configured grant | 38.306, 4.2.7.10 | Rel-15 | pc_configuredUL_GrantType2 | No |  |  |
| 12 | Support PDSCH Reception when configured with higher layer parameter aggregationFactorDL > 1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_RepetitionMultiSlots | No |  |  |
| 13 | Supports supplemental uplink with dynamic switch (DCI based selection of PUSCH carrier) | 38.306, 4.2.7.7 | Rel-15 | pc_dynamicSwitchSUL | No |  |  |
| 14 | Supports MIMO layers at the UE for PUSCH transmission with codebook precoding. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset | 38.306, 4.2.7.8 | Rel-15 | pc_nrMIMO_CB_PUSCH | No |  | Set to true if maxNumberMIMO-LayersCB-PUSCH has value different from "oneLayer" |
| 15 | Supports MIMO layers at the UE for PUSCH transmission using non-codebook precoding | 38.306, 4.2.7.8 | Rel-15 | pc_nrMIMO_NonCB_PUSCH | No |  | Set to true if maxNumberMIMO-LayersNonCB-PUSCH has value different from "oneLayer" |
| 16 | Support receiving PDSCH with interleaved VRB-to-PRB mapping | 38.306, 4.2.7.10 | Rel-15 | pc_interleavingVRB_ToPRB_PDSCH | Yes |  |  |
| 17 | Support dynamic EN-DC power sharing for at least one EN-DC band combination | 38.306, 4.2.7.9 | Rel-15 | pc_dynamicPowerSharing | Yes |  | If the UE supports this capability it will dynamically share the power between NR and LTE if P_LTE + P_NR > Pcmax. |
| 18 | Supports up to 10 search spaces in a SCell per BWP | 38.306, 4.2.7.10 | Rel-15 | pc_maxNumberSearchSpaces | No |  |  |
| 19 | Supports spatial bundling of HARQ-ACK bits carried on PUCCH or PUSCH per PUCCH group. With spatial bundling, two HARQ-ACK bits for a DL MIMO data is bundled into a single bit by logical "AND" operation | 38.306, 4.2.7.10 | Rel-15 | pc_spatialBundlingHARQ_ACK | Yes |  |  |
| 20 | Support alternative additional DMRS position for co-existence with LTE CRS | 38.306, 4.2.7.5 | Rel-15 | pc_additionalDMRS_DL_Alt | No |  |  |
| 21 | Supports transmitting PUSCH scheduled by DCI format 0_0 or 0_1 when configured with higher layer parameter aggregationFactorIUL > 1 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_RepetitionMultiSlots | Yes |  |  |
| 22 | Support beam correspondence without UL beam sweeping | 38.306, 4.2.7.2 | Rel-15 | pc_beamCorrespondenceWithoutUL_BeamSweeping | Yes |  | A UE that can fulfil the requirements without UL beam sweeping then set the bit to 1. A UE that can fulfil the requirements with UL beam sweeping then set the bit to 0. |
| 23 | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier | 38.306, 4.2.7.6 | Rel-15 | pc_maxNumberMIMO_LayersPDSCH | CY |  |  |
| 24 | Supports DCI and timer based active BWP switching delay type1 | 38.306, 4.2.7.10 | Rel-15 | pc_bwp_SwitchingDelay_Type1 | No |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |
| 24A | Supports DCI and timer based active BWP switching delay type2 | 38.306, 4.2.7.10 | Rel-15 | pc_bwp_SwitchingDelay_Type2 | No |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |
| 25 | Support modified MPR behaviour | 38.306 4.2.7.2 | Rel-15 | pc_modifiedMPR_behaviour | No |  |  |
| 26 | Support dynamic switching between resource allocation Types 0 and 1 for PDSCH | 38.306, 4.2.7.10 | Rel-15 | pc_dynamicSwitchRA_Type0_1_PDSCH | No |  |  |
| 27 | Support dynamic switching between resource allocation Types 0 and 1 for PUSCH | 38.306, 4.2.7.10 | Rel-15 | pc_dynamicSwitchRA_Type0_1_PUSCH | No |  |  |
| 28 | Support almost contiguous UL CP-OFDM transmissions in FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_almostContiguousCP_OFDM_UL_FR1 | No |  |  |
| 29 | Support almost contiguous UL CP-OFDM transmissions in FR2 | 38.306, 4.2.7.10 | Rel-15 | pc_almostContiguousCP_OFDM_UL_FR2 | No |  |  |
| 30 | Support dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1, and the minimum scheduling offset for PDSCH and aperiodic CSI-RS triggering offset (K0), and PUSCH (K2), and the extended value range for aperiodic CSI-RS triggering offset | 38.306, 4.2.7.10 | Rel-16 | pc_crossSlotScheduling | No |  |  |
| 31 | Supports pi/2-BPSK modulation scheme for PUSCH in FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_halfpiBPSK | No |  |  |
| 32 | Support multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency | 38.306, 4.2.7.6 | Rel-16 | pc_multiDCI_MultiTRP_r16 | No |  |  |
| 33 | Support receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS | 38.306, 4.2.7.2 | Rel-15 | pc_rateMatchingLTE_CRS | Yes |  |  |
| 34 | Support of BWP operation without bandwidth restriction | 38.306, 4.2.7.2 | Rel-15 | pc_bwp-WithoutRestriction | No |  |  |

### <<< Text Skipped Here>>>>

### A.4.3.7	General Capabilities

Table A.4.3.7-1: UE General Capabilities

| Item | UE General Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support UL transmission via either MCG path or SCG path for the split SRB as specified in TS 37.340[20] | 38.306, 4.2.2 | Rel-15 | pc_splitSRB_WithOneUL_Path | No |  |  |
| 2 | Support UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340[20] | 38.306, 4.2.2 | Rel-15 | pc_splitDRB_withUL_Both_MCG_SCG | Yes |  |  |
| 3 | Support direct SRB between the SN and the UE as specified in TS 37.340[20] | 38.306, 4.2.2 | Rel-15 | pc_srb3 | Yes |  |  |
| 4 | Support of reflective QoS | 38.306, 4.2.2 | Rel-15 | pc_as_ReflectiveQoS | No |  |  |
| 5 | Support of NAS reflective QoS | 24.501, 6.2.5.1.4.1, 9.11.4.1 | Rel-15 | pc_nas_ReflectiveQoS | No |  |  |
| 6 | Support of SMS over NAS | 24.501, 5.5.1.2 | Rel-15 | pc_sms_over_NAS | No |  |  |
| 7 | Support of CMAS message on NR | 38.331, 5.2.2.2.2 | Rel-15 | pc_CMAS_NR | No |  |  |
| 8 | Support of ETWS message on NR | 38.331, 5.2.2.2.2 | Rel-15 | pc_ETWS_NR | No |  |  |
| 9 | The UE supports additional UE-requested PDU establishment | 24.501, 6.4.1.5 | Rel-15 | pc_Additional_PDU_establishment | No |  | pc_ExpectedNoOfPDUSessionsAtRegistration +1 |
| 10 | The UE includes the SM PDU DN request container IE in the PDU SESSION ESTABLISHMENT REQUEST message | 24.501, 6.4.1.2 | Rel-15 | pc_SM_PDU_DN_RequestContainer | No |  |  |
| 11 | Support of emergency services fallback in NR connected to 5GCN | 24.501 | Rel-15 | pc_NR_5GC_EmergencyService_fallback | No |  |  |
| 12 | Support of EPS fallback | 24.501, | Rel-15 | pc_EPS_fallback | No |  |  |
| 13 | Support of UE requested PDU session modification | 24.501, 6.4.2.2 | Rel-15 | pc_MO_PDU_Session_Modification | Yes |  |  |
| 14 | Support of emergency services in NR connected to 5GCN | 24.501 | Rel-15 | pc_NR_5GC_EmergencyServices | No |  |  |
| 15 | Support of voiceFallbackIndication | 38.306, 4.2.13 | Rel-16 | pc_voiceFallbackIndication | No |  |  |
| 16 | Support provision of referenceTimeInfo | 38.306, 4.2.2 | Rel-16 | pc_referenceTimeProvision_r16 | No |  | specifically for TSC (time sensitive communication) services |
| 17 | Support of RACS | 24.501, 9.11.3.1 | Rel-16 | pc_5GC_RACS | No |  |  |
| 18 | Support of RRC message Segmentation in the UL | 38.306, 5.4 | Rel-16 | pc_NR_UL_Segmentation | No |  | UE supports segmenation of UECapabilityInformation message, IF size > maximum supported size of a PDCP SDU |
| 19 | Support of RRC_INACTIVE as specified in TS 38.331 [9]. | 38.306, 4.2.2 | Rel-15 | pc_inactiveState | Yes |  |  |