3GPP TSG-RAN5 Meeting #90-e	R5-210433r1

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1024 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Annex F for 5.4.3.1 and 7.4.3.1 with TT analysis results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The TT analysis for 5.4.3.1 and 7.4.3.1 has been provided in a separate contribution. The results of this TT analysis are not captured in Annex F of TS 38.533 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the TT analysis results to Annex F of TS 38.533 Removed the [.] of the timing advance measurement uncertainty |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5.4.3.1 and 7.4.3.1 cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: editorial change to remove highlight from the revision process. |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1 and Table F.1.1.2-2. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tighter measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 and Table F.1.1.2-2 shall be handled case by case.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc | dB | ±1.5 |  |
| Ratio of cell X signal / AWGN, Êsx / Noc | dB | ±0.3 | Same as in LTE |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.101-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.101-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 |  |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 |  |
| Timing Advance Adjustment accuracy | Tc | ±88 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2:These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Void |  |  |  |

Table F.1.1.2-2: Maximum allowed measurement uncertainty values for the test system for FR2 (up to 40GHz) and Cell BW ≤ 400 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | TBD |  |
| AWGN absolute power, Noc for Measurement PRB | dB | TBD | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 |  |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| ÊsxNote 3 absolute power, Noc averaged over BWConfig | dB | TBD |  |
| ÊsxNote 3 absolute power, Noc for Measurement PRB | dB | TBD | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| Relative power between any two Ês levels at the same frequency | dB | TBD |  |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx |  |
| AWGN and signal flatness* | dB | TBD |  |
| Uplink absolute power measurement | dB | TBD |  |
| Uplink relative power measurement | dB | TBD |  |
| Uplink signal transmit timing relative to downlink | Tc | ±[48] |  |
| Relative transmit timing during UE timing adjustment | Tc | ±[40] |  |
| Timing Advance Adjustment | Tc | ±40 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combine these values in the TT analysis in TR 38.903 Note 2: These values apply for cell BW ≤ 400 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Applies for test cases that use Ês without AWGN |  |  |  |

# < Unchanged sections omitted >

Table F.1.1.2-5: Maximum test system uncertainty for RRM requirements for EN-DC FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.3.2.2.1 Contention based random access test in FR2 for PSCell in EN-DC | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.3.2.2.2 Non-contention based random access test in FR2 for PSCell in EN-DC | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Noc ±6.0 dB Ês1 / Noc ±[0.3] dB ±[48]Tc Uplink signal transmit timing relative to downlink ±[40]Tc Relative during UE timing adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3.2.2.1 Contention based random access test in FR2 for NR standalone | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.3.2.2.2 Non-contention based random access test in FR2 for NR standalone | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Noc ±6.0 dB Ês1 / Noc ±[0.3] dB ±[48]Tc Uplink signal transmit timing relative to downlink ±[40]Tc Relative during UE timing adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |

# < Unchanged sections omitted >

Table F.1.3.2-4: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 Contention based random access test in FR2 for PSCell in EN-DC | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 5.3.2.2.2 Non-contention based random access test in FR2 for PSCell in EN-DC | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | TBD | TBD | TBD |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |

Table F.1.3.2-5: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 7.3.2.2.1 Contention based random access test in FR2 for NR standalone | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 7.3.2.2.2 Non-contention based random access test in FR2 for NR standalone | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | TBD | TBD | TBD |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |

< End of changes >