3GPP TSG-RAN5 Meeting #90-e	R5-210454r2

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0199 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update RRM MU values for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz, Anritsu, Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The MU values for RRM FR2 in E.3 of TR 38.903 are not finalized. RRM TT analyses cannot be completed without finalizing the MU. The alternative solutions to increase the downlink dB range for RRM FR2 test cases were presented in R5-203321, but never captured in the TR. It is useful information to handle the TT analysis in the future and it is worth to capture it |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR captures the agreements of the MU discussion in Annex E.3: Added a new A.4 with the methods outlined in R5-203321 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MU values for RRM FR2 will remain TBD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A, E.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: added section A.4 and reviewed wording in Table E.3.1-1, added Anritsu as co-source. R2: added KS as co-source |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# A.4	Design of radiated test cases defined in TS 38.533

The design of radiated test cases defined in TS 38.533 is more challenging than for conducted test cases, because the over-the-air path loss reduces the downlink power seen by the UE, and reduces the uplink power received by the test system.

The achievable downlink power in a practical test system is restricted, and there is less dB range between the lowest and highest power that can be applied within the UE Core requirement side conditions.

The range of uplink power that can be measured by a practical test system is also restricted, by signal-to-noise ratio considerations at the low end, and by the UE output power at the high end.

For both downlink and uplink, the achievable dB range is most restricted when the signal arrives from the UE Spherical coverage direction. In a test case it is further restricted by downlink power level uncertainty or uplink power measurement uncertainty, which are both larger for radiated signals than for conducted signals. This Annex considers the effect of restricted dB range on radiated RRM test case design.

## A.4.1	Downlink considerations

### A.4.1.1	Side conditions for Rx Beam Peak angle of arrival

Side conditions for Rx Beam Peak angle of arrival are less stringent than for Spherical Coverage angle of arrival. They are not directly analysed here, but the same principles apply as for Spherical Coverage in clause A.4.1.2.

### A.4.1.2	Side conditions for Spherical Coverage angle of arrival

As an example, consider a test case where the UE makes measurements on the downlink signal, for example in Event-triggered reporting. Some side condition values are band-dependent, and also depend on whether the cell being measured is intra-frequency or inter-frequency. An adverse case is chosen for illustration:

- Spherical Coverage angle of arrival

- Inter-frequency

- UE Power Class 3

- Band n259

- Maximum multi-band relaxation

- Full RB allocation

- UE is required to make measurements, so side conditions apply

The scenario is however best case on two points:

- Applied Es only, without Noc (only UE internal noise)

- Only one cell on the frequency (no other intra-frequency cells)

Figure A.4.1.2-1: Example side conditions when UE is making measurements

It can be seen that when uncertainties are taken into account (an indicative value of ±6dB is used), the applied level range can be from -87dBm/SCS to -85dBm/SCS, a range of only 2dB. This is very restrictive for the test case design.

### A.4.1.3	Test case design options to increase downlink dB range

A number of options are available to increase the dB range of the applied downlink signal, if necessary. For a specific test case, only some may be available, for example in a test case such as Radio Link Monitoring, it would not be possible to use applied Es only, because the test case relies on a well-defined SNR at the UE baseband.

Table A.4.1.3-1: Options to increase downlink dB range

| Option | Applicability | Comments |
| --- | --- | --- |
| Reduce number of allocated RBs | All Test cases | 120kHz SSB SCS, use 24RBs 240kHz SSB SCS, use 48RBs Allocated RBs must include CORESET |
| Use applied Es only, without applied Noc | Test cases where Es/IotBB requirement is to be ≥ defined value | Maximises Es contribution to Io. Not suitable for test cases where Es/IotBB is intended as well-defined SNR. |
| If applied Noc is used, reduce margin above UE internal noise | Test cases where Es/IotBB requirement is to be ≥ defined value | Not suitable for test cases where Es/IotBB is intended as well-defined SNR. |
| Use time-division multiplexing with SSB#0, SSB#1 | Test cases using SSB#0, SSB#1 | Avoids intra-frequency interference and degradation of Es/IotBB |
| Use Rx Beam Peak AoA instead of Spherical Coverage | Test cases where using Rx Beam Peak for one or both AoA, instead of Spherical Coverage, would provide adequate test coverage | Adequate test coverage may be achieveable across several Test cases |
| Allow Io > -50dBm | Test cases where measurement side conditions are not applicable | Test system may not be able to deliver >-50dBm |
| Restrict to 120kHz SSB SCS | Test cases with a 240kHz SSB SCS Configuration | Adequate test coverage of 120kHz and 240kHz SSB SCS may be achieveable across several Test cases |

Figure A.4.1.3-1: Example illustration of selected options to increase downlink dB range

# < Unchanged sections omitted >

# E.3	Uncertainty assessment for RRM MU quantities.

RRM measurement uncertainty analysis shall define the values for the following MU quantities:

-	DL AWGN absolute power or wanted DL signal absolute power

-	DL applied SNR

-	DL Fading profile uncertainty

-	DL AWGN and signal flatness

-	UL absolute power measurement

-	UL relative power measurement

-	UL signal transmit timing relative to DL

-	Relative transmit timing accuracy during UE timing adjustment

## E.3.1	Uncertainty assessment for DL AWGN absolute power or wanted DL signal absolute power

Table E.3.1-1 summarizes the MU threshold for DL AWGN absolute power for RRM FR2 test cases. The origin MU values for different test setups with varies parameters can be found in following subclauses.

Table E.3.1-1: MU threshold for DL AWGN absolute power for RRM FR2

| Power Class | Frequency | MBW | Power | Threshold MU value (NOTE 1) |
| --- | --- | --- | --- | --- |
| PC3 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | As configured in the test case | 5.65 dB2 |
|  | 32.125GHz < f <= 40.8GHz |  |  | 5.65 dB2 |
| PC1 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | As configured in the test case | FFS |
|  | 32.125GHz < f <= 40.8GHz |  |  | FFS |
| NOTE 1: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm in Table E.3.1.3-2 for PC3 UEs and Table FFS for PC1 UEs NOTE 2: If the TT analysis for a specific test case based on this MU value results in an unsolvable conflict, making the test case untestable, even after the alternative solutions listed in clause A.4 have been considered for the test case in TS 38.133 [6] Annex A, the TT analysis shall be repeated using a lower MU value, taking into account the lower values defined in this clause. The test case will be applicable for the subset of the test systems meeting this reduced MU Threshold. |  |  |  |  |

The types of test setup are defined in clause 7.1.3.2 of TS 38.508-1 [18]

### E.3.1.1	Uncertainty budget format and assessment for DFF test setup

The uncertainty contributions that may impact the overall MU value are listed in Table E.3.1.1-1.

Table E.3.1.1-1: Uncertainty contributions for DL AWGN absolute power or wanted DL signal absolute power

| UID |  | Description of uncertainty contribution |  | Details in annex |  |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 |  | Positioning misalignment |  | B.2.1.1 |  |
| 2 |  | Measure distance uncertainty |  | B.2.1.2 |  |
| 3 |  | Quality of Quiet Zone |  | B.2.1.3 |  |
| 4 |  | Mismatch |  | B.2.1.4 |  |
| 5 |  | Standing wave between the DUT and measurement antenna |  | B.2.1.5 |  |
| 6 |  | gNB emulator uncertainty |  | B.2.1.17 |  |
| 7 |  | Phase curvature |  | B.2.1.7 |  |
| 8 |  | Amplifier uncertainties |  | B.2.1.8 |  |
| 9 |  | Random uncertainty |  | B.2.1.9 |  |
| 10 |  | Influence of the XPD |  | B.2.1.10 |  |
| 11 |  | Insertion Loss Variation |  | B.2.1.11 |  |
| 12 |  | RF leakage (from measurement antenna to the receiver/transmitter) |  | B.2.1.12 |  |
| 13 |  | Multiple measurement antenna uncertainty |  | B.2.1.25 |  |
| 14 |  | DUT repositioning |  | B.2.1.26 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 |  | Mismatch |  | B.2.1.4 |  |
| 16 |  | Amplifier Uncertainties |  | B.2.1.8 |  |
| 17 |  | Misalignment of positioning System |  | B.2.1.13 |  |
| 18 |  | Uncertainty of the Network Analyzer |  | B.2.1.14 |  |
| 19 |  | Uncertainty of the absolute gain of the calibration antenna |  | B.2.1.15 |  |
| 20 |  | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  | B.2.1.16 |  |
| 21 |  | Phase centre offset of calibration antenna |  | B.2.1.18 |  |
| 22 |  | Quality of quiet zone for calibration process |  | B.2.1.19 |  |
| 23 |  | Standing wave between reference calibration antenna and measurement antenna |  | B.2.1.20 |  |
| 24 |  | Influence of the calibration antenna feed cable |  | B.2.1.21 |  |
| 25 |  | Insertion Loss Variation |  | B.2.1.11 |  |
| Systematic uncertainties |  |  |  |  |  |
| 26 |  | Systematic error related to beam peak search |  | B.2.1.28 |  |

The uncertainty assessment tables are organized as follows:

-	For the purpose of uncertainty assessment, the radiating antenna aperture of the DUT is denoted as D

-	The uncertainty assessment has been derived for the case of Quiet Zone size ≤ [30 cm], f = {23.45GHz, 32.125GHz, 40.8GHz}.

-	The uncertainty assessment is applicable for 1AoA and 2AoA test cases

-	The uncertainty assessment is provided in Table E.3.1.1-2.

Table E.3.1.1-2: Uncertainty assessment for DL AWGN absolute power or wanted DL signal absolute power (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm)

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.15 | Rectangular | 1.73 | 0.08 |
| 3 | Quality of Quiet Zone (NOTE 4) | [1.1] | Actual | 1.00 | [1.1] |
| 4 | Mismatch | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | 1.45 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | [0.03] | U-shaped | 1.41 | [0.02] |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Multiple measurement antenna uncertainty (NOTE 3) | 0.15 | Actual | 1.00 | 0.15 |
| 14 | DUT repositioning | 0.08 | Rectangular | 1.73 | 0.05 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 16 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 17 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 18 | Uncertainty of the Network Analyzer | 0.73 | Normal | 2.00 | 0.37 |
| 19 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 21 | Phase centre offset of calibration antenna | 0.47 | Rectangular | 1.73 | 0.27 |
| 22 | Quality of quiet zone for calibration process (NOTE 4) | [0.3] | Actual | 1.00 | [0.3] |
| 23 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 24 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 25 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Systematic uncertainties (NOTE 2) |  |  |  | Value |
| 26 | Systematic error related to beam peak search |  |  |  | 0.5 |
| Total measurement uncertainty |  |  |  |  | Value |
| DL AWGN absolute power expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [5.53] |
| NOTE 1: The analysis was done only for the case of operating in-band, non-CA. NOTE 2: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 3: Applies to the system which has a structure of mechanical feed antenna positioning. NOTE 4: Value based on procedure defined in Annex D.2 of TR 38.810 [13] for Quiet Zone size less or equal to 30 cm. NOTE 5: The values in this table have been derived for DL powers above and equal to REFSENS. The values might need to be revisited for power levels below REFSENS |  |  |  |  |  |

### E.3.1.2	Uncertainty budget format and assessment for Simplified DFF test setup

[FFS]

### E.3.1.3	Uncertainty budget format and assessment for IFF test setup

The uncertainty contributions that may impact the overall MU value are listed in Table E.3.1.3-1.

Table E.3.1.3-1: Uncertainty contributions for DL AWGN absolute power or wanted DL signal absolute power

| UID |  | Description of uncertainty contribution |  | Details in annex |  |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 |  | Positioning misalignment |  | B.2.2.1 |  |
| 2 |  | Measure distance uncertainty |  | B.2.2.2 |  |
| 3 |  | Quality of Quiet Zone |  | B.2.2.3 |  |
| 4 |  | Mismatch |  | B.2.2.4 |  |
| 5 |  | Standing wave between the DUT and measurement antenna |  | B.2.2.5 |  |
| 6 |  | gNB emulator uncertainty |  | B.2.2.17 |  |
| 7 |  | Phase curvature |  | B.2.2.7 |  |
| 8 |  | Amplifier uncertainties |  | B.2.2.8 |  |
| 9 |  | Random uncertainty |  | B.2.2.9 |  |
| 10 |  | Influence of the XPD |  | B.2.2.10 |  |
| 11 |  | Insertion Loss Variation |  | B.2.2.11 |  |
| 12 |  | RF leakage (from measurement antenna to the receiver/transmitter) |  | B.2.2.12 |  |
| 13 |  | Multiple measurement antenna uncertainty |  | B.2.2.25 |  |
| 14 |  | DUT repositioning |  | B.2.2.26 |  |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 |  | Mismatch |  | B.2.2.4 |  |
| 16 |  | Amplifier Uncertainties |  | B.2.2.8 |  |
| 17 |  | Misalignment of positioning System |  | B.2.2.13 |  |
| 18 |  | Uncertainty of the Network Analyzer |  | B.2.2.14 |  |
| 19 |  | Uncertainty of the absolute gain of the calibration antenna |  | B.2.2.15 |  |
| 20 |  | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  | B.2.2.16 |  |
| 21 |  | Phase centre offset of calibration antenna |  | B.2.2.18 |  |
| 22 |  | Quality of quiet zone for calibration process |  | B.2.2.19 |  |
| 23 |  | Standing wave between reference calibration antenna and measurement antenna |  | B.2.2.20 |  |
| 24 |  | Influence of the calibration antenna feed cable |  | B.2.2.21 |  |
| 25 |  | Insertion Loss Variation |  | B.2.2.11 |  |
| Systematic uncertainties |  |  |  |  |  |
| 26 |  | Systematic error related to beam peak search |  | B.2.2.28 |  |

The uncertainty assessment tables are organized as follows:

-	For the purpose of uncertainty assessment, the radiating antenna aperture of the DUT is denoted as D

-	The uncertainty assessment has been derived for the case of Quiet Zone size ≤ [30 cm], f = {23.45GHz, 32.125GHz, 40.8GHz}.

-	The uncertainty assessment is applicable for 1AoA test cases-	The uncertainty assessment is provided in Table E.3.1.3-2.

Table E.3.1.3-2: Uncertainty assessment for DL AWGN absolute power or wanted DL signal absolute power (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm)

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 4) | 0.6 | Actual | 1.00 | 0.6 |
| 4 | Mismatch | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | 1.45 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | 0.00 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Multiple measurement antenna uncertainty (NOTE 3) | 0.15 | Actual | 1.00 | 0.15 |
| 14 | DUT repositioning | 0.08 | Rectangular | 1.73 | 0.05 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 16 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 17 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 18 | Uncertainty of the Network Analyzer | 0.73 | Normal | 2.00 | 0.37 |
| 19 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 21 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 22 | Quality of quiet zone for calibration process (NOTE 4) | 0.4 | Actual | 1.00 | 0.4 |
| 23 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 24 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 25 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Systematic uncertainties (NOTE 2) |  |  |  | Value |
| 26 | Systematic error related to beam peak search |  |  |  | 0.5 |
| Total measurement uncertainty |  |  |  |  | Value |
| DL AWGN absolute power expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | 5.19 |
| NOTE 1: The analysis was done only for the case of operating in-band, non-CA. NOTE 2: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 3: Applies to the system which has a structure of mechanical feed antenna positioning. NOTE 4: Value based on procedure defined in Annex D.2 of TR 38.810 [13] for Quiet Zone size less or equal to 30 cm. NOTE 5: The values in this table have been derived for DL powers above and equal to REFSENS. The values might need to be revisited for power levels below REFSENS |  |  |  |  |  |

### E.3.1.4	Uncertainty budget format and assessment for Enhanced IFF test setup

The uncertainty contributions that may impact the overall MU value are listed in Table E.3.1.4-1.

Table E.3.1.4-1: Uncertainty contributions for DL AWGN absolute power or wanted DL signal absolute power

| UID |  | Description of uncertainty contribution |  | Details in annex |  |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 to 14 |  | See 1-14 of Table E.3.1.3-1 |  | N/A |  |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 15 to 25 |  | See 15-25 of Table E.3.1.3-1 |  | N/A |  |
| Systematic uncertainties |  |  |  |  |  |
| 26 |  | See 26 of Table E.3.1.3-1 |  | N/A |  |

The uncertainty assessment tables are organized as follows:

-	For the purpose of uncertainty assessment, the radiating antenna aperture of the DUT is denoted as D

-	The uncertainty assessment has been derived for the case of Quiet Zone size ≤ [30 cm], f = {23.45GHz, 32.125GHz, 40.8GHz}.

-	The uncertainty assessment is applicable for 1AoA and 2AoA test cases

-	The uncertainty assessment is provided in Table E.3.1.4-2.

Table E.3.1.4-2: Uncertainty assessment for fDL AWGN absolute power or wanted DL signal absolute power (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm)

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.00 | Normal | 2.00 | 0.00 |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | 0.00 |
| 3 | Quality of Quiet Zone (NOTE 4) | 0.7 | Actual | 1.00 | 0.7 |
| 4 | Mismatch | 1.30 | Actual | 1.00 | 1.30 |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 6 | gNB uncertainty on absolute level | 2.9 | Normal | 2.00 | 1.45 |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | 0.00 |
| 8 | Amplifier uncertainties | 2.1 | Normal | 2.00 | 1.05 |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | 0.25 |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | 0.00 |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | 0.00 |
| 13 | Multiple measurement antenna uncertainty (NOTE 3) | 0.15 | Actual | 1.00 | 0.15 |
| 14 | DUT repositioning | 0.08 | Rectangular | 1.73 | 0.05 |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 17 | Mismatch | 0.00 | U-shaped | 1.41 | 0.00 |
| 18 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | 0.00 |
| 19 | Misalignment of positioning System | 0.00 | Normal | 2.00 | 0.00 |
| 20 | Uncertainty of the Network Analyzer | 0.73 | Normal | 2.00 | 0.37 |
| 21 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | 0.30 |
| 22 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | 0.00 |
| 23 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | 0.00 |
| 24 | Quality of quiet zone for calibration process (NOTE 4) | 0.4 | Actual | 1.00 | 0.4 |
| 25 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 26 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | 0.07 |
| 27 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | 0.00 |
|  | Systematic uncertainties (NOTE 2) |  |  |  | Value |
| 28 | Systematic error related to beam peak search |  |  |  | 0.5 |
| Total measurement uncertainty |  |  |  |  | Value |
| DL AWGN absolute power expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | 5.25 |
| NOTE 1: The analysis was done only for the case of operating in-band, non-CA. NOTE 2: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 3: Applies to the system which has a structure of mechanical feed antenna positioning. NOTE 4: Value based on procedure defined in Annex M of TR 38.521-2 [7] for Quiet Zone size less or equal to 30 cm. NOTE 5: The values in this table have been derived for DL powers above and equal to REFSENS. The values might need to be revisited for power levels below REFSENS |  |  |  |  |  |

### E.3.1.5	Uncertainty budget format and assessment for IFF+DFF Hybrid test setup

[FFS]

## E.3.2	Uncertainty assessment for DL applied SNR

## E.3.3	Uncertainty assessment for DL Fading profile uncertainty

## E.3.4	Uncertainty assessment for DL AWGN and signal flatness

## E.3.5	Uncertainty assessment for UL absolute power measurement

## E.3.6	Uncertainty assessment for UL relative power measurement

## E.3.7	Uncertainty assessment for UL signal transmit timing relative to DL

## E.3.8	Uncertainty assessment for Relative transmit timing accuracy during UE timing adjustment

Annex F: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-09 | RAN5 #76 | R5-174706 |  |  |  | Initial skeleton | 0.0.1 |
| 2018-04 | RAN5 #2-5G-NR-Adhoc | R5-182093 |  |  |  | Implementation of pCRs to TS 38.903 V0.0.1 | 0.1.0 |
| 2018-05 | RAN5#79 | R5-182670 |  |  |  | Editorial update of TR 38.903. | 0.2.0 |
| 2018-09 | RAN5#80 | R5-185213 |  |  |  | Making Measurement Uncertainty Terms Common between methods in TR 38.90 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185214 |  |  |  | TP on Measurement Uncertainty Contributions in FR2 | 1.0.0 |
| 2018-09 | RAN5#80 | R5-185212 |  |  |  | Adding MU values for EIRPTRP measurements with Near Field test range (NFTF) at mmWave | 1.0.0 |
| 2018-09 | RAN#81 | - | - | - | - | raised to v15.0.0 with editorial changes only | 15.0.0 |
| 2018-12 | RAN#82 | R5-187023 | 0010 | - | F | Editorial update of Annex B | 15.1.0 |
| 2018-12 | RAN#82 | R5-187024 | 0011 | - | F | Addition of MU contribution for demodulation test cases | 15.1.0 |
| 2018-12 | RAN#82 | R5-187025 | 0012 | - | F | Addition of MU contribution for RRM test cases | 15.1.0 |
| 2018-12 | RAN#82 | R5-187148 | 0013 | - | F | General clauses updated for TR38.903 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187848 | 0008 | 1 | F | FR2 Spurious Emission measurement grids and offset values | 15.1.0 |
| 2018-12 | RAN#82 | R5-188060 | 0019 | 1 | F | Update of MU budget and contributor description to TR 38.903 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188224 | 0009 | 1 | F | Update MU budget in TR 38.903 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188225 | 0016 | 1 | F | Update of MU budget tables in TR 38.903 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188226 | 0017 | 2 | F | Addition of descriptions on new MU contributions | 15.1.0 |
| 2019-03 | RAN#83 | R5-192476 | 0030 | 1 | F | Addition of Test Tolerance analysis for FR1 PRACH Test cases | 15.2.0 |
| 2019-03 | RAN#83 | R5-192504 | 0038 | 1 | F | Addition of TT analysis for Transmit timing accuracy Tests | 15.2.0 |
| 2019-03 | RAN#83 | R5-192505 | 0031 | 1 | F | Addition common text for RRM | 15.2.0 |
| 2019-03 | RAN#83 | R5-192534 | 0039 | - | F | Addition of TT Analysis for Timing Advance Adjustment Accuracy 4.4.3.1 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192671 | 0033 | 1 | F | Addition of TT analysis for event triggered test cases | 15.2.0 |
| 2019-03 | RAN#83 | R5-192679 | 0036 | 1 | F | Addition of TT analysis for handover with known cell | 15.2.0 |
| 2019-03 | RAN#83 | R5-192845 | 0029 | 1 | F | CR to update TR 38.903 | 15.2.0 |
| 2019-06 | RAN#84 | R5-193799 | 0048 | - | F | FR1 Test tolerance analysis for intra re-selection 6.1.1.1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193800 | 0049 | - | F | FR1 Test tolerance analysis for inter re-selection 6.1.1.2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193801 | 0050 | - | F | FR1 Test tolerance analysis for interRAT higher priority re-selection 6.1.2.1 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193802 | 0051 | - | F | FR1 Test tolerance analysis for interRAT lower priority re-selection 6.1.2.2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-193803 | 0052 | - | F | FR1 Test tolerance analysis for interRAT known handover 6.3.1.4 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194027 | 0057 | - | F | CR on spurious emission MU in FR2 | 15.3.0 |
| 2019-06 | RAN#84 | R5-194123 | 0058 | - | F | Definition of MU terminologies in TR 38.903 | 15.3.0 |
| 2019-06 | RAN#84 | R5-195014 | 0054 | 1 | F | FR1 Test tolerance analysis for EN-DC SCell activation 4.5.3.1-4.5.3.3 | 15.3.0 |
| 2019-06 | RAN#84 | R5-195015 | 0060 | 1 | F | Test Tolerance analysis for Inter-Freq measurement Test Cases | 15.3.0 |
| 2019-06 | RAN#84 | R5-195159 | 0059 | 1 | F | CR to update TR 38.903 after RAN5#5-5GNR Adhoc | 15.3.0 |
| 2019-06 | RAN#84 | R5-195181 | 0055 | 1 | F | FR1 Test tolerance analysis for EN-DC measurement reporting 4.6.1.1-4.6.1.4 | 15.3.0 |
| 2019-06 | RAN#84 | - | - | - | - | Administrative release upgrade to match the release of 3GPP TS 38.521-1 which was upgraded at RAN#84 to Rel-16 due to Rel-16 relevant CR(s) | 16.0.0 |
| 2019-09 | RAN#85 | R5-195583 | 0061 | - | F | Update FR1 Test tolerance of 4.5.3.1-4.5.3.3 Scell activation | 16.1.0 |
| 2019-09 | RAN#85 | R5-195584 | 0062 | - | F | Update FR1 Test tolerance of 6.1.1.1 FR1 cell re-selection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195585 | 0063 | - | F | Update FR1 Test tolerance of 6.1.1.2 FR1-FR1 cell re-selection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195586 | 0064 | - | F | Update FR1 Test tolerance of 6.1.2.1 inter-RAT cell re-selection to higher priority | 16.1.0 |
| 2019-09 | RAN#85 | R5-195587 | 0065 | - | F | Update FR1 Test tolerance of 6.1.2.2 inter-RAT cell re-selection to lower priority | 16.1.0 |
| 2019-09 | RAN#85 | R5-195588 | 0066 | - | F | Update FR1 Test tolerance of 6.3.1.4 inter-RAT handover to known cell | 16.1.0 |
| 2019-09 | RAN#85 | R5-195589 | 0067 | - | F | Addition FR1 Test tolerance of 6.3.1.5 inter-RAT handover to unknown cell | 16.1.0 |
| 2019-09 | RAN#85 | R5-195590 | 0068 | - | F | Addition FR1 Test tolerance of 6.3.2.1.1 intra-freq RRC re-establishment | 16.1.0 |
| 2019-09 | RAN#85 | R5-195591 | 0069 | - | F | Addition FR1 Test tolerance of 6.3.2.1.2 inter-freq RRC re-establishment | 16.1.0 |
| 2019-09 | RAN#85 | R5-195592 | 0070 | - | F | Addition FR1 Test tolerance of 6.3.2.3.1 NR RRC redirection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195593 | 0071 | - | F | Addition FR1 Test tolerance of 6.3.2.3.2 inter-RAT RRC redirection | 16.1.0 |
| 2019-09 | RAN#85 | R5-197362 | 0072 | 1 | F | FR1 Test Tolerance Analysis for SSB-based RLM IS Tests | 16.1.0 |
| 2019-09 | RAN#85 | R5-197363 | 0073 | 1 | F | FR1 Test Tolerance Analysis for SA Tx Timing Accuracy 6.4.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197365 | 0083 | 1 | F | TT_Analysis_ENDC_FR1_RLM_OOS | 16.1.0 |
| 2019-09 | RAN#85 | R5-197369 | 0087 | 1 | F | TT_Analysis_SA_FR1_TAAA | 16.1.0 |
| 2019-09 | RAN#85 | R5-197494 | 0077 | 1 | F | CR on DUT turnover and relations with QoQZ MU | 16.1.0 |
| 2019-09 | RAN#85 | R5-197505 | 0081 | 1 | F | Update of FR2 MUs in TR 38.903 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197571 | 0078 | 1 | F | TT Analysis for SS-RSRP FR1 tests | 16.1.0 |
| 2019-09 | RAN#85 | R5-197625 | 0080 | 1 | F | CR on FR2 OFF Power MU | 16.1.0 |
| 2019-09 | RAN#85 | R5-197659 | 0076 | 2 | F | CR on spurious emission MU in FR2 | 16.1.0 |
| 2019-12 | RAN#86 | R5-198260 | 0099 | - | F | CR to 38.903 to define Reference Methodology for SE | 16.2.0 |
| 2019-12 | RAN#86 | R5-198285 | 0100 | - | F | FR1 Test tolerance analysis for interRAT measurement | 16.2.0 |
| 2019-12 | RAN#86 | R5-198427 | 0101 | - | F | Correction to uncertainty budget calculation principles | 16.2.0 |
| 2019-12 | RAN#86 | R5-199070 | 0098 | 1 | F | Editorial corrections to FR1 Test Tolerance files | 16.2.0 |
| 2019-12 | RAN#86 | R5-199082 | 0104 | 2 | F | FR1 Test Tolerance : Addition of TT Analysis for 6.3.1.1 NR SA FR1 Intra-Freq Handover | 16.2.0 |
| 2019-12 | RAN#86 | R5-199083 | 0105 | 2 | F | FR1 Test Tolerance : Addition of TT Analysis for 6.3.1.2 NR SA FR1 Intra-Freq Handover | 16.2.0 |
| 2019-12 | RAN#86 | R5-199084 | 0106 | 2 | F | FR1 Test Tolerance : Addition of TT Analysis for 6.3.1.3 NR SA FR1 Inter-Freq Handover | 16.2.0 |
| 2019-12 | RAN#86 | R5-199091 | 0102 | 1 | F | Update on FR2 MUs in 38.903 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199092 | 0103 | 1 | F | Update on FR2 Spurious MUs in 38.903 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199362 | 0091 | 1 | F | FR1 Test tolerance analysis for interruptions active and non-active | 16.2.0 |
| 2019-12 | RAN#86 | R5-199363 | 0094 | 1 | F | FR1 Test tolerance analysis for CSI-RS based RLM | 16.2.0 |
| 2020-03 | RAN#87 | R5-200163 | 0107 | - | F | Add Annex A.2 handling of common Test Tolerance Topics for FR2 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200329 | 0112 | - | F | CR to 38.903 on XPD Verification | 16.3.0 |
| 2020-03 | RAN#87 | R5-200470 | 0114 | - | F | FR1 Test tolerance analysis for interruptions deactivated NR SCC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200918 | 0116 | 1 | F | Update to FR2 TRx Measurement Uncertainties | 16.3.0 |
| 2020-03 | RAN#87 | R5-201037 | 0108 | 1 | F | Test tolerance analysis inter-frequency SS-RSRP and intra-frequency SS-SINR | 16.3.0 |
| 2020-03 | RAN#87 | R5-201038 | 0109 | 1 | F | Test tolerance analysis SS-RSRQ and inter-frequency SS-SINR | 16.3.0 |
| 2020-03 | RAN#87 | R5-201042 | 0110 | 1 | F | Test Tolerance analysis for CSI-RS-Based L1-RSRP measurement test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-201043 | 0111 | 1 | F | Test Tolerance analysis for SSB-Based L1-RSRP measurement test cases | 16.3.0 |
| 2020-06 | RAN#88 | R5-201662 | 0118 | - | F | FR1 Test tolerance analysis for interruptions deactivated E-UTRAN SCC | 16.4.0 |
| 2020-06 | RAN#88 | R5-203094 | 0119 | 1 | F | FR1 Test tolerance analysis for SCell activation | 16.4.0 |
| 2020-06 | RAN#88 | R5-202104 | 0124 | - | F | Test tolerance correction for event triggered measurement test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202105 | 0125 | - | F | Test tolerance correction for CSI-RS-based L1-RSRP measurement test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202702 | 0130 | 1 | F | Test Tolerance analysis TC 4.5.4 and 6.5.4 RRC reconfiguration delay | 16.4.0 |
| 2020-06 | RAN#88 | R5-202769 | 0127 | 1 | F | CR to 38.903 to introduce baseline Demod MU tables | 16.4.0 |
| 2020-06 | RAN#88 | R5-202915 | 0117 | 1 | F | MU contributors for RRM FR2 TC 7.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202916 | 0126 | 1 | F | CR to 38.903 to introduce PC1 MU Tables | 16.4.0 |
| 2020-06 | RAN#88 | R5-202917 | 0128 | 1 | F | Update to FR2 Measurement Uncertainties | 16.4.0 |
| 2020-06 | RAN#88 | R5-202938 | 0129 | 1 | F | Addition of EIRP to Transmit OFF power MU analysis | 16.4.0 |
| 2020-09 | RAN#89 | R5-203231 | 0131 | - | F | TT analysis for RRM TC 8.5.2.1.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203232 | 0132 | - | F | TT analysis for RRM TC 8.5.2.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203233 | 0133 | - | F | TT analysis for RRM TC 8.5.2.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203237 | 0134 | - | F | TT analysis for RRM TC 4.7.4.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203238 | 0135 | - | F | TT analysis for RRM TC 4.7.4.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203323 | 0140 | - | F | Add Draft Test Tolerance analysis for FR2 Tx Timing Test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203324 | 0141 | - | F | Add Draft Test Tolerance analysis for FR2 Inter-freq Event-trig Test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203325 | 0142 | - | F | Add Draft Test Tolerance analysis for FR2 Intra-freq SS-RSRP Test case | 16.5.0 |
| 2020-09 | RAN#89 | R5-203825 | 0149 | - | F | Addition of FR1 Test tolerance analysis for 6.3.2.1.3 RRC Re-establishment | 16.5.0 |
| 2020-09 | RAN#89 | R5-203826 | 0150 | - | F | Update of grouping of test cases in clause 8 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204190 | 0154 | - | F | On Standard Deviation Definition in 38.903 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204788 | 0138 | 1 | F | Correction to the extreme conditions in TT analysis of 4.7.1.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204887 | 0139 | 1 | F | CR to update the DL AWGN absolute power for RRM test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204888 | 0143 | 1 | F | Adjacent Channel Selectivity FR2 MU definition in 38.903 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204889 | 0144 | 1 | F | In-band Blocking FR2 MU definition in 38.903 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204890 | 0153 | 1 | F | CR to update MU in 38.903 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204891 | 0155 | 1 | F | FR2 Minimum output power measurement uncertainty | 16.5.0 |
| 2020-09 | RAN#89 | R5-204945 | 0152 | 1 | F | CR to 38.903 on some of the Transmit OFF power MU parameters | 16.5.0 |
| 2020-09 | RAN#89 | R5-204946 | 0156 | 1 | F | Update of AWGN flatness in TR 38.903 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204947 | 0157 | 1 | F | FR2 EIRP OFF power measurement uncertainty | 16.5.0 |
| 2020-09 | RAN#89 | R5-205001 | 0145 | 1 | F | Addition of FR1 Test tolerance analysis for DCI based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-205002 | 0146 | 1 | F | Addition of FR1 Test tolerance analysis for RRC based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-205003 | 0147 | 1 | F | Addition of FR1 Test tolerance analysis for SSB based BFR | 16.5.0 |
| 2020-09 | RAN#89 | R5-205004 | 0148 | 1 | F | Addition of FR1 Test tolerance analysis for CSI-RS based BFR | 16.5.0 |
| 2020-12 | RAN#90 | R5-205628 | 0174 | - | F | RRM FR2 DL AWGN absolute power MU | 16.6.0 |
| 2020-12 | RAN#90 | R5-205704 | 0176 | - | F | Update of demod MU | 16.6.0 |
| 2020-12 | RAN#90 | R5-205831 | 0177 | - | F | Editorial correction of clause 5.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205949 | 0183 | - | F | Update of grouping of test cases in clause 8 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206809 | 0158 | 1 | F | TT analysis for RRM 6.7.5.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206810 | 0159 | 1 | F | TT analysis for RRM 6.7.6.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206811 | 0160 | 1 | F | TT analysis for RRM 6.7.7.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206812 | 0161 | 1 | F | TT analysis for RRM 4.7.5.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206813 | 0163 | 1 | F | TT analysis for RRM 8.5.2.1.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206814 | 0166 | 1 | F | Add Draft Test Tolerance analysis for FR2 PRACH Test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206815 | 0178 | 1 | F | Addition of FR1 TT analysis for inter-RAT cell reselection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206816 | 0179 | 1 | F | Addition of FR1 TT analysis for inter-RAT handover | 16.6.0 |
| 2020-12 | RAN#90 | R5-206817 | 0180 | 1 | F | Addition of FR1 TT analysis for inter-RAT SFTD measurement | 16.6.0 |
| 2020-12 | RAN#90 | R5-206818 | 0181 | 1 | F | Addition of FR1 TT analysis for inter-RAT event-triggered reporting | 16.6.0 |
| 2020-12 | RAN#90 | R5-206835 | 0184 | 1 | F | CR to 38.903 on ETC Testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206836 | 0185 | 1 | F | CR to add DFF MU Tables in 38.903 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206837 | 0187 | 1 | F | Update FR2 TRx MU in 38.903 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206838 | 0189 | 1 | F | FR2 Time masks updates | 16.6.0 |
| 2020-12 | RAN#90 | R5-206845 | 0164 | 1 | F | TT analysis for RRM 8.5.2.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206846 | 0167 | 1 | F | Update Draft Test Tolerance analysis for FR2 Tx Timing Test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206847 | 0169 | 1 | F | Add Draft Test Tolerance analysis FR2 RLM Peak Test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206848 | 0170 | 1 | F | Add Draft Test Tolerance analysis for FR2 Intra-freq Event-trig Test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206849 | 0171 | 1 | F | Update Draft Test Tolerance analysis for FR2 Inter-freq Event-trig Test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206850 | 0172 | 1 | F | Update Draft Test Tolerance analysis for FR2 Intra-freq SS-RSRP Test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206851 | 0173 | 1 | F | Add Draft Test Tolerance analysis for FR2 Inter-freq SS-RSRP Test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206852 | 0182 | 1 | F | Update of FR1 TT analysis for 6.1.1.1 intra-freq cell re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206911 | 0165 | 1 | F | TT analysis for RRM 8.5.2.3.2 | 16.6.0 |

< End of changes >