3GPP TSG-RAN5 Meeting #90-e	R5-210463r1

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1044 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Serving Cell Config for RRM timing test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRM Timing Accuracy tests need modify startSymbolAndLength for PUSCH. A new table PUSCH-TimeDomainResourceAllocationList is added to Annex H |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new clause H.3.9 and a new table for PUSCH-TimeDomainResourceAllocationList |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM timing accuracy test cases will not use the correct message contens |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: revise to remove table ServingCellConfig and add table PUSCH-TimeDomainResourceAllocationList |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

## H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay

ServingCellConfigCommonSIB: information elements content exception for RRC reconfiguration delay test cases to specific both NR uplink and supplementary uplink for SCell.

Table H.3.8-1: ServingCellConfigCommonSIB-Procedure Delay

| Derivation Path: TS 38.508-1, table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | NR uplink |  |
|  | UplinkConfigCommonSIB with condition SUL_NUL |  | SUL |
| supplementaryUplink | UplinkConfigCommonSIB with condition SUL_SUL |  | SUL |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’0100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid; |  |
| groupPresence | Not present |  |  |
| } |  |  |  |
| ssb-PeriodicityServingCell | ms20 |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | FR1_TDD |
| ss-PBCH-BlockPower | 0 |  |  |
| } |  |  |  |

RRCReconfiguration: information elements contents exception for RRC reconfiguration delay test cases to specific radio bearers and cell groups.

Table H.3.8-2: RRCReconfiguration-Procedure Delay

| Derivation Path: TS 38.508-1, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |
| } |  |  |  |
| secondaryCellGroup | CellGroupConfig with condition EN-DC_SCell_add | OCTET STRING (CONTAINING CellGroupConfig) |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with conditions EN-DC | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.9	RRC messages and information elements contents exceptions for UL timing

Table H.3.9-1: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1, Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| startSymbolAndLength | 41 | Start symbol(S)=0, Length(L)=13 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| startSymbolAndLength | 41 | Start symbol(S)=0, Length(L)=13 |  |
| } |  |  |  |
| } |  |  |  |

< End of changes >