3GPP TSG-RAN5 Meeting #90-e	R5-210504r1

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0868 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test configuration tables in section 7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are misalignments, errors or missing information in the test configuration tables in section 7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test Environment ‘NC’ was unified to ‘Normal’. - Wrong table number was corrected. - Test Frequency ‘High’ was unified to ‘High range’. - ‘[ ]’ were removed for completed test cases. - Missing note for NRB_agg was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The misalignments, errors and missing information will remain in the test configuration tables in section 7. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.1.4.1, 7.3B.2.2.4.1, 7.3B.2.3.4.2.1, 7.3B.2.3_1.1.4.1, 7.3B.2.3_1.2.4.1, 7.3B.2.3_1.3.4.1, 7.4B.1.4.1, 7.6B.2.2.4, 7.6B.3.2.4, 7.6B.4.2.4, 7.8B.2.2.4, 7.9B.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: The revision in Table 7.8B.2.6.4.1-1 was canceled to avoid overlapping with R5-211000. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.3B.2.1.4	Test description

7.3B.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations for NR consist of environmental conditions test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of TS 38.521-1 [8]. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2, and Table 7.3.2.4.1-3 of TS 38.521-1 [8]. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2 of TS 38.521-1 [8]. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 38.521-1 [8].

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.4.2.1-1 of TS 36.521-1 [10]. All of these configurations shall be tested with applicable test parameters for each channel bandwidth are shown in tables 7.3.4.1-1 and 7.3.4.1-2 of TS 36.521-1 [10]. The details of the downlink and uplink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3 of TS 36.521-1 [10]. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 36.521-1 [10].

The channel bandwidths for E-UTRA and NR component carriers shall follow the values specified in Table 5.3B.1.2-1 for a given EN-DC combination.

Table 7.3B.2.1.4.1-1: Test configurations table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [5] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508-1 [11] clause 4.3.1 |  |  |  |  | Low, mid and high range |  |  |  |  |  |  |  |  |
| E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  | Specified below |  |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [5] clause 4.3.1 |  |  |  |  | Specified below |  |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  |  | Uplink Configuration |  |  |  |  |  |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |  |  | NR Modulation |  | NR RB allocation |  | E-UTRA Modulation |  | E-UTRA RB allocation |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB |  |  | DFT-s-OFDM QPSK |  | Specified in Table 7.3B.2.1.4.1-2 |  | QPSK |  | Specified in Table 7.3B.2.1.4.1-2 |
| Test Point configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_(n)71AA dual UL UE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | E-UTRA/NR band | FC (UL) (MHz) NUL |  | UL Channel BW (MHz) |  |  | UL allocation (LCRB) |  | FC (DL) (MHz) NDL |  | Duplex mode |  |  |  |
| 1 | 71 | 665.5MHz, EUL= 133147 |  | 5 |  |  | 5@19 |  | 619.5 MHz EDL = 68611 |  | FDD |  |  |  |
|  | n71 | 675.5 NUL= 135100 |  | 15 |  |  | 15@0 |  | 629.5 NDL = 125900 |  |  |  |  |  |
| 2 | 71 | 670.5 EUL= 133197 |  | 15 |  |  | 15@59 |  | 624.5 EDL = 68661 |  |  |  |  |  |
|  | n71 | 680.5 NUL= 136100 |  | 5 |  |  | 5@0 |  | 634.5 NDL = 126900 |  |  |  |  |  |
| 3 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@39 |  | 622 EDL = 68636 |  |  |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 10 |  |  | 10@0 |  | 632 NDL = 126400 |  |  |  |  |  |
| 4 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@0 |  | 622 EDL = 68636 |  |  |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 10 |  |  | 10@41 |  | 632 NDL = 126400 |  |  |  |  |  |
| 5 | 71 | 665.5 EUL= 133147 |  | 5 |  |  | 5@19 |  | 619.5 EDL = 68611 |  | FDD |  |  |  |
|  | n71 | 675.5 NUL= 135100 |  | 153 |  |  | 15@0 |  | 6323 NUL= 126400 |  |  |  |  |  |
| 6 | 71 | 670.5 EUL= 133197 |  | 15 |  |  | 15@59 |  | 624.5 EDL = 68661 |  |  |  |  |  |
|  | n71 | 680.5 NUL= 136100 |  | 53 |  |  | 5@0 |  | 6373 NUL= 127400 |  |  |  |  |  |
| 7 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@39 |  | 622 EDL = 68636 |  |  |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 103 |  |  | 10@0 |  | 634.53 NUL= 126900 |  |  |  |  |  |
| 8 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@0 |  | 622 EDL = 68636 |  |  |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 103 |  |  | 10@41 |  | 634.53 NUL= 126900 |  |  |  |  |  |
| Test Settings for DC_(n)71AA for single UL UE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 71 | default |  | Highest |  |  | 0 |  | default |  | FDD |  |  |  |
|  | n71 | default |  | Highest |  |  | REFSENS_NR |  | default |  |  |  |  |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 4: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.1.4.1-2: Test configurations table for EN-DC configurations not affected by Reference sensitivity exceptions, EN-DC configuration other than DC_(n)71AA

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
|  | NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Low range, High range |  |  |  |
|  | Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
|  | Network signalling value |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_(n)XAA Configuration (Intra-band contiguous EN-DC) – Note 2 |  |  |  |  |  |  |  |  |
| 1 (Note 4) | X | default |  |  | nX | default |  |  |
|  | QPSK | QPSK | Highest NRB | All RBs / REFSENS_LTE | CP-OFDM QPSK | QPSK /CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| 2 (Note 5) | X | default |  |  | nX | default |  |  |
|  | N/A | QPSK | Highest NRB | All RBs / 0 | CP-OFDM QPSK | QPSK /CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| Note 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. Note 2: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.3B.2.2.4	Test Description

7.3B.2.2.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations for E-UTRA consist of the test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1, with the exception that the E-UTRA channel bandwidth is the lowest supported value in Table 5.3B.1.3-1 for the EN-DC non-contiguous configuration under test.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2B.2.1-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing are shown in Table 7.3B.2.2.4.1-1 for NR band. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [8] Annex C2.

Table 7.3B.2.2.4.1-1: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [5] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508-1 [11] clause 4.3.1 |  |  |  |  |  | Low, mid and high range |  |  |  |  |  |  |
| E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | Specified below |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [5] clause 4.3.1 |  |  |  |  |  | Specified below |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  |  |  | Uplink Configuration |  |  |  |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation |  | E-UTRA RB allocation |  | NR Modulation |  | NR RB allocation |  | E-UTRA Modulation | E-UTRA RB allocation |  |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK |  | Full RB |  | DFT-s-OFDM QPSK |  |  |  | QPSK |  |  |
| Test Point configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_3A_n3A dual UL UE |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | E-UTRA/NR band |  | FC (UL) (MHz) NUL |  | UL Channel BW (MHz) |  | UL allocation (LCRB) |  | FC (DL) (MHz) NDL |  | Duplex mode |  |
| 1 | 3 |  | 1782.5, NUL= TBD |  | 5 |  | 12@0 |  | 1877.5 NDL = TBD |  | FDD |  |
|  | n3 |  | 1772.5 NUL= TBD |  | 5 |  | 12@13 |  | 1867.5 NDL = TBD |  |  |  |
| 2 | 3 |  | 1782.5, NUL= TBD |  | 5 |  | 12@9 |  | 1877.5 NDL = TBD |  |  |  |
|  | n3 |  | 1752.5 NUL= TBD |  | 5 |  | 12@0 |  | 1847.5 NDL = TBD |  |  |  |
| 3 | 3 |  | 1782.5 NUL= TBD |  | 5 |  | 12@12 |  | 1877.5 NDL = TBD |  |  |  |
|  | n3 |  | 1737.5 NUL= TBD |  | 5 |  | 12@0 |  | 1832.5 NDL = TBD |  |  |  |
| 4 | 3 |  | 1737.5 NUL= TBD |  | 5 |  | 12@0 |  | 1832.5 NDL = TBD |  |  |  |
|  | n3 |  | 1782.5 NUL= TBD |  | 5 |  | 12@12 |  | 1877.5 NDL = TBD |  |  |  |
| Test Settings for DC_3A_n3A for single UL UE |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | default |  | Highest |  | 0 |  | default |  |  | FDD |
|  | n3 |  | default |  | Highest |  | REFSENS_NR |  | default |  |  |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE2: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.2.4.1-2: Test configurations table for EN-DC configurations not affected by Reference sensitivity exceptions, EN-DC configuration other than DC_3A_n3A

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
|  | NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | TBD |  |  |  |
|  | Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
|  | Network signalling value |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nXA Configuration (Intra-band non-contiguous EN-DC) – Note 2 |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nX | default |  |  |
|  | N/A | N/A | 5 MHz | N/A | CP-OFDM QPSK | QPSK /CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| Note 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. Note 2: LTE anchor agnostic configuration Note 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations for E-UTRA consist of the test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1, with the exception that the E-UTRA channel bandwidth is the lowest supported value in Table 5.3B.1.3-1 for the EN-DC non-contiguous configuration under test.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2B.2.1-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing are shown in Tables 7.3B.2.3.4.2.1-0 in anchor agnostic mode and Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-5 for exceptional test cases in anchor specific mode. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [8] Annex C2.

In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3.4.2.1-0: Test configurations table for EN-DC configuration without any reference sensitivity exception

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
|  | NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Mid range |  |  |  |
|  | Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
|  | Network signalling value |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nYA Configuration (Inter-band EN-DC) – Note 2 |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | N/A | CP-OFDM QPSK | QPSK /CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| Note 1: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. Note 2: LTE anchor agnostic configuration |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-1: Initial test conditions for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 38.508-1 [11] clause4.3.1 |  |  |  | Mid range for E-UTRA and NR, unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28 |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Highest NRB_agg, unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28 |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest supported SCS otherwise specified |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| E-UTRA Cell |  | NR Cell |  | E-UTRA Cell |  | NR Cell |  |
| Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation |
| QPSK | Full RB | CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |

<< Unchanged contents omitted >>

Table 7.3B.2.3.4.2.1-5: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to "Range" columns. |  |  |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | DC Configuration / NRB_agg |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |
|  | DC Configuration |  |  |  |  |  | E-UTRA Ch BW |  | NR Ch BW |  | CC MOD E-UTRA/NR | E-UTRA & NR RB allocation |  | CC MOD E-UTRA/NR |  | E-UTRA & NR allocations (LCRB @ RBstart) |  |  |  |
|  | E-UTRA |  |  |  | NR |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |  |
|  | Band |  | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  | Mid |  | Y | Mid | 5 |  | Mid |  | QPSK /CP-OFDM QPSK | All RBs |  | QPSK/ DFT-s-OFDM QPSK |  | REFSENS |  |  | REFSENS |
| Test Settings for DC_1A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Note 5 |  | 3 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_1A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_1A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_2A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | Note 5 |  | 66 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_2A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_3A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 5 | Note 5 | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_3A-n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 7 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_3A-n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 20 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_3A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 77/ 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_5A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | Note 5 |  | 66 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_5A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| Test Settings for DC_7A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Low |  | 5 | High | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_7A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Note 5 |  | 66 | Note 5 | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_7A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Low |  | 77 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A-n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 1 | High | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_8A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 3 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 41 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A-n77A/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| 2 | 8 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| Test Settings for DC_8A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Note 5 |  | 79 | Note 5 | 5 |  | 40 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 216@0 |
| Test Settings for DC_12A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | All RBs |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_20A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 3 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_20A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 8 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_20A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 2 | 20 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_21A- n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Note 5 |  | 79 |  | Note 5 | 5 | 40 |  | Note 7 |  | All RBs |  |  | Note 7 |  | 25@0 | 216@0 |  |
| Test Settings for DC_26- n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 |  | Note 5 |  | 41 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_26A- n77/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 1 | 26 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_48A- n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 48 |  | Note 5 |  | 66 | Note 5 | 20 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 100@0 |  |  | 25@0 |
| Test Settings for DC_28A- n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 41 | Note 5 | 10 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 50@0 |
| Test Settings for DC_28A- n50A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 50 | Note 5 | 10 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 50@0 |
| Test Settings for DC_28A- n51A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 51 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_28A- n77/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 2 | 28 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_66A-n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 2 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A-n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 71 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for CA_7A-20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Note 5 |  | 20 | Note 5 | 10 |  | 5 |  | QPSK | All RBs |  | QPSK |  | 50@0 |  |  | 25@0 |
| Note 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. Note 2: Use DC Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the DC Configuration. E.g. for DC_1A-n3A, X=1, Y=3. Note 4: REFSENS refers to the E_UTRA bands and NR band NRB 's single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2 of TS 36.521-1 [10] and Table 7.3.2.4.1-3 of TS 38.521-1 [8], respectively Note 5: Test frequency for each DC configuration shall follow Table 7.3B.2.0.3.5.1-1. If test configurations of each ID in a DC configuration are same, test frequency shall follow the order of Table 7.3B.2.0.3.5.1-1. Note 6: Not applicable if the UE only supports Bandwidth Combination Set 1. Note 7: Same as default. Note 8: RBSTART = 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  | NRB |  |  | Band |  |  |  | Range |  | NRB |  |  |
|  | UL MOD | DL MOD |  | CH BW |  | DLalloc / UL alloc | UL MOD |  |  |  | DL MOD |  | UL/DL Ch BW | DLalloc / UL alloc |  |
| Test Settings for a DC_1A_n78 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  | FFS |  |  |  |  |  |  |  |  |
| 24 | 1 | DL 2140 |  |  |  |  | n78 |  |  |  | 3710 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3_n1 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 3 | Mid |  |  |  |  | n1 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_NR |  |
| 26 | 3 | Mid |  |  |  |  | n1 |  |  |  | Mid |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |
| 34 | 3 | DL 1855 |  |  |  |  | n1 |  |  |  | 2140 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n41 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 | Low |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | QPSK |  | 20 MHz | All RBs / 0 |  |
| 26 | 3 | Mid |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | QPSK |  | 20 MHz | All RBs / 0 |  |
| 36 | 3 | High |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | QPSK |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| 46 | 3 | High |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | QPSK |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| 54 | 3 | DL 1835 |  |  |  |  | n41 |  |  |  | 2657.5 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | QP |  |  |  | QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n78 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3495 |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / 0 |  |
| 23 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3525 |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 20 MHz | All RBs / 0 |  |
| 35 | 3 | High |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | CP-OFDM QPSK |  |  |  | QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |
| 44 | 3 | DL 1835 |  |  |  |  | n78 |  |  |  | 3575 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 54 | 3 | DL 1860 |  |  |  |  | n78 |  |  |  | 3435 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 20 | Mid |  |  |  |  | n3 |  |  |  | High |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / 0 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_NR |  |
| 24 | 20 | DL 799 |  |  |  |  | n3 |  |  |  | 1870 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 34 | 20 | DL 806 |  |  |  |  | n3 |  |  |  | 1830 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  | FFS |  |  |  |  |  |  |  |  |
| 23 | 20 | Mid |  |  |  |  | n78 |  |  |  | 3388 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / 0 |  |
| 34 | 20 | DL 809 MHz |  |  |  |  | n78 |  |  |  | 3359 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_40_n78 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 | Mid |  |  |  |  |  | n78 |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz |  | All RBs / REFSENS_ENDC_2 |  |  | N/A |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation according to table 7.3B.2.0.3.5.1-1. NOTE 2: Test ID with no exception NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Test ID for UE supporting single UL. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.3B.2.3_1.1.4	Test description

7.3B.2.3_1.1.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.1.4.1-1 and 7.3B.2.3_1.1.4.1-2.

# Table 7.3B.2.3_1.1.4.1-0: Test Configuration Table for EN-DC configurations not affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  | DC_XA_nYA_nZA and DC_XA-YA_nZA: Mid range for MCG and SCG DC_XA_nYC: Mid range for XA Low range, High Range for nXC DC_(n)XCA: Low range, High Range for nYC For non-contiguous carriers: TBD |  |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  | CG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range |  |  | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc/UL alloc | UL MOD | DL MOD |  |  | UL/DL Ch BW | DLalloc/ ULalloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc/UL alloc |
| Test Settings for a DC_(n)XCA Configuration (Intra-band contiguous EN-DC with LTE CA)– Note 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | X | default |  |  |  |  | nX | default |  |  |
|  | QPSK | REFSENS | Highest | All RBs / REFSENS | N/A | QPSK |  |  | Highest | All RBs / 0 | CP-OFDM QPSK | QPSK | Highest | All RBs / REFSENS |
| Test Settings for a DC_XC_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2) |  |  |  |  |  |
| Test Settings for a DC_XA-XA_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2) |  |  |  |  |  |
| Test Settings for a DC_XA_nYC Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | QPSK /CP-OFDM QPSK |  |  | Highest | All RBs / 0 | CP-OFDM QPSK | QPSK | Highest | All RBs / REFSENS |
| Test Settings for a DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3) |  |  |  |  |  |
| Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3) |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3) |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA_nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid |  |  | nY | default |  |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | QPSK /CP-OFDM QPSK |  |  | Highest | All RBs / 0 | CP-OFDM QPSK | QPSK | Highest | All RBs / REFSENS |
| 2 | X | Mid |  |  | nY | default |  |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | CP-OFDM QPSK | QPSK /CP-OFDM QPSK |  |  | Highest | All RBs / REFSENS | N/A | QPSK | Highest | All RBs / 0 |
| Note 1: LTE anchor agnostic configuration Note 2: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 3: LTE anchor agnostic configuration with LTE CA where LTE CA fallback to 1CC is sufficient to test Note 4: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Table 7.3B.2.3_1.1.4.1-1: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  | EN-DC configurations containing the following band combinations: 20-n78: Mid in band 20 and Mid in band 78. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB" and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  |  |  |  |  | CG -NR |  |  |  |  |  |  |
|  | Band |  |  | Range | NRB |  |  |  |  | Band | Range |  |  |  | NRB |  |  |  |  | Band |  | Range | NRB |  |  |  |
|  | UL MOD |  |  | DL MOD | CH BW |  | DLalloc/UL alloc |  |  | UL MOD | DL MOD |  |  |  | UL/DL Ch BW |  |  | DLalloc/ ULalloc |  | UL MOD |  | DL MOD | UL/DL Ch BW |  | DLalloc/UL alloc |  |
| Default Test Settings for a DC_(n)XAC Configuration (Intra-band contiguous EN-DC with NR CA) – Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_(n)XCA Configuration (Intra-band contiguous EN-DC with LTE CA)– Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-XA_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYC Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  |  | default |  |  |  |  |  | nY | default |  |  |  |  |  |  |  |  | nY |  | default |  |  |  |  |
|  | QPSK |  |  | REFSENS | Highest |  | All RBs / REFSENS |  |  | N/A | CP-OFDM QPSK |  |  |  | Highest |  |  | All RBs / 0 |  | N/A |  | CP-OFDM QPSK | Highest |  | All RBs / 0 |  |
| 2 | X |  |  | default |  |  |  |  |  | nY | default |  |  |  |  |  |  |  |  | nY |  | default |  |  |  |  |
|  | N/A |  |  | REFSENS | Highest |  | All RBs / 0 |  |  | N/A | CP-OFDM QPSK |  |  |  | Highest |  |  | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK | Highest |  | All RBs / REFSENS |  |
| Default Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3) |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3) |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA_nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  |  | Mid |  |  |  |  |  | nY | default |  |  |  |  |  |  |  |  | nZ |  | default |  |  |  |  |
|  | N/A |  |  | N/A | 5 MHz |  | 0/0 |  |  | N/A | CP-OFDM QPSK |  |  |  | Highest |  |  | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK | Highest |  | All RBs / REFSENS |  |
| 2 | X |  |  | Mid |  |  |  |  |  | nY | default |  |  |  |  |  |  |  |  | nZ |  | default |  |  |  |  |
|  | N/A |  |  | N/A | 5 MHz |  | 0/0 |  |  | CP-OFDM QPSK | CP-OFDM QPSK |  |  |  | Highest |  |  | All RBs / REFSENS |  | N/A |  | CP-OFDM QPSK | Highest |  | All RBs / 0 |  |
| Test Settings for DC_1A-3A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 | DL 2140 MHz |  |  |  |  |  | 3 |  |  | DL 1807.5 MHz |  |  |  |  |  |  | n78 |  | 3757.5 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 1 | DL 2125 MHz |  |  |  |  |  | 3 |  |  | DL 1870 MHz |  |  |  |  |  |  | n78 |  | 3725.5 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_1A-5A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 | DL 2122 MHz |  |  |  |  |  | 5 |  |  | DL 874 MHz |  |  |  |  |  |  | n78 |  | 3780 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 1 | DL 2165 MHz |  |  |  |  |  | 5 |  |  | DL 885 MHz |  |  |  |  |  |  | n78 |  | 3405 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_1A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 | DL 2167.5 MHz |  |  |  |  |  | 7 |  |  | DL 2627.5 MHz |  |  |  |  |  |  | n78 |  | 3305 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 1 | DL 2140 MHz |  |  |  |  |  | 7 |  |  | DL 2630 MHz |  |  |  |  |  |  | n78 |  | 3580 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_1A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 | DL 2120 MHz |  |  |  |  |  | 20 |  |  | DL 794 MHz |  |  |  |  |  |  | n78 |  | 3790 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 1 | DL 2140 MHz |  |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  | n78 |  | 3330 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_3A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 | DL 1820 MHz |  |  |  |  |  | 7 |  |  | DL 2685 MHz |  |  |  |  |  |  | n78 |  | 3310 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 3 | DL 1820 MHz |  |  |  |  |  | 7 |  |  | DL 2685 MHz |  |  |  |  |  |  | n78 |  | 3475 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_3A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 | DL 1820 MHz |  |  |  |  |  | 20 |  |  | DL 804 MHz |  |  |  |  |  |  | n78 |  | 3510 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_3A-40A_n1A Configuration – Note 3, 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 3 | DL 1830 MHz |  |  |  |  |  | 40 |  |  | DL 2380 MHz |  |  |  |  |  |  | n1A |  | 2140 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| Test Settings for DC_5A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 5 | DL 889 MHz |  |  |  |  |  | 7 |  |  | DL 2645 MHz |  |  |  |  |  |  | n78 |  | 3489 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 5 | DL 879 MHz |  |  |  |  |  | 7 |  |  | DL 2670 MHz |  |  |  |  |  |  | n78 |  | 3429 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 3 |  | 5 | DL 875 MHz |  |  |  |  |  | 7 |  |  | DL 2645 MHz |  |  |  |  |  |  | n78 |  | 3350 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_7A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 | DL 2680 MHz |  |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  | n78 |  | 3370 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 7 | DL 2680 MHz |  |  |  |  |  | 20 |  |  | DL 810 MHz |  |  |  |  |  |  | n78 |  | 3435 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 3 |  | 7 | DL 2675 MHz |  |  |  |  |  | 20 |  |  | DL 804 MHz |  |  |  |  |  |  | n78 |  | 3520 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_7A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 7 | DL 2687.5 MHz |  |  |  |  |  | 20 |  |  | DL 782.5 MHz |  |  |  |  |  |  | n78 |  | 3350 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 2 |  | 7 | DL 2687.5 MHz |  |  |  |  |  | 20 |  |  | DL 782.5 MHz |  |  |  |  |  |  | n78 |  | 3460 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| 3 |  | 7 | DL 2650 MHz |  |  |  |  |  | 20 |  |  | DL 795 MHz |  |  |  |  |  |  | n78 |  | 3390 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 10 MHz |  | All RBs / All RBs |
| Test Settings for DC_2A-14A_n66A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 2 | DL 1954 MHz |  |  |  |  |  | 14 |  |  | DL 763 MHz |  |  |  |  |  |  | n66 |  | DL 2170 MHz |  |  |  |  |  |
|  |  | N/A | QPSK |  |  | 5 MHz |  | All RBs / 0 | QPSK |  |  | QPSK |  |  |  | 5 MHz | All RBs / All RBs |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| Test Settings for DC_14A-66A_n2A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 14 | DL 763 MHz |  |  |  |  |  | 66 |  |  | DL 2162 MHz |  |  |  |  |  |  | n2 |  | DL 1954 MHz |  |  |  |  |  |
|  |  | QPSK | QPSK |  |  | 5 MHz |  | All RBs / All RBs | N/A |  |  | QPSK |  |  |  | 5 MHz | All RBs / 0 |  | CP-OFDM QPSK |  | CP-OFDM QPSK |  |  | 5 MHz |  | All RBs / All RBs |
| Note 1: No refsens exceptions with this EN-DC configuration exist in TS38.101-3 Note 2: This test setting is valid for EN-DC configurations affected by UL harmonics, harmonic mixing or cross band isolation exceptions when the exception is avoided by selecting specific test frequency. Test only one of the test points. UL to be active on the cell that form the aggressor of the exception Note 3: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. Note 4: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 5: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 6: Test only applicable for 2UL UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.3B.2.3_1.2.4	Test description

7.3B.2.3_1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations for E-UTRA consist of the test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1, with the exception that the E-UTRA channel bandwidth is the lowest supported value in Table 5.3B.1.3-1 for the EN-DC non-contiguous configuration under test.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2B.2.1-1. The 4CC EN-DC configurations listed in table 7.3B.2.3_1.2.4.1-0 shall not be tested according to TR 38.905 test point analysis. The other 4CC configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing are shown in Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-1 for NR band. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [8] Annex C2.

In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.Table 7.3B.2.3_1.2.4.1-0: 4CC EN-DC configurations that shall not be tested

| EN-DC type |  | E-UTRA CA | NR CA | Notation |
| --- | --- | --- | --- | --- |
| Intra-band contiguous EN-DC (1 band) |  | Yes | No | DC_(n)XDA |
| Intra-band non-contiguous EN-DC (1 band) |  | Yes (cont) | No | DC_XD_nXA |
|  |  | Yes (non-cont) | No | DC_XA-XC_nXA |
|  |  | No | Yes (non-cont) | DC_XA-nY(2A)-nZA |
|  |  | No | Yes (inter) | DC_XA-nYA-nZC |
|  | Yes (all types) | No | DC_XD-nYA, DC_XA-YC_nZA, DC_XA-XA-YA-nZA, DC_XA-YA-ZA_nRA |
|  | Yes (all types) | Yes (cont) | DC_XC_nYC, DC_XA-XA_nYC, DC_XA-YA_nYC |
|  |  | Yes (cont) | DC_XC_nY(2A), DC_XC_nY(2A), DC_XA-YA_nY(2A) |
|  |  | Yes (inter) | DC_XC-nYA-nZA, DC_XA-XA_nYA-nZA, DC_XA-YA_nZA-nRA |
| Intra-band contiguous EN-DC (2-3 band) | Yes | No | DC_XA-YA_(n)ZAA, DC_XC_(n)YAA |
|  | Yes | Yes | DC_(n)XCA-nYA |
| Intra-band non contiguous EN-DC (2-3 band) | Yes | No | DC_XA-YA-ZA_nZA |
|  | Yes | Yes | DC_XA-YA_nYA-nZA |
| NOTE 1: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 2: The table apply to all band numbers, i.e. all values of X, Y and Z defined in TS38.101-3 NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. |  |  |  |

Table 7.3B.2.3_1.2.4.1-1: Test Configuration Table for EN-DC configurations not affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | DC_(n)XDA: Low range, High Range DC_XA-nYA-nZC: Mid range for X Low range, High Range for nY and nZ DC_XA-nY(2A)-nZA: Mid range for X TBD for nY Mid range for nZ |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_(n)XDA Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 (Note 6) | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | X |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 (Note 7) | PCC(M) | X |  | Highest NRB | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | X |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nY(2A)-nZA Configuration – Note 4 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nYA-nZC Configuration– Note 4 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. Note 4: LTE anchor agnostic configuration Note 5: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 6: Test point for UE supporting dual UL Note 7: Test point for UE supporting single UL |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.2.4.1-2: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | DC_XA-nYA-nZC: Mid range for X Mid Range for nY and nZ DC_XA-nY(2A)-nZA: Mid range for X TBD for nY Mid range for nZ |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_(n)XDA Configuration – Note 4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | N/A |  |  |  |  |  |
| Default Test Settings for a DC_XA-nY(2A)-nZA Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nYA-nZC Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Y |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | Z |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | Y |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | Z |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. NOTE 4: No refsens exceptions with this EN-DC configuration exist in TS38.101-3 NOTE 5: This test setting is valid for EN-DC configurations affected by UL harmonics, harmonic mixing or cross band isolation exceptions when the exception is avoided by selecting specific test frequency. Test only one of the test points. UL to be active on the cell that form the aggressor of the exception NOTE 6: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. NOTE 7: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). |  |  |  |  |  |  |  |  |  |  |

# Table 7.3B.2.3_1.2.4.1-3: Void

<< Unchanged sections omitted >>

# 7.3B.2.3_1.3.4	Test description

# 7.3B.2.3_1.3.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.3.4.1-1 and Table 7.3B.2.3_1.3.4.1-2

Table 7.3B.2.3_1.3.4.1-1: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (four bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB" column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-YA_ZC_nRA Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Highest NRB | Mid |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC3(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | R | 15 kHz | Highest NRB | Mid |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.3.4.1-2: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (five bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB" column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-YA-ZA-SA_nRA Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Highest NRB | Mid |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC3(M) | S | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | R | 15 kHz | Highest NRB | Mid |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different E-UTRA bands i.e. X != Y != Z, R corresponds to NR band. NOTE 3: For inter-band EN-DC 4CCs configuration with four bands, if there is no additional exceptional test point is defined besides the requirement for default uplink EN-DC configurations defined in Table 5.5B.4.3-1, testing for 4CC with four band can be skipped. |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.4B.1.4	Test Description

7.4B.1.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.2B.2, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2.All of these configurations shall be tested with applicable test parameters for each intra-band contiguous EN-DC configuration specified in clause 5.3B.1.2, and are shown in table 7.4B.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.for E-UTRA RMC for TDD, TS 36.521-1 [10] Annex A.2 for E-UTRA RMC for FDD , and TS 38.521-1 [8] Annex A.2 for NR RMC Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.4B.1.4.1-1: Test configuration table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | Mid range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 5) |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  | Lowest |  |  |  |
| Test Parameters for Intra-band Contiguous EN-DC Configuration |  |  |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  |  | Uplink Configuration |  |  |  |  |  |
| Test ID | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation |  | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |  |
| 1 | CP-OFDM 64QAM | Full RB (NOTE 1) | CP-OFDM 64QAM | Full RB | DFT-s-OFDM QPSK |  | NOTE 2 | QPSK | NOTE 3 |  |
| 2 | CP-OFDM 256QAM | Full RB (NOTE 1) | CP-OFDM 256QAM | Full RB | DFT-s-OFDM QPSK |  | NOTE 2 | QPSK | NOTE 3 |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Same RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 3: Same RB allocation shall be used per the E-UTRA band and channel BW as specified in Table 7.3.3-2 of TS 36.521 [10]. NOTE 4: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE5: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [6] TS 38.508-1 A.3.1.1 for SS diagram and A.3.2 for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3, and the parameter settings for the NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0, C.1,C.2,C3.1 and TS 38.521-1 [8] Annex C.0,C.1,C.2,C3.1for E-UTRA CG and NR CG respectively, and uplink signals according to TS 36.521-1 [10] Annex H.0,H.1,H.2,H.3.1 and TS 38.521-1 [8] Annex G.0,G.1,G.2,G.3.1 for E-UTRA CG and NR CG respectively.

4.	The UL Reference Measurement channels are TS 36.521-1 [10] Annex A.2 and TS 38.521-1 [8] Annex A.2 for E-UTRA CG and NR CG respectively.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.4B.1.4.3.

<< Unchanged sections omitted >>

7.6B.2.2.4	Test Description

Same test description as in clause 7.6.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 7.6B.2.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | High range with maxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

<< Unchanged sections omitted >>

7.6B.3.2.4	Test Description

Same test description as in clause 7.6.3.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 7.6B.3.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | High range with maxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

<< Unchanged sections omitted >>

7.6B.4.2.4	Test Description

Same test description as in clause 7.6.4.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

Table 7.6B.4.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | High range with maxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

<< Unchanged sections omitted >>

7.8B.2.2.4	Test Description

Same test description as in clause 7.8.2.4 in TS 38.521-1 [8] with the following exceptions:

Table 7.8B.2.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes | High range with maxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

<< Unchanged sections omitted >>

7.9B.2.4	Test description

Same test description as in clause 7.9.4 in TS 38.521-1 [8] with the following exceptions:

Table 7.9B.2.4-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different DC bandwidth classes | MaxWGap |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE | Highest NRB_agg (NOTE 1) |
| NOTE 1: If the UE supports multiple CC Combinations in the EN-DC Configuration with the same NRB_agg , only the combination with the highest NRB_SCG is tested. |  |

<< End of changes >>