3GPP TSG-RAN WG5 Meeting #90-e	R5-210551r2

# Electronic Meeting, 22th Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0119 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCPTT Info Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UPV/EHU, Nemergent Solutions, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - MCPTT Info body is not only included in the INVITE message in the call establishment process. In the case of mobile terminating call (initiated by the SS) the UE shall include an MCPTT Info body in the 200 OK message. The issue is that MCPTT Info body defining table (Table 5.5.3.2.1-1) does not distinguish between INVTE and 200 OK bodies, or mobile-originated and mobile-terminated calls. In fields as mcptt-called-party-id this causes a mismatch. According to TS 24.379 Section F1.3: “the <mcptt-called-party-id> can be included, set to the MCPTT ID of the terminating user;”. However, the terminating user is not the same in mobile-originated and mobile-terminated calls, so this condition needs to be introduced in the MCPTT Info body defining table. - MCPTT Info body session type element shall only be introduced in call-initiating INVITE. However, in call establishment 200 OK this field should not be introduced. According to TS 24.379 Section F1.3: “2) the session-type element can be included with: a) a value of "chat" to indicate that the MCPTT client wants to join a chat group call b) a value of "prearranged" to indicate the MCPTT client wants to make a prearranged group call; […]”. Therefore, the session type is only introduced in call-initiating message. - Unlike group calls, in private call it is optional to include both mcptt-client-id and mcptt-request-uri. Attending to both TS 24.379 Section 11.1.1.2.1.1 (where private call mcptt info MIME contain is defined) and TS 24.379 F.1.3 there are no references that make these fields mandatory. In this last section: “[…]3) the <mcptt-request-uri> can be included with: […] b) a value set to the MCPTT ID of the called MCPTT user when the <session-type> is set to a value of "private"; […]” - MCPTT-Client Id field content correction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In mcptt-called-party-id element value of Table 5.5.3.2.1-1, mobile-originated and mobile-terminated calls are distinguished. Default value in MCPTT-Info from UE corrected. - Session-type element of Table 5.5.3.2.1-1 is removed for mobile terminated calls case. - mcptt-client-id and mcptt-request-uri elements of Table 5.5.3.2.1-1 are changed to optional in case of private calls. - MCPTT-Client Id field content correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Several test cases will remain incorrectly implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: ambient-listening-type removed (wrong structure and is handled in specific message contents of test cases), typo corrected mcptt-called-party-id default value corrected, typos corrected in session-type (lower case) and “true” value (ending quotation mark) for broadcast-ind R2: mcptt-called-party-id could be introduced in call establishment messages from the terminating user in both private and group calls. |  |  |  |  |  |  |  |  |

<Start of modification>

#### 5.5.3.2	MCS Info Lists

##### 5.5.3.2.1	MCS Info Lists from the UE

##### -	MCPTT

Table 5.5.3.2.1-1: MCPTT-Info from the UE

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-access-token | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcptt-access-token> with mcpttString set to access token as assigned to the UE in the Token Response | The access token is opaque to the MCPTT client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG, GROUPCONFIG |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | (GROUP-CALL OR BROADCAST-GROUP-CALL) AND INVITE_REFER |
|  | "private" |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | “chat” |  |  | CHAT-GROUP-CALL AND INVITE_REFER |
|  | "ambient-listening" |  |  | AMBIENT-LISTENING AND INVITE_REFER |
| mcptt-request-uri | not present |  |  |  |
|  | Encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_A_ID | The URI of the group |  | (GROUP-CALL OR BROADCAST-GROUP-CALL OR CHAT-GROUP-CALL) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_Client_B_ID | The URI of the invited MCPTT Client |  | PRIVATE-CALL AND INVITE_REFER |
|  | encrypted (NOTE 2) <mcptt-request-uri> with mcpttURI set to px_MCPTT_ID_User_A |  |  | POC-SETTINGS-EVENT |
| mcptt-calling-user-id | not present or encrypted (NOTE 2) <mcptt-calling-user-id> with mcpttURI set to px_MCPTT_ID_User_A |  |  |  |
|  | not present |  |  | CONFIG, GROUPCONFIG, POC-SETTINGS-EVENT |
| mcptt-called-party-id | not present |  |  |  |
|  | not present or encrypted (NOTE 2) <mcptt-called-party-id> with mcpttURI set to px_MCPTT_ID_User_B |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-called-party-id> with mcpttURI set to px_MCPTT_ID_User_A |  |  | NOT INVITE_REFER |
| mcptt-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "true" |  |  | EMERGENCY-CALL AND INVITE_REFER |
| alert-ind | not present or encrypted (NOTE 2) <alert-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "true" |  |  | EMERGENCY-ALERT AND INVITE_REFER |
| imminentperil-ind | not present or encrypted (NOTE 2) <imminentperil-ind> with mcpttBoolean set to "false" |  |  |  |
|  | Encrypted (NOTE 2) <emergency-ind> with mcpttBoolean set to "true" |  |  | IMMPERIL-CALL AND INVITE_REFER |
| broadcast-ind | not present or “false” |  |  |  |
|  | "true” |  |  | BROADCAST-GROUP-CALL |
| mc-org" | not present |  |  |  |
| floor-state | not present |  |  |  |
| associated-group-id | px_MCPTT_Group_A_ID if mcptt-request-uri contains a temporary group identity; otherwise, not present | if the <mcptt-request-uri> element contains a group identity then this element can include an MCPTT group ID associated with the group identity in the <mcptt-request-uri> element. E.g. if the <mcptt-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCPTT group ID | TS 24.379 [9] clause F.1.3 | GROUP-CALL |
|  | not present |  |  | PRIVATE-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcptt-client-id | not present |  |  |  |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | The UUID URN of the MCPTT Client | RFC 4122 [106] TS 24.379 [9] clause 4.10 | ( GROUP-CALL OR BROADCAST-GROUP-CALL OR CHAT-GROUP-CALLOR EMERGENCY-CALL OR IMMPERIL-CALL OR EMERGENCY-ALERT) AND INVITE_REFER |
|  | not present or encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) if present | in general mcptt-client-id is not mandatory (e.g. for SIP SUBSCRIBE) | RFC 4122 [106] TS 24.379 [9] clause 4.10 | CONFIG, GROUPCONFIG |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | mcptt-client-id is mandatory in the SIP REGISTER or SIP PUBLISH for service authorisation according to TS 24.379 [9] clauses 7.2.1 and 7.2.2 | RFC 4122 [106] TS 24.379 [9] clause 4.10 | CONFIG AND REGISTER_PUBLISH |
|  | encrypted (NOTE 2) <mcptt-client-id> with mcpttString set to valid UUID URN (NOTE 1) | mcptt-client-id is mandatory in SIP PUBLISH for MCPTT service settings only, according to TS 24.379 [9] clause 7.2.3 | RFC 4122 [106] TS 24.379 [9] clause 4.10 | POC-SETTINGS-EVENT |
| alert-ind-rcvd | not present |  |  |  |
| anyExt | not present or any allowed value |  | TS 24.379 [9], clause F.1.3 |  |
| NOTE 1: The SS shall check the mcptt-client-id - at the first time being sent by the UE to be a valid UUID URN with a format like "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" according to RFC 4122 [106] - to be all the same UUID URN in subsequent messages. NOTE 2: Encrypted element as described in Table 5.5.3.2.1-1A |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| REGISTER_PUBLISH | MCPTT-Info in SIP REGISTER or SIP PUBLISH request for service authorisation |
| INVITE_REFER | MCPTT-Info in SIP INVITE or SIP REFER request for call establishment |
| For further conditions see table 5.5.1-1 |  |

Table 5.5.3.2.1-1A: Encrypted MCPTT info parameter sent by the UE

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| type attribute | "Encrypted" |  |  |  |
| EncryptedData | EncryptedData as described in Table 5.5.13.2-1 containing encrypted element content of the mcptt parameter |  |  |  |

##### -	MCVideo

Table 5.5.3.2.1-2: MCVideo-Info from the UE

| Derivation Path: TS 24.281 [86] Clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-access-token | not present |  |  |  |
|  | "eyJhbGciOiJSUzI1NiJ9.eyJtY3B0dF9pZCI6ImFsaWNlQG9yZy5jb20iLCJleHAiOjE0NTM1MDYxMjEsInNjb3BlIjpbIm9wZW5pZCIsIjNncHA6bWNwdHQ6cHR0X3NlcnZlciJdLCJjbGllbnRfaWQiOiJtY3B0dF9jbGllbnQifQ.XYIqai4YKSZCKRNMLipGC_5nV4BE79IJpvjexWjIqqcqiEx6AmHHIRo0mhcxeCESrXei9krom9e8Goxr_hgF3szvgbwl8JRbFuv97XgepDLjEq4jL3Cbu41Q9b0WdXAdFmeEbiB8wo_xggiGwv6IDR1b3TgAAsdjkRxSK4ctIKPaOJSRmM7MKMcKhIug3BEkSC9-aXBTSIv5fAGN-ShDbPvHycBpjzKWXBvMIR5PaCg-9fwjELXZXdRwz8C6JbRM8aqzhdt4CVhQ3-Arip-S9CKd0tu-qhHfF2rvJDRlg8ZBiihdPH8mJs-qpTFep_1-kON3mL0_g54xVmlMwN0XQA" | The access token is opaque to the MCVideo client | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL AND INVITE_REFER |
|  | "private" |  |  | PRIVATE-CALL AND INVITE_REFER |
| mcvideo-request-uri | px_MCVideo_Group_A_ID | The URI of the group |  | GROUP-CALL |
|  | not present or px_MCVideo_Client_B_ID | The URI of the invited MCVideo Client |  | PRIVATE-CALL |
| mcvideo-calling-user-id | not present or px_MCVideo_ID_User_A |  |  |  |
| mcvideo-called-party-id | not present |  |  |  |
|  | not present or px_MCVideo_ID_User_B |  |  | PRIVATE-CALL AND INVITE_REFER |
|  | not present or px_MCVideo_ID_User_A |  |  | NOT INVITE_REFER |
| mcvideo-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or if present then="false" |  |  |  |
|  | "true" |  |  | EMERGENCY-CALL |
| alert-ind | not present or if present then="false" |  |  |  |
|  | "true" |  |  | EMERGENCY-ALERT |
| imminentperil-ind | not present or if present then="false" |  |  |  |
|  | "true" |  |  | IMMPERIL-CALL |
| broadcast-ind | not present |  |  |  |
| mc-org" | not present |  |  |  |
| transmission-state | not present |  |  |  |
| associated-group-id | px_MCVideo_Group_A_ID if mcvideo-request-uri contains a temporary group identity; otherwise, not present | if the <mcvideo-request-uri> element contains a group identity then this element can include an MCVideo group ID associated with the group identity in the <mcvideo-request-uri> element. E.g. if the <mcvideo-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCVideo group ID | TS 24.281 [86] clause F.1.3 | GROUP-CALL |
|  | not present |  |  | PRIVATE-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcvideo-client-id | px_MCVideo_Client_A_ID | The URI of the MCVideo Client |  | PRIVATE-CALL GROUP-CALL EMERGENCY-CALL IMMPERIL-CALL EMERGENCY-ALERT |
|  | "eyJhbGciOiJSUzI1NiJ9.eyJzdWIiOiIxMjM0NTY3ODkwIiwiYXVkIjoibWNwdHRfY2xpZW50IiwiaXNzIjoiSWRNUy5zZXJ2ZXIuY29tOjkwMzEiLCJpYXQiOjE0NTM0OTgxNTgsImV4cCI6MTQ1MzQ5ODQ1OCwibWNwdHRfaWQiOiJhbGljZUBvcmcuY29tIn0.Dpn7AhIMaqMEgg12NYUUfJGSFJMPG8M2li9FLtPotDlHvwU2emBws8z5JLw81SXQnoLqZ8ZF8tIhZ1W7uuMbufF4Wsr7PAadZixz3CnV2wxFV9qR_VA1-0ccDTPukUsRHsic0SgZ3aIbcYKd6VsehFe_GDwfqysYzD7yPwCfPZo" | The MCVideo client may validate the user with the ID token and configure itself for the user | TS 33.180 [94], clause B.4 RFC 6749 [77] | CONFIG |
| alert-ind-rcvd | not present |  |  |  |
| anyExt | not present or any allowed value |  | TS 24.281 [86] clause F.1.3 |  |

##### -	MCData

Table 5.5.3.2.1-3: MCData-Info from the UE

| Derivation Path: TS 24.282 [87], Clause D.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-access-token | not present |  |  |  |
| request-type | "one-to-one-sds" |  |  | MCD_1to1 |
| request-type | “group-sds” |  |  | MCD_grp |
| mcdata-request-uri | px_MCData_Group_A |  |  | MCD_grp |
| mcdata-calling-user-id | not present |  |  |  |
| mcdata-called-party-id | not present |  |  |  |
| mcdata-calling-group-id | not present |  |  |  |
| alert-ind | not present |  |  |  |
| originated-by | not present |  |  |  |
| mcdata-client-id | px_MCData_Client_A_ID |  |  | MCD_grp |
| mcdata-controller-psi | not present |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCD_1to1 | A one-to-one MCData call |
| MCD_grp | A goup MCData call |
| For further conditions see table 5.5.1-1 |  |

##### 5.5.3.2.2	MCPTT-Info from the SS

##### -	MCPTT

Table 5.5.3.2.2-1: MCPTT-Info from the SS

| Derivation Path: TS 24.379 [9] clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-access-token | not present |  |  |  |
| session-type | not present |  |  |  |
|  | "prearranged" |  |  | GROUP-CALL |
|  | "private" |  |  | PRIVATE-CALL |
|  | “chat” |  |  | CHAT-GROUP-CALL |
|  | "ambient-listening" |  |  | AMBIENT-LISTENING |
| mcptt-request-uri | px_MCPTT_ID_User_A | The URI of the called user |  |  |
| mcptt-calling-user-id | px_MCPTT_ID_User_B | The URI of the calling user |  |  |
| mcptt-called-party-id | not present |  |  |  |
| mcptt-calling-group-id | not present |  |  |  |
|  | px_MCPTT_Group_A_ID | The URI of the group |  | GROUP-CALL OR BROADCAST-GROUP-CALL OR CHAT-GROUP-CALL |
| required | not present |  |  |  |
| emergency-ind | not present |  |  |  |
|  | "true" |  |  | EMERGENCY-CALL |
| alert-ind | not present |  |  |  |
|  | "true" |  |  | EMERGENCY-ALERT |
| imminentperil-ind | not present |  |  |  |
|  | "true" |  |  | IMMPERIL-CALL |
| broadcast-ind | not present |  |  |  |
|  | “true” |  |  | BROADCAST-GROUP-CALL |
| mc-org" | not present |  |  |  |
| floor-state | not present |  |  |  |
| associated-group-id | not present |  |  |  |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcptt-client-id | not present |  |  |  |
| alert-ind-rcvd | not present |  |  |  |
| anyExt | not present |  | TS 24.379 [9], clause F.1.3 |  |

##### -	MCVideo

Table 5.5.3.2.2-2: MCVideo-Info from the SS

| Derivation Path: TS 24.281 [86] Clause F.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-access-token | not present |  |  |  |
|  | "eyJhbGciOiJSUzI1NiJ9.eyJtY3B0dF9pZCI6ImFsaWNlQG9yZy5jb20iLCJleHAiOjE0NTM1MDYxMjEsInNjb3BlIjpbIm9wZW5pZCIsIjNncHA6bWNwdHQ6cHR0X3NlcnZlciJdLCJjbGllbnRfaWQiOiJtY3B0dF9jbGllbnQifQ.XYIqai4YKSZCKRNMLipGC_5nV4BE79IJpvjexWjIqqcqiEx6AmHHIRo0mhcxeCESrXei9krom9e8Goxr_hgF3szvgbwl8JRbFuv97XgepDLjEq4jL3Cbu41Q9b0WdXAdFmeEbiB8wo_xggiGwv6IDR1b3TgAAsdjkRxSK4ctIKPaOJSRmM7MKMcKhIug3BEkSC9-aXBTSIv5fAGN-ShDbPvHycBpjzKWXBvMIR5PaCg-9fwjELXZXdRwz8C6JbRM8aqzhdt4CVhQ3-Arip-S9CKd0tu-qhHfF2rvJDRlg8ZBiihdPH8mJs-qpTFep_1-kON3mL0_g54xVmlMwN0XQA" | The access token is opaque to the MCVideo client | TS 33.180 [94] clause B.4 RFC 6749 [77] | CONFIG |
| session-type | "prearranged" |  |  | GROUP-CALL |
|  | "private" |  |  | PRIVATE-CALL |
| mcvideo-request-uri | px_MCVideo_Group_A_ID | The URI of the group |  | GROUP-CALL |
|  | px_MCVideo_Client_B_ID | The URI of the invited MCVideo Client |  | PRIVATE-CALL |
| mcvideo-calling-user-id | not present or px_MCVideo_ID_User_A |  |  |  |
| mcvideo-called-party-id | not present or px_MCVideo_ID_User_B |  |  |  |
| mcvideo-calling-group-id | not present |  |  |  |
| required | not present |  |  |  |
| emergency-ind | not present or if present then="false" |  |  |  |
|  | "true" |  |  | EMERGENCY-CALL |
| alert-ind | not present or if present then="false" |  |  |  |
|  | "true" |  |  | EMERGENCY-ALERT |
| imminentperil-ind | not present or if present then="false" |  |  |  |
|  | "true" |  |  | IMMPERIL-CALL |
| broadcast-ind | not present |  |  |  |
| mc-org" | not present |  |  |  |
| floor-state | not present |  |  |  |
| associated-group-id | px_MCVideo_Group_A_ID if mcvideo-request-uri contains a temporary group identity; otherwise, not present | if the <mcvideo-request-uri> element contains a group identity then this element can include an MCVideo group ID associated with the group identity in the <mcvideo-request-uri> element. E.g. if the <mcvideo-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCVideo group ID | TS 24.281 [86] clause F.1.3 | GROUP-CALL |
|  | not present |  |  | PRIVATE-CALL |
| originated-by | not present |  |  |  |
| MKFC-GKTPs | not present |  |  |  |
| mcvideo-client-id | px_MCVideo_Client_A_ID | The URI of the MCVideo Client |  | PRIVATE-CALL GROUP-CALL EMERGENCY-CALL IMMPERIL-CALL EMERGENCY-ALERT |
|  | "eyJhbGciOiJSUzI1NiJ9.eyJzdWIiOiIxMjM0NTY3ODkwIiwiYXVkIjoibWNwdHRfY2xpZW50IiwiaXNzIjoiSWRNUy5zZXJ2ZXIuY29tOjkwMzEiLCJpYXQiOjE0NTM0OTgxNTgsImV4cCI6MTQ1MzQ5ODQ1OCwibWNwdHRfaWQiOiJhbGljZUBvcmcuY29tIn0.Dpn7AhIMaqMEgg12NYUUfJGSFJMPG8M2li9FLtPotDlHvwU2emBws8z5JLw81SXQnoLqZ8ZF8tIhZ1W7uuMbufF4Wsr7PAadZixz3CnV2wxFV9qR_VA1-0ccDTPukUsRHsic0SgZ3aIbcYKd6VsehFe_GDwfqysYzD7yPwCfPZo" | The MCVideo client may validate the user with the ID token and configure itself for the user | TS 33.180 [94] clause B.4 RFC 6749 [77] | CONFIG |
| alert-ind-rcvd | not present |  |  |  |
| anyExt | not present or any allowed value |  | TS 24.281 [86] clause F.1.3 |  |

##### -	MCData

Table 5.5.3.2.2-3: MCData-Info from the SS

| Derivation Path: TS 24.282 [87], Clause D.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-access-token | not present |  |  |  |
| request-type | "one-to-one-sds" |  |  | MCD_1to1 |
| request-type | “group-sds” |  |  | MCD_grp |
| mcdata-request-uri | px_MCData_Group_A |  |  | MCD_grp |
| mcdata-calling-user-id | px_MCData_ID_User_B |  |  |  |
| mcdata-called-party-id | px_MCData_ID_User_A |  |  |  |
| mcdata-calling-group-id | not present |  |  |  |
| alert-ind | not present |  |  |  |
| originated-by | not present |  |  |  |
| mcdata-client-id | px_MCData_Client_B_ID |  |  |  |
| mcdata-controller-psi | not present |  |  |  |

| Condition | Explanation |
| --- | --- |
| MCD_1to1 | A one-to-one MCData call |
| MCD_grp | A goup MCData call |
| For further conditions see table 5.5.1-1 |  |

##### <End of modification>