3GPP TSG-RAN5 Meeting #90-e	R5-210658r1

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0103 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS A.9.2-Optional UPDATE after EPS fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Qualcomm, Rohde&Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause A.9.2 for EPS fallback requires the UE to send UPDATE after falling back to E-UTRA. From a core specification point of view, it is not entirely clear though if such UPDATE is mandatory when preconditions are not used. As RAN5 is planning for a corresponding LS to CT1 (see R5-211311), and will have to wait for a response, it is proposed to make UPDATE optional for the time being. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Made the UPDATE and consequencial 200 OK optional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Legit UEs might fail EPS fallback relevent TCs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Revised “Reason for change” text. - Added Qualcomm and Rohde & Schwarz as co-sources. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

## A.9.2	EPS Fallback for Voice Call / steps after fallback / 5GS

Expected sequence

| Step | Direction |  | Message/Procedure | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UPDATE | Optional: The UE sends an UPDATE request containing a second SDP offer. |
| 2 |  |  | 200 OK | If the UE sent UPDATE, the SS sends a 200 OK response for UPDATE containing an SDP answer. |
| 3 |  |  | 180 Ringing | SS sends a 180 Ringing provisional response. |
| 4 |  |  | 200 OK | SS responds to INVITE with 200 OK. |
| 5 |  |  | ACK | UE acknowledges. |

<End of modified section 1>