3GPP TSG-RAN5 Meeting #90-e	R5-210662r1

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0107 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR IMS TC 8.26-MO hold without announcement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently there is no test coverage to verfiy that an UE could correctly handle MO hold without announcement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New TC 8.26 for MO hold without announcement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No testing of UE compliance for such use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.26 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2... CR 0278 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Editorial changes: adjusted the editorial style of the Conformance Requirements and referenced steps in the main behavior; added “affected” CR in the coverpge; subclause format changed to “H6”. - Re-arragned the test procedures. - Chenged the media attribute conent in SDP as the same with the initial session. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.26	MO Call Hold without announcement / 5GS

8.26.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and having set up a voice call }

ensure that {

when { UE is being made to hold the call }

then { UE sends re-INVITE or UPDATE, and completes the call hold procedure }

}

(2)

with { UE having put the voice call on hold }

ensure that {

when { UE is being made to resume the call }

then { UE sends re-INVITE or UPDATE, and completes the call resume procedure }

}

8.26.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.610 clause 4.5.2.1]:

In addition to the application of procedures according to 3GPP TS 24.229 [1], the following procedures shall be applied at the invoking UE in accordance with RFC 3264 [4].

A UE shall not invoke the HOLD service on a dialog associated with an emergency call the UE has initiated.

If not all the media streams are affected, the invoking UE shall generate a new SDP offer where:

1)	for each media stream that is to be held, the SDP offer contains:

-	an "inactive" SDP attribute if the stream was previously set to "recvonly"; or

-	a "sendonly" SDP attribute if the stream was previously set to "sendrecv";

NOTE 1:	If the directionality attribute of the media stream is currently "sendonly" or "inactive", then that media stream is not put on hold and, in the SDP offer, the directionality for that media stream remains unchanged.

2)	for each held media stream that is to be resumed, the SDP offer contains:

-	a "recvonly" SDP attribute if the stream was previously an inactive media stream; or

-	a "sendrecv" SDP attribute if the stream was previously a sendonly media stream, or the attribute may be omitted, since sendrecv is the default; and

3)	for each media stream that is unaffected, the media parameters in the SDP offer remain unchanged from the previous SDP.

If all the media streams are to be held:

-	if they all have identical directionality, the invoking UE shall generate an SDP offer containing a session level direction attribute, or separate media level direction attributes, in the SDP that is set to:

1)	"inactive" if the streams were previously set to "recvonly"; or

2)	"sendonly" if the streams were previously set to "sendrecv"; and

NOTE 2:	If the directionality attribute of all the media streams is currently "sendonly" or "inactive", then all these media streams are not put on hold and, in the SDP offer, the directionality for these media streams will remain unchanged.

-	if they all do not have identical directionality, then for each media stream in the session, the invoking UE shall follow the procedure listed above for individual media streams.

If all the media streams were previously on hold and are to be resumed:

-	if they all have identical directionality, the invoking UE shall generate a session level direction attribute, or separate media level direction attributes, in the SDP that is set to:

1)	"recvonly" if the streams were previously inactive media streams; or

2)	"sendrecv" if the streams were previously sendonly media streams, or the attribute may be omitted, since sendrecv is the default; and

-	if they all do not have identical directionality, then for each media stream in the session, the invoking UE shall follow the procedure listed above for individual media streams.

If, in the generated SDP offer, there is at least one media stream whose directionality has changed from the previous SDP, the UE shall send the generated SDP offer in a re-INVITE request (or UPDATE request) to the remote UE.

[TS 26.114 clause 7.3.1]:

RTCP packets should be sent for all types of multimedia sessions to enable synchronization with other RTP transported media, remote end-point aliveness information, monitoring of the transmission quality, and carriage of feedback messages such as TMMBR for video and RTCP APP for speech. The RR value should be set greater than zero to enable RTCP packets to be sent when media is put on hold and during active RTP media transmission, including real-time text sessions which may have infrequent RTP media transmissions.

[TS 24.229 clause 6.1.1]:

If the media line in the SDP message body indicates the usage of RTP/RTCP, and if the UE is configured to request an RTCP bandwidth level for the session is different than the default RTCP bandwidth as specified in RFC 3556 [56], then in addition to the "AS" bandwidth modifier in the media-level "b=" line, the UE shall include two media-level "b=" lines, one with the "RS" bandwidth modifier and the other with the "RR" bandwidth modifier as described in RFC 3556 [56] to specify the required bandwidth allocation for RTCP. The bandwidth-value in the b=RS: and b=RR: lines may include transport overhead as described in subclause 6.1 of RFC 3890 [152].

For other media streams the "b=" media descriptor may be included. The value or absence of the "b=" parameter will affect the assigned QoS which is defined in or 3GPP 29.213 [13C].

NOTE 3:	In a two-party session where both participants are active, the RTCP receiver reports are not sent, therefore, the RR bandwidth modifier will typically get the value of zero.

8.26.3	Test description

8.26.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

- 	UE is configured to use preconditions

Preamble:

-	The UE has registered to IMS and set up the MO call, by executing the generic test procedure in Annex A.2 up to the last step and thereafter executing the generic test procedure in A4.1.

8.26.3.2	Test procedure sequence

Table 8.26.3.2-1: Main Behaviour

Table 8.26.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is made to hold the call | - | - |  |  |
| 2 | Check: Does the UE send INVITE or UPDATE with a SDP offer to hold the call? | --> | INVITE or UPDATE | 1 | P |
| 3 | Optional: The SS responds to the INVITE with a 100 Trying provisional response | <-- | 100 Trying |  |  |
| 4 | The SS responds to INVITE or UPDATE with 200 OK to indicate that the remote UE is no more sending any media (call hold) or resumes sending media (call resume) | <-- | 200 OK |  |  |
| 5 | Optional: If the UE sent INVITE in step 1 then UE acknowledges the receipt of 200 OK for INVITE | --> | ACK |  |  |
| 6 | The UE is made to resume the call | - | - |  |  |
| 7 | Check: Does the UE send INVITE or UPDATE with a SDP offer to resume the call? | --> | INVITE or UPDATE | 2 | P |
| 8 | Optional: The SS responds to the INVITE with a 100 Trying provisional response | <-- | 100 Trying |  |  |
| 9 | The SS responds to INVITE or UPDATE with 200 OK to indicate that the remote UE is no more sending any media (call hold) or resumes sending media (call resume) | <-- | 200 OK |  |  |
| 10 | Optional: If the UE sent INVITE in step 1 then UE acknowledges the receipt of 200 OK for INVITE | --> | ACK |  |  |
| 11 | The SS releases the call with BYE | <-- | BYE | - | - |
| 12 | The UE sends 200 OK for BYE | --> | 200 OK | - | - |

8.26.3.3	Specific message contents

Table 8.26.3.3-1: INVITE (step 2, table 8.26.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in annex A.2.1, Conditions A1, A3 and A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Request-Line Request-URI |  | the same URI in the request line as the SS has sent in the Contact header of an earlier message within the same dialog |  |  |
| Supported Option-tag |  | precondition |  |  |
| Message-body |  | Header optional Contents if present: The following SDP types and values shall be present. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 1] s=(session name) c=IN (addrtype) (connection-address for UE) [Note 2] b=AS: (bandwidth-value) Time description: t=0 0 Media description: The same media description with the initial session. Attributes for media: The same codec with the initial session. Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 2: At least one "c=" field shall be present. |  | TS 24.229 [7] |

Table 8.26.3.3-2: UPDATE (step 2, table 8.26.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in annex A.2.5, Conditions A1, A3 and A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Request-Line Request-URI |  | the same URI in the request line as the SS has sent in the Contact header of an earlier message within the same dialog |  |  |
| Supported Option-tag |  | precondition |  |  |
| Message-body |  | Header optional Contents if present: The following SDP types and values shall be present. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 1] s=(session name) c=IN (addrtype) (connection-address for UE) [Note 2] b=AS: (bandwidth-value) Time description: t=0 0 Media description: The same media description with the initial session. Attributes for media: The same codec with the initial session. Attributes for preconditions: a=curr:qos local sendrecv a=curr:qos remote sendrecv a=des:qos mandatory local sendrecv a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 2: At least one "c=" field shall be present. |  | TS 24.229 [7] |

<End of modified section 1>