3GPP TSG-RAN5 Meeting #90-e	R5-210727r3

# Electronic, 22nd February - 5th March, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0469 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Inner_partial allocation in general section and a few test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For PC1, replacing Inner_1RB allocation by Inner_Partial allocation In LTE or FR1 testing, there are several test cases that requires 1RB allocation, such as carrier leakage, In-band emission and Spurious emission. However in FR2 the MPRnarrow would apply to 1RB allocation, which is quite high especially for PC1. In order to avoid possible testability issue caused by large MPR value, it can be considered to use partial allocation in mentioned test cases. For Carrier leakage, In-band emission, Inner_4RB/Inner_16RB are already used in test configuration tables but specific RB position is not provided; For Spurious emission, 1RB is used and need to be replaced. - For PC3, Inner_4RB/Inner_16RB are used in Carrier leakage, In-band emission,but specific RB allocation is not defined. Adding Inner_partial allocation in 6.1-1. - Correcting the general RB allocation table in section 6. There are some mistakes in table 6.1-1 and 6.1-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For PC1, replacing Inner_1RB allocation by Inner_Partial allocation - Inner_Partial allocations are defined in 6.1-1 and 6.1-2 - In Carrier leakage, In-band emission and Spurious emission, Inner_Partial allocations are used. - For PC3 - Inner_Partial allocations are defined in 6.1-1 and 6.1-2 - In Carrier leakage, In-band emission, Inner_Partial allocations are used. - Correcting the general RB allocation table in section 6. - Change inner full configuration of 100MHz, 120kHz, DFT-s-OFDM to 20@23 to reflect the reference waveform defined in 6.2.2.0. - Change Inner_full of 300MHz to 66RB. - Adding notes to reflect the difference in requirements between R15 and R16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test condition is not set up properly. UE might fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.4.2.2, 6.4.2.3, 6.4A.2.2.1, 6.4A.2.2.2, 6.4A.2.2.3, 6.4A.2.2.4, 6.4A.2.2.5, 6.4A.2.2.6, 6.4A.2.2.7, 6.4A.2.3.1, 6.4A.2.3.2, 6.4A.2.3.3, 6.4A.2.3.4, 6.4A.2.3.5, 6.4A.2.3.6, 6.4A.2.3.7, 6.5.3.1, 6.5.3.2, 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Mis-uploaded without any change R2: Adding Keysight as co-source Merging changes in 6.5.3.3 into R5-211231 R3：Undo Remova of Inner_1RB allocation for PC2,3,4 Adding Inner_partial allocation for PC2,3,4 In Spurious emission, keeping Inner_1RB test points for PC2,3,4 |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.1	General

Editor’s Note: Test configurations/environments that require new spherical scan shall be included in test procedure section and identifying such scenarios is currently FFS and owned by RAN5.

Unless otherwise stated, the transmitter characteristics are specified over the air (OTA) with a single or multiple transmit chains.

Unless otherwise stated, for power class 3 UEs, the beam correspondence side condition for SSB and CSI-RS specified in subclause 6.6.4 shall apply to the transmission tests.

Transmitter requirements for CA operation apply only when the DMRS initialization parameters (including the case when the UE applies cell ID as DMRS scrambling ID) are different across all CCs. The UE may use higher MPR values outside this limitation.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

For Tx test cases the identified beam peak direction can be stored and reused for a device under test in various configurations/environments for the full duration of device testing as long as beam peak direction is the same.

Unless otherwise stated, Channel Bandwidth shall be prioritized in the selecting of test points. Subcarrier spacing shall be selected after Test Channel Bandwidth is selected.

Uplink RB allocations given in Table 6.1-1 are used throughout this section, unless otherwise stated by the test case.

The UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

Table 6.1-1: Common Uplink Configuration PC2, for PC3 and PC4

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Outer_Full | Outer_1RB_Left | Outer_1RB_Right | Inner_Full (Note 1) | Inner_1RB_Left | Inner_1RB_Right | Inner_Partial_Left | Inner_Partial_Right |
| 50MHz |  | 60 |  | DFT-s |  | 64@0 | 1@0 | 1@65 | 20@22 | 1@22 | 1@43 | 4@22 | 4@40 |
|  |  |  |  | CP |  | 66@0 | 1@0 | 1@65 | 22@22 | 1@22 | 1@43 | 4@22 | 4@40 |
|  |  | 120 |  | DFT-s |  | 32@0 | 1@0 | 1@31 | 10@11 | 1@11 | 1@21 | 4@11 | 4@18 |
|  |  |  |  | CP |  | 32@0 | 1@0 | 1@31 | 11@113 10@104 | 1@11 | 1@21 | 4@11 | 4@18 |
| 100MHz |  | 60 |  | DFT-s |  | 128@0 | 1@0 | 1@131 | 40@44 | 1@44 | 1@87 | 4@44 | 4@84 |
|  |  |  |  | CP |  | 132@0 | 1@0 | 1@131 | 44@44 | 1@44 | 1@87 | 4@44 | 4@84 |
|  |  | 120 |  | DFT-s |  | 64@0 | 1@0 | 1@65 | 20@23 | 1@22 | 1@43 | 4@22 | 4@40 |
|  |  |  |  | CP |  | 66@0 | 1@0 | 1@65 | 22@22 | 1@22 | 1@43 | 4@22 | 4@40 |
| 200MHz |  | 60 |  | DFT-s |  | 256@0 | 1@0 | 1@263 | 81@88 | 1@88 | 1@175 | 4@88 | 4@172 |
|  |  |  |  | CP |  | 264@0 | 1@0 | 1@263 | 88@88 | 1@88 | 1@175 | 4@88 | 4@172 |
|  |  | 120 |  | DFT-s |  | 128@0 | 1@0 | 1@131 | 40@44 | 1@44 | 1@87 | 4@44 | 4@84 |
|  |  |  |  | CP |  | 132@0 | 1@0 | 1@131 | 44@44 | 1@44 | 1@87 | 4@44 | 4@84 |
| 400MHz |  | 60 |  | DFT-s |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  |  |  | CP |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 120 |  | DFT-s |  | 256@0 | 1@0 | 1@263 | 64@66 | 1@66 | 1@197 | 4@66 | 4@194 |
|  |  |  |  | CP |  | 264@0 | 1@0 | 1@263 | 66@66 | 1@66 | 1@197 | 4@66 | 4@194 |
| Note 1: RB allocation is left aligned within inner region Note 2: Inner_Full allocation is selected as the largest RB allocation within Region 1 inner allocation defined in 6.2.2.3.3; Inner_Partial_Left and Inner_Partial_Right are selected as partial allocation within Region 1 inner allocation which are not impacted by MPRnarrow defined in 6.2.2.3.3. Note 3: Applicable to Rel-15 only. Note 4: Applicable to Rel-16 and forward. |  |  |  |  |  |  |  |  |  |  |

Table 6.1-2: Common Uplink Configuration for PC1

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_1RB_Left | Outer_1RB_Right | Inner_Full_Region1 | Innner_partial_Left_Region1 | Inner_Partial_Right_Region1 | Inner_Full_Region2 | Innner_Partial_Left_Region2 | Inner_Partial_Right_Region2 |
| 50MHz | 60 | DFT-s | 64@0 | 1@0 | 1@65 | 20@22 | 16@22 | 16@28 | 32@16 | 16@8 | 16@42 |
|  |  | CP | 66@0 | 1@0 | 1@65 | 22@22 | 16@22 | 16@28 | 33@16 | 16@8 | 16@42 |
|  | 120 | DFT-s | 32@0 | 1@0 | 1@31 | 10@11 | 8@11 | 8@14 | 16@8 | 8@4 | 8@54 |
|  |  | CP | 32@0 | 1@0 | 1@31 | 11@11 | 8@11 | 8@14 | 16@8 | 8@4 | 8@54 |
| 100MHz | 60 | DFT-s | 128@0 | 1@0 | 1@131 | 40@44 | 16@44 | 16@72 | 64@32 | 16@8 | 16@108 |
|  |  | CP | 132@0 | 1@0 | 1@131 | 44@44 | 16@44 | 16@72 | 66@33 | 16@8 | 16@108 |
|  | 120 | DFT-s | 64@0 | 1@0 | 1@65 | 20@23 | 8@22 | 8@36 | 32@16 | 8@4 | 8@120 |
|  |  | CP | 66@0 | 1@0 | 1@65 | 22@22 | 8@22 | 8@36 | 33@16 | 8@4 | 8@120 |
| 200MHz | 60 | DFT-s | 256@0 | 1@0 | 1@263 | 81@88 | 16@88 | 16@161 | 128@64 | 16@8 | 16@240 |
|  |  | CP | 264@0 | 1@0 | 1@263 | 88@88 | 16@88 | 16@161 | 132@66 | 16@8 | 16@240 |
|  | 120 | DFT-s | 128@0 | 1@0 | 1@131 | 40@44 | 8@44 | 8@36 | 64@32 | 8@4 | 8@252 |
|  |  | CP | 132@0 | 1@0 | 1@131 | 44@44 | 8@44 | 8@36 | 66@33 | 8@4 | 8@252 |
| 400MHz | 60 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 120 | DFT-s | 256@0 | 1@0 | 1@263 | 64@66 | 8@66 | 8@190 | 128@64 | 8@4 | 8@252 |
|  |  | CP | 264@0 | 1@0 | 1@263 | 66@66 | 8@66 | 8@190 | 132@66 | 8@4 | 8@252 |
| Note 1: RB allocation is left aligned within inner region 1 or inner region 2 as defined in clause 6.2.2.3.1. Note 2: Inner_Full allocation is selected as the largest RB allocation within Region 1 or Region 2 inner allocation defined in 6.2.2.3.1; Inner_partial_Left and Inner_partial_Right are selected as minimum allocation within Region 1 or Region 2 inner allocation which are not impacted by MPRnarrow defined in 6.2.2.3.1. |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4.2.2	Carrier leakage

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainty and Test Tolerance are FFS.

6.4.2.2.1	Test purpose

Carrier leakage expresses itself as unmodulated sine wave with the carrier frequency. It is an interference of approximately constant amplitude and independent of the amplitude of the wanted signal. Carrier leakage interferes with the sub carriers at its position (if allocated), especially, when their amplitude is small.

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of carrier leakage.

<Unchanged Text Skipped>

6.4.2.2.4	Test description

6.4.2.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4.2.2.4.1-1: Test Configuration

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Mid |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1, 3) |
| 1 |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1. NOTE 3: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_Partial_Left_Region2 for PC1, use Inner_Partial_Right_Region2 for UL RB allocation. |  |  |  |

<Unchanged Text Skipped>

6.4.2.3	In-band emissions

<Unchanged Text Skipped>

6.4.2.3.4	Test description

6.4.2.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4.2.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex [TBD].

Table 6.4.2.3.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
| 3 |  | CP-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
| 4 |  | CP-OFDM QPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1. |  |  |  |

<Unchanged Text Skipped>

6.4A.2.2.1	Carrier leakage for CA (2UL CA)

<Unchanged Text Skipped>

6.4A.2.2.1.4	Test description

6.4A.2.2.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.4A.2.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.2.1.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 5) | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | 50 | N/A for this test |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC/CC2 | 50 |  |  |  |  |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 4: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_Partial_Left_Region2 for PC1, use Inner_Partial_Right_Region2 for UL RB allocation. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.2.2	Carrier leakage for CA (3UL CA)

<Unchanged Text Skipped>

6.4A.2.2.2.4	Test description

6.4A.2.2.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.4A.2.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.2.2.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 5) | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | 50 | N/A for this test |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC/CC2 | 50 |  |  |  |  |
|  | SCC/CC3 | 50 |  |  |  |  |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 4: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_16RB_Left_Region2 for PC1, use Inner_16RB_Right_Region2 for UL RB allocation. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.2.3	Carrier leakage for CA (4UL CA)

<Unchanged Text Skipped>

6.4A.2.2.3.4	Test description

6.4A.2.2.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.4A.2.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.2.3.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCC are mapped onto physical frequencies according to Table 6.1-2 |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 5) | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | 50 | N/A for this test |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC/CC2 | 50 |  |  | DFT-s-OFDM QPSK |  |
|  | SCC/CC3 | 50 |  |  | DFT-s-OFDM QPSK |  |
|  | SCC/CC4 | 50 |  |  | DFT-s-OFDM QPSK |  |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 4: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_Partial_Left_Region2 for PC1, use Inner_Partial_Right_Region2 for UL RB allocation. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS38.508-1 [10]. |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.2.4	Carrier leakage for CA (5UL CA)

<Unchanged Text Skipped>

6.4A.2.2.4.4	Test description

6.4A.2.2.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.4A.2.2.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.2.4.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2 |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 5) | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | 50 | N/A for this test |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC/CC2 | 50 |  |  | - | - |
|  | SCC/CC3 | 50 |  |  | - | - |
|  | SCC/CC4 | 50 |  |  | - | - |
|  | SCC/CC5 | 50 |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 4: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_Partial_Left_Region2 for PC1, use Inner_Partial_Right_Region2 for UL RB allocation. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS 38.508-1 [10]. |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.2.5	Carrier leakage for CA (6UL CA)

<Unchanged Text Skipped>

6.4A.2.2.5.4	Test description

6.4A.2.2.5.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.4A.2.2.5.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.2.5.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2 |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 5) | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | 50 | N/A for this test |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC/CC2 | 50 |  |  | - | - |
|  | SCC/CC3 | 50 |  |  | - | - |
|  | SCC/CC4 | 50 |  |  | - | - |
|  | SCC/CC5 | 50 |  |  | - | - |
|  | SCC/CC6 | 50 |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 4: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_Partial_Left_Region2 for PC1, use Inner_Partial_Right_Region2 for UL RB allocation. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS 38.508-1 [10]. |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.2.6	Carrier leakage for CA (7UL CA)

<Unchanged Text Skipped>

6.4A.2.2.6.4	Test description

6.4A.2.2.6.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.4A.2.2.6.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.2.6.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2 |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 5) | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | 50 | N/A for this test |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC/CC2 | 50 |  |  | - | - |
|  | SCC/CC3 | 50 |  |  | - | - |
|  | SCC/CC4 | 50 |  |  | - | - |
|  | SCC/CC5 | 50 |  |  | - | - |
|  | SCC/CC6 | 50 |  |  | - | - |
|  | SCC/CC7 | 50 |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 4: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_Partial_Left_Region2 for PC1, use Inner_Partial_Right_Region2 for UL RB allocation. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS 38.508-1 [10]. |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.2.7	Carrier leakage for CA (8UL CA)

<Unchanged Text Skipped>

6.4A.2.2.7.4	Test description

6.4A.2.2.7.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in table 6.4A.2.2.7.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.2.7.4.1-1: Intra-band Contiguous CA Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause [4.1] |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2 |  |  |  | Low and High range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  | Lowest aggregated BW |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 5) | CBW (MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | 50 | N/A for this test |  | DFT-s-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC/CC2 | 50 |  |  | - | - |
|  | SCC/CC3 | 50 |  |  | - | - |
|  | SCC/CC4 | 50 |  |  | - | - |
|  | SCC/CC5 | 50 |  |  | - | - |
|  | SCC/CC6 | 50 |  |  | - | - |
|  | SCC/CC7 | 50 |  |  | - | - |
|  | SCC/CC8 | 50 |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same cumulative aggregated BW, only the combination with the lowest PCC ChBW is tested. NOTE 4: When the signalled DC carrier position is at Inner_Partial_Left for PC2, PC3, PC4, use Inner_Partial_Right for UL RB allocation. When the signalled DC carrier position is in Inner_Partial_Left_Region2 for PC1, use Inner_Partial_Right_Region2 for UL RB allocation. NOTE 5: PCC/CCi and SCC/CCj means PCC is on component carrier CCi and SCC is on component carrier CCj, with CCi or CCj frequencies defined in TS 38.508-1 [10]. |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.3.1	In-band emissions for CA (2UL CA)

<Unchanged Text Skipped>

6.4A.2.3.1.4	Test description

6.4A.2.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in Table 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in table 6.4A.2.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.3.1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping | ChBw(MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | Default | N/A |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
| 2 | PCC |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
| 3 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
| 4 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1 |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.3.2	In-band emissions for CA (3UL CA)

<Unchanged Text Skipped>

6.4A.2.3.2.4	Test description

Same as in clause 6.4A.2.3.1.4 with following exceptions:

-	Instead of Table 6.4A.2.3.1.4.1-1  use Table 6.4A.2.3.2.4.1-1.

Table 6.4A.2.3.2.4.1-1: Test Configuration Table for 3UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping | ChBw(MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | Default | N/A |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
| 2 | PCC |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
| 3 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
| 4 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1 |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.3.3	In-band emissions for CA (4UL CA)

<Unchanged Text Skipped>

6.4A.2.3.3.4	Test description

Same as in clause 6.4A.2.3.1.4 with following exceptions:

-	Instead of Table 6.4A.2.3.1.4.1-1  use Table 6.4A.2.3.3.4.1-1.

Table 6.4A.2.3.3.4.1-1: Test Configuration Table for 4UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping | ChBw(MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | Default | N/A |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
| 2 | PCC |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
| 3 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
| 4 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1 |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.3.4	In-band emissions for CA (5UL CA)

<Unchanged Text Skipped>

6.4A.2.3.4.4	Test description

Same as in clause 6.4A.2.3.1.4 with following exceptions:

-	Instead of Table 6.4A.2.3.1.4.1-1  use Table 6.4A.2.3.4.4.1-1.

Table 6.4A.2.3.4.4.1-1: Test Configuration Table for 5UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping | ChBw(MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | Default | N/A |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
| 2 | PCC |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
| 3 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
| 4 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1 |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.3.5	In-band emissions for CA (6UL CA)

<Unchanged Text Skipped>

6.4A.2.3.5.4	Test description

Same as in clause 6.4A.2.3.1.4 with following exceptions:

-	Instead of Table 6.4A.2.3.1.4.1-1  use Table 6.4A.2.3.5.4.1-1.

Table 6.4A.2.3.5.4.1-1: Test Configuration Table for 6UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping | ChBw(MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | Default | N/A |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
| 2 | PCC |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
| 3 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
| 4 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1 |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.3.6	In-band emissions for CA (7UL CA)

<Unchanged Text Skipped>

6.4A.2.3.6.4	Test description

Same as in clause 6.4A.2.3.1.4 with following exceptions:

-	Instead of Table 6.4A.2.3.1.4.1-1  use Table 6.4A.2.3.6.4.1-1.

Table 6.4A.2.3.6.4.1-1: Test Configuration Table for 7UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping | ChBw(MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | Default | N/A |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
|  | SCC6 |  |  |  | - | - |
| 2 | PCC |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
|  | SCC6 |  |  |  | - | - |
| 3 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
|  | SCC6 |  |  |  | - | - |
| 4 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
|  | SCC6 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1 |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.4A.2.3.7	In-band emissions for CA (8UL CA)

<Unchanged Text Skipped>

6.4A.2.3.7.4	Test description

Same as in clause 6.4A.2.3.1.4 with following exceptions:

-	Instead of Table 6.4A.2.3.1.4.1-1  use Table 6.4A.2.3.7.4.1-1.

Table 6.4A.2.3.7.4.1-1: Test Configuration Table for 8UL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause [4.3.1.2.3] for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low and High range |  |  |
| Test CC combination setting as specified in subclause 5.5A.1-1, 5.5A.2-1 and 5.5A.2-2 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest aggregated BW of the CA configuration Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1. |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | CC & Mapping | ChBw(MHz) | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC | Default | N/A |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
|  | SCC6 |  |  |  | - | - |
|  | SCC7 |  |  |  | - | - |
| 2 | PCC |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
|  | SCC6 |  |  |  | - | - |
|  | SCC7 |  |  |  | - | - |
| 3 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Left for PC2, PC3, PC4 Inner_Partial_Left_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
|  | SCC6 |  |  |  | - | - |
|  | SCC7 |  |  |  | - | - |
| 4 | PCC |  |  |  | CP-OFDM QPSK | Inner_Partial_Right for PC2, PC3, PC4 Inner_Partial_Right_Region2 for PC1 |
|  | SCC1 |  |  |  | - | - |
|  | SCC2 |  |  |  | - | - |
|  | SCC3 |  |  |  | - | - |
|  | SCC4 |  |  |  | - | - |
|  | SCC5 |  |  |  | - | - |
|  | SCC6 |  |  |  | - | - |
|  | SCC7 |  |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: CA Configuration Test cumulative aggregated BW settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1 |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.5.3	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions. The spurious emission limits are specified in terms of general requirements in line with SM.329 [7] and NR operating band requirement to address UE co-existence. Spurious emissions are measured as TRP.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5.3.1	Transmitter Spurious emissions

<Unchanged Text Skipped>

6.5.3.1.4	Test description

6.5.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the Subscriber Station (SS) to take with the UE to reach the correct measurement state.

Table 6.5.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s -OFDM QPSK | Inner_Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
| 2 |  | DFT-s -OFDM QPSK | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_Partial_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_Partial_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. |  |  |  |

<Unchanged Text Skipped>

6.5.3.2	Spurious emission band UE co-existence

<Unchanged Text Skipped>

6.5.3.2.4	Test description

6.5.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the Subscriber Station (SS) to take with the UE to reach the correct measurement state.

Table 6.5.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
| 2 |  | DFT-s-OFDM QPSK | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_Partial_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_Partial_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. |  |  |  |

<Unchanged Text Skipped>

6.5.3.3	Additional Spurious emission

<Unchanged Text Skipped>

6.5.3.3.4	Test description

6.5.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the Subscriber Station (SS) to take with the UE to reach the correct measurement state.

Table 6.5.3.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
| 2 |  | DFT-s-OFDM QPSK | Inner_1RB_Left for PC2, PC3 and PC4 Inner_1RB_Left_Region1 for PC1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |

<End of Changes >