3GPP TSG-RAN5 Meeting #90-e	R5-210728r1

# Electronic, 22nd February - 5th March, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0470 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of parameter configuration for open loop power control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Power Configuration The power configurations based on open loop power control in several test cases are incorrect. The UE transmitted power is decided by P0 and alpha, where P0 = P0_nominal+P0_UE. Given the PUSCH is scheduled with DCI not including SRI, the P0_nominal = P0-NominalWithGrant, and P0_UE / alpha are determined from first P0-PUSCH-ALphaSet. The default P0-NominalWithGrant=-90dBm in 38.508-1. In the first P0-PUSCH-ALphaSet, P0=0dBm, Alpha-=0.8. In several test cases, the P0 and Alpha need to be configured differently from default value, however the configurations are not correct. - SSB SCS In FR2, SSB SCS can only be 120kHz and 240kHz, but in several test cases 60kHz SSB is considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The power configuration - In 6.3.3.2, 6.3.4.2, 6.3A.4.2.1.4.3, 6.3D.3.1.4.3 The ‘tpc-Accumulation’ and ‘alpha’ should be configured in PUSCH-PowerControl rather than PUSCH-ConfigCommon. - In 6.3.4.2, 6.3A.4.2.1.4.3-3 Changing P0-Nominal in PUSCH-ConfigCommon to reach different test points instead of changing P0-UE in PUSCH-PowerControl, otherwise the P0 would be ultra low by adding P0-Nominal and P0-UE together. - SSB SCS Replacing 60kHz by 240kHz for SSB SCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test condition is not set up properly. UE might fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.2, 6.3.4.2, 6.3A.4.2.1, 6.3D.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Adding Clauses affected in coversheet. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.3.3.2	General ON/OFF time mask

<Unchanged Text Skipped>

6.3.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with following exceptions.

Table 6.3.3.2.4.3-1: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -102 |  | 50MHz |
|  | -106 |  | 100MHz |
|  | -108 |  | 200MHz |
|  | -112 |  | 400MHz |
| } |  |  |  |

Table 6.3.3.2.4.3-2: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 4 |  | SCS_120kHz |
|  | 7 |  | SCS_240kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_120kHz | SCS=120kHz for SS/PBCH block |
| SCS_240kHz | SCS=240kHz for SS/PBCH block |

Table 6.3.3.2.4.3-3: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| tpc-Accumulation | disabled |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

6.3.4.2	Absolute power tolerance

<Unchanged Text Skipped>

6.3.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with following exceptions:

Table 6.3.4.2.4.3-1: PUSCH-ConfigCommon (Test point 1) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -116 |  | 50MHz |
|  | -120 |  | 100MHz |
|  | -122 |  | 200MHz |
|  | -126 |  | 400MHz |
| } |  |  |  |

Table 6.3.4.2.4.3-2: PUSCH-ConfigCommon (Test point 2) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -112 |  | 50MHz |
|  | -116 |  | 100MHz |
|  | -118 |  | 200MHz |
|  | -122 |  | 400MHz |
| } |  |  |  |

Table 6.3.4.2.4.3-3: PUSCH-PowerControl (Test point 3) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -102 |  | 50MHz |
|  | -106 |  | 100MHz |
|  | -108 |  | 200MHz |
|  | -112 |  | 400MHz |
| } |  |  |  |
| } |  |  |  |

Table 6.3.4.2.4.3-4: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 4 |  | SCS_120kHz |
|  | 7 |  | SCS_240kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_120kHz | SCS=120kHz for SS/PBCH block |
| SCS_240kHz | SCS=240kHz for SS/PBCH block |

Table 6.3.4.2.4.3-5: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| tpc-Accumulation | disabled |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

6.3A.4.2.1	Absolute power tolerance for CA (2UL CA)

<Unchanged Text Skipped>

6.3A.4.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with following exceptions:

Table 6.3A.4.2.1.4.3-1: PUSCH-ConfigCommon (Test point 1) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -116 |  | 50MHz |
|  | -120 |  | 100MHz |
|  | -122 |  | 200MHz |
|  | -126 |  | 400MHz |
| } |  |  |  |

Table 6.3A.4.2.1.4.3-2: PUSCH-ConfigCommon (Test point 2) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -112 |  | 50MHz |
|  | -116 |  | 100MHz |
|  | -118 |  | 200MHz |
|  | -122 |  | 400MHz |
| } |  |  |  |

Table 6.3A.4.2.1.4.3-3: PUSCH-ConfigCommon (Test point 3) for power class 3

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -102 |  | 50MHz |
|  | -106 |  | 100MHz |
|  | -108 |  | 200MHz |
|  | -112 |  | 400MHz |
| } |  |  |  |

Table 6.3A.4.2.1.4.3-5: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 4 |  | SCS_120kHz |
|  | 7 |  | SCS_240kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_120kHz | SCS=120kHz for SS/PBCH block |
| SCS_240kHz | SCS=240kHz for SS/PBCH block |

Table 6.3A.4.2.1.4.3-6: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| tpc-Accumulation | disabled |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<Unchanged Text Skipped>

6.3D.3.1	General ON/OFF time mask for UL MIMO

<Unchanged Text Skipped>

6.3D.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

Table 6.3D.3.1.4.3-1: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -102 |  | 50MHz |
|  | -106 |  | 100MHz |
|  | -108 |  | 200MHz |
|  | -112 |  | 400MHz |
| } |  |  |  |

Table 6.3D.3.1.4.3-2: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 4 |  | SCS_120kHz |
|  | 7 |  | SCS_240kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_120kHz | SCS=120kHz for SS/PBCH block |
| SCS_240kHz | SCS=240kHz for SS/PBCH block |

Table 6.3D.3.1.4.3-3: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| tpc-Accumulation | disabled |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changes >