3GPP TSG-RAN5 Meeting #90-e	R5-210744r1

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0155 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding PICS for UL switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test cases for UL switching between carriers are introduced to RF specifications. The necessary PICS need to be added so that applicability can be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding PICS for UL switching in A.4.2.1. Adding new section A.4.3.2C for SUL baseline Implementation capabilities. Adding new column for UE to indicate supported ULTxSwitching Band pair in CA/EN-DC/SUL configuration tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test applicabilty for UL switching test cases can’t be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2, A.4.3.2A.4, A.4.3.2B.2.3, A.4.3.2.C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0057 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Adding Clause affected in coversheet. Correcting numbering of A.4.3.2C |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| 2 | Support 256QAM for PDSCH for FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_256QAM_FR1 | Yes |  |  |
| 3 | Support 256QAM for PDSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pdsch_256QAM_FR2 | No |  |  |
| 4 | Support 256QAM for PUSCH for at least one NR FR1 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR1 | No |  |  |
| 4a | Support 256QAM for PUSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR2 | No |  |  |
| 5 | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeA | Yes |  |  |
| 6 | Support receiving PDSCH using PDSCH mapping type B | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeB | Yes |  |  |
| 7 | Support resource allocation Type 0 for PUSCH | 38.306, 4.2.7.10 | Rel-15 | pc_ra_Type0_PUSCH | No |  |  |
| 8 | Support scaling factor 0.75 is applied to the band in the max data rate calculation | 38.306, 4.2.7 | Rel-15 | pc_scalingFactor0dot75 |  |  |  |
| 9 | Support reconfiguration with sync using a contention free random access on PRACH resources that are associated with CSI-RS resources of the target cell | 38.306, 4.2.7.10 | Rel-15 | pc_csi_RS_CFRA_ForHO | No |  |  |
| 10 | Support Type 1 PUSCH transmissions with configured grant | 38.306, 4.2.7.10 | Rel-15 | pc_configuredUL_GrantType1 | No |  |  |
| 11 | Support Type 2 PUSCH transmissions with configured grant | 38.306, 4.2.7.10 | Rel-15 | pc_configuredUL_GrantType2 | No |  |  |
| 12 | Support PDSCH Reception when configured with higher layer parameter aggregationFactorDL > 1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_RepetitionMultiSlots | No |  |  |
| 13 | Supports supplemental uplink with dynamic switch (DCI based selection of PUSCH carrier) | 38.306, 4.2.7.7 | Rel-15 | pc_dynamicSwitchSUL | No |  |  |
| 14 | Supports MIMO layers at the UE for PUSCH transmission with codebook precoding. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset | 38.306, 4.2.7.8 | Rel-15 | pc_nrMIMO_CB_PUSCH | No |  | Set to true if maxNumberMIMO-LayersCB-PUSCH has value |
| 15 | Supports MIMO layers at the UE for PUSCH transmission using non-codebook precoding | 38.306, 4.2.7.8 | Rel-15 | pc_nrMIMO_NonCB_PUSCH | No |  | Set to true if maxNumberMIMO-LayersNonCB-PUSCH has value |
| 16 | Support receiving PDSCH with interleaved VRB-to-PRB mapping | 38.306, 4.2.7.10 | Rel-15 | pc_interleavingVRB_ToPRB_PDSCH | Yes |  |  |
| 17 | Support dynamic EN-DC power sharing for at least one EN-DC band combination | 38.306, 4.2.7.9 | Rel-15 | pc_dynamicPowerSharing | Yes |  | If the UE supports this capability it will dynamically share the power between NR and LTE if P_LTE + P_NR > Pcmax. |
| 18 | Supports up to 10 search spaces in a SCell per BWP | 38.306, 4.2.7.10 | Rel-15 | pc_maxNumberSearchSpaces | No |  |  |
| 19 | Supports spatial bundling of HARQ-ACK bits carried on PUCCH or PUSCH per PUCCH group. With spatial bundling, two HARQ-ACK bits for a DL MIMO data is bundled into a single bit by logical "AND" operation | 38.306, 4.2.7.10 | Rel-15 | pc_spatialBundlingHARQ_ACK | Yes |  |  |
| 20 | Support alternative additional DMRS position for co-existence with LTE CRS | 38.306, 4.2.7.5 | Rel-15 | pc_additionalDMRS_DL_Alt | No |  |  |
| 21 | Supports transmitting PUSCH scheduled by DCI format 0_0 or 0_1 when configured with higher layer parameter aggregationFactorIUL > 1 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_RepetitionMultiSlots | Yes |  |  |
| 22 | Support beam correspondence without UL beam sweeping | 38.306, 4.2.7.2 | Rel-15 | pc_beamCorrespondenceWithoutUL_BeamSweeping | Yes |  | A UE that can fulfil the requirements without UL beam sweeping then set the bit to 1. A UE that can fulfil the requirements with UL beam sweeping then set the bit to 0. |
| 23 | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier | 38.306, 4.2.7.6 | Rel-15 | pc_maxNumberMIMO_LayersPDSCH | CY |  |  |
| 24 | Supports DCI and timer based active BWP switching delay type1 or type2 | 38.306, 4.2.7.10 | Rel-15 | pc_bwp_SwitchingDelay | Yes |  |  |
| 25 | Support modified MPR behaviour | 38.306 4.2.7.2 | Rel-15 | pc_modifiedMPR_behaviour | No |  |  |
| 26 | Support dynamic switching between resource allocation Types 0 and 1 for PDSCH | 38.306, 4.2.7.10 | Rel-15 | pc_dynamicSwitchRA_Type0_1_PDSCH | No |  |  |
| 27 | Support dynamic switching between resource allocation Types 0 and 1 for PUSCH | 38.306, 4.2.7.10 | Rel-15 | pc_dynamicSwitchRA_Type0_1_PUSCH | No |  |  |
| 28 | Support almost contiguous UL CP-OFDM transmissions in FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_almostContiguousCP_OFDM_UL_FR1 | No |  |  |
| 29 | Support almost contiguous UL CP-OFDM transmissions in FR2 | 38.306, 4.2.7.10 | Rel-15 | pc_almostContiguousCP_OFDM_UL_FR2 | No |  |  |
| 30 | Support dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1, and the minimum scheduling offset for PDSCH and aperiodic CSI-RS triggering offset (K0), and PUSCH (K2), and the extended value range for aperiodic CSI-RS triggering offset | 38.306, 4.2.7.10 | Rel-16 | pc_crossSlotScheduling | No |  |  |
| 31 | Supports pi/2-BPSK modulation scheme for PUSCH in FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_halfpiBPSK | No |  |  |
| 32 | Support multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency | 38.306, 4.2.7.6 | Rel-16 | pc_multiDCI_MultiTRP_r16 | No |  |  |
| 33 | Support receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS | 38.306, 4.2.7.2 | Rel-15 | pc_rateMatchingLTE_CRS | Yes |  |  |
| 34 | Support of BWP operation without bandwidth restriction | 38.306, 4.2.7.2 | Rel-15 | pc_bwp-WithoutRestriction | No |  |  |
| aa | Support of dynamic UL Tx switching | 38.306, 4.2.7.1 | Rel-16 | pc_ULTxSwitchingBandPair | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2A.4.2-3, Table A.4.3.2A.4.3-3, Table A.4.3.2B.2.3.1-2, Table A.4.3.2B.2.3.2-2, Table A.4.3.2B.2.3.3-2, Table A.4.3.2B.2.3.4-2, Table A.4.3.2B.2.3.5-2 and Table A.4.3.2C.2-1 |

<Unchanged Text Skipped>

A.4.3.2A.4	NR Inter-band CA within FR1

A.4.3.2A.4.1	NR Inter-band CA within FR1 (two bands)

Table A.4.3.2A.4.1-1: Downlink Bandwidth Class Combination capabilities for NR Inter-band CA configuration within FR1 and two bands (for one or more of the supported CA configurations in Table A.4.3.2A.4.1-3)

| Item | DL NR FR1 Inter-band CA Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR FR1 Inter-band CA BW Class Combination A-A (two bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_2B_Class_A-A |  |
| 2 | DL NR FR1 Inter-band CA BW Class Combination A-(2A) (two bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_2B_Class_A-(2A) |  |
| 3 | DL NR FR1 Inter-band CA BW Class Combination A-B (two bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_2B_Class_A-B |  |
| 4 | DL NR FR1 Inter-band CA BW Class Combination A-C (two bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_2B_Class_A-C |  |
| 5 | DL NR FR1 Inter-band CA BW Class Combination (2A)-A (two bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_2B_Class_(2A)-A |  |
| 6 | DL NR FR1 Inter-band CA BW Class Combination (2A)-(2A) (two bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_2B_Class_(2A)-(2A) |  |
| 7 | DL NR FR1 Inter-band CA BW Class Combination (2A)-B (two bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_2B_Class_(2A)-B |  |
| 8 | DL NR FR1 Inter-band CA BW Class Combination B-A (two bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_2B_Class_B-A |  |
| 9 | DL NR FR1 Inter-band CA BW Class Combination C-A (two bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_2B_Class_C-A |  |
| 10 | DL NR FR1 Inter-band CA BW Class Combination C-B (two bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_2B_Class_C-B |  |

Table A.4.3.2A.4.1-2: Uplink Bandwidth Class Combination capabilities for NR Inter-band CA within FR1 and two bands (for one or more of the supported CA configurations in Table A.4.3.2A.4.1-3)

| Item | UL NR FR1 Inter-band CA Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR FR1 Inter-band CA BW Class Combination A-A (two bands) | 38.101-1, 5.3A.5 | pc_UL_inter_band_CANR_FR1_2B_Class_A-A |  |

Table A.4.3.2A.4.1-3: Supported configurations for NR Inter-band CA within FR1 and two bands

| NR FR1 Inter-band CA configuration / Item (Note 1) | Release | Supported | Supported CA Bandwidth Class(es) in UL (Note 2,5) | Supported Bandwidth Combination Set(s) (Note 3) | Supported ULTxSwitching Band Pair (Note 7, 8) |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n77A | Rel-16 |  |  |  |  |
| CA_n1A-n78A | Rel-16 |  |  |  |  |
| CA_n1A-n78C | Rel-16 |  |  |  |  |
| CA_n3A-n78A | Rel-15 |  |  |  |  |
| CA_n8A-n78A | Rel-15 |  |  |  |  |
| CA_n29A-n66A | Rel-16 |  |  |  |  |
| CA_n29A-n66B | Rel-16 |  |  |  |  |
| CA_n29A-n66(2A) | Rel-16 |  |  |  |  |
| CA_n29A-n70A | Rel-16 |  |  |  |  |
| CA_n41A-n79A | Rel-16 |  |  |  |  |
| CA_n66A-n70A (Note 6) | Rel-16 |  |  |  |  |
| CA_n66B-n70A (Note 6) | Rel-16 |  |  |  |  |
| CA_n66(2A)-n70A (Note 6) | Rel-16 |  |  |  |  |
| CA_n66A-n71A (Note 6) | Rel-16 |  |  |  |  |
| CA_n66B-n71A (Note 6) | Rel-16 |  |  |  |  |
| CA_n66(2A)-n71A (Note 6) | Rel-16 |  |  |  |  |
| CA_n70A-n71A | Rel-16 |  |  |  |  |
| CA_n78A-n79A | Rel-15 |  |  |  |  |
| Note 1: Notation used for inter-band CA Bands is according to TS 38.101-1 [23] Table 5.5A.3-1, e.g. ‘CA_n1A-n78C’ indicates CA operation on NR band n1 and n78 with DL CA Bandwidth Class A and C respectively. Note 2: The UL CA capabilities as per Table A.4.3.2A.4.1-2 can be supported on a single or multiple CA Band(s). The UE supplier shall indicate all supported UL CA Bandwidth Class(es), in uplink of the supported CA Band(s), as per TS 38.101-1 [23] Table 5.5A.1-1. For this release of specification valid choices are ’N’, ‘nXB’ and ‘nXC’, where nX is the NR band. For example, for CA_n1B, N would mean only DL CA, ‘n1B’ would mean both DL and UL CA. Note 3: The UE supplier shall indicate the supported Bandwidth Combination Set(s) as per TS 38.101-1 [23] Table 5.5A.3-1. Note 4: Reference to all items is 38.101-1 [23], 5.5A.3 and 38.331, 6.3.4 Note 5: UL(Table A.4.3.2A.4.1-3) shall return all supported CA Configurations where at least one UL CA Bandwidth Class was declared in column "Supported CA Bandwidth Class(es) in UL". UL_2CC(Table A.4.3.2A.4.1-3) shall return all supported CA Configurations where at least one 2 Carrier UL CA Bandwidth Class was declared in column "Supported CA Bandwidth Class(es) in UL". UL_3CC(Table A.4.3.2A.4.1-3) shall return all supported CA Configurations where at least one 3 Carrier UL CA Bandwidth Class was declared. Note 6: A UE that supports NR Band n66 (Table A.4.3.1-1) and CA operation in any CA band shall also support the DL CA configurations CA_n66B and CA_n66(2A), as per Note 7, in Table 5.2-1, in TS 38.521-1 [5]. Note 7: The ULTxSwitching capability can be reported on inter-band CA band combinations. The UE supplier shall indicate inter-band CA band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for CA_n1A-n77A, N would mean not supporting ULTxSwitching, ‘n1-n77’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at lease one inter-band UL CA configuration on the same band pair in the column “Supported CA Bandwidth Class(es) in UL”. Note 8: ULSwitching(Table A.4.3.2A.4.1-3) shall return all supported CA Configurations where at least one UL CA band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

A.4.3.2A.4.2	NR Inter-band CA within FR1 (three bands)

Table A.4.3.2A.4.2-1: Downlink Bandwidth Class Combination capabilities for NR Inter-band CA configuration within FR1 and three bands (for one or more of the supported CA configurations in Table A.4.3.2A.4.2-3)

| Item | DL NR FR1 Inter-band CA Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR FR1 Inter-band CA BW Class Combination A-A-A (three bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_3B_Class_A-A-A |  |
| 2 | DL NR FR1 Inter-band CA BW Class Combination A-A-(2A) (three bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_Ccqf3qFYW%J A_NR_FR1_3B_Class_A-A-(2A) |  |
| 3 | DL NR FR1 Inter-band CA BW Class Combination A-A-B (three bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_3B_Class_A-A-B |  |
| 4 | DL NR FR1 Inter-band CA BW Class Combination A-(2A)-A (three bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_3B_Class_A-(2A)-A |  |
| 5 | DL NR FR1 Inter-band CA BW Class Combination A-B-A (three bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_3B_Class_A-B-A |  |
| 6 | DL NR FR1 Inter-band CA BW Class Combination A-C-A (three bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_3B_Class_A-C-A |  |
| 7 | DL NR FR1 Inter-band CA BW Class Combination (2A)-A-A (three bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_3B_Class_(2A)-A-A |  |
| 8 | DL NR FR1 Inter-band CA BW Class Combination B-A-A (three bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_3B_Class_B-A-A |  |
| 9 | DL NR FR1 Inter-band CA BW Class Combination C-A-A (three bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_3B_Class_C-A-A |  |

Table A.4.3.2A.4.2-2: Uplink Bandwidth Class Combination capabilities for NR Inter-band CA within FR1 and three bands (for one or more of the supported CA configurations in Table A.4.3.2A.4.2-3)

| Item | UL NR FR1 Inter-band CA Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR FR1 Inter-band CA BW Class Combination A-A-A (three bands) | 38.101-1, 5.3A.5 | pc_UL_inter_band_CA_NR_FR1_3B_Class_A-A-A |  |

Table A.4.3.2A.4.2-3: Supported configurations for NR Inter-band CA within FR1 and three bands

| NR FR1 Inter-band CA configuration / Item (Note 1) | Release | Supported | Supported CA Bandwidth Class(es) in UL (Note 2,5) | Supported Bandwidth Combination Set(s) (Note 3) | Supported ULTxSwitching Band Pair (Note 7, 8) |
| --- | --- | --- | --- | --- | --- |
| CA_n29A-n66A-n70A | Rel-16 |  |  |  |  |
| CA_n66A-n70A-n71A (Note 6) | Rel-16 |  |  |  |  |
| CA_n66B-n70A-n71A (Note 6) | Rel-16 |  |  |  |  |
| CA_n66(2A)-n70A-n71A (Note 6) | Rel-16 |  |  |  |  |
| Note 1: Notation used for inter-band CA Bands is according to TS 38.101-1 [23] Table 5.5A.3-2, e.g. ‘CA_n66B-n70A-n71A’ indicates CA operation on NR band n66, n70 and n71 with DL CA Bandwidth Class B, A and A respectively. Note 2: The UL CA capabilities as per Table A.4.3.2A.4.2-2 can be supported on a single or multiple CA Band(s). The UE supplier shall indicate all supported UL CA Bandwidth Class(es), in uplink of the supported CA Band(s), as per TS 38.101-1 [23] Table 5.5A.1-1. For this release of specification valid choices are ’N’, ‘nXB’ and ‘nXC’, where nX is the NR band. For example, for CA_n1B, N would mean only DL CA, ‘n1B’ would mean both DL and UL CA. Note 3: The UE supplier shall indicate the supported Bandwidth Combination Set(s) as per TS 38.101-1 [23] Table 5.5A.3-2. Note 4: Reference to all items is 38.101-1 [23], 5.5A.3 and 38.331, 6.3.4 Note 5: UL(Table A.4.3.2A.4.2-3) shall return all supported CA Configurations where at least one UL CA Bandwidth Class was declared in column "Supported CA Bandwidth Class(es) in UL". UL_2CC(Table A.4.3.2A.4.2-3) shall return all supported CA Configurations where at least one 2 Carrier UL CA Bandwidth Class was declared in column "Supported CA Bandwidth Class(es) in UL". UL_3CC(Table A.4.3.2A.4.2-3) shall return all supported CA Configurations where at least one 3 Carrier UL CA Bandwidth Class was declared. Note 6: A UE that supports NR Band n66 (Table A.4.3.1-1) and CA operation in any CA band shall also support the DL CA configurations CA_n66B and CA_n66(2A), as per Note 7, in Table 5.2-1, in TS 38.521-1 [5]. Note 7: The ULTxSwitching capability can be reported on inter-band CA band combinations. The UE supplier shall indicate inter-band CA band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for CA_n1A-n77A, N would mean not supporting ULTxSwitching, ‘n1-n77’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at lease one UL CA configuration on the same band pair in the column “Supported CA Bandwidth Class(es) in UL”. Note 8: ULSwitching(Table A.4.3.2A.4.2-3) shall return all supported CA Configurations where at least one UL CA band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

A.4.3.2A.4.3	NR Inter-band CA within FR1 (four bands)

Table A.4.3.2A.4.3-1: Downlink Bandwidth Class Combination capabilities for NR Inter-band CA configuration within FR1 and four bands (for one or more of the supported CA configurations in Table A.4.3.2A.4.3-3)

| Item | DL NR FR1 Inter-band CA Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR FR1 Inter-band CA BW Class Combination A-A-A-A (four bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_4B_Class_A-A-A-A |  |
| 2 | DL NR FR1 Inter-band CA BW Class Combination A-A-B-A (four bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_4B_Class_A-A-B-A |  |
| 3 | DL NR FR1 Inter-band CA BW Class Combination A-B-A-A (four bands) | 38.101-1, 5.3A.5 | pc_DL_inter_band_CA_NR_FR1_4B_Class_A-B-A-A |  |

Table A.4.3.2A.4.3-2: Uplink Bandwidth Class Combination capabilities for NR Inter-band CA within FR1 and four bands (for one or more of the supported CA configurations in Table A.4.3.2A.4.3-3)

TBD

Table A.4.3.2A.4.2-3: Supported configurations for NR Inter-band CA within FR1 and four bands

TBD

<Unchanged Text Skipped>

A.4.3.2B.2.3	Inter-band EN-DC

A.4.3.2B.2.3.1	Inter-band EN-DC within FR1 (two bands)

Table A.4.3.2B.2.3.1-1: Downlink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and two bands (for one or more of the supported DC configurations in Table A.4.3.2B.2.3.1-2)

| Item | DL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | Inter-band EN-DC within FR1 BW Class Combination A_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_A |  |
| 2 | Inter-band EN-DC within FR1 BW Class Combination A_(2A) (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_(2A) |  |
| 3 | Inter-band EN-DC within FR1 BW Class Combination A_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_B |  |
| 4 | Inter-band EN-DC within FR1 BW Class Combination A_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_A_C |  |
| 5 | Inter-band EN-DC within FR1 BW Class Combination (2A)_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(2A)_A |  |
| 6 | Inter-band EN-DC within FR1 BW Class Combination (2A)_(2A) (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(2A)_(2A) |  |
| 7 | Inter-band EN-DC within FR1 BW Class Combination (2A)_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(2A)_B |  |
| 8 | Inter-band EN-DC within FR1 BW Class Combination (3A)_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_(3A)_A |  |
| 9 | Inter-band EN-DC within FR1 BW Class Combination B_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_B_A |  |
| 10 | Inter-band EN-DC within FR1 BW Class Combination C_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_A |  |
| 11 | Inter-band EN-DC within FR1 BW Class Combination C_(2A) (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_(2A) |  |
| 12 | Inter-band EN-DC within FR1 BW Class Combination C_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_B |  |
| 13 | Inter-band EN-DC within FR1 BW Class Combination C_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_C_C |  |
| 14 | Inter-band EN-DC within FR1 BW Class Combination D_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_D_A |  |
| 15 | Inter-band EN-DC within FR1 BW Class Combination D_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_D_C |  |
| 16 | Inter-band EN-DC within FR1 BW Class Combination E_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_E_A |  |
| 17 | Inter-band EN-DC within FR1 BW Class Combination E_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_DL_inter_band_EN_DC_FR1_2B_Class_E_C |  |

Table A.4.3.2B.2.3.1-1a: Uplink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and two bands (for one or more of the supported configurations in Table A.4.3.2B.2.3.1-2 )

| Item | UL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Inter-band EN-DC within FR1 BW Class Combination A_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_A_A |  |
| 2 | UL Inter-band EN-DC within FR1 BW Class Combination A_B (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_A_B |  |
| 3 | UL Inter-band EN-DC within FR1 BW Class Combination A_C (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_A_C |  |
| 4 | UL Inter-band EN-DC within FR1 BW Class Combination (2A)_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_(2A)_A |  |
| 5 | UL Inter-band EN-DC within FR1 BW Class Combination C_A (two bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.1 | pc_UL_inter_band_EN_DC_FR1_2B_Class_C_A |  |

Table A.4.3.2B.2.3.1-2: Supported Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration / Item (Note 1) | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- |
| DC_1A_n3A | Rel-16 |  |  |  |  |
| DC_1A_n28A | Rel-15 |  |  |  |  |
| DC_1A_n77A | Rel-15 |  |  |  |  |
| DC_1A_n78A | Rel-15 |  |  |  |  |
| DC_1A_n78C | Rel-15 |  |  |  |  |
| DC_1A_n79A | Rel-15 |  |  |  |  |
| DC_2A_n5A | Rel-15 |  |  |  |  |
| DC_2A_n41A | Rel-16 |  |  |  |  |
| DC_2C_n41A | Rel-16 |  |  |  |  |
| DC_2A_n71A | Rel-15 |  |  |  |  |
| DC_3A_n1A | Rel-16 |  |  |  |  |
| DC_3A_n7A | Rel-15 |  |  |  |  |
| DC_3A_n28A | Rel-15 |  |  |  |  |
| DC_3A_n41A | Rel-16 |  |  |  |  |
| DC_3A_n77A | Rel-15 |  |  |  |  |
| DC_3A_n78A | Rel-15 |  |  |  |  |
| DC_3A_n79A | Rel-15 |  |  |  |  |
| DC_3C_n78A | Rel-15 |  |  |  |  |
| DC_5A_n2A | Rel-16 |  |  |  |  |
| DC_5A_n66A | Rel-15 |  |  |  |  |
| DC_5A_n78A | Rel-15 |  |  |  |  |
| DC_7A_n1A | Rel-16 |  |  |  |  |
| DC_7A_n3A | Rel-16 |  |  |  |  |
| DC_7A_n28A | Rel-15 |  |  |  |  |
| DC_7A_n78A | Rel-15 |  |  |  |  |
| DC_7A_n66A | Rel-15 |  |  |  |  |
| DC_7C_n66A | Rel-15 |  |  |  |  |
| DC_7C_n78A | Rel-15 |  |  |  |  |
| DC_8A_n1A | Rel-16 |  |  |  |  |
| DC_8A_n3A | Rel-16 |  |  |  |  |
| DC_8A_n41A | Rel-16 |  |  |  |  |
| DC_8A_n78A | Rel-15 |  |  |  |  |
| DC_12A_n66A | Rel-15 |  |  |  |  |
| DC_12A_n78A | Rel-16 |  |  |  |  |
| DC_13A_n2A | Rel-16 |  |  |  |  |
| DC_13A_n66A | Rel-15 |  |  |  |  |
| DC_14A_n2A | Rel-16 |  |  |  |  |
| DC_14A_n66A | Rel-16 |  |  |  |  |
| DC_28A_n3A | Rel-16 |  |  |  |  |
| DC_48A_n5A | Rel-16 |  |  |  |  |
| DC_48A_n66A | Rel-16 |  |  |  |  |
| DC_19A_n77A | Rel-15 |  |  |  |  |
| DC_19A_n78A | Rel-15 |  |  |  |  |
| DC_19A_n79A | Rel-15 |  |  |  |  |
| DC_20A_n28A | Rel-15 |  |  |  |  |
| DC_20A_n78A | Rel-15 |  |  |  |  |
| DC_21A_n77A | Rel-15 |  |  |  |  |
| DC_21A_n78A | Rel-15 |  |  |  |  |
| DC_21A_n79A | Rel-15 |  |  |  |  |
| DC_25A_n41A | Rel-15 |  |  |  |  |
| DC_28A_n77A | Rel-15 |  |  |  |  |
| DC_28A_n78A | Rel-15 |  |  |  |  |
| DC_28A_n79A | Rel-15 |  |  |  |  |
| DC_30A_n5A | Rel-15 |  |  |  |  |
| DC_39A_n41A | Rel-16 |  |  |  |  |
| DC_39A_n79A | Rel-15 |  |  |  |  |
| DC_40A_n1A | Rel-16 |  |  |  |  |
| DC_40A_n41A | Rel-16 |  |  |  |  |
| DC_40A_n78A | Rel-16 |  |  |  |  |
| DC_40C_n78A | Rel-16 |  |  |  |  |
| DC_41A_n79A | Rel-15 |  |  |  |  |
| DC_66A_n2A | Rel-16 |  |  |  |  |
| DC_66A_n5A | Rel-15 |  |  |  |  |
| DC_66A_n41A | Rel-16 |  |  |  |  |
| DC_66A_n71A | Rel-15 |  |  |  |  |
| DC_66A_n78A | Rel-15 |  |  |  |  |
| Note 1: Notation used for inter-band EN-DC Bands is according to TS 38.101-3 [25] Table 5.5B.4.1-1, e.g. ‘DC_1A_n28A’ indicates EN-DC operation on E-UTRA band 1 with E-UTRA DL Bandwidth Class A and NR band n28 with NR DL CA Bandwidth Class A. Note 2: The ULTxSwitching capability can be reported on EN-DC band combinations. The UE supplier shall indicate E-UTRA/NR band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘A-nB’, where A is LTE band and nB is NR band. For example, for DC_3A_n77A, N would mean not supporting ULTxSwitching, ‘3-n77’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at lease one UL EN-DC configuration on the same band pair in the column “Supported EN-DC Bandwidth Class(es) in UL”. Note 3: ULSwitching(Table A.4.3.2B.2.3.1-2) shall return all supported EN-DC Configurations where at least one E-UTRA/NR band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

Table A.4.3.2B.2.3.1-3: Inter-band EN-DC within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities

| Item | Inter-band EN-DC within FR1 (two bands) PC2 UE RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | LTE Frequency band: 1880-1920 MHz NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band39_nrBand41_ PC2_Supp | DC_39A-n41A |
| 2 | LTE Frequency band: 1880-1920 MHz NR Frequency band: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band39_nrBand79_ PC2_Supp | DC_39A-n79A |
| 3 | LTE Frequency band: 2496-2690 MHz NR Frequency band: 4400-5000 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band41_nrBand79_ PC2_Supp | DC_41A-n79A |
| 4 | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 3300-3800 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band3_nrBand78_ PC2_Supp | DC_3A-n78A |
| 5 | LTE Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) NR Frequency band: 2496-2690 MHz | 38.101-3, 6.2B.1.3 | Rel-16 | pc_Band3_nrBand41_ PC2_Supp | DC_3A-n41A DC_3A-n41C |

Table A.4.3.2B.2.3.1-4: UE Power Class implementation Capabilities for inter-band EN-DC within FR1 (two bands)

| Item | UE Power Class implementation Capabilities | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | UE Power Class 2 for Inter-band EN-DC within FR1 (two bands) | 38.101-3, 6.2B.1.3 | Applicable to the bands in Table A.4.3.2B.2.3.1-3 |
| 2 | UE Power Class 3 for Inter-band EN-DC within FR1 (two bands) | 38.101-3, 6.2B.1.3 | Applicable to the bands in Table A.4.3.2B.2.3.1-2 |

A.4.3.2B.2.3.2	Inter-band EN-DC within FR1 (three bands)

Table A.4.3.2B.2.3.2-1: Downlink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and three bands (for one or more of the supported DC configurations in Table A.4.3.2B.2.3.2-2)

| Item | DL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic |
| --- | --- | --- | --- |
| 1 | Inter-band EN-DC within FR1 BW Class Combination A-A_A (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-A_A |
| 2 | Inter-band EN-DC withinFR1 BW Class Combination A-A_B (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-A_B |
| 3 | Inter-band EN-DC within FR1 BW Class Combination A-A_C (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-A_C |
| 4 | Inter-band EN-DC within FR1 BW Class Combination A-C_A (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-C_A |
| 5 | Inter-band EN-DC within FR1 BW Class Combination A-C_C (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-C_C |
| 6 | Inter-band EN-DC withinFR1 BW Class Combination A-D_A (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-D_A |
| 7 | Inter-band EN-DC within FR1 BW Class Combination A-E_A (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A-E_A |
| 8 | Inter-band EN-DC within FR1 BW Class Combination A_A-A (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A_A-A |
| 9 | Inter-band EN-DC within FR1 BW Class Combination C-A_A (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_C-A_A |
| 10 | Inter-band EN-DC within FR1 BW Class Combination C-C_A (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_C-C_A |
| 11 | Inter-band EN-DC within FR1 BW Class Combination A_(n)AA (three bands) | 36.101, 5.6A.1 38.101-3. 5.5B.4.2 | pc_DL_inter_band_EN_DC_FR1_3B_Class_A_(n)AA |

Table A.4.3.2B.2.3.2-1a: Uplink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and three bands (for one or more of the supported configurations in Table A.4.3.2B.2.3.2-2)

| Item | UL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Inter-band EN-DC within FR1 BW Class Combination A_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_UL_inter_band_EN_DC_FR1_3B_Class_A_A |  |
| 2 | UL Inter-band EN-DC within FR1 BW Class Combination C_A (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_UL_inter_band_EN_DC_FR1_3B_Class_C_A |  |
| 3 | UL Inter-band EN-DC within FR1 BW Class Combination C_B (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_UL_inter_band_EN_DC_FR1_3B_Class_C_B |  |
| 4 | UL Inter-band EN-DC within FR1 BW Class Combination (n)AA (three bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.2 | pc_UL_inter_band_EN_DC_FR1_3B_Class_(n)AA |  |

Table A.4.3.2B.2.3.2-2: Supported Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration / Item (Note 1) | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A | Rel-15 |  |  |  |  |
| DC_1A-3A_n78A | Rel-15 |  |  |  |  |
| DC_1A-3C_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A_n79A | Rel-15 |  |  |  |  |
| DC_1A-7A_n3A | Rel-16 |  |  |  |  |
| DC_1A-7A_n78A | Rel-15 |  |  |  |  |
| DC_1A-8A_n3A | Rel-16 |  |  |  |  |
| DC_1A-19A_n78A | Rel-15 |  |  |  |  |
| DC_1A-19A_n79A | Rel-15 |  |  |  |  |
| DC_1A-20A_n3A | Rel-16 |  |  |  |  |
| DC_1A-20A_n78A | Rel-15 |  |  |  |  |
| DC_1A-21A_n78A | Rel-15 |  |  |  |  |
| DC_1A-21A_n79A | Rel-15 |  |  |  |  |
| DC_1A-28A_n3A | Rel-16 |  |  |  |  |
| DC_1A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_1A-42A_n78A | Rel-15 |  |  |  |  |
| DC_1A-42C_n78A | Rel-15 |  |  |  |  |
| DC_1A-42D_n78A | Rel-15 |  |  |  |  |
| DC_1A-42E_n78A | Rel-15 |  |  |  |  |
| DC_1A-42A_n79A | Rel-15 |  |  |  |  |
| DC_1A-42C_n79A | Rel-15 |  |  |  |  |
| DC_1A-42D_n79A | Rel-15 |  |  |  |  |
| DC_1A-42E_n79A | Rel-15 |  |  |  |  |
| DC_1A_n78A-n79A | Rel-15 |  |  |  |  |
| DC_2A-2A-14A_n66A | Rel-16 |  |  |  |  |
| DC_2A-14A_n2A | Rel-16 |  |  |  |  |
| DC_2A-14A_n66A | Rel-16 |  |  |  |  |
| DC_2A-66A_n41A | Rel-16 |  |  |  |  |
| DC_2A-66A_n5A | Rel-16 |  |  |  |  |
| DC_2A-66A_n71A | Rel-15 |  |  |  |  |
| DC_2A-(n)71AA | Rel-15 |  |  |  |  |
| DC_3A-7A_n1A | Rel-16 |  |  |  |  |
| DC_3A-7A_n78A | Rel-15 |  |  |  |  |
| DC_3A-8A_n1A | Rel-16 |  |  |  |  |
| DC_3A-19A_n78A | Rel-15 |  |  |  |  |
| DC_3A-19A_n79A | Rel-15 |  |  |  |  |
| DC_3A-20A_n1A | Rel-16 |  |  |  |  |
| DC_3A-20A_n78A | Rel-15 |  |  |  |  |
| DC_3A-21A_n78A | Rel-15 |  |  |  |  |
| DC_3A-21A_n79A | Rel-15 |  |  |  |  |
| DC_3A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_3A-40A_n1A | Rel-16 |  |  |  |  |
| DC_3A-42A_n78A | Rel-15 |  |  |  |  |
| DC_3A-42C_n78A | Rel-15 |  |  |  |  |
| DC_3A-42D_n78A | Rel-15 |  |  |  |  |
| DC_3A-42E_n78A | Rel-15 |  |  |  |  |
| DC_3A-42A_n79A | Rel-15 |  |  |  |  |
| DC_3A-42C_n79A | Rel-15 |  |  |  |  |
| DC_3A-42D_n79A | Rel-15 |  |  |  |  |
| DC_3A-42E_n79A | Rel-15 |  |  |  |  |
| DC_3A_n78A-n79A | Rel-15 |  |  |  |  |
| DC_5A-7A_n78A | Rel-15 |  |  |  |  |
| DC_7A-8A_n1A | Rel-16 |  |  |  |  |
| DC_7A-20A_n1A | Rel-16 |  |  |  |  |
| DC_7A-20A_n3A | Rel-16 |  |  |  |  |
| DC_7A-20A_n78A | Rel-15 |  |  |  |  |
| DC_7A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_14A-66A_n2A | Rel-16 |  |  |  |  |
| DC_14A-66A-66A_n2A | Rel-16 |  |  |  |  |
| DC_14A-66A_n66A | Rel-16 |  |  |  |  |
| DC_19A-21A_n78A | Rel-15 |  |  |  |  |
| DC_19A-21A_n79A | Rel-15 |  |  |  |  |
| DC_19A-42A_n78A | Rel-15 |  |  |  |  |
| DC_19A-42A_n79A | Rel-15 |  |  |  |  |
| DC_19A-42C_n78A | Rel-15 |  |  |  |  |
| DC_19A-42C_n79A | Rel-15 |  |  |  |  |
| DC_19A_n78A-n79A | Rel-15 |  |  |  |  |
| DC_20A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_21A-42A_n78A | Rel-15 |  |  |  |  |
| DC_21A-42C_n78A | Rel-15 |  |  |  |  |
| DC_21A-42A_n79A | Rel-15 |  |  |  |  |
| DC_21A-42C_n79A | Rel-15 |  |  |  |  |
| DC_21A_n78A-n79A | Rel-15 |  |  |  |  |
| DC_66A_(n)71AA | Rel-15 |  |  |  |  |
| Note 1: Notation used for inter-band EN-DC Bands is according to TS 38.101-3 [25] Table 5.5B.4.2-1, e.g. ‘DC_1A-3C_n78A’ indicates EN-DC operation on E-UTRA CA configuration CA_1A-3C with E-UTRA DL Bandwidth Classes A, C for the E-UTRA bands 1 and 3 respectively and NR band n78 with NR DL CA Bandwidth Class A. Note 2: The ULTxSwitching capability can be reported on EN-DC band combinations. The UE supplier shall indicate E-UTRA/NR band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘A-nB’, where A is LTE band and nB is NR band. For example, for DC_3A_n77A, N would mean not supporting ULTxSwitching, ‘3-n77’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at lease one UL EN-DC configuration on the same band pair in the column “Supported EN-DC Bandwidth Class(es) in UL”. Note 3: ULSwitching(Table A.4.3.2B.2.3.2-2) shall return all supported EN-DC Configurations where at least one E-UTRA/NR band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

A.4.3.2B.2.3.3	Inter-band EN-DC within FR1 (four bands)

Table A.4.3.2B.2.3.3-1: Downlink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and four bands (for one or more of the supported DC configurations in Table A.4.3.2B.2.3.3-2)

| Item | DL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | Inter-band EN-DC within FR1 BW Class Combination A-A-A_A (four bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.3 | pc_DL_inter_band_EN_DC_FR1_4B_Class_A-A-A_A |  |
| 2 | Inter-band EN-DC within FR1 BW Class Combination A-A-C_A (four bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.3 | pc_DL_inter_band_EN_DC_FR1_4B_Class_A-A-C_A |  |
| 3 | Inter-band EN-DC within FR1 BW Class Combination A-A-D_A (four bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.3 | pc_DL_inter_band_EN_DC_FR1_4B_Class_A-A-D_A |  |
| 4 | Inter-band EN-DC within FR1 BW Class Combination A-C-A_A (four bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.3 | pc_DL_inter_band_EN_DC_FR1_4B_Class_A-C-A_A |  |
| 5 | Inter-band EN-DC within FR1 BW Class Combination A-(2A)-A_A (four bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.3 | pc_DL_inter_band_EN_DC_FR1_4B_Class_A-(2A)-A_A |  |
| 6 | Inter-band EN-DC within FR1 BW Class Combination A-A_A-A (four bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.3 | pc_DL_inter_band_EN_DC_FR1_4B_Class_A-A_A-A |  |
| 7 | Inter-band EN-DC within FR1 BW Class Combination A-A_(n)AA (four bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.3 | pc_DL_inter_band_EN_DC_FR1_4B_Class_A-A_(n)AA |  |

Table A.4.3.2B.2.3.3-1a: Uplink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and four bands (for one or more of the supported configurations in Table A.4.3.2B.2.3.3-2 )

| Item | UL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Inter-band EN-DC within FR1 BW Class Combination A_A (four bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.3 | pc_UL_inter_band_EN_DC_FR1_4B_Class_A_A |  |
| 2 | UL Inter-band EN-DC within FR1 BW Class Combination A_B (four bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.3 | pc_UL_inter_band_EN_DC_FR1_4B_Class_A_B |  |
| 3 | UL Inter-band EN-DC within FR1 BW Class Combination (n)AA (four bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.3 | pc_UL_inter_band_EN_DC_FR1_4B_Class_(n)AA |  |
| 4 | UL Inter-band EN-DC within FR1 BW Class Combination C_A (four bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.3 | pc_UL_inter_band_EN_DC_FR1_4B_Class_C_A |  |
| 5 | UL Inter-band EN-DC within FR1 BW Class Combination C_B (four bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.3 | pc_UL_inter_band_EN_DC_FR1_4B_Class_C_B |  |

Table A.4.3.2B.2.3.3-2: Supported Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration / Item (Note 1) | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- |
| DC_1A-3A-7A_n28A | Rel-15 |  |  |  |  |
| DC_1A-3A-7A_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-19A_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-19A_n79A | Rel-15 |  |  |  |  |
| DC_1A-3A-20A_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-21A_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-21A_n79A | Rel-15 |  |  |  |  |
| DC_1A-3A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-42A_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-42C_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-42D_n78A | Rel-16 |  |  |  |  |
| DC_1A-3A-42D_n79A | Rel-16 |  |  |  |  |
| DC_1A-3A-42A_n79A | Rel-15 |  |  |  |  |
| DC_1A-3A-42C_n79A | Rel-15 |  |  |  |  |
| DC_1A-7A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_1A-7A-20A_n78A | Rel-15 |  |  |  |  |
| DC_1A-19A-21A_n78A | Rel-15 |  |  |  |  |
| DC_1A-19A-21A_n79A | Rel-15 |  |  |  |  |
| DC_1A-19A-42A_n78A | Rel-15 |  |  |  |  |
| DC_1A-19A-42C_n78A | Rel-15 |  |  |  |  |
| DC_1A-19A-42A_n79A | Rel-15 |  |  |  |  |
| DC_1A-19A-42C_n79A | Rel-15 |  |  |  |  |
| DC_1A-20A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_1A-21A-42A_n78A | Rel-15 |  |  |  |  |
| DC_1A-21A-42C_n78A | Rel-15 |  |  |  |  |
| DC_1A-21A-42A_n79A | Rel-15 |  |  |  |  |
| DC_1A-21A-42C_n79A | Rel-15 |  |  |  |  |
| DC_2A-2A-14A-66A_n66A | Rel-16 |  |  |  |  |
| DC_2A-7A-7A-13A_n66A | Rel-16 |  |  |  |  |
| DC_2A-7A-7A-66A_n66A | Rel-16 |  |  |  |  |
| DC_2A-7A-7A-66A_n78A | Rel-16 |  |  |  |  |
| DC_2A-7A-13A_n66A | Rel-16 |  |  |  |  |
| DC_2A-7A-66A_n66A | Rel-16 |  |  |  |  |
| DC_2A-7C-13A_n66A | Rel-16 |  |  |  |  |
| DC_2A-7C-66A_n66A | Rel-16 |  |  |  |  |
| DC_2A-7C-66A_n78A | Rel-16 |  |  |  |  |
| DC_2A-14A-66A_n2A | Rel-16 |  |  |  |  |
| DC_2A-14A-66A_n66A | Rel-16 |  |  |  |  |
| DC_2A-14A-66A-66A_n2A | Rel-16 |  |  |  |  |
| DC_2A-66A-(n)71AA | Rel-15 |  |  |  |  |
| DC_3A-7A-20A_n1A | Rel-16 |  |  |  |  |
| DC_3A-7A-20A_n78A | Rel-15 |  |  |  |  |
| DC_3A-7A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_3A-19A-21A_n78A | Rel-15 |  |  |  |  |
| DC_3A-19A-21A_n79A | Rel-15 |  |  |  |  |
| DC_3A-19A-42A_n78A | Rel-15 |  |  |  |  |
| DC_3A-19A-42C_n78A | Rel-15 |  |  |  |  |
| DC_3A-19A-42A_n79A | Rel-15 |  |  |  |  |
| DC_3A-19A-42C_n79A | Rel-15 |  |  |  |  |
| DC_3A-20A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_3A-21A-42A_n78A | Rel-15 |  |  |  |  |
| DC_3A-21A-42C_n78A | Rel-15 |  |  |  |  |
| DC_3A-21A-42A_n79A | Rel-15 |  |  |  |  |
| DC_3A-21A-42C_n79A | Rel-15 |  |  |  |  |
| DC_7A-20A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_19A-21A-42A_n78A | Rel-15 |  |  |  |  |
| DC_19A-21A-42C_n78A | Rel-15 |  |  |  |  |
| DC_19A-21A-42A_n79A | Rel-15 |  |  |  |  |
| DC_19A-21A-42C_n79A | Rel-15 |  |  |  |  |
| Note 1: Notation used for inter-band EN-DC Bands is according to TS 38.101-3 [25] Table 5.5B.4.3-1, e.g. ‘DC_2A-7C-13A_n66A’ indicates EN-DC operation on E-UTRA CA configuration CA_2A-7C-13A with E-UTRA DL Bandwidth Classes A, C, A for the E-UTRA bands 2, 7 and 13 respectively and NR band n66 with NR DL CA Bandwidth Class A. Note 2: The ULTxSwitching capability can be reported on EN-DC band combinations. The UE supplier shall indicate E-UTRA/NR band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘A-nB’, where A is LTE band and nB is NR band. For example, for DC_3A_n77A, N would mean not supporting ULTxSwitching, ‘3-n77’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at lease one UL EN-DC configuration on the same band pair in the column “Supported EN-DC Bandwidth Class(es) in UL”. Note 3: ULSwitching(Table A.4.3.2B.2.3.3-2) shall return all supported EN-DC Configurations where at least one E-UTRA/NR band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

A.4.3.2B.2.3.4	Inter-band EN-DC within FR1 (five bands)

Table A.4.3.2B.2.3.4-1: Downlink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and five bands (for one or more of the supported DC configurations in Table A.4.3.2B.2.3.4-2)

| Item | DL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | Inter-band EN-DC within FR1 BW Class Combination A-A-A-A_A (five bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.4 | pc_DL_inter_band_EN_DC_FR1_5B_Class_A-A-A-A_A |  |
| 2 | Inter-band EN-DC within FR1 BW Class Combination A-A-A_A-A (five bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.4 | pc_DL_inter_band_EN_DC_FR1_5B_Class_A-A-A_A-A |  |
| 3 | Inter-band EN-DC within FR1 BW Class Combination A-A-A-C_A (five bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.4 | pc_DL_inter_band_EN_DC_FR1_5B_Class_A-A-A-C_A |  |
| 4 | Inter-band EN-DC within FR1 BW Class Combination A-A-C-A_A (five bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.4 | pc_DL_inter_band_EN_DC_FR1_5B_Class_A-A-C-A_A |  |
| 5 | Inter-band EN-DC within FR1 BW Class Combination A-C-A-A_A (five bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.4 | pc_DL_inter_band_EN_DC_FR1_5B_Class_A-C-A-A_A |  |
| 6 | Inter-band EN-DC within FR1 BW Class Combination C-A-A-A_A (five bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.4 | pc_DL_inter_band_EN_DC_FR1_5B_Class_C-A-A-A_A |  |

Table A.4.3.2B.2.3.4-1a: Uplink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and five bands (for one or more of the supported configurations in Table A.4.3.2B.2.3.4-2 )

| Item | UL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Inter-band EN-DC within FR1 BW Class Combination A_A (five bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.4 | pc_UL_inter_band_EN_DC_FR1_5B_Class_A_A |  |
| 2 | UL Inter-band EN-DC within FR1 BW Class Combination A_B (five bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.4 | pc_UL_inter_band_EN_DC_FR1_5B_Class_A_B |  |
| 3 | UL Inter-band EN-DC within FR1 BW Class Combination C_A (five bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.4 | pc_UL_inter_band_EN_DC_FR1_5B_Class_C_A |  |
| 4 | UL Inter-band EN-DC within FR1 BW Class Combination C_B (five bands) | 36.101, 5.6A.1 38.101-3, 5.5B.4.4 | pc_UL_inter_band_EN_DC_FR1_5B_Class_C_B |  |

Table A.4.3.2B.2.3.4-2: Supported Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration / Item (Note 1) | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- |
| DC_1A-3A-5A-41A_n79A | Rel-16 |  |  |  |  |
| DC_1A-3A-7A-20A_n28A | Rel-15 |  |  |  |  |
| DC_1A-3A-7A-20A_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-7A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-19A-42A_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-19A-42C_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-19A-42A_n79A | Rel-15 |  |  |  |  |
| DC_1A-3A-19A-42C_n79A | Rel-15 |  |  |  |  |
| DC_1A-3A-19A-42C_n78A | Rel-16 |  |  |  |  |
| DC_1A-3A-19A-42C_n79A | Rel-16 |  |  |  |  |
| DC_1A-3A-20A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-21A-42A_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-21A-42C_n78A | Rel-15 |  |  |  |  |
| DC_1A-3A-21A-42A_n79A | Rel-15 |  |  |  |  |
| DC_1A-3A-21A-42C_n79A | Rel-15 |  |  |  |  |
| DC_1A-3A-21A-42C_n78A | Rel-16 |  |  |  |  |
| DC_1A-3A-21A-42C_n79A | Rel-16 |  |  |  |  |
| DC_1A-7A-20A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_1A-19A-21A-42A_n78A | Rel-15 |  |  |  |  |
| DC_1A-19A-21A-42C_n78A | Rel-15 |  |  |  |  |
| DC_1A-19A-21A-42A_n79A | Rel-15 |  |  |  |  |
| DC_1A-19A-21A-42C_n78A | Rel-16 |  |  |  |  |
| DC_1A-19A-21A-42C_n79A | Rel-15 |  |  |  |  |
| DC_1A-19A-21A-42C_n79A | Rel-16 |  |  |  |  |
| DC_3A-7A-20A_n28A-n78A | Rel-15 |  |  |  |  |
| DC_3A-19A-21A-42A_n78A | Rel-16 |  |  |  |  |
| DC_3A-19A-21A-42C_n78A | Rel-16 |  |  |  |  |
| DC_3A-19A-21A-42A_n79A | Rel-16 |  |  |  |  |
| DC_3A-19A-21A-42C_n79A | Rel-16 |  |  |  |  |
| Note 1: Notation used for inter-band EN-DC Bands is according to TS 38.101-3 [25] Table 5.5B.4.4-1, e.g. ‘DC_1A-3A-5A-41A_n79A’ indicates EN-DC operation on E-UTRA CA configuration CA_1A-3A-5A-41A with E-UTRA DL Bandwidth Classes A for all the E-UTRA bands 1, 3, 5 and 41 and NR band n79 with NR DL CA Bandwidth Class A. Note 2: The ULTxSwitching capability can be reported on EN-DC band combinations. The UE supplier shall indicate E-UTRA/NR band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘A-nB’, where A is LTE band and nB is NR band. For example, for DC_3A_n77A, N would mean not supporting ULTxSwitching, ‘3-n77’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at lease one UL EN-DC configuration on the same band pair in the column “Supported EN-DC Bandwidth Class(es) in UL”. Note 3: ULSwitching(Table A.4.3.2B.2.3.4-2) shall return all supported EN-DC Configurations where at least one E-UTRA/NR band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

A.4.3.2B.2.3.5	Inter-band EN-DC within FR1 (six bands)

Table A.4.3.2B.2.3.5-1: Downlink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and six bands (for one or more of the supported DC configurations in Table A.4.3.2B.2.3.5-2)

| Item | DL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | EN-DC Inter-band with NR FR1 BW Class Combination A-A-A-A_A-A (six bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.5 | pc_DL_inter_band_EN_DC_FR1_6B_Class_A-A-A-A_A-A |  |
| 2 | EN-DC Inter-band with NR FR1 BW Class Combination A-A-C-A_A-A (six bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.5 | pc_DL_inter_band_EN_DC_FR1_6B_Class_A-A-C-A_A-A |  |
| 3 | EN-DC Inter-band with NR FR1 BW Class Combination A-C-A-A_A-A (six bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.5 | pc_DL_inter_band_EN_DC_FR1_6B_Class_A-C-A-A_A-A |  |
| 4 | EN-DC Inter-band with NR FR1 BW Class Combination A-C-C-A_A-A (six bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.5 | pc_DL_inter_band_EN_DC_FR1_6B_Class_A-C-C-A_A-A |  |

Table A.4.3.2B.2.3.5-1a: Uplink Bandwidth Class Combination capabilities for Inter-band EN-DC within FR1 and six bands (for one or more of the supported configurations in Table A.4.3.2B.2.3.5-2)

| Item | UL inter-band EN-DC within FR1 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL Inter-band EN-DC within FR1 BW Class Combination A_A (six bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.5 | pc_UL_inter_band_EN_DC_FR1_6B_Class_A_A |  |
| 2 | UL Inter-band EN-DC within FR1 BW Class Combination C_A (six bands) | 36.101, 5.6A.1 38.101-1, 5.5B.4.5 | pc_UL_inter_band_EN_DC_FR1_6B_Class_C_A |  |

Table A.4.3.2B.2.3.5-2: Supported Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration / Item (Note 1) | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- | --- |
| DC_1A-3A-7A-20A_n28A-n78A | Rel-15 |  |  |  |  |
| Note 1: Notation used for inter-band EN-DC Bands is according to TS 38.101-3 [25] Table 5.5B.4.5-1, e.g. ‘DC_1A-3A-7A-20A_n28A-n78A’ indicates EN-DC operation on E-UTRA CA configuration CA_1A-3A-7A-20A with E-UTRA DL Bandwidth Class A for all the E-UTRA bands 1, 3, 7 and 20 and NR CA configuration CA_n28A-n78A with NR DL CA Bandwidth Class A for all the NR bands n28 and n78. Note 2: The ULTxSwitching capability can be reported on EN-DC band combinations. The UE supplier shall indicate E-UTRA/NR band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘A-nB’, where A is LTE band and nB is NR band. For example, for DC_3A_n77A, N would mean not supporting ULTxSwitching, ‘3-n77’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at lease one UL EN-DC configuration on the same band pair in the column “Supported EN-DC Bandwidth Class(es) in UL”. Note 3: ULSwitching(Table A.4.3.2B.2.3.5-2) shall return all supported EN-DC Configurations where at least one E-UTRA/NR band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |  |

<Unchanged Text Skipped>

A.4.3.2B.2.3.14	Inter-band EN-DC including FR1 and FR2 (six bands)

Table A.4.3.2B.2.3.14-1: Downlink Bandwidth Class Combination capabilities for Inter-band EN-DC including FR1 and FR2, and six bands (for one or more of the supported DC configurations in Table A.4.3.2B.2.3.14-2)

| Item | DL inter-band EN-DC including FR1 and FR2 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | TBD | TBD | TBD |  |

Table A.4.3.2B.2.3.14-1a: Uplink Bandwidth Class Combination capabilities for Inter-band EN-DC including FR1 and FR2, and six bands (for one or more of the supported DC configurations in Table A.4.3.2B.2.3.14-2)

| Item | UL inter-band EN-DC including FR1 and FR2 Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | TBD | TBD | TBD |  |

Table A.4.3.2B.2.3.14-2: Supported Inter-band EN-DC configurations including FR1 and FR2 (six bands)

| EN-DC configuration / Item | Release | Supported | Supported EN-DC Bandwidth Class(es) in UL | Supported Bandwidth Combination Set(s) |
| --- | --- | --- | --- | --- |
| TBD | TBD |  |  |  |

A.4.3.2C	NR SUL Physical Layer Baseline Implementation Capabilities

A.4.3.2C.1	General NR-DC capabilities

FFS

A.4.3.2C.2		SUL band combinations without CA

Table A.4.3.2C.2-1: Supported SUL configurations without CA

| SUL configuration / Item (Note 1) | Release | Supported | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 2, 3) |
| --- | --- | --- | --- | --- |
| SUL_n78A-n80A | Rel-15 |  |  |  |
| SUL_n78A-n81A | Rel-15 |  |  |  |
| SUL_n78A-n82A | Rel-15 |  |  |  |
| SUL_n78A-n83A | Rel-15 |  |  |  |
| SUL_n78A-n84A | Rel-15 |  |  |  |
| SUL_n78A-n86A | Rel-15 |  |  |  |
| SUL_n79A-n80A | Rel-15 |  |  |  |
| SUL_n79A-n81A | Rel-15 |  |  |  |
| Note 1: Notation used for SUL configurations is according to TS 38.101-1 [23] Table 5.5C-1, e.g. ‘SUL_n78A-n80A’ indicates SUL operation on NR bands n78 and n80 with UL CA Bandwidth Class A on both bands. Note 2: The ULTxSwitching capability can be reported on SUL band combinations. The UE supplier shall indicate SUL band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for SUL_n78A-n80A, N would mean not supporting ULTxSwitching, ‘n78-n80’ would mean supporting of ULTxSwitching on this band pair. Note 3: ULSwitching(Table A.4.3.2C.2-1) shall return all supported SUL Configurations where at least one SUL band pair was declared in column “Supported ULTxSwitching Band Pair". |  |  |  |  |

<End of Changes >