3GPP TSG-RAN5 Meeting #90-e	R5-210753r1

# Electronic, 22nd February - 5th March, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0314 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of TDD NB-IOT RSTD measurement test case 9.7.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The TDD NB-IOT RSTD measurement test case 9.7.3 (A.4.7.3 in TS 38.133) is defined in RAN4. Corresponding RAN5 test case needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defining the test case 9.7.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case can’t be performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0133 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updating format of subclauses to H6 |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

9.6.4	TDD Inter frequency RSTD Measurement Reporting Delay for NB-IOT Standalone Mode in enhanced coverage

FFS.

9.7	TDD RSTD Intra-Frequency Measurements for NB-IOT

9.7.3	TDD Intra Frequency RSTD Measurement Reporting Delay for NB-IOT Standalone Mode in enhanced coverage

9.7.3.1	Test purpose

To verify that the RSTD TDD intra-frequency measurement reporting delay is within the specified limits for NB-IOT Standalone Mode in enhanced coverage.

9.7.3.2	Test applicability

This test applies to all types of NB-IoT TDD UE release 15 and forward of UE Category NB1 and higher that supports UE-assisted OTDOA.

9.7.3.3	Minimum conformance requirements

The UE shall follow the procedure for RRC_IDLE state positioning measurement as defined in TS 36.305 [41] section 7.1.3.

When the physical layer cell identities of neighbour cells together with the OTDOA assistance data have been provided and the UE has entered the RRC_IDLE state, the UE shall be able to detect and measure intra-frequency RSTD, specified in TS 36.214 [6], for at least n = 16 cells, including the reference cell, on the same carrier frequency f1 as that of the reference cell within  ms as given below:

,

where

is the total time for detecting and measuring at least n cells;

is the cell-specific positioning subframe configuration period as defined in TS 36.355 [4] if Part B subframe configuration is provided; otherwise if only Part A subframe configuration is provided, the  equals to the length of the subframe pattern,

is the number of NPRS positioning occasions as defined in Table 9.7.3.3-1,

=  ms is the measurement time for a single NPRS positioning occasion which includes the sampling time and the processing time;

is the cell-specific number of NPRS subframes within a NPRS occasion as defined in TS 36.355 [4] if Part B subframe configuration is provided; if only Part A subframe configuration is provided, the NPRS occasion length is 10 ms,

is the minimum number of NPRS subframes per cell measurement as defined in Table 9.7.3.3-2.

, , and ,  are the parameters of the same cell, for which  is the largest among all the measured cells.

Table 9.7.3.3-1: Number of NPRS positioning occasions within

| Positioning subframe configuration period | Number of NPRS positioning occasions |  |
| --- | --- | --- |
|  | f1 Note1 | f1 and f2 Note2 |
| 160 ms | 16* | 32* |
| >160 ms | 8* | 16* |
| Note 1: When only intra-frequency RSTD measurements are performed over cells belonging to the serving carrier frequency f1. Note 2: When intra-frequency RSTD and inter-frequency RSTD measurements are performed over cells belonging to the serving carrier frequency f1 and one inter-frequency carrier frequency f2, respectively. |  |  |

The UE physical layer shall be capable of reporting RSTD for the reference cell and all the neighbour cells i out of at least (n-1) neighbour cells within  provided:

-15 dB for all Frequency Bands for the reference cell,

-15 dB for all Frequency Bands for neighbour cell i,

and  conditions apply for all subframes of at least  NPRS positioning occasions,

NPRP 1,2|dBm according to Annex E.5 for a corresponding Band

is defined as the ratio of the average received energy per PRS RE during the useful part of the symbol to the average received power spectral density of the total noise and interference for this RE, where the ratio is measured over all REs which carry NPRS.

The time  starts from the point when the UE has received both the OTDOA-RequestLocationInformation message and the OTDOA assistance data in the OTDOA-ProvideAssistanceData message as specified in TS 36.355 [4], and the message and data have been delivered to the physical layer of the UE, and the UE has entered the RRC_IDLE state.

After receiving both the OTDOA-RequestLocationInformation message and the OTDOA assistance data in the OTDOA-ProvideAssistanceData message as specified in TS 36.355 [4], the UE shall be sent to RRC IDLE state. The maximum allowed RSTD reporting delay is defined as the time between the point when the UE has entered the RRC_IDLE state, and the point when the UE is ready to transmit the measurement report over the air interface and starts to establish a signalling connection with the MME.

The maximum allowed RSTD reporting delay shall be less than +TRandomAccess_NB-IoT-EC.

The normative reference for this requirement is TS 36.133 [23] clauses 4.8.2, 4.8.2.1 and A.4.7.3.

9.7.3.4	Test description

9.7.3.4.1	Initial conditions

Test Environment: Normal; as defined in TS 36.508 [18] clause 8.1.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [18] clause 8.1.3.1.1.

Channel bandwidth to be tested: 200 kHz as defined in TS 36.508 [18] clause 8.1.3.1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.3 but using only the main TX/RX antenna.

2.	The general test parameter settings are set up according to Table 9.7.3.4.1-1.

3.	Propagation conditions are set according to clause 4.7.2.1.

4.	Message contents are defined in clause 9.7.3.4.3.

5.	There are three synchronous cells: nCell 1, nCell 2 and nCell 3. nCell 1 is the reference as well as the serving cell. nCell 2 and nCell 3 are the neighbour cells. All cells are on the same RF channel. The assistance data neighbour cell list includes in total 15 cells, where 13 of the cells are not simulated (dummy cells; as defined in TS 37.571-5 [20], clause 7.2.2).

6.	The true RSTD (which is the receive time difference for frame 0 between nCell 2 and nCell 1 as seen at the UE antenna connector) is set to 31 Ts (about 1 s between neighbour nCell 2 and serving nCell 1; and set to -31 Ts (about -1 s) between neighbour nCell 3 and serving nCell 1

Table 9.7.3.4-1: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Standalone |  |
| Reference cell |  | nCell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [6] and TS 36.355 [4]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | nCell 2 and nCell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| nprsID |  | (nprsID of Cell 1 – nprsID of Cell 2)mod6=1 and (nprsID of Cell 1 – nprsID of Cell 3)mod6=2 | As defined in TS 36.355 [4] |
| nprs-period | ms | 1280 | As defined in TS 36.355 [4] |
| nprs-startSF |  | sf0 | Subframe offset of the NPRS positioning occasion as defined in TS 36.355 [4] |
| Number of consecutive downlink positioning subframes nprs-NumSF |  | 640 | As defined in TS 36.355 [4] |
| NPRS muting info |  | nCell 1: ‘11110000’ nCell 2: ‘00001111’ nCell 3: ‘11110000’ | Corresponds to nprs-MutingInfoB defined in TS 36.355 [4] |
| Part A Configuration |  | N/A | NPRS is configured based on Part B but not Part A. |
| CP length |  | Normal |  |
| NPRACH Configuration |  | NPRACH.R-2 | Refer to A.3.18 |
| DRX cycle length |  | 1.28 | The value shall be used for all cells in the test. |
| Radio frame receive time offset between the cells at the UE antenna connector | s | nCell 2 to nCell 1: 1 nCell 3 to nCell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | nCell 2: 3 nCell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [4] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [4] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [26] |
| Uplink-downlink configuration |  | 2 | As specified in table 4.2-2 in TS 36.211 [26] |
| T2 | s | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| T4 | s | 5.12 | The length of the time interval that follows immediately after time interval T3 |
| T5 | s | 5.12 | The length of the time interval that follows immediately after time interval T4 |
| T6 | s | ≥ 58.2 | The length of the time interval that follows immediately after time interval T5 |

9.7.3.4.2	Test procedure

Same as in clause 9.5.3.4.2.

9.7.3.4.3	Message contents

Same as in clause 9.5.3.4.3.

9.7.3.5	Test requirement

Table 9.7.3.5-1 and 9.7.3.5-2 define the primary level settings including test tolerances for the test.

Table 9.7.3.5-1: Cell Specific Test Parameters for intra frequency RSTD Tests for T1 and T6

| Parameter | Unit | nCell 1 |  | nCell 2 |  | nCell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| NB-IoT RF Channel Number |  | 1 |  | 1 |  | 1 |
| NB-IoT Channel Bandwidth (BWchannel) | kHz | 200 |  | 200 |  | 200 |
| OCNG Pattern Note 1 |  | NOP.3 TDD |  | N/A |  | N/A |
| NPDSCH parameters Note 2 |  | R.18 NB-TDD |  | N/A |  | N/A |
| NPDCCH parameters Note 2 |  | R.30 NB-TDD |  | N/A |  | N/A |
| NPBCH_RA | dB | 0 |  | N/A |  | N/A |
| NPBCH_RB |  |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -98 |  |  |  |  |
| NPRS | dB | -Infinity |  | -Infinity |  | -Infinity |
|  | dB | -2 |  | -Infinity |  | -Infinity |
| Propagation Condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |
| Timing offset to nCell 1 | s | N/A | 1 |  | -1 |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |  |

Table 9.7.3.5-2: Cell Specific Test Parameters for intra frequency RSTD Tests for T2 to T5

| Parameter | Unit | nCell 1 |  | nCell 2 |  |  |  | nCell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 and T4 | T3 and T5 | T2 and T4 |  | T3 and T5 |  | T2 and T4 | T3 and T5 |
| BWchannel | kHz | 200 |  | 200 |  |  |  | 200 |  |
| NB-IoT RF Channel Number |  | 1 |  | 1 |  |  |  | 1 |  |
| OCNG patterns |  | NOP.3 TDD |  | N/A | NOP.3 TDD |  |  | NOP.3 TDD | N/A |
| NPBCH_RA | dB | 0 |  | 0 |  |  |  | 0 | N/A |
| NPBCH_RB |  |  |  |  |  |  |  |  |  |
| NPSS_RA |  |  |  |  |  |  |  |  |  |
| NSSS_RA |  |  |  |  |  |  |  |  |  |
| NPDCCH_RA |  |  |  |  |  |  |  |  |  |
| NPDCCH_RB |  |  |  |  |  |  |  |  |  |
| NPDSCH_RA |  |  |  |  |  |  |  |  |  |
| NPDSCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |  |  |  |
| NPRS_RA | dB | -3 | N/A | N/A |  | 0 |  | 0 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 |  | -95 |  | -98 | -95 |
| NPRS | dB | -15 | -Infinity | -Infinity |  | -15 |  | -15 | -Infinity |
| NPRS Note 4 | dB | -15 | -Infinity | -Infinity |  | -15 |  | -15 | -Infinity |
| Io Note 4 | dBm/ 180kHz | -87.14 | -87.12 | -87.14 |  | -87.12 |  | -87.14 | -87.12 |
| NPRP Note 4 | dBm/ 15 kHz | -113 | -Infinity | -Infinity |  | -110 |  | -113 | -Infinity |
| NRSRP Note 4 | dBm/ 15 kHz | -110 | -107 | -113 |  | -110 |  | -113 | -Infinity |
| Note 4 | dB | -12 | -12 | -15 |  | -15 |  | -15 | -Infinity |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x1 |  |  |  |  |  |  |  |
| Timing offset to nCell 1 | s | N/A |  | 1 |  |  | -1 |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted NPRS. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If NPRS_RA is not “N/A”, , NPRS , Io, NRSRP and NPRP levels have been derived from other parameters and are given for information purpose. If NPRS_RA is “N/A”, Io and NRSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |  |  |

The response time including test tolerance is 79.3 s. The response time is equal to the LPP time IE value plus the test tolerance.

The LPP time IE value is derived from the maximum allowed RSTD reporting delay plus T2 and T3 plus T, where T = 150 ms, giving a value of 78830 ms. This is rounded up to the next allowed LPP value of 79 seconds.

The maximum allowed RSTD reporting delay is 68.44 s. This time is measured starting from the beginning of time interval T4, to the moment when the UE starts to send preambles on the PRACH for sending the positioning measurement report message to nCell1. The maximum allowed RSTD reporting delay is equal to TRSTD _intra_NB-IoT-EC +TRandomAccess_NB-IoT-EC.

The RSTD measurement time TRSTD _intra_NB-IoT-EC in the test is derived according to section 9.7.3.3 where it is equal to . This gives the total RSTD measurement time of 11.52s for nCell 2 and nCell 3 with respect to the reference nCell 1

The random access to an already detected cell TRandomAccess_NB-IoT-EC can be expressed as: Tevaluate, NB_intra_NB-IoT-EC + TSI + TPRACH_NB-IoT,

Where:

Tevaluate, NB_intra_NB-IoT-EC = 12800 ms: see Table 4.6.2.4-1 in clause 4.6.2.4 in TS 36.133 [23].

TSI = 41560 ms and is the time required for receiving all the relevant system information as defined in TS 36.331 [22] for the target NB-IoT FDD cell.

TPRACH_NB-IoT = 2560 ms and is the additional delay caused by the random access procedure.

This gives TRandomAccess_NB-IoT-EC = 56.92 s for the random access delay to an already detected cell in the test case.

The test tolerances are defined in clauses C.1.3 and C4.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90% with a confidence level of 95%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

<End of Changes >