3GPP TSG-RAN5 Meeting #90-e	R5-210769r1

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2041 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to DL Multi SPS test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIoT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clarification on specific message contents for condition in which IE is not present is required Missing clarification on the SP-index of 2 active SPS grants The test sequence mentions the DL SPS assignment on PDCCH and DL MAC PDU transmission on PDCCH happens in same slot. This is possible because PDSCH offset K0 is 0 in 38.508-1. Note for the same is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Conformance requirements from TS 38.213, clause 10.2 added Clarificaiton on not present of IE under condition is added. Added clarification on SPS index of grants Added Note 6 to clarify the timing between DL SPS assignment and corresponding DL PDSCH MAC PDU slot timing Revision 1: WIC changed Step 1 sps-ConfigIndex changed to 1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conforment UE may Fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.1.6.4	Correct handling of DL assignment / Multi Semi-persistent configuration

7.1.1.6.4.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and sps-Configuration in DL is enabled }

ensure that {

when { UE receives a DL assignment addressed to its stored CS-RNTI in slot y and with NDI set as 0 associated associated with sps-ConfigIndex=0}

then {UE starts receiving DL MAC PDU in slots y+n*[semiPersistSchedIntervalDL] where ‘n’ is positive integer starting at zero }

}

(2)

with { UE in RRC_Connected state with DRB established and stored DL SPS assignment to receive MAC PDU in slot y+n*[semiPersistSchedIntervalDL] }

ensure that {

when { UE receives another DL assignment addressed to its CS-RNTI in slot p and with NDI set as 0 associated with sps-ConfigIndex=1, where p!= y+n*[semiPersistSchedIntervalDL] }

then { UE starts receiving DL MAC PDU in slots p+n*[semiPersistSchedIntervalDL] and continue receiving DL MAC PDU at slots y+n*[semiPersistSchedIntervalDL]where ‘n’ is positive integer starting at zero }

}

7.1.1.6.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in TS 38.321, clause 5.3.1, 5.8.1 TS 38.300, clause 10.2, and TS 38.213, clause 10.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.321, clause 5.3.1]

Downlink assignments received on the PDCCH both indicate that there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI:

2>	if this is the first downlink assignment for this Temporary C-RNTI:

3>	consider the NDI to have been toggled.

2>	if the downlink assignment is for the MAC entity's C-RNTI, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity's CS-RNTI or a configured downlink assignment:

3>	consider the NDI to have been toggled regardless of the value of the NDI.

2>	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity.

1>	else if a downlink assignment for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	indicate the presence of a downlink assignment for this Serving Cell and deliver the associated HARQ information to the HARQ entity.

2>	if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate SPS deactivation:

4>	clear the configured downlink assignment for this Serving Cell (if any);

4>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is running:

5>	indicate a positive acknowledgement for the SPS deactivation to the physical layer.

3>	else if PDCCH content indicates SPS activation:

4>	store the downlink assignment for this Serving Cell and the associated HARQ information as configured downlink assignment;

4>	initialise or re-initialise the configured downlink assignment for this Serving Cell to start in the associated PDSCH duration and to recur according to rules in clause 5.8.1;

For each Serving Cell and each configured downlink assignment, if configured and activated, the MAC entity shall:

1>	if the PDSCH duration of the configured downlink assignment does not overlap with the PDSCH duration of a downlink assignment received on the PDCCH for this Serving Cell:

2>	instruct the physical layer to receive, in this PDSCH duration, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

2>	consider the NDI bit for the corresponding HARQ process to have been toggled;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity.

For configured downlink assignments without harq-ProcID-Offset, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

NOTE 1:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the HARQ Process ID used for configured downlink assignments.

For configured downlink assignments with harq-ProcID-Offset, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes + harq-ProcID-Offset

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

NOTE 2:	CURRENT_slot refers to the slot index of the first transmission occasion of a bundle of configured downlink assignment.

When the MAC entity needs to read BCCH, the MAC entity may, based on the scheduling information from RRC:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the SI-RNTI;

2>	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity.

[TS 38.321, clause 5.8.1]

Semi-Persistent Scheduling (SPS) is configured by RRC per Serving Cell and per BWP. Multiple assignments can be active simultaneously in the same BWP. Activation and deactivation of the DL SPS are independent among the Serving Cells.

For the DL SPS, a DL assignment is provided by PDCCH, and stored or cleared based on L1 signalling indicating SPS activation or deactivation.

RRC configures the following parameters when the SPS is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	nrofHARQ-Processes: the number of configured HARQ processes for SPS;

-	harq-ProcID-Offset: Offset of HARQ process for SPS;

-	periodicity: periodicity of configured downlink assignment for SPS.

When the SPS is released by upper layers, all the corresponding configurations shall be released.

After a downlink assignment is configured for SPS, the MAC entity shall consider sequentially that the Nth downlink assignment occurs in the slot for which:

(numberOfSlotsPerFrame × SFN + slot number in the frame) =
[(numberOfSlotsPerFrame × SFNstart time + slotstart time) + N × periodicity × numberOfSlotsPerFrame / 10] modulo (1024 × numberOfSlotsPerFrame)

where SFNstart time and slotstart time are the SFN and slot, respectively, of the first transmission of PDSCH where the configured downlink assignment was (re-)initialised.

NOTE:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the occurrences of configured downlink assignments.

[TS 38.300, clause 10.2]

In the downlink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible assignments when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

The gNB may pre-empt an ongoing PDSCH transmission to one UE with a latency-critical transmission to another UE. The gNB can configure UEs to monitor interrupted transmission indications using INT-RNTI on a PDCCH. If a UE receives the interrupted transmission indication, the UE may assume that no useful information to that UE was carried by the resource elements included in the indication, even if some of those resource elements were already scheduled to this UE.

In addition, with Semi-Persistent Scheduling (SPS), the gNB can allocate downlink resources for the initial HARQ transmissions to UEs: RRC defines the periodicity of the configured downlink assignments while PDCCH addressed to CS-RNTI can either signal and activate the configured downlink assignment, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the downlink assignment can be implicitly reused according to the periodicity defined by RRC, until deactivated.

NOTE:	When required, retransmissions are explicitly scheduled on PDCCH(s).

The dynamically allocated downlink reception overrides the configured downlink assignment in the same serving cell, if they overlap in time. Otherwise a downlink reception according to the configured downlink assignment is assumed, if activated.

The UE may be configured with up to 8 active configured downlink assignments for a given BWP of a serving cell. When more than one is configured:

-	The network decides which of these configured downlink assignments are active at a time (including all of them); and

-	Each configured downlink assignment is activated separately using a DCI command and deactivation of configured downlink assignments is done using a DCI command, which can either deactivate a single configured downlink assignment or multiple configured downlink assignments jointly.

[TS 38.213, clause 10.2]

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and

-	the new data indicator field in the DCI format for the enabled transport block is set to '0', and

-	the DFI flag field, if present, in the DCI format is set to '0', and

-	if validation is for scheduling activation and if the PDSCH-to-HARQ_feedback timing indicator field in the DCI format is present, the PDSCH-to-HARQ_feedback timing indicator field does not provide an inapplicable value from dl-DataToUL-ACK-r16.

If a UE is provided a single configuration for UL grant Type 2 PUSCH or for SPS PDSCH, validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

If a UE is provided more than one configurations for UL grant Type 2 PUSCH or for SPS PDSCH, a value of the HARQ process number field in a DCI format indicates an activation for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by ConfiguredGrantConfigIndex or by sps-ConfigIndex, respectively. Validation of the DCI format is achieved if the RV field for the DCI format is set as in Table 10.2-3.

If a UE is provided more than one configuration for UL grant Type 2 PUSCH or for SPS PDSCH

-	if the UE is provided ConfiguredGrantConfigType2DeactivationStateList or sps-ConfigDeactivationStateList, a value of the HARQ process number field in a DCI format indicates a corresponding entry for scheduling release of one or more UL grant Type 2 PUSCH or SPS PDSCH configurations

-	if the UE is not provided ConfiguredGrantConfigType2DeactivationStateList or sps-ConfigDeactivationStateList, a value of the HARQ process number field in a DCI format indicates a release for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by ConfiguredGrantConfigIndex or by sps-ConfigIndex, respectively

Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-4.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format.

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2 | DCI format 1_1 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with μ=1 set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

Table 10.2-3: Special fields for a single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2 | DCI format 1_1 |
| --- | --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-4: Special fields for a single or multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with μ=1 set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after N symbols from the last symbol of a PDCCH providing the SPS PDSCH release. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the SPS PDSCH release, N=5 for μ=0, N=5.5 for μ=1, and N=11 for μ=2, otherwise, N=10 for μ=0, N=12 for μ=1, N=22 for μ=2, and N=25 for μ=3, wherein μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH providing the SPS PDSCH release and the SCS configuration of a PUCCH carrying the HARQ-ACK information in response to a SPS PDSCH release.

7.1.1.6.4.3	Test description

7.1.1.6.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink and that the UM DRB is configured according to Table 7.1.1.6.4.3.1-1.

Table 7.1.1.6.4.3.1-1: RLC parameters

| Uplink UM RLC sn-FieldLength | IF (pc_um_WithShortSN ) size6 ELSE size12 |
| --- | --- |
| Downlink UM RLC sn-FieldLength | F (pc_um_WithShortSN ) size6 ELSE size12 |

7.1.1.6.4.3.2	Test procedure sequence

Table 7.1.1.6.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration to config SPS-ConfigurationDL.(Note 2) | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete.(Note 3) | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS transmits a DL assignment using UE’s CS-RNTI associated with sps-ConfigIndex=0 in Slot ‘Y’, NDI=0 | <-- | (DL SPS Grant) | - | - |
| 4 | The SS transmits in Slot ‘Y’, a DL MAC PDU containing a RLC PDU on UM DRB. | <-- | MAC PDU | - | - |
| 5 | Check: does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 6 | The SS transmits in Slot ‘Y+X’, a DL MAC PDU containing a RLC PDU on DRB. (Note 1) | <-- | MAC PDU | - | - |
| 7 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 8 | SS transmits NR RRCReconfiguration to add another SPS-ConfigurationDL.(Note 2) | <-- | RRCConnectionReconfiguration | - | - |
| 9 | The UE transmits NR RRCReconfigurationComplete.(Note 3) | --> | RRCConnectionReconfigurationComplete | - | - |
| 10 | The SS transmits a DL assignment using UE’s CS-RNTI associated with sps-ConfigIndex=1 in Slot ‘P’, NDI=0; (Where Y+X<P<Y+2X) | <-- | (DL SPS Grant) | - | - |
| 11 | The SS transmits in Slot ‘P’, a DL MAC PDU containing a RLC PDU on UM DRB. | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2 | P |
| 13 | The SS transmits in Slot ‘Y+2X’, a DL MAC PDU containing a RLC PDU on UM DRB. | <-- | MAC PDU | - | - |
| 14 | Check: Does the UE transmit a HARQ Feedback? | --> | HARQ ACK | 2 | P |
| 15 | The SS transmits in Slot ‘P+X’, a DL MAC PDU containing a RLC PDU (DL-SQN=1) on UM DRB. | <-- | MAC PDU | - | - |
| 16 | Check: Does the UE transmit a HARQ Feedback? | --> | HARQ ACK | 2 | P |
| 17 | SS transmits NR RRCReconfiguration to disable SPS-ConfigurationDL.(Note 2) | <-- | RRCConnectionReconfiguration | - | - |
| 18 | The UE transmits NR RRCReconfigurationComplete.(Note 3) | --> | RRCConnectionReconfigurationComplete | - | - |
| Note 1: X is equal to semiPersistSchedIntervalDL in this document. Note 2: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 3: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 6: As per TS 38.508-1[4], the default value for PDSCH slot offset (K0) is 0, hence the DL MAC PDU’s associated with DL SPS grant in Slot X are sent insame slot X. |  |  |  |  |  |

7.1.1.6.4.3.3	Specific message contents

Table 7.1.1.6.4.3.3-1: RRCReconfiguration (step 1/8, Table 7.1.1.6.4.3.2-1)

| Derivation path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension := SEQUENCE{} | Not present |  | EN-DC |
| nonCriticalExtension := SEQUENCE{ |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.4.3.3-2: CellGroupConfig (in Table 7.1.1.6.4.3.3-1)

| Derivation path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| physicalCellGroupConfig | PhysicalCellGroupConfig | Table 7.1.1.6.4.3.3-3 |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | 1 |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| sps-ConfigToAddModList-r16 ::= SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-Config { |  |  |  |
| SPS-Config ::= SEQUENCE [1] { |  | The first SPS configuration entry | STEP 1 |
| setup SEQUENCE { |  |  |  |
| periodicity | ms160 |  |  |
| nrofHARQ-Processes | 8 |  |  |
| sps-ConfigIndex-r16 | 0 |  |  |
| harq-ProcID-Offset-r16 | 0 |  |  |
| sps-ConfigIndex-r16 | 0 |  |  |
| } |  |  |  |
| SPS-Config ::= SEQUENCE [2] { |  | The second SPS configuration entry | STEP 8 |
| setup SEQUENCE { |  |  |  |
| periodicity | ms160 |  |  |
| nrofHARQ-Processes | 8 |  |  |
| sps-ConfigIndex-r16 | 1 |  |  |
| harq-ProcID-Offset-r16 | 8 |  |  |
| sps-ConfigIndex-r16 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.4.3.3-3: PhysicalCellGroupConfig (Table 7.1.1.6.4.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| cs-RNTI | ‘FFE0’H |  |  |
| } |  |  |  |

Table 7.1.1.6.4.3.3-4: RRCReconfiguration (step 17 of Table 7.1.1.6.4.3.2-1)

| Derivation path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension := SEQUENCE{} | Not present |  | EN-DC |
| nonCriticalExtension := SEQUENCE{ |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.4.3.3-5: CellGroupConfig (Table 7.1.1.6.4.3.3-4)

| Derivation path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| servCellIndex |  | 1 |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |  |
| sps-ConfigToReleaseList-r16 SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16 { |  | 2 entries | Release all the SPS entries configured previously |  |
| SPS-ConfigIndex-r16[1] |  | 0 |  |  |
| SPS-ConfigIndex-r16[2] |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |