3GPP TSG-RAN5 Meeting #90-e	R5-210800r1

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2045 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new MDT TC 8.1.6.1.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Tejet, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the SON and MDT WP, the new TC 8.1.6.1.3.3 Radio Link Failure / Reporting at RRC connection establishment and reestablishment is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New MDT TC 8.1.6.1.3.3 was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0128 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: 1. TP description updated to reflect the trigger conditions. 2. NR cells updated in power level tables. 3. Generic procedure 4.9.5 was used to trigger the RRC setup request. 4. RRCReconfiguration/RRCReconfiguration added to resume SRB/DRB. 5. Values added for ssbRLMConfigBitmap-r16 and c-RNTI-r16. 6. IEs not included in the UEInformationResponse message set to Not present while mandatory IE timeSinceFailure-r16set to Not checked. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

8.1.6.1.3.3	Radio Link Failure / Reporting at RRC connection establishment and reestablishment

8.1.6.1.3.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state detecting radio link failure }

ensure that {

when { T301 expires after UE having sent an RRCReestablishmentRequest message, the UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCSetupComplete message with rlf-InfoAvailable included after receiving an RRCSetup message }

}

(2)

with { UE in NR RRC_CONNECTED state detecting radio link failure }

ensure that {

when { UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCReestablishmentComplete message with rlf-InfoAvailable included after receiving an RRCReestablishment message }

}

(3)

with { UE in NR RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq set to true }

then { UE sends the UEInformationResponse message with rlf-Report included }

}

8.1.6.1.3.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.3.4, 5.3.7.5, 5.3.10.3, 5.3.7.4 and 5.7.10.3.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

…

1>	perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

…

1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

…

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	if reconnectCellId in VarRLF-Report is not set:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link or handover failure;

4>	set nrReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

3>	include rlf-InfoAvailable in the RRCSetupComplete message;

…

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReestablishmentComplete message;

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	else:

2>	during a DAPS handover: the following only applies for the target PCell;

2>	upon T310 expiry in PCell; or

…

2>	upon consistent uplink LBT failure indication from MCG MAC while T304 is not running:

…

3>	else:

4>	consider radio link failure to be detected for the MCG i.e. RLF;

4>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

NOTE:	Void.

4>	if AS security has not been activated:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

4>	else if AS security has been activated but SRB2 and at least one DRB or, for IAB, SRB2, have not been setup:

5>	store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5;

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5;

…

5>	else:

6>	initiate the connection re-establishment procedure as specified in 5.3.7.

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected.

[TS 38.331, clause 5.3.7.4]

The UE shall set the contents of RRCReestablishmentRequest message as follows:

1>	if the procedure was initiated due to radio link failure as specified in 5.3.10.3 or handover failure as specified in 5.3.5.8.3:

2>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.3.3	Test description

8.1.6.1.3.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 (TAI-1) is the serving cell.

-	NR Cell 3 and Cell 6 are the inter-frequency neighbor cells.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

NOTE: For NR Cell 3 and Cell 6 TAI is set to TAI-2.

UE:

None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.

8.1.6.1.3.3.3.2	Test procedure sequence

Table 8.1.6.1.3.3.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.3.3.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | "Off" | Only Cell 1 is available. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | "Off" | Only Cell 3 is available. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | -88 | Only Cell 6 is available. |

Table 8.1.6.1.3.3.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" | "Off" | Only Cell 1 is available. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -82 | "Off" | Only Cell 3 is available. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | -82 | Only Cell 6 is available. |

Table 8.1.6.1.3.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 1 and Cell 3 parameters according to the row "T1" in Table 8.1.6.1.3.3.3.2-1/2. | - | - | - | - |
| 2 | The UE transmits an RRCReestablishmentRequest message on NR Cell 3. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 3 | The SS does not respond to any RRCReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to NR RRC_IDLE state on NR Cell 3. | - | - | - | - |
| 4-5 | Steps 1-2 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. | - | - | - | - |
| 6 | Check: Does the UE transmit an RRCSetupComplete message with rlf-InfoAvailable included and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 1 | P |
| 7-8 | Steps 4-5 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 9 | The SS changes NR Cell 3 and Cell 6 parameters according to the row "T2" in Table 8.1.6.1.3.3.3.2-1/2. | - | - | - | - |
| 10 | The UE transmits an RRCReestablishmentRequest message on NR Cell 6. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 11 | The SS transmits an RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 12 | Check: Does the UE transmit an RRCReestablishmentComplete message with rlf-InfoAvailable included? | --> | NR RRC: RRCReestablishmentComplete | 2 | P |
| 13 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 14 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 15 | The SS transmits a UEInformationRequest message with rlf-ReportReq set to true. | <-- | NR RRC: UEInformationRequest | - | - |
| 16 | Check: Does the UE transmit a UEInformationResponse message on Cell 6 with rlf-Report included? | --> | NR RRC: UEInformationResponse | 3 | P |

8.1.6.1.3.3.3.3	Specific message contents

Table 8.1.6.1.3.3.3.3-1: RRCReestablishmentRequest (steps 2 and 10, Table 8.1.6.1.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PCI of NR Cell 1 |  | Step 2 |
|  | PCI of NR Cell 3 |  | Step 14 |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |

Table 8.1.6.1.3.3.3.3-2: RRCSetupComplete (step 6, Table 8.1.6.1.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.3.3.3-3: RRCReestablishmentComplete (step 12, Table 8.1.6.1.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.3.3.3-4: UEInformationRequest (step 15, Table 8.1.6.1.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |
| rlf-ReportReq-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.3.3.3-5: UEInformationResponse (step 16, Table 8.1.6.1.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| c-RNTI-r16 | the value of the C-RNTI of the UE |  |  |
| previousPCellId-r16 | Not present |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 CHOICE { |  |  |  |
| pci-arfcn-r16 SEQUENCE { |  |  |  |
| physCellId-r16 | PCI of NR Cell 3 |  |  |
| carrierFreq-r16 | NARFCN of NR Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 6 |  |  |
| cellIdentity-r16 | CGI of NR Cell 6 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 6 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Not present |  |  |
| timeSinceFailure-r16 | Not checked |  |  |
| connectionFailureType-r16 | rlf |  |  |
| rlf-Cause-r16 | t310-Expiry |  |  |
| locationInfo-r16 | Not present or any allowed value |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]