3GPP TSG-RAN5 Meeting #90-e	R5-210808r1

# Electronic Meeting, 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2047 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GS Non-3GPP Access Test Case 9.2.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_Ph1-CT_n3GPPA-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no specifying about how to achieve the pre-test condition "The UE does not have a valid 5G NAS security context". |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed the pre-test condition and instead extend the expected sequence to include the relevant steps. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE can not pass the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Updated to refer to the procedure defined in 38.508-1 clause 4.9.8, which the expected sequence are all defined in. |  |  |  |  |  |  |  |  |

9.2.2.2	Protection of initial NAS signalling messages

9.2.2.2.1	Test Purpose (TP)

Same Test purpose as in clause 9.1.2.2.1

9.2.2.2.2	Conformance requirements

Same conformance requirements as in clause 9.1.2.2.2

9.2.2.2.3	Test description

9.2.2.2.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27.

UE:

-	None.

Preamble:

-	The UE is in state 0W-B on WLAN Cell 27 according to TS 38.508-1 [4].

-	The UE does not have a valid 5G NAS security context by performing the procedure defined in subclause 4.9.8 in 38.508-1 [4].

9.2.2.2.3.2	Test procedure sequence

Table 9.2.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-3 | Steps 1-2 of the generic procedure for UE registration specified in TS 38.508-1 [4] table 4.5.2.2-3 are performed. | - | - | - | - |
| - | Exception: The UE establishes an IPsec tunnel in parallel to 5GC registration steps 4 to 8 as per the IKEv2 protocol as defined in 3GPP TS 23.502 [31] clause 4.12.2.2 figure 4.12.2.2-1. | - | - | - | - |
| 4 | The UE transmits a REGISTRATION REQUEST message. | --> | REGISTRATION REQUEST | 1 | P |
| 5 | The SS transmits a DLInformationTransfer message and an AUTHENTICATION REQUEST message. | <-- | AUTHENTICATION REQUEST | - | - |
| 6 | The UE transmits an ULInformationTransfer message and an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 7 | The SS transmits a DLInformationTransfer message and a SECURITY MODE COMMAND message. | <-- | SECURITY MODE COMMAND |  |  |
| 8 | The UE transmits an ULInformationTransfer message and a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | 2 | P |
| 9-11 | Steps 8-10 of the generic procedure for UE registration specified in TS 38.508-1 [4] table 4.5.2.2-3 are performed. | - | - | - | - |

9.2.2.2.3.3	Specific message contents

Table 9.2.2.2.3.3-1: REGISTRATION REQUEST (Step 4, Table 9.2.2.2.3.2-1)

| Derivation path: TS 38.508-1 [4],table 4.7.1-6 using condition NON_CLEARTEXT_IE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |

Table 9.2.2.2.3.3-2: SECURITY MODE COMPLETE (Step 8, Table 9.2.2.2.3.2-1)

| Derivation path: TS 38.508-1 [4],table 4.7.1-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| NAS message container | Contents of Table 9.2.2.2.3.3-3 |  |  |

Table 9.2.2.2.3.3-3: REGISTRATION REQUEST (Step 8, Table 9.2.2.2.3.2-1)

| Derivation path: TS 38.508-1 [4],table 4.7.1-6 using condition CIPHERED_MESSAGE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |