3GPP TSG-RAN5 Meeting #90-e	R5-210872r2

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1751 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default configuration of CSI-IM for RRM tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add CSI-IM resource related default configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CSI-IM resource related default configurations are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Added “entry” to the tables - Changed hardcode to “xx” 2nd revision: - missing reference for CSI-ResourceConfig for CSI-IM is added. - titles are added. - Positions of CSI-ResourceConfig and CSI-ResourceConfigId are moved. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 7.3.1-6: CSI-MeasConfig for RRM

| Derivation Path: Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n1+n2+n3+n4+n5 entries | n1=1 if CSI-RS for CSI is configured in test case, n1=0 otherwise; n2=2 if CSI-RS for BM is configured in test case, n2=0 otherwise; n3=4 if TRS is configured in test case, n3=0 otherwise; n4=4 if second resource set of TRS is configured in test case, n4=0 otherwise; n5=2 if aperiodic CSI-RS for BM is configured in test case, n5=0 otherwise; |  |
| NZP-CSI-RS-Resource[k, k=1..n1] | NZP-CSI-RS-Resource for CSI | entry 1 | n1>0 |
| NZP-CSI-RS-Resource[k, k=n1+1.. n1+n2] | NZP-CSI-RS-Resource for BM (k-n1-1) | entry ... | n2>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+1.. n1+n2+n3] | NZP-CSI-RS-Resource for TRS (k-n1-n2) | entry ... | n3>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+1.. n1+n2+n3+n4] | NZP-CSI-RS-Resource for TRS (k-n1-n2-n3) with condition SECOND_SET | entry ... | n4>0 |
| NZP-CSI-RS-Resource[k, k= n1+n2+n3+n4+1.. n1+n2+n3+n4+n5] | NZP-CSI-RS-Resource for BM (k-n1-n2-n3-n4) with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | m1+m2+m3+m4+m5 entries | mi=1 if ni>0, mi=0 otherwise |  |
| NZP-CSI-RS-ResourceSet[k, k=1..m1] | NZP-CSI-RS-ResourceSet for CSI | entry 1 | n1>0 |
| NZP-CSI-RS-ResourceSet[k, k=m1+1.. m1+m2] | NZP-CSI-RS-ResourceSet for BM | entry ... | n2>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+1.. m1+m2+m3] | NZP-CSI-RS-ResourceSet for TRS | entry ... | n3>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+1.. m1+m2+m3+m4] | NZP-CSI-RS-ResourceSet for TRS with condition SECOND_SET | entry ... | n4>0 |
| NZP-CSI-RS-ResourceSet[k, k= m1+m2+m3+m4+1.. m1+m2+m3+m4+m5] | NZP-CSI-RS-ResourceSet for BM with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| csi-IM-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource { | 1 entry |  | n1>0 |
| CSI-IM-Resource[1] | CSI-IM-Resource-RRM | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  | n1=0 |
| csi-IM-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet { | 1 entry |  | n1>0 |
| CSI-IM-ResourceSet[1] | CSI-IM-ResourceSet-RRM | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  | n1=0 |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | p1+m2+m3+m5 entries | p1=m1+1 if n1>0, p1=0 otherwise. |  |
| CSI-ResourceConfig[k, k=1..m1] | CSI-ResourceConfig for CSI | entry 1 | n1>0 |
| CSI-ResourceConfig[k, k=p1] | CSI-ResourceConfig for CSI-IM | entry ... | n1>0 |
| CSI-ResourceConfig[k, k=p1+1.. p1+m2] | CSI-ResourceConfig for BM | entry ... | n2>0 |
| CSI-ResourceConfig[k, k= p1+m2+1.. p1+m2+m3] | CSI-ResourceConfig for TRS | entry ... | n3>0 and n4=0 |
|  | CSI-ResourceConfig for TRS with condition SECOND_SET |  | n4>0 |
| CSI-ResourceConfig[k, k= p1+m2+m3+1.. p1+m2+m3+m5] | CSI-ResourceConfig for BM with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | m1+m2+m5 entries |  |  |
| CSI-ReportConfig[k, k=1..m1] | CSI-ReportConfig for CSI | entry 1 | n1>0 |
| CSI-ReportConfig[k, k=m1+1.. m1+m2] | CSI-ResourceConfig for BM | entry ... | n2>0 |
| CSI-ReportConfig[k, k=m1+m2+1.. m1+m2+m5] | CSI-ResourceConfig for BM with condition APERIODIC | entry ... | n5>0 |
| } |  |  |  |
| reportTriggerSize | Not present |  |  |
|  | 1 |  | n5>0 |
| aperiodicTriggerStateList CHOICE { | Not present |  |  |
| setup | CSI-AperiodicTriggerStateList |  | n5>0 |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>

<Start of modified section 2>

Table 7.3.1-12B: CSI-ResourceConfig for BM

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-BM |  |  |
|  | CSI-ResourceConfigId-BM with condition APERIODIC |  | APERIODIC |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-BM |  |  |
|  | NZP-CSI-RS-ResourceSetId-BM with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| resourceType | aperiodic |  | APERIODIC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-ResourceConfig for CSI-IM

Table 7.3.1-BA: CSI-ResourceConfig for CSI-IM

| Derivation Path: Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-CSI-IM |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId { | 1 entry |  |  |
| CSI-IM-ResourceSetId[1] | CSI-IM-ResourceSetId-RRM | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

<End of modified section 2>

<Start of modified section 3>

Table 7.3.1-12E: CSI-ResourceConfigId-BM

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the CSI-ResourceConfigId allocated for resource config of CSI-RS for BM. Value of n is left to internal implementation |  |
|  | m | m is the CSI-ResourceConfigId allocated for resource config of aperiodic CSI-RS for BM. Value of m is left to internal implementation | APERIODIC |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

#### –	CSI-ResourceConfigId-CSI-IM

Table 7.3.1-12EA: CSI-ResourceConfigId-CSI-IM

| Derivation Path: Table 4.6.3-42 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfigId | n | n is the first CSI-ResourceConfigId allocated for CSI-IM resource configuration Value of n is left to internal implementation |  |

<End of modified section 3>

<Start of modified section 4>

Table 7.3.1-12F: CSI-ReportConfig for CSI

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-CSI |  |  |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-CSI |  |  |
| csi-IM-ResourcesForInterference | CSI-ResourceConfigId-CSI-IM |  |  |
| nzp-CSI-RS-ResourcesForInterference | Not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic | Set according to parameters given in test cases |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 4>

<Start of modified section 5>

#### – 	ServingCellConfig

Table 7.3.1-23: ServingCellConfig

| Derivation Path: Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig CHOICE { |  |  |  |
| setup | csi-MeasConfig |  |  |
| } |  |  |  |

#### –	CSI-IM-Resource-RRM

Table 7.3.1-xx: CSI-IM-Resource-RRM

| Derivation Path: Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId-RRM |  |  |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| pattern1 SEQUENCE { |  |  |  |
| subcarrierLocation-p1 | s8 |  |  |
| symbolLocation-p1 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for CSI |  |  |
| } |  |  |  |

#### –	CSI-IM-ResourceId-RRM

Table 7.3.1-xx: CSI-IM-ResourceId-RRM

| Derivation Path: Table 4.6.3-35 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceId | n | n is the first CSI-IM-ResourceId allocated for CSI-IM resource. Value of n is left to internal implementation |  |

#### –	CSI-IM-ResourceSet-RRM

Table 7.3.1-xx: CSI-IM-ResourceSet-RRM

| Derivation Path: Table 4.6.3-36 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId-RRM |  |  |
| csi-IM-Resources SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId { | 1 entry |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId-RRM | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

#### –	CSI-IM-ResourceSetId-RRM

Table 7.3.1-xx: CSI-IM-ResourceSetId-RRM

| Derivation Path: Table 4.6.3-37 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | n | n is the first CSI-IM-ResourceSetId allocated for CSI-IM resource set. Value of n is left to internal implementation |  |

<End of modified section 5>