3GPP TSG-RAN5 Meeting #90-e	R5-210885r1

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2563 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 7.1.28 E-UTRAN TDD-UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRM Test Case 7.1.28 E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage has been introduced in RAN4 [4] core-spec, but not adding in RAN5 TS 36.521-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Test Case 7.1.28 and TT reused from HD-FDD Test Cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case 7.1.28 will be still missed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.28 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Change CR title to modify the Invalid characters. - Put all non-test case titles to H6 - Adding infomration in summary of change |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage

7.1.28.1	Test purpose

To verify that the Category NB1 UE under enhanced coverage is capable of following the frame timing change of the connected eNode B, that the UE initial transmit timing accuracy is within the specified limits and that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission.

7.1.28.2	Test applicability

This test applies to all types of E-UTRA TDD UE release 15 and forward of UE Category NB1 and NB2.

7.1.28.3	Minimum conformance requirements

Same minimum conformance requirements as defined in 7.1.18.3.

7.1.28.4	Test description

7.1.28.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: EUTRA Bandwidth is 10 MHz, as defined in TS 36.508 [7] clause 4.3.1.1, except for bands within band group FDD_N where 5 MHz bandwidth is used. Ncell bandwidth is as specified in Table 7.1.28.5-1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.10 using only main Tx/Rx antenna and without using the faders.

2.	The general test parameter settings are set according to Table 7.1.28.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	There is one E-UTRA TDD carrier and one cell specified in the test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

Table 7.1.28.4.1-1: General Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Category NB1 UE in In-Band mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| NB-IoT Operation mode |  | In-band | In-band |
| DRX |  | OFF | ON |
| Special subframe configuration |  | 6 |  |
| Uplink-downlink configuration |  | 1 |  |
| NPRACH configuration |  | NPRACH.R-2 |  |
| NPDCCH repetition level |  | 32 | 32 |
| npdcch-StartSF-USS Note 2 |  | v2 | v2 |
| npdcch-NumRepetitions-r13 Note 2 |  | r32 | r32 |
| NPUSCH resource units |  | 1 | 1 |
| NPUSCH repetition level |  | 128 | 128 |
| NPUSCH subcarrier spacing | kHz | 15 | 15 |
| NPUSCH number of subcarriers |  | 1 | 1 |
| NPUSCH modulation |  |  /4QPSK |  /4QPSK |
| NPUSCH Transport block size | Bits | 40 | 40 |
| Note 1: DRX related parameters are defined in Table A.7.1.28.1-4. Note 2: For further information see clause 6.7.3.2 in TS 36.331 [2]. |  |  |  |

7.1.28.4.2	Test procedure

Same test procedure as in clause 7.1.18.4.2.

7.1.28.4.3	Message contents

Same message contents as in clause 7.1.18.4.3.

7.1.28.5	Test requirement

Tables 7.1.28.5-1, 7.1.28.5-2 and 7.1.28.5-3 define the primary settings of Ncell and Cell for UE transmit timing accuracy for TDD Category NB1 UE transmit timing accuracy test.

Table 7.1.18.5-4 define the timing error limit with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1 include Test Tolerances.

Table 7.1.28.5-1: nCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Category NB1 UE in In-Band mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| RF Channel Number |  | 1 | 1 |
| BWchannel | kHz | 200 | 200 |
| PRB location within eCell |  | eCell1 BWchannel 10MHz: 30 | eCell1 BWchannel 10MHz: 30 |
| NPDSCH parameter |  | eCell1 BWchannel 10MHz: R.15 NB-TDD | eCell1 BWchannel 10MHz: R.15 NB-TDD |
| NPDCCH parameter |  | eCell1 BWchannel 10MHz:R.29 NB-TDD | eCell1 BWchannel 10MHz: R.29 NB-TDD |
| NPBCH_RA | dB | -3 | -3 |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| NOCNG_RA Note1 |  |  |  |
| NOCNG_RB Note1 |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.1.28.1-3 | Specified in Table A.7.1.28.1-3 |
|  | dB | -10.7 | -10.7 |
|  | dB | -10.7 | -10.7 |
| Antenna Configuration |  | 2x1 | 2x1 |
| Propagation condition | - | AWGN | AWGN |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.1.28.5-2: eCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Category NB1 UE in In-Band mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| E-UTRA RF Channel Number |  | 1 | 1 |
| BWchannel | MHz | 10 | 10 |
| NOCNG pattern |  | BWchannel 10MHz: NOP.1 TDD | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 | -3 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
|  | dBm/15 kHz | -98 | -98 |
|  | dB | -11 | -11 |
| Antenna Configuration |  | 2x1 | 2x1 |
| Propagation condition | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.1.28.5-3: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 for E-UTRAN TDD Category NB1 UE In-band mode under enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
|  | Test 2 |  |
| onDurationTimer | pp1 |  |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | sf2048 |  |
| shortDRX | disable |  |
| Note 1: For further information see clause 6.7.3 in TS 36.331 [2]. |  |  |

The UE transmit timing offset shall be within the requirements in Table 7.1.28.5-4. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission.

The reference point for the UE initial transmit timing control test requirement shall be the downlink timing minus .

Table 7.1.18.5-4: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 83*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

## << End of changes >>