3GPP TSG-RAN5 Meeting #90-e	R5-210904r1

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1149 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating test case 7.3C.2-Reference sensitivity power level for SUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case 7.3C.2 is incomplete and need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating test case 7.3C.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | REFSENS for SUL can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0356 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Adding NOTE 3 and NOTE 4 inTable 7.3C.2.4.1-1. The NOTE 4 is merged from R5-210488. CAICT is added as co-source. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.3C.2	Reference sensitivity power level for SUL

Editor’s Note:  The following aspects are either missing or not yet determined:

-	Exceptional test points for configurations except SUL_n78-n80 is FFS

7.3C.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under SUL operation and conditions of low signal level, ideal propagation and no added noise.

7.3C.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports SUL operation on the SUL bands.

7.3C.2.3	Minimum conformance requirement

The minimum conformance requirements are defined in clause 7.3C.0.

7.3C.2.4	Test description

7.3C.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3C.2.4.1-1 and 7.3C.2.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3C.2.4.1-1: Test Configuration Table for SUL without exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  | Mid range for SUL carrier. Low, Mid, High range for non-SUL carrier With following exceptions: SUL_n78-n80: High in band n78 |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  | Lowest, Mid, Highest for Non-SUL carrier For SUL band: n80: 30 MHz n81: 20 MHz n82: 20 MHz n83: 20 MHz n84: 20 MHz n86: 40 MHz n95: 15 MHz |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  |  |  |  |  |  |  | 15kHz for both SUL carrier Lowest for Non-SUL carrier |  |  |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID |  | Downlink Configuration |  |  |  |  |  | UL Configuration |  |  | SUL Configuration |  |  |  |  |  |  |
|  |  | Modulation |  |  | RB allocation |  |  |  |  |  | Modulation |  |  |  | RB allocation (NOTE 2) |  |  |
| 1 |  |  | CP-OFDM QPSK |  |  | Full RB (NOTE 1) | N/A |  |  |  |  | DFT-s-OFDM QPSK |  |  | REFSENS (NOTE 2) |  |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3C.2.4.1-1a which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: In a band where UE supports 4Rx, the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3C.2.4.1-1a: SUL configuration for reference sensitivity, LCRB @ RBstart format (without exception)

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / LCRB@RBStart of SUL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n80 | 15 |  | 160@0 | 160@0 | 160@0 |  |  | 160@0 | 160@0 | 160@0 | 160@0 | 160@0 | 160@0 |
| n41 | n81 | 15 |  | 100@0 | 100@0 | 100@0 |  |  | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 |
| n41 | n95 | 15 |  | 75@0 | 75@0 | 75@0 |  | 75@0 | 75@0 | 75@0 | 75@0 | 75@0 | 75@0 | 75@0 |
| n77 | n80 | 15 |  | 160@0 | 160@0 | 160@0 |  |  | 160@0 | 160@0 | 160@0 | 160@0 | 160@0 | 160@0 |
| n77 | n84 | 15 |  | 100@0 | 100@0 | 100@0 |  |  | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 |
| n78 | n80 | 15 |  | 160@0 | 160@0 | 160@0 |  |  | 160@0 | 160@0 | 160@0 | 160@0 | 160@0 | 160@0 |
| n78 | n81 | 15 |  | 100@0 | 100@0 | 100@0 |  |  | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 |
| n78 | n82 | 15 |  | 100@0 | 100@0 | 100@0 |  |  | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 |
| n78 | n83 | 15 |  | 100@0 | 100@0 | 100@0 |  |  | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 |
| n78 | n84 | 15 |  | 100@0 | 100@0 | 100@0 |  |  | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 | 100@0 |
| n78 | n86 | 15 |  | 216@0 | 216@0 | 216@0 |  |  | 216@0 | 216@0 | 216@0 | 216@0 | 216@0 | 216@0 |
| n79 | n80 | 15 |  |  |  |  |  |  | 160@0 | 160@0 | 160@0 | 160@0 |  | 160@0 |
| n79 | n81 | 15 |  |  |  |  |  |  | 100@0 | 100@0 | 100@0 | 100@0 |  | 100@0 |
| n79 | n84 | 15 |  |  |  |  |  |  | 100@0 | 100@0 | 100@0 | 100@0 |  | 100@0 |
| n79 | n95 | 15 |  |  |  |  |  |  | 75@0 | 75@0 | 75@0 | 75@0 |  | 75@0 |

Table 7.3C.2.4.1-2: Test configurations table for SUL operation exceptions due to UL harmonic issue

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | See range column for each CC |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | See CBW column for each CC |  |  |  |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  |  |  |  | 15kHz for SUL carrier Lowest for Non-SUL carrier |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | Downlink Configuration |  |  |  |  | UL Configuration |  | SUL Configuration |  |  |  |  |
|  | Band | Range | CBW | Mod | RB alloc (NOTE 1) |  |  | Band | Range | CBW | Mod | RB alloc (NOTE 2) |
| Test settings for SUL_n78-n80 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3560 | Highest | CP-OFDM QPSK | Full RB | N/A |  | n80 | High | 10 MHz | DFT-s-OFDM QPSK | REFSENS_SUL |
| 2 | n78 | 3530 | 20 MHz | CP-OFDM QPSK | Full RB | N/A |  | n80 | High | 10 MHz | DFT-s-OFDM QPSK | REFSENS_SUL |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS_SUL refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3C.0.2-3. |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1, C.2, C3.1, and uplink signals according to Annex G.0, G.1, G.2, G.3.0 with consideration of supplementary uplink physical channels.

4.	The UL and DL Reference Measurement Channel shall be set according to Table 7.3C.2.4.1-1 or 7.3C.2.4.1-2.

5.	The UL Reference Measurement Channel shall be set according to Table 7.3C.2.4.1-1 for REFSENS without exceptions and Table 7.3C.2.4.1-2 when testing is performed with SUL/DL band combination listed in Table 7.3C.0.2-2 for exceptions due to harmonic issue.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3C.2.4.3

7.3C.2.4.2	Test procedure

1	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3C.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC on SUL band according to Tables 7.3C.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3C.2.5-1 for 2Rx and table 7.3C.2.5-2 for 4Rx. Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

5.	For configurations listed in table 7.3C.2.4.1-2, repeat step 1-4 with table 7.3C.2.4.1-2 replacing table 7.3C.2.4.1-1 in step1 and step 2, table 7.3C.2.5.1-1 replacing 7.3C.2.5-1 and table 7.3C.2.5-2 in step 3.

7.3C.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Tables 4.6.3-14, 4.6.3-15 with condition SUL, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL, additionally the following exception shown in Table 7.3C.2.4.3-1 is considered.

Table 7.3C.2.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

7.3C.2.5	Test requirement

The throughput measured in step 4 shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 for REFSENS without exception testing with receive power level specified in Tables 7.3C.2.5-1 for 2Rx antenna port and Tables 7.3C.2.5-2 for 4 Rx antenna port, and parameters specified in table 7.3C.2.4.1-1.

Table 7.3C.2.5-0: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Table 7.3.2.5-1: Reference sensitivity QPSK PREFSENS for 2Rx

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT | -90.6 +TT | -89.6 +TT |  |  |  |  | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT | -90.7 +TT | -89.7 +TT |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT | -90.9 +TT | -89.7 +TT |  |  |  |  |  |
| n2 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |  |  |  |  |  |  |  |  |
| n3 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | -89.7 +TT | -88.9 +TT | -87.6 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -91.0 +TT | -89.8 +TT | -89.0 +TT | -87.7 +TT |  |  |  |  |  |  |
|  | 60 |  | -94.5 +TT | -92.4 +TT | -91.2 +TT | -90.0 +TT | -89.1 +TT | -87.9 +TT |  |  |  |  |  |  |
| n5 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -90.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -91.0 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |  |  |  |  |  |  |  |  |
| n8 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.0 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -90.2 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -93.6 +TT | -91.6 +TT | -90.5 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -94.0 +TT | -91.9 +TT | -90.7 +TT |  |  |  |  |  |  |  |  |  |
| n26 | 15 | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT |  |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |  |  |  |  |  |  |  |  |
| n28 | 15 | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT |  | -78.5 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  | -78.6 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | -99.0 +TT | -95.8 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.1 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT |  |  | -90.6 +TT |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT |  |  | -90.7 +TT |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |  | -90.9 +TT |  |  |  |  |  |  |
| n39 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT | -90.6 +TT |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT | -90.7 +TT |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT | -90.9 +TT |  |  |  |  |  |  |
| n40 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT | -90.6 +TT | -89.6 +TT |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT | -90.7 +TT | -89.7 +TT | -88.9 +TT | -87.6 +TT |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT | -90.9 +TT | -89.8 +TT | -89.1 +TT | -87.6 +TT |  |  |  |
| n411 | 15 |  | -94.8 +TT | -93.0 +TT | -91.8 +TT |  | -89.9 +TT | -88.6 +TT | -87.6 +TT |  |  |  |  | TDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  | -90.0 +TT | -88.7 +TT | -87.7 +TT | -86.9 +TT | -85.6 +TT | -85.1 +TT | -84.7 +TT |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  | -90.1 +TT | -88.9 +TT | -87.8 +TT | -87.1 +TT | -85.6 +TT | -85.1 +TT | -84.7 +TT |  |
| n481 | 15 | -99.0 +TT | -95.8 +TT | -94.0 +TT | -92.7 +TT |  |  | -89.6 +TT | -88.65 +TT |  |  |  |  | TDD |
|  | 30 |  | -96.1 +TT | -94.1 +TT | -92.9 +TT |  |  | -89.7 +TT | -88.75 +TT | -87.95 +TT | -86.65 +TT | -86.15 +TT | -85.65 +TT |  |
|  | 60 |  | -96.5 +TT | -94.4 +TT | -93.1 +TT |  |  | -89.9 +TT | -88.85 +TT | -88.05 +TT | -86.75 +TT | -86.25 +TT | -85.75 +TT |  |
| n50 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT |  | -91.9 +TT | -90.6 +TT | -89.6 +TT |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT |  | -92.0 +TT | -90.7 +TT | -89.7 +TT | -88.9 +TT | -87.6 +TT |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  | -92.1 +TT | -90.9 +TT | -89.8 +TT | -89.1 +TT | -87.6 +TT |  |  |  |
| n51 | 15 | -100.0 +TT |  |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 15 | -100.0 +TT | -96.8 +TT |  |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | -97.1 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.6+TT | -94.6+TT | -93.5+TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.0+TT | -94.9+TT | -93.7+TT |  |  |  |  |  |  |  |  |  |
| n66 | 15 | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | -92.2 +TT | -91.4 +TT | -90.1 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -96.6 +TT | -94.6 +TT | -93.5 +TT | -92.3 +TT | -91.5 +TT | -90.2 +TT |  |  |  |  |  |  |
|  | 60 |  | -97.0 +TT | -94.9 +TT | -93.7 +TT | -92.5 +TT | -91.6 +TT | -90.4 +TT |  |  |  |  |  |  |
| n70 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT |  |  |  |  |  |  |  |  |
| n71 | 15 | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.3 +TT | -91.9 +TT | -87.4 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 | - |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | -99.53 +TT | -96.33 +TT | -94.53 +TT | -93.33 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.63 +TT | -94.63 +TT | -93.53 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.03 +TT | -94.93 +TT | -93.73 +TT |  |  |  |  |  |  |  |  |  |
| n771,4 | 15 |  | -95.3 +TT | -93.5 +TT | -92.2 +TT |  |  | -89.1 +TT | -88.1 +TT |  |  |  |  | TDD |
|  | 30 |  | -95.6 +TT | -93.6 +TT | -92.4 +TT |  |  | -89.2 +TT | -88.2 +TT | -87.4 +TT | -86.1 +TT | -85.6 +TT | -85.1 +TT |  |
|  | 60 | - | -96.0 +TT | -93.9 +TT | -92.6 +TT |  |  | -89.4 +TT | -88.3 +TT | -87.5 +TT | -86.2 +TT | -85.7 +TT | -85.2 +TT |  |
| n781 | 15 |  | -95.8 +TT | -94.0 +TT | -92.7 +TT |  |  | -89.6 +TT | -88.6 +TT |  |  |  |  | TDD |
|  | 30 |  | -96.1 +TT | -94.1 +TT | -92.9 +TT |  |  | -89.7 +TT | -88.7 +TT | -87.9 +TT | -86.6 +TT | -86.1 +TT | -85.6 +TT |  |
|  | 60 |  | -96.5 +TT | -94.4 +TT | -93.1 +TT |  |  | -89.9 +TT | -88.8 +TT | -88.0 +TT | -86.7 +TT | -86.2 +TT | -85.7 +TT |  |
| n791 | 15 |  |  |  |  |  |  | -89.6 +TT | -88.6 +TT |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  | -89.7 +TT | -88.7 +TT | -87.9 +TT | -86.6 +TT |  | -85.6 +TT |  |
|  | 60 |  |  |  |  |  |  | -89.9 +TT | -88.8 +TT | -88.0 +TT | -86.7 +TT |  | -85.7 +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2C.1 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: TT for each frequency and channel bandwidth is specified in Table 7.3C.2.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3C.2.5-2: Reference sensitivity QPSK PREFSENS for Four Rx antenna ports

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n1 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT | -94.6 +TT | -93.3 +TT | -92.3 +TT |  |  |  |  | FDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT | -94.7 +TT | -93.4 +TT | -92.4 +TT |  |  |  |  |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT | -94.8 +TT | -93.6 +TT | -92.4 +TT |  |  |  |  |  |
| n2 | 15 | -100.7 +TT | -97.5 +TT | -95.7 +TT | -94.5 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.8 +TT | -95.8 +TT | -94.7 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -98.2 +TT | -96.1 +TT | -94.9 +TT |  |  |  |  |  |  |  |  |  |
| n3 | 15 | -99.7 +TT | -96.5 +TT | -94.7 +TT | -93.5 +TT | -92.4 +TT | -91.6 +TT | -90.3 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -96.8 +TT | -94.8 +TT | -93.7 +TT | -92.5 +TT | -91.7 +TT | -90.4 +TT |  |  |  |  |  |  |
|  | 60 |  | -97.2 +TT | -95.1 +TT | -93.9 +TT | -92.7 +TT | -91.8 +TT | -90.6 +TT |  |  |  |  |  |  |
| n7 | 15 | -100.7 +TT | -97.5 +TT | -95.7 +TT | -94.5 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.8 +TT | -95.8 +TT | -94.7 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -98.2 +TT | -97.1 +TT | -94.9 +TT |  |  |  |  |  |  |  |  |  |
| n30 | 15 | -101.7 +TT | -98.5 +TT |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -98.8 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT |  |  |  |  |  |  |  |  |  | TDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT |  |  |  |  |  |  |  |  |  |  |
| n38 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT |  |  | -93.3 +TT |  |  |  |  |  | TDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT |  |  | -93.4 +TT |  |  |  |  |  |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT |  |  | -93.6 +TT |  |  |  |  |  |  |
| n39 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT | -94.6 +TT | -93.3 +TT |  |  |  |  |  | TDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT | -94.7 +TT | -93.4 +TT |  |  |  |  |  |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT | -94.8 +TT | -93.6 +TT |  |  |  |  |  |  |
| n40 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT | -94.6 +TT | -93.3 +TT | -92.3 +TT |  |  |  |  | TDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT | -94.7 +TT | -93.4 +TT | -92.4 +TT | -91.6 +TT | -90.3 +TT |  |  |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT | -94.8 +TT | -93.6 +TT | -92.5 +TT | -91.8 +TT | -90.3 +TT |  |  |  |
| n41 | 15 |  | -97.5 +TT | -95.7 +TT | -94.5 +TT |  | -92.6 +TT | -91.3 +TT | -90.3 +TT |  |  |  |  | TDD |
|  | 30 |  | -97.8 +TT | -95.8 +TT | -94.7 +TT |  | -92.7 +TT | -91.4 +TT | -90.4 +TT | -89.6 +TT | -88.3 +TT | -87.8 +TT | -87.4 +TT |  |
|  | 60 |  | -98.2 +TT | -96.1 +TT | -94.9 +TT |  | -92.8 +TT | -91.6 +TT | -90.5 +TT | -89.8 +TT | -88.3 +TT | -87.8 +TT | -87.4 +TT |  |
| n48 | 15 | -101.2 +TT | -98.0 +TT | -96.2 +TT | -94.9 +TT |  |  | -91.8 +TT | -90.83 +TT |  |  |  |  | TDD |
|  | 30 |  | -98.3 +TT | -96.3 +TT | -95.1 +TT |  |  | -91.9 +TT | -90.93 +TT | -90.13 +TT | -88.83 +TT | -88.33 +TT | -87.83 +TT |  |
|  | 60 |  | -98.7 +TT | -96.6 +TT | -95.3 +TT |  |  | -92.1 +TT | -91.03 +TT | -90.23 +TT | -88.93 +TT | -88.43 +TT | -87.93 +TT |  |
| n66 | 15 | -102.2 +TT | -99.0 +TT | -97.2 +TT | -96.0 +TT | -94.9 +TT | -94.1 +TT | -92.8 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -99.3 +TT | -97.3 +TT | -96.2 +TT | -95.0 +TT | -94.2 +TT | -92.9 +TT |  |  |  |  |  |  |
|  | 60 |  | -99.7 +TT | -97.6 +TT | -96.4 +TT | -95.2 +TT | -94.3 +TT | -93.1 +TT |  |  |  |  |  |  |
| n70 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT |  |  |  |  |  |  |  |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT |  |  |  |  |  |  |  |  |
| n774 | 15 |  | -97.5 +TT | -95.7 +TT | -94.4 +TT |  |  | -91.3 +TT | -90.3 +TT |  |  |  |  | TDD |
|  | 30 |  | -97.8 +TT | -95.8 +TT | -94.6 +TT |  |  | -91.4 +TT | -90.4 +TT | -89.6 +TT | -88.3 +TT | -87.8 +TT | -87.3 +TT |  |
|  | 60 | - | -98.2 +TT | -96.1 +TT | -94.8 +TT |  |  | -91.6 +TT | -90.5 +TT | -89.7 +TT | -88.4 +TT | -87.9 +TT | -87.4 +TT |  |
| n78 | 15 |  | -98.0 +TT | -96.2 +TT | -94.9 +TT |  |  | -91.8 +TT | -90.8 +TT |  |  |  |  | TDD |
|  | 30 |  | -98.3 +TT | -96.3 +TT | -95.1 +TT |  |  | -91.9 +TT | -90.9 +TT | -90.1 +TT | -88.8 +TT | -88.3 +TT | -87.8 +TT |  |
|  | 60 |  | -98.7 +TT | -96.6 +TT | -95.3 +TT |  |  | -92.1 +TT | -91.0 +TT | -90.2 +TT | -88.9 +TT | -88.4 +TT | -87.9 +TT |  |
| n79 | 15 |  |  |  |  |  |  | -91.8 +TT | -90.8 +TT |  |  |  |  | TDD |
|  | 30 |  |  |  |  |  |  | -91.9 +TT | -90.9 +TT | -90.1 +TT | -88.8 +TT |  | -87.8 +TT |  |
|  | 60 |  |  |  |  |  |  | -92.1 +TT | -91.0 +TT | -90.2 +TT | -88.9 +TT |  | -87.9 +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for above listed operating band except for two Rx vehicular UE. NOTE 2 The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 3: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3C.2.5-0. NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE that supports any of the SUL operation given in Table 7.3C.0.2-2, exceptions to the requirements specified in Table 7.3C.2.5-1 or Table 7.3C.2.5-2 are allowed when the uplink is active in a lower frequency band and is within a specified frequency range such that transmitter harmonics fall within the downlink transmission bandwidth assigned in a higher band as noted in Table 7.3C.0.2-2. For these exceptions, the UE shall meet the requirements specified in clause 7.3C.2.5.1.

7.3C.2.5.1	Reference sensitivity exceptions due to harmonic issue

For SUL operation with DL band listed in Table 7.3C.0.2.3-2 with supplementary uplink transmission bandwidth less than or equal to that specified in Table 7.3C.0.2.3-1, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in Table 7.3C.2.5.1-1 due to harmonic exceptions.

Table 7.3C.2.5.1-1: Reference sensitivity for SUL operation (exceptions due to harmonic issue)

| SUL band |  | DL band |  | Test ID |  |  | SCS |  |  | Downlink Channel Bandwidth / REFSENS requirement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | 10 MHz |  |  | 15 MHz |  |  | 20 MHz |  |  |  | 25 MHz |  |  | 30 MHz |  |  | 40 MHz |  |  | 50 MHz |  |  | 60 MHz |  |  | 80 MHz |  |  | 90 MHz |  |  | 100 MHz |
|  |  |  |  |  |  | kHz |  |  | dBm |  |  |  | dBm |  |  | dBm |  |  | dBm |  |  |  | dBm |  |  | dBm |  |  | dBm |  |  | dBm |  |  | dBm |  |  | dBm |  |  | dBm |  |  | dBm |
| n80 |  | n781 |  | 1 |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 +13.8 +TT |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -85.6 -2.2 +13.8 +TT3 |  |
|  |  |  |  | 2 |  |  | 15 |  |  |  |  |  |  |  |  |  |  | -92.7 +0.3 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -92.7 -2.2 +0.3 +TT3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2C.1 NOTE 3: Applicable only if operation with 4 antenna ports is supported in the band with SUL configured |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3C.2.5.1-2: Void

For the UE which supports SUL band combination, the test requirement for reference sensitivity in Tables 7.3C.2.5-1, 7.3C.2.5-2 and 7.3C.2.5.1-1shall be increased by the amount given in ΔRIB,c defined in subclause 7.3C.0.3.

<End of modified section 1>