3GPP TSG-RAN5 Meeting #90-e	R5-210948r1

# Electronic Meeting, 22nd February – 5th March 2021

Title:	Discussion on the uplink power configuration for EN-DC RF cases

Source:	Huawei, Hisilicon

Agenda item:	5.3.2.17

Document for:	Endorsement

Introduction

For EN-DC Tx test cases, UEs need to be configured a maximum power PLTE for transmissions on the MCG by p-MaxEUTRA and a maximum power PNR for transmissions in FR1 on the SCG by p-NR-FR1. In TS 38.521-3, there are a lot of EN-DC Tx test cases with LTE anchor agnostic applied, where the value of PLTE and PNR follows the default configuration of TS 38.508-1. However, the default value of PLTE and PNR may not be appropriate for critical testing.

The purpose of this document is to derive the appropriate value of PLTE and PNR for EN-DC Tx test cases applying LTE anchor agnostic to acheive critical testing.

Background

According to TS 38.101-3 clause 6.2B.4.1, for NR carrier in EN-DC

The configured maximum output power PCMAX,f,c,NR (q) in physical channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified  as follows:

PCMAX_L,f,c,,NR = MIN {MIN(PEMAX,c , PEMAX, EN-DC, PNR) - TC_NR, c, (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ),  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MAX(MPRc,A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC, PNR, PPowerClass, EN-DC, PPowerClass,NR – ΔPPowerClass,NR }

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8];

-	PLTE signalled by RRC as p-MaxEUTRA-r15 in TS 36.331 [8]

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in  [9] and signalled by RRC;

Except for power class and MPR, PCMAX_L,f,c,,NR is limited by PEMAX,c, PEMAX, EN-DC, and PNR, where PEMAX,c is the value given by the p-Max IE as defined in TS 38.331. From 38.331, p-Max is used to indicate the maximum transmitted power allowed in this serving cell and p-NR-FR1 (PNR) is used to indicate the maximum total power for NR cell group. Given that uplink EN-DC configuration defined in 38.101-3 only include at most 1 NR cell, actually p-Max and p-NR-FR1 have the same indication of the maximum transmitted power for NR carrier, which is the reason that both p-Max and p-NR-FR1 refer to the value P-Max in Table 4.6.3-89 in TS 38.508-1. Consequently, unless stated in 38.521-3 test cases, the maximum transmitted power for NR carrier is determined by the value P-Max in Table 4.6.3-89.

Table 4.6.3-89: P-Max

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | FR1 AND pc_dynamicPowerSharing |
|  | 23 |  | FR1_RF_PC2_Testing_PC3 |
|  | Not present |  | FR1_RF_PC3 |
|  | Not present |  | FR1_RF_PC2 |
|  | 26 |  | FR2 AND pc_dynamicPowerSharing |
|  | 20 | P-Max value when pc_dynamicPowerSharing is set to FALSE | NOT pc_dynamicPowerSharing |

LTE anchor agnostic applied

The value of PNR

For EN-DC Tx test cases with LTE anchor agnostic applied, the maximum transmitted power for NR carrier follows P-Max in TS 38.508-1 since there are no message contents exception in test cases. According to Table 4.6.3-89, the value of PNR is set to 23 dBm for PC3 UEs supporting DPS while PNR is set to 20 dBm for PC3 UEs not supporting DPS, which is to ensure that the measured maximum power would not exceed the PTotalEN-DC as defined in 6.2B.4 in 38.101-3.

However, for non-DPS UE the maximum transmitted power for the NR carrier is limited to 20 dBm, which may not be critical enough for EN-DC Tx testing since the active uplink carrier is always expected to achieve as higher transmitted power as possible.

According to 38.101-3,

If the UE does not support dynamic power sharing,

PTotalEN-DC = MIN { PEMAX, EN-DC , PPowerClass, EN-DC - ΔPPowerClass,EN-DC } + 0.3 dB

Observation 1: For PC3 UEs not supporting DPS, the value of PTotalEN-DC is 23.3 dBm.

Actually, when LTE anchor agnostic is applied, only NR carrier is used for the uplink transmission while LTE carrier is used for signaling and setting up the connection. After the connection is set up, the transmitted power on the LTE carrier is OFF Power due to no uplink transmission in MCG. Therefore, if the value of PNR is set to 23 dBm, the NR carrier can achieve higher transmitted power and the total measured power would not exceed PTotalEN-DC as in Observation 1.

Proposal 1: In EN-DC RF test cases with LTE anchor agnostic applied, PNR shall be set to the supported power class so that the active NR carrier could achieve the maximum transmitted power.

The value of PLTE

According to TS 38.213 clause 7.6.1, for UEs configured with EN-DC the mechanism of uplink power control is as follows:

If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power  for transmissions on the MCG by p-MaxEUTRA and a maximum power  for transmissions in FR1 on the SCG by p-NR-FR1.

…

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows.

-	If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213])

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE does not transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability tdm-Pattern-dualTx in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and

-	if UE transmission(s) in subframe  of the MCG overlap in time with UE transmission(s) in slot  of the SCG in FR1, and

-	if  in any portion of slot  of the SCG,

the UE reduces transmission power in any portion of slot  of the SCG so that  in any portion of slot , where  and  are the linear values of the total UE transmission powers in subframe  of the MCG and in slot  of the SCG in FR1, respectively. The UE is not required to transmit in any portion of slot  of the SCG if  would need to be reduced by more than the value provided by XSCALE in order for  in any portion of slot  of the SCG. The UE is required to transmit in slot  of the SCG if  would not need to be reduced by more than the value provided by XSCALE in order for  in all portions of slot .

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE expects to be configured with reference TDD configuration for E-UTRA by tdm-PatternConfig-r15 in [13, TS 36.213].

Observation 2: For UEs in EN-DC mode, PLTE and PNR shall be configured to indicate the maximum transmission power on MCG and SCG, respectively.

Observation 3: If UE is configured with , non-DPS UEs shall be configured with TDM pattern for limiting the total UE transmission power.

Based on PNR = 23 dBm for PC3 UEs in Proposal 1, regarding how to configure PLTE there are two options:

- Option 1: PLTE is set to 23 dBm for PC3 UE, which could guarantee the more stable connection but TDM pattern needs to be configured for non-DPS UE since  is met.

- Option 2: PLTE is set to 10 dBm for PC3 UE as in Table 2.1.2-1, which can make  not hold so that TDM pattern is not needed for non-DPS UEs.

Table 2.1.2-1: PLTE in Option 2

| Power Class | PtotalEN-DC (dBm) | PNR(dBm) | PLTE(dBm) |
| --- | --- | --- | --- |
| PC3 | 23.3 | 23 | 10 |
| PC2 | 26.3 | 26 | 13 |

Given that some EN-DC test cases have been validated, Option 1 may lead to the re-validation of those TCs. Hence, Option 2 is more preferred than Option 1.

Proposal 2: In EN-DC RF Tx test cases that LTE anchor agnostic applied, it is suggested to set PLTE as 10 dBm for PC3 and 13 dBm for PC2 to ensure that TDM pattern configuration is not needed.

Proposal

For EN-DC Tx test cases, it is proposed RAN5 consider below proposals:

Proposal 1: In EN-DC RF test cases with LTE anchor agnostic applied, PNR shall be set to the supported power class so that the active NR carrier could achieve the maximum transmitted power.

Proposal 2: In EN-DC RF Tx test cases that LTE anchor agnostic applied, it is suggested to set PLTE as 10 dBm for PC3 and 13 dBm for PC2 to ensure that TDM pattern configuration is not needed.

References

[1] 3GPP TS 38.213: “NR; Physical layer procedures for control”.

[2] 3GPP TS 38.508-1 “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”.

[3] 3GPP TS 38.521-3 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 16)”.