3GPP TSG-RAN5 Meeting #90-e	R5-210949r1

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1758 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating the value of P-Max for EN-DC and NR SA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In 38.508, the value of P-Max in Table 4.6.3-89 indicates the maximum transmitted power allowed in the NR cell for both NR SA and EN-DC cases. Due to the fact that SIG and RF test cases need different configurations of P-Max, for easier reading it is considered to move the P-Max setting for RF cases to clause 5. Moreover, P-Max for EN-DC RF test cases needs to be updated for critical testing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update Table 4.6.3-89 to make P-Max configured as SIG needs. - Update the value of P-Max for EN-DC RF cases to the supported power class so that the active NR uplink carrier could achieve the maximum transmitted power when LTE anchor agnostic applied, which is based on the proposal in related discussion. - Delete the condition PC2 testing PC3 since it has been configured in 38. 521-3 test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The value of PNR would not be optimal for EN-DC and NR SA test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3, 5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Given that Power Class is not specified in SIG test cases, the conditions in Table 4.6.3-89 are corrected to delete power class information. - The conditions in RF tables are slightly corrected to look neater. |  |  |  |  |  |  |  |  |

<Start of Changes>

–	P-Max

Table 4.6.3-89: P-Max

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | SA OR (EN-DC AND pc_dynamicPowerSharing) |
|  | 20 |  | EN-DC AND (NOT pc_dynamicPowerSharing) |

| Condition | Explanation |
| --- | --- |
| SA | NR standalone test cases |
| EN-DC | E-UTRA-NR Dual Connectivity |

<Unchanged Text Skipped>

5.4.1	Radio resource control information elements

As defined in clause 4.6.3 with the following exceptions:

For Tx test cases in which Power Class 3 UEs verifying Power Class 3 requirements, refer to Table 5.4.1-1; For Tx test cases in which Power Class 2 UEs verifying Power Class 2 requirements, refer to Table 5.4.1-2.

Table 5.4.1-1: P-Max-PC3

| Derivation Path: Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | Not present |  | SA |
|  | 23 |  | EN-DC |

Table 5.4.1-2: P-Max-PC2

| Derivation Path: Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | Not present |  | SA |
|  | 26 |  | EN-DC |

| Condition | Explanation |
| --- | --- |
| SA | NR standalone RF tests specified in 38.521-1 |
| EN-DC | EN-DC RF tests specified in 38.521-3 |

Table 5.4.1-3: Void

<End of Changes>