3GPP TSG-RAN5 Meeting #90-e	R5-210961r1

# Electronic Meeting, 22nd Feb – 5th Mar 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1115 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Netherlands B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Recently concluded RAN4 added TRS Configuration to this test |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added TRS Configuration to Table 6.6.3.2.5-1 - Below Table 6.6.3.2.4.3-1, there was a text “FFS” which I deleted since the test is marked as 100% complete in WP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test case 6.6.3.2 will not be in sync with core RAN4 Spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Reference RAN4 Tdoc is R4-2104086 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updated Table 6.6.3.2.5-1 with TRS Config instead of deleting and adding new Table. |  |  |  |  |  |  |  |  |

<<Start of Changes>>

#### 6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX

6.6.3.2.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used under the cell search and measurement requirements.

6.6.3.2.2	Test applicability

This test applies to all types of NR UE supporting SA FR1 from Release 15 onwards.

6.6.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 6.6.3.0.1 and 6.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.3.2.

6.6.3.2.4	Test description

6.6.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.3.2.4.1-1.

Table 6.6.3.2.4.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Test Case ID | Description |
| --- | --- |
| 6.6.3.2-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.3.2-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.2-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.2-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.3.2-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.3.2-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.3.2.4.1-2 and Table 6.6.3.2.4.1-3.

Table 6.6.3.2.4.1-2: Initial conditions for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.3.2.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1 |  |  | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 |  |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables 6.6.3.2.5-1 and 6.6.3.2.5-2. |  |  |  |
| Active cell |  | Cell 1 |  |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  |  | As specified in Clause TS 38.133 [6] Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP |  |  | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP |  |  | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | note 1 |  |  | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -97 |  |  | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |  |  |
| TimeToTrigger | s | 0 |  |  |  |
| Filter coefficient |  | 0 |  |  | L3 filtering is not used |
| DRX |  | DRX.1 |  | DRX.2 | DRX cycle configurations DRX.1 and DRX.2 are defined in Table A.3.3.1-1 and Table A.3.3.2-1 respectively. |
| T1 | s | 5 |  |  |  |
| T2 | s | 5 | 15 |  |  |
| NOTE 1: Values are defined in Table 6.6.3.2.5-1 |  |  |  |  |  |

1.	Message contents are defined in clause 6.6.3.2.4.3.

2.	There are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

6.6.3.2.4.2	Test procedure

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

In each test the UE shall be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore the UE shall be allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.3.2.5-1 and 6.6.3.2.5-2. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Tables 6.6.3.2.5-1 and 6.6.3.2.5-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than 3.48 s for Test 1 or less than 12.8 s for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off .

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.3.2.4.3-1: Common Exception messages NR SA FR1 – E-UTRAN event-triggered reporting in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-RAT Table H.3.1-3 with Condition SSB.1 FR1 and Asynchronous cells cells for configuration 6.6.3.2-1, 6.6.3.2-2, 6.6.3.2-4, 6.6.3.2-5 Table H.3.1-3 with Condition SSB.1 FR2 and Asynchronous cells cells for configuration 6.6.2.2-3, 6.6.3.2-6 Table H.3.1-3A Table H.3.1-4A Table H.3.1-5 with Condition INTER-RAT Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-RAT Table H.3.7-1 with Condition DRX.1 for Test 1 Table H.3.7-1 with Condition DRX.2 for Test 2 |

6.6.3.2.5	Test requirement

Table 6.6.3.2.5-1 and Table 6.6.3.2.5-2 defines the primary level settings including test tolerances for all tests.

Table 6.6.3.2.5-1: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |
|  |  |  |  | 3, 6 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |
|  |  |  |  | 3, 6 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 19.65 | -3.65 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 19.65 | -3.65 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.35 | -109.65 |
|  |  |  |  | 3, 6 | -83.35 | -106.65 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.35 | -109.65 |
|  |  |  |  | 3, 6 | -83.35 | -106.65 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -58.35 | -76.49 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -52.25 | -70.39 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | TDLA30 |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.3.2.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 20.65 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 20.65 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.35 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.35 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -78.22+10log (NRB,c /50) | -57.53+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

In test 1, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

In test 2, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 12.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

<<End of Changes>>