3GPP TSG-RAN5 Meeting #90-e	R5-211012r1

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0897 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of reference sensitivity for inter-band 2CC EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The initial condition for E-UTRA carrier incorrectly referred to LTE anchor agnostic, which is not applicable for EN-DC configs affected by exceptions. - In Table 7.3B.2.3.4.1-6, for DC_3A_n1A and DC_40A_n78A the UL RB allocation is incorrect since NR band n1 and n78 are aggressor bands. - The test procedure only applies to Table 7.3B.2.3.4.1-0 EN-DC with non-exception without taking Table 7.3B.2.3.4.1-6 EN-DC with exceptions into account. - The test requirements for LTE victim band was added with TT. - UL harmonic exception between n28 and n75 couldn’t be tested in 2CC test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correcting the initial condition for E-UTRA carrier to refer to Table 7.3B.2.3.4.1-0 and Table 7.3B.2.3.4.1-6. - Correct the UL RB allocation to make NR band as the aggressor for DC_3A_n1A and DC_40A_n78A. - Updating the test procedure to cover both Table 7.3B.2.3.4.1-0 and Table 7.3B.2.3.4.1-6. - Deleting “+TT” for LTE victim band in 7.3B.2.3.5. - Deleting the row “n28 and n75” in Table 7.3B.2.3.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The refsens test case for inter-band EN-DC would remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Cancelling changes that overlap with R5-211137. |  |  |  |  |  |  |  |  |

<Start of Changes>

7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.2B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 for E-UTRA RMC respectively. The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3.4.2.1-0: Test configurations table for EN-DC configuration without any reference sensitivity exception

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
|  | NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Mid range |  |  |  |
|  | Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
|  | Network signalling value |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nYA Configuration (Inter-band EN-DC) – Note 2 |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | N/A | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| Note 1: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. Note 2: LTE anchor agnostic configuration |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-1: Initial test conditions for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 38.508-1 [11] clause4.3.1 |  |  |  | Mid range for E-UTRA and NR, unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28 |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Highest NRB_agg, unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28 |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest supported SCS otherwise specified |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| E-UTRA Cell |  | NR Cell |  | E-UTRA Cell |  | NR Cell |  |
| Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation |
| QPSK | Full RB | CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_1: Test configurations table for exceptions due to UL harmonic interference for EN-DC 1_n77

| E-UTRA Band 1 |  |  |  | NR Band n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 1925 MHz/ 18050 | 25@12 | 3850.02 MHz/ 656668 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 1925 MHz/ 18050 | 50@0 | 3850.02 MHz/ 656668 | Mid Highest | REFSENS (NOTE 2) |
| 3 | 10 | 1950 MHz/ 18300 | 25@12 | 3900 MHz/ 660000 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 1950 MHz/ 18300 | 50@0 | 3900 MHz/ 660000 | Mid Highest | REFSENS (NOTE 2) |
| 5 |  | 1975 MHz/ 18550 | 25@12 | 3950.01 MHz/ 663334 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 1975 MHz/ 18550 | 50@0 | 3950.01 MHz/ 663334 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 2 and 13 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

<Unchanged Text Skipped>

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  | NRB |  |  | Band |  |  |  | Range |  | NRB |  |  |
|  | UL MOD | DL MOD |  | CH BW |  | DLalloc / UL alloc | UL MOD |  |  |  | DL MOD |  | UL/DL Ch BW | DLalloc / UL alloc |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  | FFS |  |  |  |  |  |  |  |  |
| 24 | 1 | DL 2140 |  |  |  |  | n78 |  |  |  | 3710 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 3 | Mid |  |  |  |  | n1 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_NR |  |
| 26 | 3 | Mid |  |  |  |  | n1 |  |  |  | Mid |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |
| 34 | 3 | DL 1855 |  |  |  |  | n1 |  |  |  | 2140 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 | Low |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |
| 26 | 3 | Mid |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |
| 36 | 3 | High |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| 46 | 3 | High |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| 54 | 3 | DL 1835 |  |  |  |  | n41 |  |  |  | 2657.5 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3495 |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |
| 23 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3525 |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |
| 35 | 3 | High |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |
| 44 | 3 | DL 1835 |  |  |  |  | n78 |  |  |  | 3575 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 54 | 3 | DL 1860 |  |  |  |  | n78 |  |  |  | 3435 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 20 | Mid |  |  |  |  | n3 |  |  |  | High |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / 0 | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_NR |  |
| 24 | 20 | DL 799 |  |  |  |  | n3 |  |  |  | 1870 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 34 | 20 | DL 806 |  |  |  |  | n3 |  |  |  | 1830 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  | FFS |  |  |  |  |  |  |  |  |
| 23 | 20 | Mid |  |  |  |  | n78 |  |  |  | 3388 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / 0 |  |
| 34 | 20 | DL 809 MHz |  |  |  |  | n78 |  |  |  | 3359 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 | Mid |  |  |  |  |  | n78 |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz |  | All RBs / REFSENS_ENDC_2 |  |  | N/A |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation according to table 7.3B.2.0.3.5.1-1. NOTE 2: Test ID with no exception NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Test ID for UE supporting single UL. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

7.3B.2.3.4.2.2	Test procedure

For test points in Table 7.3B.2.3.4.2.1-0:

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-0 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-0 Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8], Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

For test points in Table 7.3B.2.3.4.2.1-6:

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level for the E-UTRA CC and NR CC to the appropriate REFSENS value defined in Tables 7.3B.2.3.5-1 to 7.3B.2.3.5-4. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

7.3B.2.3.4.2.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6.

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations, the throughput of each CG shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with reference receive power level specified in Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

Reference sensitivity test requirements for EN-DC configurations without any reference sensitivity exception are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC with uplink configuration specified in Table 7.3B.2.3.4.2.1-0.

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable for two, three, four and five bands operation.

Table 7.3B.2.3.5-1: Reference sensitivity due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n771,2 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 2 | n781,2 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT |  |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT |  |  |  |  |  |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT |  |  |  |  |  |
| 2 | n783 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 3 | n781,2 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| 5 | n78 | 15 |  | -84.2 +TT | -84.4 +TT |  |  |  | -84.2 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -84.3 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.3 +TT | -84.5 +TT | -84.4 +TT | -84.3 +TT | -84.0 +TT | -84.2 +TT |
|  |  | 60 |  | -84.5 +TT | -84.6 +TT | -84.5 +TT |  |  | -84.5 +TT | -84.6 +TT | -84.5 +TT | -84.4 +TT | -84.1 +TT | -84.3 +TT |
| 8 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -84.0 +TT | -83.9 +TT |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.8 +TT | -83.5 +TT | -83.7 +TT |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -84.3 +TT | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.6 +TT | -83.8 +TT |
| 8 | n41 | 15 |  | -81.8 +TT | -81.7 +TT | -81.7 +TT |  |  | -81.6 +TT | -81.5 +TT |  |  |  |  |
|  |  | 30 |  | -82.1 +TT | -81.8 +TT | -81.9 +TT |  |  | -81.7 +TT | -81.6 +TT | -81.4 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
|  |  | 60 |  | -82.5 +TT | -82.1 +TT | -81.1 +TT |  |  | -81.9 +TT | -81.7 +TT | -81.6 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
| 8 | n794,5 | 15 |  |  |  |  |  |  | -82.8 +TT | -82.4 +TT |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  | -82.9 +TT | -82.5 +TT | -82.3 +TT | -81.7 +TT |  | -81.2 +TT |
|  |  | 60 |  |  |  |  |  |  | -83.1 +TT | -82.6 +TT | -82.4 +TT | -81.8 +TT |  | -81.3 +TT |
| 12 | n66 | 15 | -89.5 +TT | -88.8 +TT | -88.3 +TT | -87.8 +TT |  |  | -87.7 +TT |  |  |  |  |  |
|  |  | 30 |  | -89.1 +TT | -88.4 +TT | -88.0 +TT |  |  | -87.8 +TT |  |  |  |  |  |
|  |  | 60 |  | -89.5 +TT | -88.7 +TT | -88.2 +TT |  |  | -87.9 +TT |  |  |  |  |  |
| 12 | n78 | 15 |  | -85.4 +TT | -85.1 +TT | -84.9 +TT |  |  | -84.9 +TT | -84.9 +TT |  |  |  |  |
|  |  | 30 |  | -85.7 +TT | -85.2 +TT | -85.1 +TT |  |  | -85.0 +TT | -85.0 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT |
|  |  | 60 |  | -86.1 +TT | -85.5 +TT | -85.3 +TT |  |  | -85.2 +TT | -85.1 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT |
| 18， 19 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT |  | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT |  | -84.5 +TT |
| 28 | n51 | 15 | -72.2 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n774,5 n784,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT |  | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT |  | -84.5 +TT |
| 20 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -83.1 +TT |  |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -83.2 +TT |  |  |  |  |  |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -83.4 +TT |  |  |  |  |  |
| 26 | n41 | 15 |  | -84.5 +TT | -84.6 +TT | -84.4 +TT |  |  | -83.6 +TT | -83.3 +TT | 3.9 +TT | 3.1 +TT | 2.7 +TT |  |
|  |  | 30 |  | -84.8 +TT | -84.7 +TT | -84.6 +TT |  |  | -83.7 +TT | -83.4 +TT | -83.0 +TT | -82.5 +TT | -82.4 +TT |  |
|  |  | 60 |  | -85.2 +TT | -85.0 +TT | -84.8 +TT |  |  | -83.9 +TT | -83.5 +TT | -83.2 +TT | -82.5 +TT | -82.4 +TT |  |
| 26 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -83.1 +TT |  |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -83.2 +TT |  |  |  |  |  |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -83.4 +TT |  |  |  |  |  |
| 26 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -85.6 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -85.7 +TT | -84.5 +TT |
| n28 | 18,9,10 | N/A | -89.1 | -88.7 | -88.3 | -88.0 |  |  |  |  |  |  |  |  |
| n71 | 211 | N/A | -92.7 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 212 | N/A | -95.6 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 7 | N/A | -82.7 | -82.6 | -82.4 | -82.3 |  |  |  |  |  |  |  |  |
| 66 | n781,2 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT |  |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT |  |  |  |  |  |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT |  |  |  |  |  |
|  | n783 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| n66 | 48,2, 13 | N/A | -70.7 | -70.6 | -70.8 | -70.8 |  |  |  |  |  |  |  |  |
| n66 | 48,3 | N/A | -96.1 | -93.6 | -92.3 | -91.6 |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n77, DC_66_n77, DC_3_n77 and DC_3_n78. NOTE 14: TT is the same as defined in Table 7.3B.2.3.5-1a. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-1a: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-2 with uplink configuration specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k for each specific EN-DC combination scenario.

Table 7.3B.2.3.5-2: Reference sensitivity due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n714 | 15 | -70.4 +TT | -70.4 +TT | -70.4 +TT | -70.4 +TT |  |  |  |  |  |  |  |
|  |  | 30 |  | -70.7 +TT | -70.7 +TT | -71.8 +TT |  |  |  |  |  |  |  |
|  |  | 60 |  | -72.4 +TT | -72.7 +TT | -77.0 +TT |  |  |  |  |  |  |  |
| n41 | 264 | N/A | -72.5 | -69.5 | -69.5 | N/A |  |  |  |  |  |  |  |
| n77 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 282 | N/A | -69.8 | -69.8 | -69.8 | -68.3 |  |  |  |  |  |  |  |
| n78 | 3 | N/A | -91.3 | -89.8 | -89.0 | -88.1 |  |  |  |  |  |  |  |
| n78 | 40 | N/A | -86.9 | -83.9 | -82.1 | -80.9 |  |  |  |  |  |  |  |
| n78 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n79 | 192 | N/A | -69.8 | -69.8 | -69.8 |  |  |  |  |  |  |  |  |
| n79 | 262 | N/A | -69.8 | -69.8 | -69.8 | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-3 with uplink configuration specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n.

Table 7.3B.2.3.5-3: Reference sensitivity exceptions due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 1 | n40 | 15 | -93.4 +TT | -90.2 +TT | -88.4 +TT | -87.2 +TT | -86.1 +TT | -85.3 +TT | -84.0 +TT |  |  |  |  |
| n40 | 1 | NA | -91.0 | -88.0 | -86.2 | -85.0 |  |  |  |  |  |  |  |
| n41 | 2 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |
| n41 | 25 | N/A | -95.2 | -92.2 | -90.4 | -89.2 |  |  |  |  |  |  |  |
| n77 | 41 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |
| n41 | 66 | N/A | -95.3 | -92.3 | -90.5 | -89.3 |  |  |  |  |  |  |  |
| 41 | n77 | 30 |  | -87.3 +TT | -85.3 +TT | -84.1 +TT |  | -82.9 +TT | -82.9 +TT | -82.9 +TT | -82.1 +TT | -81.7 +TT | -81.3 +TT |
| 3 | n51 | 15 | -93.6 +TT |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 15 | -91.2 +TT | -88.0 +TT | -86.2 +TT | -85.0 +TT |  |  | -81.8 +TT |  |  |  |  |
| n78 | 71 | N/A | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |
| n78 | 38 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |
| n78 | 411 | N/A | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |
| 41 | n78 | 30 |  | -87.8 +TT | -85.8 +TT | -84.6 +TT |  | -83.4 +TT | -83.4 +TT | -83.4 +TT | -82.6 +TT | -82.2 +TT | -81.8 +TT |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-3a: Reference sensitivity exceptions due to cross band isolation for PC2 EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -90.0 +TT | -88.0 +TT | -87.0 +TT | -86.2 +TT | -84.9 +TT | -84.4 +TT | -84.0 +TT |
| n41 | 3 | N/A | -94.0 | -91.0 | -89.2 | -88.0 |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-4 with uplink configuration specified in Table 7.3B.2.3.4.2.1-6.

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n5A | 1 | N/A | -93.3 | - | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS |  | - | - | - | N/A | FDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -91.3 | - | - | - | - | IMD4- | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | -91.3 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n66A | 2 | N/A | -77.3 | - | - | - | - | IMD3 |  |
|  | n66 | 15 | REFSENS | - | - | - | - | N/A |  |
| DC_2A_n66A | 2 | N/A | REFSENS | - | - | - | - | N/A |  |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 |  |
| DC_2A_n78A | 2 | N/A | -71.3 | - | - | - | - | IMD23 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_2A_n78A | 2 | N/A | -89.3 | - | - | - | - | N/A |  |
|  | n78 | 15 | - | REFSENS | - | - | - | IMD43 | TDD |
| DC_3A_n1A | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -74.0+TT | - | - | - | - | IMD3 | FDD |
| DC_3A_n5A | 3 | N/A |  | -89.3 | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -74.0+TT | - | - | - | - | IMD23 | FDD |
| DC_3A_n7A | 3 | N/A | REFSENS | - | - | - | - | - |  |
|  | n7 | 15 | - | -84.6 +TT5 | - | - | - | - |  |
| CA_3A-n20A | 3 | N/A | -92.3 | - | - | - | - | IMD4 | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | -88.0+TT | - | - | - | - | IMD4 | FDD |
| DC_3A_n41A | 3 | N/A | -88.1 | - | - | - | - | IMD4 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | IMD4 | TDD |
| DC_3A_n77A, DC_3A_n78A, DC_3A-SUL_n78A-n80A, DC_3C_n78A | 3 | N/A | -70.3 | - | - | - | - | IMD23 |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_3A_n77A, DC_3A_n78A, DC_3A-SUL_n78A-n80A, DC_3C_n78A | 3 | N/A | -88.3 | - | - | - | - | IMD43 |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n78A | 5 | N/A | -89.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_7A_n5A | 7 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -86.0+TT | - | - | - | - | IMD33 | FDD |
| DC_7A_n66A | 7 | N/A | - | -79.3 | - | - | - | IMD4 | FDD |
|  | n66 | 15 | REFSENS | - |  | - | - | N/A | TDD |
| DC_7A_n77A | 7 | N/A | -90.2 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n1A | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -94.0 + TT | - | - | -- | - | IMD4 | FDD |
| DC_8A_n3A | 8 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n3 | 15 | - | REFSENS | - | - | - | N/A | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | - | -87.4+ TT | - | - | - | IMD5 | FDD |
| DC_8A_n41A | 8 | N/A | -84.2 | - | - | - | - | IMD33 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | N/A | -88.0 | - | - | - | - | IMD4 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | H4 | TDD |
| DC_8A_n79A DC_8A-SUL_n79A-n81A | 8 | N/A | -91.5 | - | - | - | - | IMD5 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_12_n78 | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n3A | 20 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | -93.0 +TT | - | - | - | - | IMD4 | FDD |
|  | 20 | N/A | -87.3 | - | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20A_n77A | 20 | N/A | -85.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 20 | N/A | -89.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | N/A | -74.6 | - | - | - | - | IMD44 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n79A | 21 | N/A | -80.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_28A_n41A | 28 | N/A | - | -82.1 | - | - | - | IMD3 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_28_n50 | 28 | N/A | - | -79.5 | - | - | - | IMD 2 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -88.8 | - | - | - | IMD 4 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -94.3 | - | - | - | IMD 5 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_48A_n66A | 48 | N/A | - | REFSENS | - | - | - | N/A | TDD |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 | FDD |
| DC_66A_n2A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n2 | 15 | -78.0+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n2 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n5A | 66 | N/A | -68.8 | - | - | - | - | IMD23 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n25A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n25 | 15 | -76.5+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -75.8 | - | - | - | - | IMD3 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n71A | 66 | N/A | -93.8 | - | - | - | - | IMD4 | FDD |
|  | n71 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n78A | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-2 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2_1.5-1 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-5: Reference sensitivity exceptions due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm)) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order) | Duplex mode |
| DC_3A_n41A | 3 | N/A | -77.9 | - | - | - | - | IMD43 |  |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -64.4 | - | - | - | - | IMD23 |  |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_3A_n78A | 3 | N/A | -77.8 | - | - | - | - | IMD43 |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-2 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2_1.5-1 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. |  |  |  |  |  |  |  |  |  |

<End of Changes>