3GPP TSG-RAN WG5 Meeting #90-e	R5-211043r1

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0115 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new IMS over 5GS TC 8.38 Communication Waiting and cancelling the call / 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Fulfilling the work plan by introducing new test case 8.38. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 8.38. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.38 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2 CR 0278 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-211043r1: 1. Changed style formatting of sub clauses. 2. Added applicability CR number in cover page. 3. Updated clause 8.38.2 to align with TS 24.615. 4. Modified Conditional PRACK representation and accordingly step numbering in Table 8.38.3.2-1. Also, modified ‘subclause’ to ‘Annex’ in Table 8.38.3.2-1. 5. Modified Derivation path in Table 8.38.3.3-1 from TS 34.229-1, A.2.6 to TS 34.229-5, Step 8 of Annex A.5.1 6. Added conditions to Tables 8.38.3.3-3 and 8.38.3.3-5. |  |  |  |  |  |  |  |  |

<Start of modified section>

## 8.37	FFS

## 8.38	Communication Waiting and cancelling the call / 5GS

### 8.38.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and having initiated an MO voice call with preconditions }

ensure that {

when { UE receives INVITE for MT voice call with preconditions }

then { UE continues voice call initiation until 180 Ringing (including conditional PRACK/200 OK) }

}

(2)

with { UE having continued initiation of incoming voice call until 180 Ringing }

ensure that {

when { UE receives CANCEL for incoming voice call }

then { UE responds with 200 OK and 487 Request Terminated responses }

}

### 8.38.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.615 clause 1]:

The Communication Waiting (CW) service enables a user to be informed, that very limited resources are available for an incoming communication. The user then has the choice of accepting, rejecting or ignoring the waiting call (as per basic call procedures).

[TS 24.615 clause 4.2.1]:

When a communication arrives at the destination user, the UE validates the status of the user. If the user is already involved in one or more communications, the terminal notifies the served user of a communication waiting situation.

[TS 24.615 clause 4.5.5.3.2]:

The UE shall insert an Alert-Info header field set to "<urn:alert:service:call-waiting>", specified in RFC 7462 [8] in the 180 (Ringing) response, in accordance with the provisional response procedures described in 3GPP TS 24.229.

[TS 24.615 clause 4.5.5.3.4]:

If user B's UE receives a CANCEL request or BYE request from User C during a CW condition, user B's UE shall:

-	stop timer TUE-CW (if necessary);

-	stop providing the CW indication to User B; and

-	apply the terminating UE procedures upon receipt of CANCEL or BYE as described in 3GPP TS 24.229.

### 8.38.3	Test description

#### 8.38.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

Preamble:

- UE is in state 1N-A, registered to IMS and has set up an MO call with preconditions, by executing the generic test procedure in Table 4.9.15.2.2-1 of TS 38.508-1.

#### 8.38.3.2	Test procedure sequence

Table 8.38.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-7 | MT Voice call setup takes place according to steps 1-7 of Annex A.5.1 | - | - | - | - |
| 8 | Step 8 of Annex A.5.1 happens | --> | 180 Ringing | 1 | P |
| 9 | Conditional Step: if UE sent 180 Ringing reliably, Step 9 of Annex A.5.1 happens | <-- | PRACK | - | - |
| 10 | Conditional Step: if UE sent 180 Ringing reliably, Step 10 of Annex A.5.1 happens | --> | 200 OK | - | - |
| 11 | SS sends CANCEL request to terminate INVITE transaction | <-- | CANCEL | - | - |
| 12 | UE acknowledges CANCEL with 200 OK | --> | 200 OK | - | - |
| 13 | The UE responds to INVITE with a 487 Request Terminated final response after transaction was terminated. | --> | 487 Request Terminated | 2 | P |
| 14 | SS acknowledges the receipt of 487 Request Terminated | <-- | ACK | - | - |

#### 8.38.3.3	Specific message contents

Table 8.38.3.3-1: 180 Ringing (step 8, table 8.38.3.2-1)

| Derivation path: Step 8 of Annex A.5.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Alert-Info |  | <urn:alert:service:call-waiting> |  | RFC 7462 [39] |

Table 8.38.3.3-2: CANCEL (step 11, table 8.38.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.2.15

Table 8.38.3.3-3: 200 OK (step 12, table 8.38.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.3.1 applying conditions A5 and A11

Table 8.38.3.3-4: 487 Request Terminated (step 13, table 8.38.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.2.16

Table 8.38.3.3-5: ACK (step 14, table 8.38.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.2.7 applying conditions A2 and A4

<End of modifed section>