3GPP TSG-RAN WG5 Meeting #90-e	R5-211151r1

# Electronic Meeting, 22nd February – 5th March, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0932 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of DC_41A_n77A and DC_41A_n78A in test case 7.3B.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | DC_41A_n77A and DC_41A_n78A test coverage is missing in EN-DC refsens test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | DC_41A_n77A and DC_41A_n78A added with test points according to TP analysis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test points |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR .38.905.. CR .0387.. |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-211151r1: - Removing unintended changes automatically added by Word - Correction of test frequency for band 41 - Removing the non-exception test point due to that the exception cannot be avoided with aggressor still active. Since the victim is LTE, NR can still be sufficiently tested even for NSA only UE with the HM and CBI test points |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

###### 7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations for E-UTRA consist of the test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1, with the exception that the E-UTRA channel bandwidth is the lowest supported value in Table 5.3B.1.3-1 for the EN-DC non-contiguous configuration under test.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2B.2.1-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing are shown in Tables 7.3B.2.3.4.2.1-0 in anchor agnostic mode and Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-5 for exceptional test cases in anchor specific mode. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [8] Annex C2.

In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3.4.2.1-0: Test configurations table for EN-DC configuration without any reference sensitivity exception

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
|  | NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Mid range |  |  |  |
|  | Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
|  | Network signalling value |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nYA Configuration (Inter-band EN-DC) – Note 2 |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | N/A | CP-OFDM QPSK | QPSK /CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| Note 1: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. Note 2: LTE anchor agnostic configuration |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-1: Initial test conditions for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 38.508-1 [11] clause4.3.1 |  |  |  | Mid range for E-UTRA and NR, unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28 |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Highest NRB_agg, unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28 |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest supported SCS otherwise specified |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| E-UTRA Cell |  | NR Cell |  | E-UTRA Cell |  | NR Cell |  |
| Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation |
| QPSK | Full RB | CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_1: Test configurations table for exceptions due to UL harmonic interference for EN-DC 1_n77

| E-UTRA Band 1 |  |  |  | NR Band n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 1925 MHz/ 18050 | 25@12 | 3850.02 MHz/ 656668 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 1925 MHz/ 18050 | 50@0 | 3850.02 MHz/ 656668 | Mid Highest | REFSENS (NOTE 2) |
| 3 | 10 | 1950 MHz/ 18300 | 25@12 | 3900 MHz/ 660000 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 1950 MHz/ 18300 | 50@0 | 3900 MHz/ 660000 | Mid Highest | REFSENS (NOTE 2) |
| 5 |  | 1975 MHz/ 18550 | 25@12 | 3950.01 MHz/ 663334 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 1975 MHz/ 18550 | 50@0 | 3950.01 MHz/ 663334 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 2 and 13 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_2: Test configurations table for exceptions due to UL harmonic interference for EN-DC 1_n77 (For Note 3 in Table 7.3B.2.0.3.1-1)

| E-UTRA Band 1 |  |  |  | NR Band n77 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) / NUL | UL allocation (LCRB) | NR FC (DL) (MHz) | NR FC (DL) NUL | NR CBW (MHz) | UL allocation (LCRB) |
|  |  | 1925MHz / | 25@12 | 3825.000 | 655000 | 10 | REFSENS |
| 1 |  | 18050 | 36@7 | 3822.495 | 654833 | 15 | (NOTE 2) |
|  |  |  | 50@0 | 3820.005 | 654667 | 20 |  |
|  | 10 | 1950 MHz / | 25@12 | 3874.995 | 658333 | 10 | REFSENS |
| 2 |  | 18300 | 36@7 | 3872.505 | 658167 | 15 | (NOTE 2) |
|  |  |  | 50@0 | 3870.000 | 658000 | 20 |  |
|  |  | 1975 MHz / | 25@12 | 3975.000 | 665000 | 10 | REFSENS |
| 3 |  | 18550 | 36@7 | 3977.505 | 665167 | 15 | (NOTE 2) |
|  |  |  | 50@0 | 3979.995 | 665333 | 20 |  |
| NOTE 1: Test frequencies are selected to fulfil Note 3 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Only NR channel bandwidths supported by the UE are tested. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_3: Test configurations table for exceptions due to UL harmonic interference for EN-DC 3_n77

| E-UTRA Band 3 |  |  |  | NR Band n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 1715 MHz/ 19250 | 25@12 | 3430.02 MHz/ 628668 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 1715 MHz/ 19250 | 50@0 | 3430.02 MHz/ 628668 | Mid Highest | REFSENS (NOTE 2) |
| 3 | 10 | 1747.5 MHz/ 19575 | 25@12 | 3495 MHz/ 633000 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 1747.5 MHz/ 19575 | 50@12 | 3495 MHz/ 633000 | Mid Highest | REFSENS (NOTE 2) |
| 5 |  | 1780 MHz/ 19900 | 25@12 | 3560.01 MHz/ 637334 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 1780 MHz/ 19900 | 50@12 | 3560.01 MHz/ 637334 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil 13 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_4: Test configurations table for exceptions due to UL harmonic interference for EN-DC 3_n77 (For Note 3 in Table 7.3B.2.0.3.1-1)

| E-UTRA Band 3 |  |  |  | NR Band n77 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) / NUL | UL allocation (LCRB) | NR FC (DL) (MHz) | NR FC (DL) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 1715 MHz / | 25@12 | 3405.000 | 627000 | 10 | REFSENS |
|  |  | 19250 | 36@7 | 3402.495 | 626833 | 15 | (NOTE 2) |
|  |  |  | 50@0 | 3400.005 | 626667 | 20 |  |
| 2 | 10 | 1747.5 MHz / | 25@12 | 3469.995 | 631333 | 10 | REFSENS |
|  |  | 19575 | 36@7 | 3467.505 | 631167 | 15 | (NOTE 2) |
|  |  |  | 50@0 | 3465.000 | 631000 | 20 |  |
| 3 |  | 1780 MHz / | 25@12 | 3585.000 | 639000 | 10 | REFSENS |
|  |  | 19900 | 36@7 | 3587.505 | 639167 | 15 | (NOTE 2) |
|  |  |  | 50@0 | 3589.995 | 639333 | 20 |  |
| NOTE 1: Test frequencies are selected to fulfil Note 3 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Only NR channel bandwidths supported by the UE are tested. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_5: Test configurations table for exceptions due to UL harmonic interference for EN-DC 2_n78

| E-UTRA Band 2 |  |  |  | NR Band n78 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 1855 MHz/ 18650 | 25@12 | 3710.01 MHz/ 647334 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 1855 MHz/ 18650 | 50@0 | 3710.01 MHz/ 647334 | Mid Highest | REFSENS (NOTE 2) |
| 3 | 10 | 1865 MHz/ 18750 | 25@12 | 3730.02 MHz/ 648668 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 1865 MHz/ 18750 | 50@0 | 3730.02 MHz/ 648668 | Mid Highest | REFSENS (NOTE 2) |
| 5 |  | 1875MHz/ 18850 | 25@12 | 3750 MHz/ 650000 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 1875 MHz/ 18850 | 50@0 | 3750 MHz/ 650000 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 2 and 13 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_6: Test configurations table for exceptions due to UL harmonic interference for EN-DC 2_n78 (for Note 3 in Table 7.3B.2.0.3.1-1)

| E-UTRA Band 2 |  |  |  |  | NR Band n78 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) | FC (UL) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) | NR FC (DL) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 1855 | 18650 | 25@12 | 3685.005 | 645667 | 10 | REFSENS |
|  |  |  |  | 36@7 | 3682.500 | 645500 | 15 | (NOTE 2) |
|  |  |  |  | 50@0 | 3679.995 | 645333 | 20 |  |
| 2 | 10 | 1867.5 | 18775 | 25@12 | 3709.995 | 647333 | 10 | REFSENS |
|  |  |  |  | 36@7 | 3707.505 | 647167 | 15 | (NOTE 2) |
|  |  |  |  | 50@0 | 3705.000 | 647000 | 20 |  |
| 3 |  | 1885 | 18950 | 25@12 | 3795.000 | 653000 | 10 | REFSENS |
|  |  | 1882.5 | 18925 | 36@7 | 3792.495 | 652833 | 15 | (NOTE 2) |
|  |  | 1879.9 | 18899 | 50@0 | 3789.795 | 652653 | 20 |  |
| NOTE 1: Test frequencies are selected to fulfil Note 3 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Only NR channel bandwidths supported by the UE are tested. |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_7: Test configurations table for exceptions due to UL harmonic interference for EN-DC 3_n78 (Test frequencies are selected to fulfil Requirement for Note 1 and 2 in Table 7.3B.2.0.3.1-1)

| E-UTRA Band 3 |  |  |  | NR Band n78 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 1715 MHz/ 19250 | 25@12 | 3430.02 MHz/ 628668 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 1715 MHz/ 19250 | 50@0 | 3430.02 MHz/ 628668 | Mid Highest | REFSENS (NOTE 2) |
| 3 | 10 | 1747.5 MHz/ 19575 | 25@12 | 3495 MHz/ 633000 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 1747.5 MHz/ 19575 | 50@0 | 3495 MHz/ 633000 | Mid Highest | REFSENS (NOTE 2) |
| 5 |  | 1780 MHz/ 19900 | 25@12 | 3560.01 MHz/ 637334 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 1780 MHz/ 19900 | 50@0 | 3560.01 MHz/ 637334 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 2 and 13 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_8: Test configurations table for exceptions due to UL harmonic interference for EN-DC 3_n78 (Test frequencies are selected to fulfil Requirement for Note 3 in Table 7.3B.2.0.3.1-1)

| E-UTRA Band 3 |  |  |  | NR Band n78 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) / NUL | UL allocation (LCRB) | NR FC (DL) (MHz) | NR FC (DL) NUL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 1715 MHz / | 25@12 | 3405.000 | 627000 | 10 | REFSENS |
|  |  | 19250 | 36@7 | 3402.495 | 626833 | 15 | (NOTE 2) |
|  |  |  | 50@0 | 3400.005 | 626667 | 20 |  |
| 2 |  | 1747.5 MHz / | 25@12 | 3469.995 | 631333 | 10 | REFSENS |
|  | 10 | 19575 | 36@7 | 3467.505 | 631167 | 15 | (NOTE 2) |
|  |  |  | 50@0 | 3465.000 | 631000 | 20 |  |
| 3 |  | 1780 MHz / | 25@12 | 3585.000 | 639000 | 10 | REFSENS |
|  |  | 19900 | 36@7 | 3587.505 | 639167 | 15 | (NOTE 2) |
|  |  |  | 50@0 | 3589.995 | 639333 | 20 |  |
| NOTE 1: Test frequencies are selected to fulfil Note 3 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Only NR channel bandwidths supported by the UE are tested. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_9: Test configurations table for exceptions due to UL harmonic interference for EN-DC 5_n78 (Test frequencies are selected to fulfil Requirement of Note 6 and 7 in Table 7.3B.2.0.3.1-1)

| E-UTRA Band 5 |  |  |  | NR Band n78 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 837.5 MHz/ 20535 | 16@17 | 3350.01 MHz/ 623334 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 837.5 MHz/ 20535 | 25@12 | 3350.01 MHz/ 623334 | 40 | REFSENS (NOTE 2) |
| 3 | 10 | 840.7 MHz/ 20567 | 16@17 | 3362.82 MHz/ 624188 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 840.7 MHz/ 20567 | 25@12 | 3362.82 MHz/ 624188 | 40 | REFSENS (NOTE 2) |
| 5 |  | 844 MHz/ 20600 | 16@17 | 3376.02 MHz/ 625068 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 844 MHz/ 20600 | 25@12 | 3376.02 MHz/ 625068 | 40 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 6 and 7 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_10: Test configurations table for exceptions due to UL harmonic interference for EN-DC 8_n41 (Test frequencies are selected to fulfil Requirement of Note 8 and 9 in Table 7.3B.2.0.3.1-1)

| E-UTRA Band 8 |  |  |  | NR Band n41 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 885 MHz/ 21500 | 16@17 | 2655 MHz/ 531000 | Lowest | REFSENS (NOTE 2) |
| 2 | 10 | 885 MHz/ 21500 | 25@12 | 2655 MHz/ 531000 | Mid | REFSENS (NOTE 2) |
| 3 |  | 890 MHz/ 21550 | 16@17 | 2670 MHz/ 534000 | Lowest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8 and 9 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_11: Test configurations table for exceptions due to UL harmonic interference for EN-DC 8_n77 and 8_n78 (Test frequencies are selected to fulfil Requirement of Note 6 and 7 in Table 7.3B.2.0.3.1-1)

| E-UTRA Band 8 |  |  |  | NR Band n77/n78 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 885 MHz/ 21500 | 16@17 | 3540 MHz/ 636000 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 885 MHz/ 21500 | 25@12 | 3540 MHz/ 636000 | Mid Highest | REFSENS (NOTE 2) |
| 3 | 10 | 897.5 MHz/ 21625 | 16@17 | 3590.01 MHz/ 639334 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 897.5 MHz/ 21625 | 25@12 | 3590.01 MHz/ 639334 | Mid Highest | REFSENS (NOTE 2) |
| 5 |  | 910 MHz/ 21750 | 16@17 | 3640.02 MHz/ 642668 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 910 MHz/ 21750 | 25@12 | 3640.02 MHz/ 642668 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 6 and 7 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_12: Test configurations table for exceptions due to UL harmonic interference for EN-DC 8_n79 (Requirement of Note 4 and 5)

| E-UTRA Band 8 |  |  |  | NR Band n79 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 890 MHz/ 21550 | 25@12 | 4450.02 MHz /696668 | Lowest Mid Highest | REFSENS (NOTE 2) |
| 2 | 10 | 897.5 MHz/ 21625 | 25@12 | 4487.52 MHz/ 699168 | Lowest Mid Highest | REFSENS (NOTE 2) |
| 3 |  | 910 MHz/ 21750 | 25@12 | 4550.01 MHz/ 703334 | Lowest Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_13: Test configurations table for exceptions due to UL harmonic interference for EN-DC 12_n66 (Requirement of Note 8 and 9)

| E-UTRA Band 12 |  |  |  | NR Band n66 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 704.1 MHz/ 23061 | 8@21 | 2112.5 MHz/ 422500 | Lowest | REFSENS (NOTE 2) |
| 2 | 10 | 707.5 MHz/ 23095 | 8@21 | 2122.5 MHz/ 424500 | Lowest | REFSENS (NOTE 2) |
|  |  | 707.5 MHz/ 23095 | 20@15 | 2122.5 MHz/ 424500 | Mid | REFSENS (NOTE 2) |
| 3 |  | 711 MHz/ 23130 | 8@21 | 2133 MHz/ 426600 | Lowest | REFSENS (NOTE 2) |
|  |  | 711 MHz/ 23130 | 20@15 | 2133 MHz/ 426600 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8 and 9 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_14: Test configurations table for exceptions due to UL harmonic interference for EN-DC 18_n77 (Requirement of Note 4 and 5)

| E-UTRA Band 18 |  |  |  | NR Band n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 820 MHz/ 23900 | 16@17 | 4100.01 MHz/ 673334 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 820 MHz/ 23900 | 25@12 | 4100.01 MHz/ 673334 | Mid Highest | REFSENS (NOTE 2) |
| 3 | 10 | 822.5 MHz/ 23925 | 16@17 | 4112.52 MHz/ 674168 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 822.5 MHz/ 23925 | 25@12 | 4112.52 MHz/ 674168 | Mid Highest | REFSENS (NOTE 2) |
| 5 |  | 825 MHz/ 23950 | 16@17 | 4125 MHz/ 675000 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 825 MHz/ 23950 | 25@12 | 4125 MHz/ 675000 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_15: Test configurations table for exceptions due to UL harmonic interference for EN-DC 19_n77 (Requirement of Note 4 and 5)

| E-UTRA Band 19 |  |  |  | NR Band n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 835 MHz/ 24050 | 16@17 | 4175.01 MHz/ 678334 | Lowest | REFSENS (NOTE 2) |
| 2 | 10 | 835 MHz/ 24050 | 25@12 | 4175.01 MHz/ 678334 | 40 | REFSENS (NOTE 2) |
| 3 |  | 838.9 MHz/ 24089 | 16@17 | 4194.99 MHz/ 679666 | Lowest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_16: Test configurations table for exceptions due to UL harmonic interference for EN-DC 28_n77/n78 (Requirement of Note 4 and 5)

| E-UTRA Band 28 |  |  |  | NR Band n77/n78 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 708 MHz/ 27260 | 10@20 | 3540 MHz/ 636000 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 708 MHz/ 27260 | 25@12 | 3540 MHz/ 636000 | Mid Highest | REFSENS (NOTE 2) |
| 3 | 10 | 723 MHz/ 27410 | 10@20 | 3615 MHz/ 641000 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 723 MHz/ 27410 | 25@12 | 3615 MHz/ 641000 | Mid Highest | REFSENS (NOTE 2) |
| 5 |  | 743 MHz/ 27610 | 10@20 | 3715.02 MHz/ 647668 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 743 MHz/ 27610 | 25@12 | 3715.02 MHz/ 647668 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_17: Void

Table 7.3B.2.3.4.2.1-2_18: Test configurations table for exceptions due to UL harmonic interference for EN-DC 26_n41 (Requirement of Note 8 and 9)

| E-UTRA Band 26 |  |  |  | NR Band n41 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 833.7 MHz/ 26887 | 16@17 | 2501.1 MHz/ 500220 | Lowest | REFSENS (NOTE 2) |
| 3 |  | 844 MHz/ 26990 | 16@17 | 2532 MHz/ 506400 | Lowest | REFSENS (NOTE 2) |
|  |  | 844 MHz/ 26990 | 25@12 | 2532 MHz/ 506400 | 50 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8 and 9 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_19: Test configurations table for exceptions due to UL harmonic interference for EN-DC 26_n77/78 (Requirement of Note 6 and 7)

| E-UTRA Band 26 |  |  |  | NR Band n77/n78 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 825 MHz/ 26800 | 16@17 | 3300 MHz/ 620000 | Lowest Mid Highest | REFSENS (NOTE 2) |
| 2 | 10 | 831.5 MHz/ 26865 | 16@17 | 3326 MHz/ 621733 | Lowest Mid Highest | REFSENS (NOTE 2) |
| 3 |  | 844 MHz/ 226990 | 16@17 | 3376 MHz/ 625066 | Lowest Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 6 and 7 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_20: Test configurations table for exceptions due to UL harmonic interference for EN-DC 66_n77/78 (Requirement of Note 2 and 13)

| E-UTRA Band 66 |  |  |  | NR Band n77/n78 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 1715 MHz/ 132022 | 25@12 | 3430 MHz/ 628666 | Lowest Mid Highest | REFSENS (NOTE 2) |
| 2 | 10 | 1755 MHz/ 132422 | 25@12 | 3510 MHz/ 634000 | Lowest Mid Highest | REFSENS (NOTE 2) |
| 3 |  | 1775 MHz/ 132622 | 25@12 | 3550 MHz/ 636666 | Lowest Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 2 and 13 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_21: Test configurations table for exceptions due to UL harmonic interference for EN-DC 66_n78 (Requirement of Note 3)

| E-UTRA Band 66 |  |  |  | NR Band n78 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) / NUL | UL allocation (LCRB) | NR FC (DL) (MHz) | NR FC (DL) NUL | NR CBW (MHz) | UL allocation (LCRB) |
|  |  |  |  | 3405.000 | 627000 | 10 |  |
| 1 |  |  |  | 3402.495 | 626833 | 15 |  |
|  |  | 1715 MHz / |  | 3400.005 | 626667 | 20 | REFSENS |
|  |  | 132022 |  | 3454.995 | 630333 | 10 | (NOTE 2) |
| 2 |  |  |  | 3457.500 | 630500 | 15 |  |
|  |  |  |  | 3460.005 | 630667 | 20 |  |
|  |  |  |  | 3484.995 | 632333 | 10 |  |
| 3 |  |  |  | 3482.505 | 632167 | 15 |  |
|  | 10 | 1755 MHz / | 25@12 | 3480.000 | 632000 | 20 | REFSENS |
|  |  | 132422 |  | 3535.005 | 635667 | 10 | (NOTE 2) |
| 4 |  |  |  | 3537.495 | 635833 | 15 |  |
|  |  |  |  | 3540.000 | 636000 | 20 |  |
|  |  |  |  | 3574.995 | 638333 | 10 |  |
| 5 |  |  |  | 3577.500 | 638500 | 15 |  |
|  |  | 1775 MHz / |  | 3580.005 | 638667 | 20 | REFSENS |
|  |  | 132622 |  | 3574.995 | 638333 | 10 | (NOTE 2) |
| 6 |  |  |  | 3577.500 | 638500 | 15 |  |
|  |  |  |  | 3580.005 | 638667 | 20 |  |
| NOTE 1: Test frequencies are selected to fulfil Note 3 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Only NR channel bandwidths supported by the UE are tested. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_22: Test configurations table for exceptions due to UL harmonic interference for EN-DC 1_n28

| NR Band n28 |  |  |  | E-UTRA Band 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | E-UTRA FC (DL) (MHz) NDL | E-UTRA CBW (MHz) | UL allocation (LCRB) |
| 1 | 10 | 715 MHz/ 143000 | 8@21 | 2145 MHz/ 350 | 5 | REFSENS (NOTE 2) |
| 2 | 10 | 715 MHz/ 143000 | 16@17 | 2145 MHz/ 350 | 10 | REFSENS (NOTE 2) |
| 3 | 10 | 715 MHz/ 143000 | 25@12 | 2145 MHz/ 350 | 15, 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8, 9 and 10 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_23: Test configurations table for exceptions due to UL harmonic interference for EN-DC 7_n71 (Requirement of Note 6 and 7)

| NR Band n71 |  |  |  | E-UTRA Band 7 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 668 MHz/ 133600 | 16@17 | 2672 MHz/ 3270 | 10 | REFSENS (NOTE 2) |
| 2 | 10 |  | 25@17 |  | 15 | REFSENS (NOTE 2) |
| 3 |  |  | 25@17 |  | 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 6 and 7 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW and E-UTRA band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_24: Test configurations table for exceptions due to UL harmonic interference for EN-DC 2_n71 (Requirement of Note 11 and 12)

| NR Band n71 |  |  |  | E-UTRA Band 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
|  |  |  | 25@12 | 1985 MHz/ 1150 | 10 | REFSENS (NOTE 2) |
| 1 | 5 | 665.5 MHz/ 133100 | 20@15 | 1982.5 MHz/ 1125 | 15 | REFSENS (NOTE 2) |
|  |  |  | 20@15 | 1980 MHz/ 1100 | 20 | REFSENS (NOTE 2) |
|  |  |  |  | 1985 MHz/ 1150 | 10 | REFSENS (NOTE 2) |
| 2 | 10 | 668 MHz/ 133600 | 8@21 | 1982.5 MHz/ 1125 | 15 | REFSENS (NOTE 2) |
|  |  |  |  | 1980 MHz/ 1100 | 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 11 and 12in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW and E-UTRA band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_25: Test configurations table for exceptions due to UL harmonic interference for EN-DC 12_n78 (Test frequencies are selected to fulfil Requirement of Note 4 and 5 in Table 7.3B.2.0.3.1-1)

|  | E-UTRA Band 12 |  |  | NR Band 78 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) / NUL | UL allocation (LCRB) | NR FC (DL) (MHz) / NDL | NR CBW (MHz) | UL allocation (LCRB) |
|  |  |  | 10@10 |  | 10 |  |
| 1 | 10 | 711 MHz / 23130 | 15@22 | Mid | 15 | REFSENS (NOTE 2) |
|  |  |  | 20@15 |  | 20 |  |
|  |  |  | 25@12 |  | >=40 |  |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Only Highest NR channel bandwidths supported by the UE are tested. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_26: Test configurations table for exceptions due to UL harmonic interference for EN-DC 48_n66 (Victim LTE B48) (Requirement of Note 2 and 13)

| NR Band n66 |  |  |  | E-UTRA Band 48 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
|  |  |  | 12@6 | 3555.0 MHz/ 55290 | 5 | REFSENS (NOTE 2) |
| 1 | 5 | 1777.5 MHz/ 355500 |  | 3555.0 MHz/ 55290 | 10 | REFSENS (NOTE 2) |
|  |  |  |  | 3555.0 MHz/ 55290 | 15 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 2 and 13 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW and E-UTRA band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_27: Test configurations table for exceptions due to UL harmonic interference for EN-DC 48_n66 (Victim LTE B48) (Requirement of Note 3)

| NR Band n66 |  |  |  | E-UTRA Band 48 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
|  |  |  | 12@6 | 3577.5 MHz/ 55515 | 5 | REFSENS (NOTE 2) |
| 1 | 5 | 1777.5 MHz/ 355500 |  | 3580.0 MHz/ 55540 | 10 | REFSENS (NOTE 2) |
|  |  |  |  | 3585.0 MHz/ 55590 | 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 3 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW and E-UTRA band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_28: Test configurations table for exceptions due to UL harmonic interference for EN-DC 28_n51 (Victim n51) (Requirement of Note 2, 13)

| E-UTRA Band B28 |  |  |  | NR Band n51 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 | 5 | 710 MHz/ 27280 | 12@6 | 1420 MHz/ 284000 | 5 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 2 and 13 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios due to receiver harmonic mixing for EN-DC in NR FR1 are specified in Table 7.3B.2.3.4.2.1-3, to Table 7.3B.2.3.4.2.1-3j.

Table 7.3B.2.3.4.2.1-3: Default test conditions for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 38.508-1 [11] clause4.3.1 |  |  |  | Mid range for E-UTRA and NR, unless otherwise specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Highest NRB_agg, unless otherwise specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest supported SCS |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| Higher Frequency Band |  | Lower Frequency Band |  | Higher Frequency Band |  | Lower Frequency Band |  |
| Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation |
| (NOTE 1) | Full RB | (NOTE 1) | Full RB | (NOTE 2) | (NOTE 3) | (NOTE 2) | REFSENS (NOTE 4) |
| NOTE 1: QPSK for E-UTRA band or CP-OFDM QPSK for NR band. NOTE 2: QPSK for E-UTRA band or DFT-s-OFDM QPSK for NR band. NOTE 3: Uplink RB configuration as defined in Table 7.3B.2.0.3.2-1, unless otherwise specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k. NOTE 4: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] if higher frequency band is NR band, which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band; REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] if higher frequency band is E-UTRA band, which defines uplink RB configuration and start RB location for each channel BW, unless otherwise specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3a: Test configurations table for exceptions due to receiver harmonic mixing for EN-DC 2_n71

|  | NR Band n71 |  |  | E-UTRA Band 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | NR Channel BW | NR FC (UL) | UL allocation (LCRB) | Channel BW (MHz) | FC (UL) (MHz) / NUL | UL allocation (LCRB) |
| 1 | 5 | Low | REFSENS (NOTE 2) | 5 | 1858.50 / 18685 | 25@0 |
| 2 | 5 | Low | REFSENS (NOTE 2) | 10 | 1858.50 / 18685 | 50@0 |
| 3 | 5 | Mid | REFSENS (NOTE 2) | 5 | 1903.50 / 19135 | 25@0 |
| 4 | 5 | Mid | REFSENS (NOTE 2) | 10 | 1903.50 / 19135 | 50@0 |
| 5 | 20 | Low | REFSENS (NOTE 2) | 5 | 1881.00 / 18910 | 25@0 |
| 6 | 20 | Low | REFSENS (NOTE 2) | 10 | 1881.00 / 18910 | 50@0 |
| 7 | 20 | Low | REFSENS (NOTE 2) | 15 | 1881.00 / 18910 | 50@13 |
| 8 | 20 | Low | REFSENS (NOTE 2) | 20 | 1881.00 / 18910 | 50@25 |
| 9 | 20 | Mid | REFSENS (NOTE 2) | 5 | 1903.50 / 19135 | 25@0 |
| 10 | 20 | Mid | REFSENS (NOTE 2) | 10 | 1903.50 / 19135 | 50@0 |
| NOTE 1: Test frequencies are selected to fulfil Note 4 in Table 7.3B.2.0.3.2-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3aa: Test configurations table for exceptions due to receiver harmonic mixing for EN-DC 3_n78

|  | NR Band n78 |  |  | E-UTRA Band 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | NR Channel BW | NR FC (UL) | UL allocation (LCRB) | Channel BW (MHz) | FC (UL) (MHz) / NUL | UL allocation (LCRB) |
| 1 | 50 | Mid | 100@85 | 20 | Mid | REFSENS (NOTE 3) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 in Table 7.3B.2.0.3.2-1. NOTE 2: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. NOTE 3: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3b: Test configurations table for exceptions due to receiver harmonic mixing for EN-DC 26_n41 (Victim band LTE)

| NR Band n41 |  |  |  | E-UTRA Band 26 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 | 10 | 2593 MHz/ 518601 | 50@0 | 864.3 MHz/ 8743 | 5, 10, 15, 20 | REFSENS (NOTE 2) |
| 2 |  | 2660 MHz/ 532000 | 50@0 | 886.6 MHz/ 8966 | 5, 10, 15, 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 in Table 7.3B.2.0.3.2-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3c: Void

Table 7.3B.2.3.4.2.1-3d: Test configurations table for exceptions due to receiver harmonic mixing for EN-DC 41_n77 (Victim band LTE)

| NR Band n77 |  |  |  | E-UTRA Band 41 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | E-UTRA FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 | 20 | 3765 MHz/ 651000 | 50@28 | 2510 MHz/ 39790 | 5, 10, 15, 20 | REFSENS (NOTE 2) |
| 2 |  | 4005 MHz/ 667000 | 50@28 | 2670 MHz/ 41390 | 5, 10, 15, 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8 in Table 7.3B.2.0.3.2-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3e: Test configurations table for exceptions due to receiver harmonic mixing for EN-DC 41_n78 (Victim band LTE)

| NR Band n78 |  |  |  | E-UTRA Band 41 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | E-UTRA FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 | 20 | 3789.7 MHz/ 652644 | 50@28 | 2526.4MHz/ 39954 | 5, 10, 15, 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8 in Table 7.3B.2.0.3.2-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3f: Test configurations table for exceptions due to receiver harmonic mixing for EN-DC 28_n77 (Victim band LTE)

| NR Band n77 |  |  |  | E-UTRA Band 41 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | E-UTRA FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 | 20 | 3850 MHz/ 656666 | 100@3 | 770 MHz/ 9330 | 5, 10, 15, 20 | REFSENS (NOTE 2) |
| 2 |  | 3950 MHz/ 663333 | 100@3 | 790 MHz/ 9530 | 5, 10, 15, 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 2 in Table 7.3B.2.0.3.2-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3g: Void

Table 7.3B.2.3.4.2.1-3h: Test configurations table for exceptions due to receiver harmonic mixing for EN-DC 19_n79

|  | E-UTRA Band 19 |  |  | NR Band n79 |  |
| --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW | FC (UL) (MHz) / NUL | UL allocation (LCRB) | NR Channel BW (MHz) | NR FC (UL) |
| 1 | 5 | 840.40 / 24104 | REFSENS (NOTE 2) | 40 | Low |
| 2 | 10 | 840.40 / 24104 | REFSENS (NOTE 2) | 40 | Low |
| 3 | 15 | 840.40 / 24104 | REFSENS (NOTE 2) | 40 | Low |
| NOTE 1: Test frequencies are selected to fulfil Note 2 in Table 7.3B.2.0.3.2-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW and E-UTRA band. NOTE 3: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3i: Test configurations table for exceptions due to receiver harmonic mixing for EN-DC 21_n79

|  | E-UTRA Band 21 |  |  | NR Band n79 |  |
| --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW | FC (UL) | UL allocation (LCRB) | NR Channel BW (MHz) | NR FC (UL) (MHz) / NUL |
| 1 | 5 | Mid | REFSENS (NOTE 2) | 40 | 4510.20 / 700680 |
| 2 | 10 | Mid | REFSENS (NOTE 2) | 40 | 4510.20 / 700680 |
| 3 | 15 | Mid | REFSENS (NOTE 2) | 40 | 4510.20 / 700680 |
| NOTE 1: Test frequencies are selected to fulfil Note 3 in Table 7.3B.2.0.3.2-1. NOTE 2: Test point NR fUL/DL = 4510.2 MHz (NDL = 700680). NOTE 3: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW and E-UTRA band. NOTE 4: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3j: Test configurations table for exceptions due to receiver harmonic mixing for EN-DC 26_n79 (Victim band LTE)

| NR Band n79 |  |  |  | E-UTRA Band 26 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | E-UTRA FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 | 40 | 4427.4 MHz/ 695160 | 216@0 | 885.4 MHz/ 8954 | 5, 10, 15 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 2 in Table 7.3B.2.0.3.2-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3k: Void

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios due to cross band isolation for EN-DC in NR FR1 are specified in Table 7.3B.2.3.4.2.1-4, to Table 7.3B.2.3.4.2.1-4n.

Table 7.3B.2.3.4.2.1-4: Test configurations table for exceptions due to cross band isolation for EN-DC FR1

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n. |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n. |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 |  |  |  | 30 KHz |  |  |  |
| NR Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | Specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n | QPSK | Table 7.3.4.1-1 in TS 36.521-1 [10] |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4a: Test configurations for exceptions due to cross band isolation for EN-DC 2_n41 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B2 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | High range | High range | High range |
| NR | 100 MHz 160@0 | low range | X1- | X1- | X1 |
| n41 | 100 MHz 160@0 | mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall set per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4b: Test configurations for exceptions due to cross band isolation for EN-DC 25_n41 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B25 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | High range | High range | High range |
| NR | 100 MHz 160@0 | low range | X1- | X1- | X1 |
| n41 | 100 MHz 160@0 | mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall set per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4c: Test configurations for exceptions due to cross band isolation for EN-DC 7_n78 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B7 DL CBM/RB allocation |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | Low, Mid High range | Low, Mid High range | Low, Mid High range |
| NR Band | 100 MHz 270@0 | Low range | X | X | X |
| n78 | 100 MHz 270@0 | High range | - | - | X |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4d: Test configurations for exceptions due to cross band isolation for EN-DC 38_n78 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B38 DL CBM/RB allocation |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | Low, Mid High range | Low, Mid High range | Low, Mid High range |
| NR Band | 100 MHz 270@0 | Low range | X | X | X |
| n78 | 100 MHz 270@0 | High range | - | - | X |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4e: Test configurations table for exceptions due to cross band isolation for EN-DC 41_n77 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B41 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | High range | High range | High range |
| NR | 100 MHz 270@0 | Low range | X1 | X1 | X1 |
| n77 | 100 MHz 270@0 | Mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. NOTE 3: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4f: Test configurations table for exceptions due to cross band isolation for EN-DC 41_n77 (Victim band NR)

| Aggressor UL |  |  | n77 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 20MHz | 50 MHz | 100MHz |
|  |  |  | Low range | Mid range | High range |
| E-UTRA | 20 MHz 100@0 | High range | X1 | X1 | X1 |
| B41 | 20 MHz 100@0 | Mid range | X1 | - | - |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. NOTE 3: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4g: Test configurations table for exceptions due to cross band isolation for EN-DC 41_n78(Victim band LTE)

| Aggressor UL |  |  | E-UTRA B41 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | High range | High range | High range |
| NR | 100 MHz 270@0 | low range | X1 | X1 | X1 |
| n78 | 100 MHz 270@0 | mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. NOTE 3: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4h: Test configurations table for exceptions due to cross band isolation for EN-DC 41_n78 (Victim band NR)

| Aggressor UL |  |  | n78 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 20MHz | 50 MHz | 100MHz |
|  |  |  | Low range | Mid range | High range |
| E-UTRA | 20 MHz 100@0 | High range | X1 | X1 | X1 |
| B41 | 20 MHz 100@0 | Mid range | X1 | - | - |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. NOTE 3: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4i: Test configurations table for exceptions due to cross band isolation for EN-DC 66_n41(Victim band LTE)

| Aggressor UL |  |  | E-UTRA B66 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | High range | High range | High range |
| NR | 100 MHz 128@0 | low range | X1 | X1 | X1 |
| n41 | 100 MHz 128@0 | mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. NOTE 3: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4j: Test configurations table for exceptions due to cross band isolation for EN-DC 1_n40 (Victim band NR)

| Aggressor UL |  |  | NR n40 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 30 MHz | 80 MHz |
|  |  |  | Low range | Low range | Low range |
| E-UTRA | 20 MHz 100@0 | Mid range | X1 | X1 | X1 |
| B1 | 20 MHz 100@0 | High range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4k: Test configurations table for exceptions due to cross band isolation for EN-DC 1_n40 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B1 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | Low range | Mid range | High range |
| NR | 20 MHz 100@0 | low range | X1 | X1 | X1 |
| n40 | 20 MHz 100@0 | mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4l: Test configurations table for exceptions due to cross band isolation for EN-DC 3_n51 (Victim band NR)

| Aggressor UL |  |  | NR n51 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 5 MHz | 5 MHz | 5 MHz |
|  |  |  | Low range | Mid range | High range |
| E-UTRA | 5 MHz 25@0 | low range | X1 | - | - |
| B3 | 5 MHz 25@0 | Mid range | X1 | - | - |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4m: Test configurations table for exceptions due to cross band isolation for EN-DC 30_n66 (Victim band NR)

| Aggressor UL |  |  | NR n66 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 20 MHz | 20 MHz | 20 MHz |
|  |  |  | Low range | Mid range | High range |
| E-UTRA | 5 MHz 25@0 | low range | X1 | X1 | X1 |
| B30 | 5 MHz 25@0 | Mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4n: Test configurations table for exceptions due to cross band isolation for EN-DC 46_n78 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B46 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 20 MHz | 20 MHz | 20 MHz |
|  |  |  | Low range | Mid range | High range |
| NR | 50 MHz 270@0 | mid range | X1 | X1 | X1 |
| N78 | 50 MHz 270@0 | high range | X1 | - | - |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios due to dual uplink operation for EN-DC in NR FR1 (two bands) are specified in Table 7.3B.2.3.4.2.1-5.

Table 7.3B.2.3.4.2.1-5: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to "Range" columns. |  |  |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | DC Configuration / NRB_agg |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |
|  | DC Configuration |  |  |  |  |  | E-UTRA Ch BW |  | NR Ch BW |  | CC MOD E-UTRA/NR | E-UTRA & NR RB allocation |  | CC MOD E-UTRA/NR |  | E-UTRA & NR allocations (LCRB @ RBstart) |  |  |  |
|  | E-UTRA |  |  |  | NR |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |  |
|  | Band |  | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  | Mid |  | Y | Mid | 5 |  | Mid |  | QPSK /CP-OFDM QPSK | All RBs |  | QPSK/ DFT-s-OFDM QPSK |  | REFSENS |  |  | REFSENS |
| Test Settings for DC_1A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Note 5 |  | 3 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_1A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_1A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_2A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | Note 5 |  | 66 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_2A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_3A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 5 | Note 5 | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_3A-n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 7 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_3A-n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 20 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_3A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | Note 5 |  | 77/ 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_5A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | Note 5 |  | 66 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_5A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| Test Settings for DC_7A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Low |  | 5 | High | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_7A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Note 5 |  | 66 | Note 5 | 10 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 25@0 |
| Test Settings for DC_7A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Low |  | 77 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A-n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 1 | High | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_8A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 3 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Low |  | 41 | High | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_8A-n77A/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| 2 | 8 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 52@0 |
| Test Settings for DC_8A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 |  | Note 5 |  | 79 | Note 5 | 5 |  | 40 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 216@0 |
| Test Settings for DC_12A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | All RBs |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_20A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 3 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_20A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 8 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_20A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 2 | 20 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_21A- n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 21 | Note 5 |  | 79 |  | Note 5 | 5 | 40 |  | Note 7 |  | All RBs |  |  | Note 7 |  | 25@0 | 216@0 |  |
| Test Settings for DC_26- n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 |  | Note 5 |  | 41 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_26A- n77/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 26 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 1 | 26 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_48A- n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 48 |  | Note 5 |  | 66 | Note 5 | 20 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 100@0 |  |  | 25@0 |
| Test Settings for DC_28A- n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 41 | Note 5 | 10 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 50@0 |
| Test Settings for DC_28A- n50A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 50 | Note 5 | 10 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 50@0 |  |  | 50@0 |
| Test Settings for DC_28A- n51A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 51 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_28A- n77/n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | Note 5 |  | 77 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| 2 | 28 |  | Note 5 |  | 78 | Note 5 | 5 |  | 10 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 50@0 |
| Test Settings for DC_66A-n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 2 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A-n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 5 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for DC_66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 |  | Note 5 |  | 71 | Note 5 | 5 |  | 5 |  | Note 7 | All RBs |  | Note 7 |  | 25@0 |  |  | 25@0 |
| Test Settings for CA_7A-20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | Note 5 |  | 20 | Note 5 | 10 |  | 5 |  | QPSK | All RBs |  | QPSK |  | 50@0 |  |  | 25@0 |
| Note 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. Note 2: Use DC Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the DC Configuration. E.g. for DC_1A-n3A, X=1, Y=3. Note 4: REFSENS refers to the E_UTRA bands and NR band NRB 's single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2 of TS 36.521-1 [10] and Table 7.3.2.4.1-3 of TS 38.521-1 [8], respectively Note 5: Test frequency for each DC configuration shall follow Table 7.3B.2.0.3.5.1-1. If test configurations of each ID in a DC configuration are same, test frequency shall follow the order of Table 7.3B.2.0.3.5.1-1. Note 6: Not applicable if the UE only supports Bandwidth Combination Set 1. Note 7: Same as default. Note 8: RBSTART = 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |
|  | Band | Range |  | NRB |  |  | Band |  |  |  | Range |  | NRB |  |  |
|  | UL MOD | DL MOD |  | CH BW |  | DLalloc / UL alloc | UL MOD |  |  |  | DL MOD |  | UL/DL Ch BW | DLalloc / UL alloc |  |
| Test Settings for a DC_1A_n78 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  | FFS |  |  |  |  |  |  |  |  |
| 24 | 1 | DL 2140 |  |  |  |  | n78 |  |  |  | 3710 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3_n1 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 3 | Mid |  |  |  |  | n1 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_NR |  |
| 26 | 3 | Mid |  |  |  |  | n1 |  |  |  | Mid |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | CP-OFDM QPSK |  | 20 MHz | All RBs / 0 |  |
| 34 | 3 | DL 1855 |  |  |  |  | n1 |  |  |  | 2140 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n41 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 | Low |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | QPSK |  | 20 MHz | All RBs / 0 |  |
| 26 | 3 | Mid |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_3 | N/A |  |  |  | QPSK |  | 20 MHz | All RBs / 0 |  |
| 36 | 3 | High |  |  |  |  | n41 |  |  |  | Low |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | QPSK |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| 46 | 3 | High |  |  |  |  | n41 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | QPSK |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| 54 | 3 | DL 1835 |  |  |  |  | n41 |  |  |  | 2657.5 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | QP |  |  |  | QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_3A_n78 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3495 |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / 0 |  |
| 23 | 3 | Mid |  |  |  |  | n78 |  |  |  | 3525 |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 20 MHz | All RBs / 0 |  |
| 35 | 3 | High |  |  |  |  | n78 |  |  |  | Mid |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | CP-OFDM QPSK |  |  |  | QPSK |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |
| 44 | 3 | DL 1835 |  |  |  |  | n78 |  |  |  | 3575 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| 54 | 3 | DL 1860 |  |  |  |  | n78 |  |  |  | 3435 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | 20 | Mid |  |  |  |  | n3 |  |  |  | High |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / 0 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / REFSENS_NR |  |
| 24 | 20 | DL 799 |  |  |  |  | n3 |  |  |  | 1870 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| 34 | 20 | DL 806 |  |  |  |  | n3 |  |  |  | 1830 |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  | FFS |  |  |  |  |  |  |  |  |
| 23 | 20 | Mid |  |  |  |  | n78 |  |  |  | 3388 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 20 MHz |  | All RBs / REFSENS_ENDC_1 | N/A |  |  |  | QPSK |  | 100 MHz | All RBs / 0 |  |
| 34 | 20 | DL 809 MHz |  |  |  |  | n78 |  |  |  | 3359 MHz |  |  |  |  |
|  | QPSK | QPSK |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 | CP-OFDM QPSK |  |  |  | QPSK |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_40_n78 Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 | Mid |  |  |  |  |  | n78 |  | Mid |  |  |  |  |  |
|  | QPSK | QPSK | 20 MHz |  | All RBs / REFSENS_ENDC_2 |  |  | N/A |  | CP-OFDM QPSK |  | 20 MHz |  |  | All RBs / 0 |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 | Mid |  |  |  |  | n77/n78 |  |  |  | 3889.5 |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |
| 26 | 41 | Mid |  |  |  |  | n77/n78 |  |  |  | 3974.5 |  |  |  |  |
|  | N/A | QPSK |  | 20 MHz |  | All RBs / 0 | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation according to table 7.3B.2.0.3.5.1-1. NOTE 2: Test ID with no exception NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Test ID for UE supporting single UL. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3.4.2.2	Test procedure