3GPP TSG-RAN5 Meeting #90-e	R5-211166r1

# Electronic Meeting, 22nd Feb 2021 – 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1782 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of QoS for URLLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no QoS support for URLLC scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New QoS rules and flows, suitable for URLLC, are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | URLLC test cases will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: The QoS rule identifier was updated from 6 to 7. |  |  |  |  |  |  |  |  |

### 4.8.2	5GC configurations

#### 4.8.2.1	Reference QoS rules

Table 4.8.2.1-1: Reference QoS rule #1

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0001’B | 1 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filters |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0001’B | QFI 1 (Table 4.8.2.3-1) |  |

Table 4.8.2.1-2: Reference QoS rule #2

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0010’B | 2 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0010’B | QFI 2 (Table 4.8.2.3-2) |  |

Table 4.8.2.1-3: Reference QoS rule #3

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-2 | Packet filter list #2 |  |
| QoS rule precedence | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0001’B | QFI 1 (Table 4.8.2.3-1) |  |

Table 4.8.2.1-4: Reference QoS rule #4

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0100’B | 4 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-3 | Packet filter list #3 |  |
| QoS rule precedence | ‘0000 00100’B | 4 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0010’B | QFI 2 (Table 4.8.2.3-2) |  |

Table 4.8.2.1-4a: Reference QoS rule #4a

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 1111’B | 15 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-3a | Packet filter list #3a |  |
| QoS rule precedence | ‘0000 1111’B | 15 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0100’B | QFI 4 (Table 4.8.2.3-2a) |  |

Table 4.8.2.1-5: Reference QoS rule #5

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0101’B | 5 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-4 | Packet filter list #4 |  |
| QoS rule precedence | ‘0000 0101’B | 5 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0101’B | QFI 5 (Table 4.8.2.3-3) |  |

Table 4.8.2.1-6: Reference QoS rule #6

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0110’B | 6 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule is the non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-5 | Packet filter list #5 |  |
| QoS rule precedence | ‘0000 0110’B | 6 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0110’B | QFI 6 (Table 4.8.2.3-4) |  |

Table 4.8.2.1-7: Reference QoS rule #7

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0001’B | 1 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0010’B | QFI 2 (Table 4.8.2.3-2) |  |
| QoS rule |  |  | IMS_VOICE |
| QoS rule identifier | ‘0000 0010’B | 2 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule a non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-6 | Packet filter list #6 |  |
| QoS rule precedence | ‘0000 0001’B | 1 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0101’B | QFI 7 (Table 4.8.2.3-5) |  |
| QoS rule |  |  | IMS_VIDEO |
| QoS rule identifier | ‘0000 0011’B | 3 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘0’B | The QoS rule a non-default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-7 | Packet filter list #7 |  |
| QoS rule precedence | ‘0000 001’B | 2 (unique per PDU session) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 0110’B | QFI 8 (Table 4.8.2.3-6) |  |

| Condition | Explanation |
| --- | --- |
| IMS_VOICE | If this QoS rule is used to setup an IMS voice session |
| IMS_VIDEO | If this QoS rule is used to setup an IMS video session |

Table 4.8.2.1-8: Reference QoS rule #8

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 0111’B | 7 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 1001’B | QFI 9 (Table 4.8.2.3-7) |  |

#### 4.8.2.2	Reference packet filters

Table 4.8.2.2-1: Packet filter list #1

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0000’B | Id 0 |  |
| Component type 1 ID | ‘0000 0001’B | Match-all type |  |

Table 4.8.2.2-2: Packet filter list #2

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0010’B | Id 2 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.2 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C002 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-3: Packet filter list #3

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0011’B | Id 3 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.3 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C003 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-3a: Packet filter list #3a

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘1111’B | Id 15 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.30 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C030 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-4: Packet filter list #4

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0100’B | Id 4 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.4 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C004 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-5: Packet filter list #5

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | ‘0101’B | Id 5 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 1 | IPv6 remote address type/prefix lenght type | remoteIPv6 |
| Component type 1 Value | 10.10.10.5 255.255.255.255 | See Note 1 | remoteIPv4 |
|  | C0C0:C0C0:C0C0:C005 C0C0:C0C0:C0C0:C0C0/ 64 | See Note 1 | remoteIPv6 |
| Note 1: This IP address is also the address of an IP server able to send a flow of downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 4.8.2.2-6: Packet filter list #6

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | 0110’B | Id 6 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS speech media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: According to to TS 26.114 [45] and RFC 4566 [46] a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 4.8.2.2-7: Packet filter list #7

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Packet filter list |  |  |  |
| Packet filter direction | ‘11’B | bidirectional |  |
| Packet filter identifier | 0111’B | Id 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS video media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: According to to TS 26.114 [45] and RFC 4566 [46] a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

#### 4.8.2.3	Reference QoS flow descriptions

Table 4.8.2.3-1: Reference QoS flow #1

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0001’B | QFI 1 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 1001’B | 5QI 9 |  |
| EPS bearer identity | Any not yet assigned value different to '5' |  | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-2: Reference QoS flow #2

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0010’B | QFI 2 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0101’B | 5QI 5 |  |
| EPS bearer identity | 5 | EBI 5 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-2a: Reference QoS flow #2a

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0100’B | QFI 4 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-3: Reference QoS flow #3

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0101’B | QFI 5 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-4: Reference QoS flow #4

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0110’B | QFI 6 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 4.8.2.3-5: Reference QoS flow #5

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0111’B | QFI 7 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | 5QI 1 | 5QI 1 |  |
| EPS bearer identity | ‘0000 011 ’B | EBI 6 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-6: Reference QoS flow #6

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1000’B | QFI 8 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| Number of parameters | ’00 0010’B | 2 parameters | Interworking_with_EPS |
| 5QI | ‘0000 0010’B | 5QI 2 |  |
| EPS bearer identity | ‘0000 0111 ’B | EBI 7 | Interworking_with_EPS |

| Condition | Explanation |
| --- | --- |
| Interworking_with_EPS | If this flow is used in the Authorized QoS flow descriptions IE of a PDU SESSION ESTABLISHMENT ACCEPT message also including the Mapped EPS bearer context IE. |

Table 4.8.2.3-7: Reference QoS flow #7

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1001’B | QFI 9 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘101 0010’B | 5QI 82 |  |