3GPP TSG-RAN5 Meeting #90-e	R5-211320

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1350 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Test frequencies for NB-IoT FDD MFBI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the Table 8.3.2.3.1-1a, the test frequencies (Nul and Ndl values for f2) for the band combination, NBIOT Operating Band (25) and MFBI Overlapping Band( 2), are not assigned with overlapping DL frequency values between the bands. The EARFCNs DL Earfcn:8689(1995MHz) and UL Earfcn:26689(1915MHz) are legitimate for Band 25, but not for Band 2(DL:1930-1990MHz and UL:1850-1910). This needs to be corrected Editorial correction to band combination (OperatingBand=5, MFBIoverlapping Band=18) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the Table 8.3.2.3.1-1a for the band combination where NBIOT Operating Band is 25 and MFBI Overlapping Band is 2 Removed to ‘High’ which is redundant as explicit value provided |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Cl.8.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR: R5s210285 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updated Tdoc number in the CR Header |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.3.2.3	NB-IoT Default test frequencies

The default channel bandwidth of 200 kHz is applied to the signalling test. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

All operating bands can accommodate at least three test frequencies f1, f2 and f3 (f3<f1<f2).

For NB-IoT in-band and guard-band signalling testing, the NB-IoT frequency to be tested are low/mid/high range and are associated to the LTE frequency low/mid/high range as specified in tables 4.3.1.1.x. The E-UTRA channel bandwidth to be tested is 10MHz and NB-IoT PRB 30 for in-band and 5 MHz for guard-band for all operating bands for all test cases; unless the specific channel bandwidth is specified for the operating band below:

For Band 31 in-band testing, the E-UTRA channel bandwidth to be tested is 5 MHz.

< Skipped unmodified section>

Test frequencies for signalling test of MFBI are specified in Table 8.3.2.3.1-1a for FDD in terms of Low, Mid and High which are referred to as Low Range, Mid Range and High range in clause 8.1.3.1.

Table 8.3.2.3.1-1a: Test frequencies for NB-IoT FDD MFBI (200 kHz)

| NB-IoT Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL |
| 2 | 25 | 60 | Mid | Mid | High | High | Low | Low |
| 5 | 18 | 15 | Mid | Mid | 23999 | 5999 | Low | Low |
| 5 | 19 | 15 | Mid | Mid | 20609 | 2609 | Low | Low |
| 5 | 26 | 25 | Mid | Mid | High | High | Low | Low |
| 12 | 17 | 12 | Mid | Mid | High | High | Low | Low |
| 17 | 12 | 12 | Mid | Mid | High | High | Low | Low |
| 19 | 5 | 15 | Mid | Mid | High | High | Low | Low |
| 19 | 26 | 15 | Mid | Mid | High | High | Low | Low |
| 25 | 2 | 60 | Mid | Mid | 26639 | 8639 | Low | Low |
| 26 | 5 | 25 | Mid | Mid | High | High | Low | Low |
| 26 | 18 | 15 | Mid | Mid | High | High | Low | Low |
| 26 | 19 | 15 | Mid | Mid | High | High | Low | Low |

<End of modified section>