3GPP TSG-RAN5 Meeting #90-e	R5-xxxx

# Electronic Meeting, 22nd February - 5th March 2021

# Source:	Huawei, Hisilicon

# Title:	Discussion paper on IMS 180 Ringing before or after resource reservation

# Document for:	Endorsement

1.	Introduction

For MT UE, when preconditions are not used, according to 38.508-1 clause 4.9.16 (Generic Test Procedure for IMS MT speech call establishment in 5GC), the MT UE would send 180 Ringing after the PDU session modification procedures.

However, reading the conformance texts in 24.229 quoted below:

[TS 24.229, annex U.3.1.4]:…

Upon successful reservation of local resources, if the precondition mechanism is not used by the terminating UE, the UE can send 180 (Ringing) response to the INVITE request and can alert the user.

The word “Upon” is frequently used in 24.229, and many times it appears as not strictly distinguishing “before” or “after”, rather, it towards more to “during”. Here is an example:

- “Upon generating an initial INVITE request, the UE shall include the Accept header field with "application/sdp…”

- “Upon generating an initial INVITE request using the precondition mechanism, the UE shall not indicate the requirement for the precondition mechanism by using the Require header field.”

Thus, the word “Upon” in above quoted U.3.1.4 might cause different interpretation as the following:

- Interpretion-1: as “after”

- Interpretion-2: as “before”

- Interpretion-3: no need to strictly distinguish “after” or “before”

2.	Discussion

For test cases that referring to “MT UE without preconditions”, the above mentioned different interpretations might cause variant UE behaviors. A couple of examples of the test cases:

- TS 34.229-5 test case 7.7 (MTSI MT Voice Call without preconditions at both originating UE and terminating UE)

- TS 34.229-5 test case 7.9 (MTSI MT Voice Call with preconditions at originating UE and without preconditions at terminating UE)

In 38.508-1 clause 4.9.16, it follows Interpretion-1 (reading “Upon” as “after”). However, considering there is no preconditions in MT UE, there is no reason to wait for “local resource reservation” before 180 Ringing. Thus, Interpretion-2 (reading “Upon” as “before”) and Interpretion-3 (no matter “before” or “after”) are also adequate..

3.	Proposals

Option 1: If RAN5 could make a consensus on the interpretation of ”Upon“ in the above mentioned context, the consensus could be reflected in R5-210055, which contains revision to 38.508-1 clause 4.9.16.

Option 2: If no consensus could be reached, then issuing a Liaise to CT1 for guidance.