3GPP TSG RAN Meeting #91-e	 	R5-212030

Electronic Meeting, 17th May – 28th May 2021

Source:		Rohde & Schwarz

Title:	On the achievable SNR for demod test cases

Agenda Item:		5.3.2.17

Document for:	Discussion and Endorsement

# Introduction

In the last RAN5 meetings the testability of high SNR demod test cases has been discussed [1 –  7] and several working assumptions have been agreed. However, the maximum achievable SNR is not yet finalized since details such as the backoff from the 1dB compression point of the PA have not been concluded. In the meantime, a working assumption on the achievable SNR has been endorsed for FR2a in order to enable testing of low SNR test points. In this contribution, we analyze the impact of backoff on the EVM and propose values for the achievable DL power in the center of quiet zone at 1dB compression. Until the acceptable backoff due to fading has been decided, we propose achievable SNR values for the Rel-15 FR2 operating bands as interim solution.

# Discussion

At RAN5 #90-e the following working assumptions have been endorsed:

Proposal 1a: For the cable loss agree 6.3 dB for FR2a as a working assumption.

Proposal 1b: Apply highest Noc level among the bands of the respective frequency range for that frequency range for calculation of achievable SNR.

Proposal 1c: Apply a backoff of 17.71 dB as working assumption.

In the same meeting, action point #89e.23 has been updated to “Provide input on the impact to TxEVM (and consequently SNR) for higher probability of saturation of faded signal”.

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#89e.23 | RF | Provide input on acceptable clipping frequency due to fading and/or acceptable fading crest factor margin for FR2 Demod performance and CSI test cases Provide input on the impact to TxEVM (and consequently SNR) for higher probability of saturation of faded signal | Qualcomm, Anritsu, Keysight, E///, R&S | R5-206168, R5-205702 R5-211083 | RAN5#91e | Open |

In this contribution, we analyze the impact of gain compression of test systems power amplifier on the DL EVM. Since the EVM of a faded 5G signal cannot be directly measured, we perform our measurements with an 5G FR2 DL signal with 100 MHz channel bandwidth, SCS 120kHz,  QPSK modulation and without fading. For the determination of the compression characteristics of the PA a CW signal has been applied.

For an EVM of 6%, a backoff of 8 dB is required for the currently specified operating bands. In the case of 256QAM an EVM of 3% is required and the measured back off is in the range from 10.4 dB to 11.7 dB for the analyzed mid frequencies.

Observation 1: A backoff of approximately 12 dB from the CW 1dB compression point is required for an EVM of 3%.

Proposal 1: Take observation 1 into account in the determination of acceptable clipping frequency and the acceptable CREST factor margin.

At RAN5 #89-e the following proposals have been endorsed.

Proposal 1 : Calculate achievable SNR for each frequency range : FR2a/FR2b/FR2c.

In order to optimize the achievable SNR, we propose to change this agreement and to calculate the achievable SNR based on the mid frequencies of FR2 operating bands instead of frequency ranges. In this way, the Refsens value of the individual band can be taken into account. In order to document the testability aspect in TS 38.521-4 [8] introduce a table summarizing the applicability of each test point for each band based on the achievable SNR.

Proposal 2: Derive the achievable SNR for the mid frequencies of FR2 operating bands instead of the frequency ranges FR2a/FR2b/FR2c.

Several figures in the achievable SNR calculator spreadsheet impact the available DL power in the center of quiet zone such as the antenna gain, cable loss, free space path loss etc. During the discussion at RAN5 #90-e it was requested to assess each figure for potential optimization. However, the values of the contributors are interrelated. For example, changing the gain of the feed antenna requires an adaption of the focal length of the reflector and this changes the free space path loss. Therefore, we feel that it will be difficult to reach an agreement for each and every contributor. Instead, we propose to replace the individual figures by the available DL power at the center of quiet zone when the PA is operated at its CW 1dB compression point.

This fosters the finding of a compromise among TE vendors since only a the combined value needs to be discussed instead of the individual contributors.

Accompanying CR [9] contains an update of the spreadsheet along these lines.

Based on the characteristics of our test system, we propose the values in Table 1 for the achievable DL power at 1dB compression.

Table 1: Proposed available DL power at QZ center

| Operating band and frequency range | Test frequency (GHz) | Available DL power at center of quiet zone at CW 1dB compression (dBm) |
| --- | --- | --- |
| n258 mid | 25.875 | -32.0 |
| n257 mid n261 mid | 28 27.925 | -32.3 |
| n260 mid | 38.5 | -36.4 |
| n259 mid | 41.5 | -39.4 |

Proposal 3: Apply the values in Table 1 in the calculation of the achievable DL SNR and update the spreadsheet calculator accordingly.

Based on the working assumption of a backoff of 17.71 dB of last meeting, this results in the maximum achievable SNR for 100MHz CHBW, IFF with 30cm QZ, PC3 summarized in Table 2.

Table 2: Maximum achievable SNR for 100 MHz CHBW, IFF with 30cm QZ, PC3

| Operating band and frequency range | Test frequency (GHz) | Maximum achievable SNR (dB) |
| --- | --- | --- |
| n258 mid | 25.875 | 21.6 |
| n257 mid n261 mid | 28 27.925 | 21.3 |
| n260 mid | 38.5 | 14.5 |
| n259 mid | 41.5 | 10.3 |

The calculation is documented in the Excel sheet attached to the CR [9] associated with this paper.

Proposal 4: Use the values in Table 2 as interim working assumption of the maximum achievable SNR for 100MHz CHBW, PC3 and IFF with 30cm Quiet Zone. Enable testing of the test points with SNR less than or equal to these values for 100 MHz CHBW.

In the discussion so far, the fact that the wireless cable method has to be applied for demod test cases has not been considered. In this method the signal for one polarization also contains an additional signal in order to compensate for channel and UE cross talk of the other polarization. The power of the additional signal can impact the maximum achievable SNR and the latter needs to be analyzed.

Proposal 5: Study the impact of wireless cable method on the maximum achievable SNR.

Once there is an agreement on the acceptable backoff and the impact of wireless cable method has been assessed, the maximum achievable SNR needs to be updated.

Proposal 6: Update the Demod SNR range calculator spreadsheet based on the outcome of the discussion of acceptable clipping and of the impact of wireless cable method in order to obtain the final maximum achievable SNR.

The maximum achievable SNRBB needs to be updated in TS 38.521-4 [8]. Currently the testability for demod test cases is captured in editor’s notes. However, summarizing this information in an applicability table makes the specification easier to conceive and to maintain. Therefore, we propose to introduce an applicability table in TS 38.521-4 specifying which test point shall be tested in a specific band. An example of such a table is contained in the Annex of this paper.

Proposal 7: Introduce a table in TS 38.521-4 summarizing the applicability of each test point for the FR2 operating bands.

Accompanying CRs [9] & [10] provide a documentation of the assumptions and limits in TR 38.903 [11] and an update in TS 38.521-4 [8], respectively.

# Conclusion

In summary, the maximum achievable SNR of demod test cases has been discussed.

Observation 1: A backoff of approximately 12 dB from the CW 1dB compression point is required for an EVM of 3%.

Proposal 1: Take observation 1 into account in the determination of acceptable clipping frequency and the acceptable CREST factor margin.

Proposal 2: Derive the achievable SNR for the mid frequencies of FR2 operating bands instead of the frequency ranges FR2a/FR2b/FR2c.

Proposal 3: Apply the values in Table 1 in the calculation of the achievable DL SNR and update the spreadsheet calculator accordingly.

Proposal 4: Use the values in Table 2 as interim working assumption of the maximum achievable SNR for 100MHz CHBW, PC3 and IFF with 30cm Quiet Zone. Enable testing of the test points with SNR less than or equal to these values for 100 MHz CHBW.

Proposal 5: Study the impact of wireless cable method on the maximum achievable SNR.

Proposal 6: Update the Demod SNR range calculator spreadsheet based on the outcome of the discussion of acceptable clipping and of the impact of wireless cable method in order to obtain the final maximum achievable SNR.

Proposal 7: Introduce a table in TS 38.521-4 summarizing the applicability of each test point for the FR2 operating bands.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #5 | Temperature range of the test equipment | 20°C – 35°C |
| #6 | Channel bandwidth | 100 MHz |

# Annex

Example of change to TS 38.521-4 to define the applicability of test requirements:

7.1.1_1	Applicability of test requirements due to maximum achievable SNR

Table 7.1.1_1-1 specifies the current assumption of maximum testable SNRBB.

Table 7.1.1_1-1: Current assumption of maximum testable SNRBB

| Operating Band / Frequency | Maximum testable SNRBB (dB) |
| --- | --- |
| n257 mid | [21.3] |
| n258 mid | [21.6[ |
| n259 mid | [10.3] |
| n260 mid | [14.5] |
| n261 mid | [21.3] |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 7.1.1_1-2.

Table 7.1.1_1-2: Current assumption of maximum testable SNRBB

| Test Case | Test point | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 1.4 | x | x | x | x | x |
|  | 1-2 | 3.6 | x | x | x | x | x |
|  | 1-3 | 14.2 | x | x | - | x | x |
|  | 2-1 | 5.8 | x | x | x | x | x |
|  | 2-2 | 16.0 | x | x | - | - | x |
|  | 2-3 | 15.7 | x | x | - | - | x |
|  | 2-4 | 15.8 | x | x | - | - | x |
|  | 2-5 | 16 | x | x | - | - | x |
|  | 2-6 | 20.3 | x | x | - | - | x |
| 7.2.2.2.1_2 | 3-1 | 20.7 | x | x | - | - | x |
| 7.3.2.2.1 | 1-1 | 7.7 | x | x | x | x | x |
|  | 1-2 | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 3.2 | x | x | x | x | x |
|  | 2-2 | 0.2 | x | x | x | x | x |

# References

- R5-205702, On Fading Impact on CREST and Demod Testability Analysis, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #89-e, November 2020

- R5-206614, On the achievable SNR for FR2 demod and perf test cases, Anritsu, 3GPP TSG RAN WG 5 Meeting #89-e, November 2020

- R5-211947, Input on fading crest factor margin for FR2 Demodulation test cases, 3GPP TSG RAN WG 5 Meeting #90-e, Qualcomm, February 2021

- R5-210775, On Testability Aspects for FR2 Demodulation Testing, Keysight Technologies UK Ltd, 3GPP TSG RAN WG 5 Meeting #90-e, February 2021

- R5-211950, On the achievable SNR for demod test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #90-e, February 2021

- R5-211936, Update of demod SNR testability, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #90-e, February 2021

- R5-211929, Update of FR2 demod test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #90-e, February 2021

- TS 38.521-4, V16.7.0, 3GPP TSG RAN WG5, March 2021

- R5-212032, Update of demod SNR testability, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- R5-212031, Update of FR2 demod test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #91-e, May 2021

- TR 38.903, V16.7.0, 3GPP TSG RAN WG5, March 2021