3GPP TSG-RAN5 Meeting #91-e	R5-212094

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2090 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of RRC message in RRC TC 8.1.5.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NR RRC message in test procedure shall align with the one in message sequence. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRC message in the test procedure was updated with editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect RRC messages will be used in test steps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.6.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

8.1.5.6.1.3	Test description

8.1.5.6.1.3.1	Pre-test conditions

System Simulator:

-	3 NR Cells. NR Cell 1 is the serving cell, NR Cell 2 and NR Cell 4 are the intra-frequency neighbour cell.

None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.5.6.1.3.2	Test procedure sequence

Table 8.1.5.6.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | “Off” | “Off” |
| T0A | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” | “Off” |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -88 | -94 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | “Off” | -88 |
| Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |

Table 8.1.5.6.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | “Off” | “Off” |
| T0A | SS/PBCH SSS EPRE | dBm/SCS | -82 | “Off” | “Off” |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -82 | -91 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | “Off” | -82 |
| Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.2-2. |  |  |  |  |  |

Table 8.1.5.6.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | The SS changes NR Cell 1 parameters according to the row "T0" in table Table 8.1.5.6.1.3.2-1/2 to set NR Cell 1 to Non-suitable “Off”. | - | - | - | - |
| 0B | The SS wait for 1,5 s (T310 (2s) is transmitted in SIB1). | - | - | - | - |
| 0C | The SS changes NR Cell 1 parameters according to the row "T0A" in table 8.1.5.6.1.3.2-1/2 making NR Cell 1 serving cell. | - | - | - | - |
| 0D | Check: For the next 5 s does the UE transmit any signalling message? | - | - | 1 | F |
| 0E | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.10 indicate that the UE is in NR RRC_CONNECTED state on NR Cell 1? | - | - | 1 | P |
| 1 | The SS changes NR Cell 1, NR Cell 2 and NR Cell 4 parameters according to the row "T1" in table 8.1.5.6.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded, and NR Cell 2 is suitable for camping. | - | - | - | - |
| 2 | Check: Does the UE initiate an RRC connection re-establishment procedure on Cell 1 or Cell 2. This is checked during the time T=T310? | - | - | 2 | F |
| 3 | Check: Does the UE send RRCReestablishmentRequest message on NR Cell 2? | --> | NR RRC: RRCReestablishmentRequest | 3 | P |
| 4 | The SS transmits RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 5 | The UE transmits RRCReestablishmentComplete message. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 6 | The SS transmits an RRCReconfiguration message to resume existing radio bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| 7 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationtComplete | 4 | P |
| 8 | The SS changes NR Cell 2 and NR Cell 4 parameters according to the row "T2" in table 8.1.5.6.1.3.2-1/2 in order that the radio link quality of NR Cell 2 is degraded and cell 4 is suitable for camping. | - | - | - | - |
| 9 | Check: Does the UE send RRCReestablishmentRequest message on NR Cell 4? | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 10 | The SS transmits RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 11 | Check: Does the UE transmit a RRCSetupComplete message and a REGISTRATION REQUEST message indicating "mobility registration updating"? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 5 | P |
| 11A | The SS transmits a DLInformationTransfer message and an AUTHENTICATION REQUEST message. | <-- | RRC: DLInformationTransfer NAS: AUTHENTICATION REQUEST | - | - |
| 11B | The UE transmits an ULInformationTransfer message and an AUTHENTICATION RESPONSE message. | --> | RRC: ULInformationTransfer NAS: AUTHENTICATION RESPONSE | - | - |
| 11C | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | RRC: DLInformationTransfer NAS: SECURITY MODE COMMAND | - | - |
| 11D | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer NAS: SECURITY MODE COMPLETE | - | - |
| 12 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 13 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 14 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB and a REGISTRATION ACCEPT message. | <-- | NR RRC: RRCReconfiguration 5GMM: REGISTRATION ACCEPT | - | - |
| 15 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 16 | The UE sends a REGISTRATION COMPLETE. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |

[End of change]