3GPP TSG RAN WG5 Meeting #91-e	R5-212147

# Electronic Meeting, 17th - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0140 | rev | - | Current version: | 14.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new MCPTT test case 6.2.20 On-network / First-to-answer call / On-demand session / Client Originated (CO) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MCPTT First-to-answer call is a new feature introduced as part of Rel-15. Testing of the relevant requirements is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new test case 6.2.20 'On-network / First-to-answer call / On-demand session / Client Originated (CO)' is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of testing of UE implementation for conformance to important requirements may lead to incorrectly implemented devices to enter the market and disrupt services or/and user experience. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.20 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | With the agreement of the present CR, TS 36.579-2 needs to be upgraded to Rel-15. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2.17.3.3	Specific message contents

Table 6.2.17.3.3-1: SIP INVITE (Step 1, Table 6.2.17.3.2-1; step 2, TS 36.579-1 [2], Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.5.2-1, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Answer-Mode | not present |  |  |  |
| Priv-Answer-Mode |  |  |  |  |
| answer-mode-value | "Auto" |  |  |  |
| answer-mode-param | “require” |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | SDP Message |  |  |
| MIME-part-body | SDP Message as described in Table 6.2.17.3.3-3 |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.2.17.3.3-2 |  |  |  |

Table 6.2.17.3.3-2: MCPTT-Info in SIP INVITE (Table 6.2.17.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.2.2-1, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| session-type | "ambient-listening" |  |  |  |
| anyExt |  |  |  |  |
| ambient-listening-type | "local-init" |  |  |  |

Table 6.2.17.3.3-3: SDP Message in SIP INVITE (Table 6.2.17.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.1.2-1, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| media attribute |  | a=line attribute=sendonly |  |  |
| sendonly |  |  |  |  |

Table 6.2.17.3.3-3A: SIP 200 (OK) (Step 2, Table 6.2.17.3.2-1; step 4, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.17.1.1-1 condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | As described in Table 6.2.17.3.3-3B |  |  |  |

Table 6.2.17.3.3-3B: SDP Message in SIP 200 OK (Table 6.2.17.3.3-3A)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.1.1-1, condition PRIVATE-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| media attribute |  | a=line attribute=recvonly |  |  |
| recvonly |  |  |  |  |

Table 6.2.17.3.3-4: Floor Taken (Step 3, Table 6.2.17.3.2-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.3-7 condition ON-NETWORK. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Permission to Request the Floor |  |  |  |
| Permission to Request the Floor | "0" | The receiver is NOT permitted to request floor |  |

Table 6.2.17.3.3-5: Void

### 6.2.18	On-network / Private call / Ambient listening call / Remote initiated ambient listening call / Pre-established session / Ambient listening call release / Client Originated (CO)

### FFS

### 6.2.19	On-network / Private call / Ambient listening call / Remote initiated ambient listening call / Pre-established session / Ambient listening call release / Client Terminated (CT)

FFS

### 6.2.20	On-network / First-to-answer call / On-demand session / Client Originated (CO)

6.2.20.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorized for MCPTT Service, including authorized to initiate first-to-answer calls }

ensure that {

when { UE (MCPTT User) requests the establishment of an on-demand first-to-answer call }

then { UE (MCPTT Client) sends a SIP INVITE message requesting the establishment of an on-demand first-to-answer call providing a MIME resource-lists body with the MCPTT IDs of the potential target MCPTT users, and, after indication from the MCPTT Server that the call was established notifies the user }

}

6.2.20.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 11.1.1.2.1.1, 6.2.2. Unless otherwise stated these are Rel-15 requirements.

[TS 24.379, clause 11.1.1.2.1.1]

Upon receiving a request from an MCPTT user to establish an MCPTT private call the MCPTT client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	shall set the Request-URI of the SIP INVITE request to a public service identity of the participating MCPTT function serving the MCPTT user;

...

4)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

5)	shall include the g.3gpp.mcptt media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

6)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

7)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP INVITE request;

8)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref contain with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

...

10)	for the establishment of a first-to-answer call shall insert in the SIP INVITE request a MIME resource-lists body with the MCPTT IDs of the potential target MCPTT users, according to rules and procedures of IETF RFC 5366 [20];

...

12)	shall include an SDP offer according to 3GPP TS 24.229 [4] with the clarification given in subclause 6.2.20 and with a media stream of the offered media-floor control entity;

13)	if implicit floor control is required, shall comply with the conditions specified in subclause 6.4 and:

a)	if the <allow-location-info-when-talking> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling MCPTT user (see the MCPTT user profile document in 3GPP TS 24.484 [50]) is set to a value of "true"; and

b)	if location information has not yet been included in the SIP re-INVITE request;

then shall include an application/vnd.3gpp.mcptt-location-info+xml MIME body with a <Report> element included in the <location-info> root element;

...

15)	if the MCPTT user is initiating a first-to-answer call shall contain an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <session-type> element set to a value of "first-to-answer";

...

17)	shall send SIP INVITE request towards the MCPTT server according to 3GPP TS 24.229 [4].

Upon receiving a SIP 183(Session Progress) response to the SIP INVITE request the MCPTT client:

1)	may indicate the progress of the session establishment to the inviting MCPTT user.

Upon receiving a SIP 200 (OK) response to the SIP INVITE request the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	if the sent SIP INVITE request was for the origination of a first-to-answer call and the SDP answer contained in the received SIP 200 (OK) response contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCPTT ID of the sender of the SIP 200 (OK) response from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [78];

b)	shall convert the MCPTT ID to a UID as described in 3GPP TS 33.180 [78];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [78];

...

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the originating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [78]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [46];

NOTE:	With the PCK successfully shared between the originating MCPTT client and the terminating MCPTT client, both clients are able to use SRTP/SRTCP to create an end-to-end secure session;

...

4)	shall notify the user that the call has been successfully established.

[TS 24.379, clause 6.2.2]

When the MCPTT client receives an initial SDP offer for an MCPTT session, the MCPTT client shall process the SDP offer and shall compose an SDP answer according to 3GPP TS 24.229 [4].

When composing an SDP answer, the MCPTT client:

1)	shall accept the MCPTT speech media stream in the SDP offer;

2)	shall set the IP address of the MCPTT client for the accepted MCPTT speech media stream and, if included in the SDP offer, for the accepted media-floor control entity;

NOTE:	If the MCPTT client is behind a NAT the IP address and port included in the SDP answer can be a different IP address and port than the actual IP address and port of the MCPTT client depending on the NAT traversal method used by the SIP/IP Core.

3)	shall include an "m=audio" media-level section for the accepted MCPTT speech media stream consisting of:

a)	the port number for the media stream;

b)	media-level attributes as specified in 3GPP TS 24.229 [4];

c)	if the "a=recvonly" attribute is present in the SDP offer, include an "a=sendonly" attribute;

d)	if the "a=sendonly" attribute is present in the SDP offer, include an "a=recvonly" attribute; and

e)	"i=" field set to "speech" according to 3GPP TS 24.229 [4];

4)	if included in the SDP offer, shall include the media-level section of the offered media-floor control entity consisting of:

a)	an "m=application" media-level section as specified in 3GPP TS 24.380 [5] clause 12; and

b)	'fmtp' attributes as specified in 3GPP TS 24.380 [5] clause 14; and

5)	if end-to-end security is required for a first-to-answer call, shall include the MIKEY-SAKKE I_MESSAGE in an "a=key-mgmt" attribute as a "mikey" attribute value in the SDP answer as specified in 3GPP TS 33.180 [78].

6.2.20.3	Test description

6.2.20.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The MCPTT User is authorized to initiate first-to-answer call: <allow-request-first-to-answer-call> element of the <ruleset> element of the <ruleset> element is present in the MCPTT user profile document and is set to "true".

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.2.20.3.2	Test procedure sequence

Table 6.2.20.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT User) request the establishment of an MCPTT first-to-answer call with targeted users B and C. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 2a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.3 'Generic Test Procedure for MCPTT CO communication in E-UTRA' take place. | - | - | - | - |
| 3 | Check: Does the UE (MCPTT Client) send a SIP INVITE requesting the establishment of an MCPTT first-to-answer call providing a MIME resource-lists body with the MCPTT IDs of the potential target MCPTT users? | --> | SIP INVITE | 1 | P |
| 4 | The SS sends SIP 100 Trying. | <-- | SIP 100 (Trying) | - | - |
| 5 | The SS sends SIP 183 (Session Progress). | <-- | SIP 183 (Session Progress) | - | - |
| 6 | The SS (MCPTT server) responds with a SIP 200 (OK) including a MIKEY-SAKKE I_MESSAGE. (NOTE 2) | <-- | SIP 200 (OK) | - | - |
| 7 | Check: Does the UE (MCPTT Client) notifies the User that the call was established? (NOTE 1) | - | - | 1 | P |
| - | EXCEPTION: Steps 8a1 describes behaviour that depends on the test case requirements ; the "lower case letter" identifies a step sequence that takes place if the UE requests implicit floor control in step 3 (i.e. the "mc_implicit_request" fmtp attribute included in the SDP offer and the SS responded with the "mc_implicit_request" fmtp attribute included and the “mc_granted” fmtp attribute not present in the SDP answer. | - | - | - | - |
| 8a1 | The SS (MCPTT server) sends a Floor Granted message. | <-- | Floor Granted | - | - |
| 9 | Make the UE (MCPTT User) request termination of the MCPTT first-to-answer call. (NOTE 1) | - | - | - | - |
| 10 | The UE (MCPTT Client) perform the generic test procedure for MCPTT CO call release as described in TS 36.579-1 [2] Table 5.3.10.3-1 to end the private call. The Session is not releases. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI NOTE 2: The UE (MCPTT Client) is expected to extract and decrypt the encapsulated parameters. With the PCK successfully shared between the originating MCPTT client and the terminating MCPTT client, both clients are able to use SRTP/SRTCP to create an end-to-end secure session. |  |  |  |  |  |

6.2.20.3.3	Specific message contents

Table 6.2.20.3.3-1: SIP INVITE (Step 2, Table 6.2.20.3.2-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.5.1-1, condition PRIVATE-CALL (NOTE 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/sdp" |  |  |  |
| MIME-part-body | SDP Message as described in Table 6.2.20.3.3-2 |  |  |  |
| MIME body part |  | MCPTT Info/MCVideo/MCData |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-info+xml" |  |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.2.20.3.3-3 |  |  |  |
| MIME body part |  | Resource list |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/resource-lists+xml" |  |  |  |
| MIME-part-body | As described in Table 6.2.20.3.3-4 |  |  |  |
| NOTE 1: Whether the UE (MCPTT Client) requests a call with automatic or manual commencement, and, with or without Floor control depends on implementation. Therefore both should be allowed. |  |  |  |  |

Table 6.2.20.3.3-2: SDP message in SIP INVITE (Table 6.2.20.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.1.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Media descriptions |  |  |  |  |
| media attribute | Not included | a= line attribute = key-mgmt |  |  |
| NOTE 1: Whether the UE (MCPTT Client) requests a call with automatic or manual commencement, and, with or without Floor control depends on implementation. Therefore both should be allowed. |  |  |  |  |

Table 6.2.20.3.3-3: MCPTT-Info in SIP INVITE (Table 6.2.20.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.2.1-1, condition INVITE_REFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| session-type | "first-to-answer" |  |  |  |

Table 6.2.20.3.3-4: Resource list in SIP INVITE (Table 6.2.20.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.3.1-1. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists |  |  |  |  |
| list[1] |  |  |  |  |
| name attribute | Not present |  |  |  |
| display-name | Not present |  |  |  |
| entry[1] |  | The MCPTT IDs of the invited users in any order | [RFC 5366] |  |
| uri attribute | px_MCPTT_ID_User_B |  |  |  |
|  | px_MCPTT_ID_User_C |  |  |  |

Table 6.2.20.3.3-5: SIP 200 (OK) (step 4, Table 6.2.20.3.2-1)

| Derivation Path: Table 5.5.2.17.1.2-1 with condition INVITE-RSP and MCPTT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP message | SDP message as described in Table 6.2.20.3.3-6 |  |  |  |

Table 6.2.20.3.3-6: SDP Message in SIP 200 (OK) (Table 6.2.20.3.3-5)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.1.2-1 conditions PRIVATE-CALL, NOT WITH_FLOORCONTROL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Media descriptions |  |  |  |  |
| media attribute |  | a= line attribute = key-mgmt |  |  |
| key-mgmt |  |  |  |  |
| mikey | MIKEY-SAKKE I_MESSAGE as specified in Table 6.2.20.3.3-7 |  |  |  |

Table 6.2.20.3.3-7: MIKEY-SAKKE I_MESSAGE in SDP Message (Table 6.2.20.3.3-6)

Derivation Path: TS 36.579-1 [2], table 5.5.9.1-2

Table 6.2.20.3.3-8: Floor Granted (step 7a1, Table 6.2.20.3.2-1

| Derivation Path: 36.579-1 [2], Table 5.5.6.3-1, condition ON-NETWORK |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RTCP header |  |  |  |
| Subtype | "00001" | Acknowledgment is not required for Floor Granted message |  |