Title: 	Discussion on test point selection for General spurious emissions for Inter-band EN-DC within FR1

Source: 	oppo

Introduction

Current TP analysis paper “38.521-3_TP_analysis_38.905_6.5B.3_TX_SpurEmission_EN-DC.zip” is missing in TS 38.905. Furthermore, this TP analysis paper was uploaded in RAN5 #82. The TP was based on the LTE anchor agnostic applied test which is not aligned with current version. Current TP refers to Table 6.5B.3.1.2.4.1-1 which only define test frequency of NR as below. It is reasonable for intra-band EN-DC because test frequency of LTE is adjacent to NR. But for inter-band EN-DC, test frequency of LTE is ambiguity.

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 | Normal |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 | Low range, High range |

Therefore, we propose a new TP analysis paper for 6.5B.3.3.1 to solve the above problem.

Discussion

For reference, TPs of General Spurious Emission of NR SA and LTE-CA(inter-band DL CA and UL CA) are extracted as below:

Table 6.5.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | OuterFull |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |

Table 6.6.3.1A.2.4.1-1: Test Configuration Table for inter-band DL CA and UL CA TX spurious emission testing)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |

Test Environment

Both NR SA and LTE CA cases select NC, so it is reasonable to use the same test environment.

Proposal 1: Normal Test Environment is selected for General spurious emissions for Inter-band EN-DC within FR1.

Test Subcarrier Spacing

Test subcarrier spacing is selected as Lowest for Spurious Emission test cases in FR1. The same approach can be followed for inter-band EN-DC test cases.

Proposal 2: Select Lowest SCS supported for UE for General spurious emissions for Inter-band EN-DC within FR1.

  - Test Frequencies

Low, Mid, High range are selected as test frequencies for NR standalone. Low range for PCC and SCC, High range for PCC and SCC are selected for LTE inter-band CA. Low and high frequencies are more crucial for spurious emission test. Furthermore, to simplify test complexity and reduce test time, it is proposed to use the same test frequencies as LTE inter-band CA.

Proposal 3: Select Low range for PCC and SCC, High range for PCC and SCC as test frequency for General spurious emissions for Inter-band EN-DC within FR1.

  - Test Channel Bandwidth

Test channel bandwidth is selected as Lowest NRB_agg, Highest NRB_agg for LTE inter-band CA. It is proposed to use similar approach which is 5MHz for E-UTRA CC1 and Lowest for NR CC1, Highest for E-UTRA CC1 and Highest for NR CC1 for inter-band EN-DC.

Proposal 4: Select 5MHz for E-UTRA CC1 and Lowest for NR CC1, Highest for E-UTRA CC1 and Highest for NR CC1 as test Channel Bandwidth for Inter-band EN-DC within FR1.

Modulations and RB allocation

It is proposed to use QPSK for E-UTRA cell and CP-OFDM for NR cell which is the same as LTE and NR standalone cases.

It is proposed to use the same RB allocation as NR standalone case for NR cell. To simplify test complexity and reduce test time, it is proposed to use Outer_Full as RB allocation for E-UTRA cell.

Proposal 5: Select QPSK and Outer_Full for E-UTRA cell. Select the same modulation and RB allocation as NR SA case for NR Cell.

Number of Test Points

Based on above proposal, the total test points are tabulated below:

Table 2.6-1 Total test points

| Environment conditions | Test SCS | Test Frequencies | Test ChBW | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 2 | 3 | 12 |

Conclusion

Proposal 1: Normal Test Environment is selected for General spurious emissions for Inter-band EN-DC within FR1.

Proposal 2: Select Lowest SCS supported for UE for General spurious emissions for Inter-band EN-DC within FR1.

Proposal 3: Select Low range for PCC and SCC, High range for PCC and SCC as test frequency for General spurious emissions for Inter-band EN-DC within FR1.

Proposal 4: Select 5MHz for E-UTRA CC1 and Lowest for NR CC1, Highest for E-UTRA CC1 and Highest for NR CC1 as test Channel Bandwidth for Inter-band EN-DC within FR1.

Proposal 5: Select QPSK and Outer_Full for E-UTRA cell. Select the same modulation and RB allocation as NR SA case for NR Cell.

References

[1] 3GPP TS 38.521-1 v16.7.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

[2] 3GPP TS 36.521-1 v16.8.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing”.